Port Additional Characteristics Configuration Commands

- Table of Contents

Chapter 1 Port Additional Characteristics Configuration Commands	2

1.1 Configuring Port Isolation	2

1.1.1 port-protected	2

1.1.2 description	2

1.1.3 switchport protected	3

1.2 Configuring Storm-control	3

1.3 Configuring Switchport Rate-limit	4

1.4 Configuring Port Loop Check	5

1.5 Configuring Port MAC-address learning	5

1.6 Configuring Port Security	6

1.6.1 switchport port-security mode	6

1.6.2 switchport port-security dynamic	6

1.6.3 switchport port-security static mac-address	7

1.6.4 switchport port-security sticky	8

1.7 Configuring Port Binding	8

1.8 Configuring SVL/IVL	9

1.9 Configuring Link Scan	9

1.10 Configuring Port Enhanced Link Status Check	10

1.11 Configuring System Mtu	11

# Port Additional Characteristics Configuration Commands

## Configuring Port Isolation

### port-protected

Syntax

[no] port-protected group-id

Configure a port isolation group with the above command.

Parameter

| Parameter | Description |
| --- | --- |
| group-id | Configure a port isolation group 1-28. |

Default

None

Usage Guidelines

Configure the command in global configuration mode for the group-based isolation.

Example

Configure the port isolation group 1.

Switch_config#port-protected 1

### description

Syntax

description word

Configure description of the port isolation group with the above command.

no description

Delete description with the above command.

Parameter

| Parameter | Description |
| --- | --- |
| Word | Configure description of the port isolation group, no more than 31 characters. |

Default

None

Usage Guidelines

Configure the command in global configuration mode for the description.

Example

The following example shows how to configure description g1 of the port isolation group 1.

Switch-config-p1#description g1

### switchport protected

Syntax

switchport protected <group-id>

no switchport protected

Configure the port isolation with the above command.

Parameter

| Parameter | Description |
| --- | --- |
| group-id | Choose the port isolation 1-28 |

Default

None

Usage Guidelines

The command works in the layer-2 interface. The default command is based on the non-group isolation that does not need to configure group-id in the end. If the command is based on group-based isolation, all ports need to configure group-based isolation in global configuration mode. Only after deleting isolations in all ports, the isolation type can be configure again.

Example

The following example shows how to configure the non-group based isolation on interface g0/1.

Switch_config_g0/1#switchport protected

## Configuring Storm-control

Syntax

storm-control {broadcast | multicast | unicast} threshold count

no storm-control {broadcast | multicast | unicast} threshold Parameter

Configure the storm control function of the port.

Parameter

| Parameter | Description |
| --- | --- |
| broadcast ¦ multicast ¦ unicast | Defines the storm control of the broadcast, multicast and unicast. |
| count | Defines the maximum flow of the storm control. The value ranges from 1 to 16384. |

Default

The storm control function is not enabled.

Usage Guidelines

This command works in layer-2 port configuration mode.

Example

Set the storm control of the unknown unicast frame on interface g0/1 to 20PPS.

Switch_config#interface g0/1

Switch_config_g0/1#storm-control unicast threshold 20

## Configuring Switchport Rate-limit

Syntax

switchport rate-limit {band | bandwidth percent} { ingress|egress}

no switchport rate-limit{ ingress|egress}

Configure the flow rate limitation for the port.

Parameter

| Parameter | Description |
| --- | --- |
| Band | Flow rate. 64kbps-step rate limitation on a port |
| percent | Flow rate ratio, unit: 1%. |
| ingress | Functions at the ingress port. |
| egress | Functions at the egress port. |

Default

The port has no port rate limitation.

Usage Guidelines

This command works in layer-2 port configuration mode.

Example

Set the incoming flow rate limitation on interface g0/1 to 1Mbps.

Switch_config#interface  g0/1

Switch_config_g0/1#switchport rate-limit 16 ingress

## Configuring Port Loop Check

Syntax

[no] keepalive second

To configure the interval for a port to transmit the loop check packets, run keep alive command.

Parameter

| Parameter | Description |
| --- | --- |
| Second | Time interval. Unit: second. |

Default

The time interval is 12 seconds by default.

Usage Guidelines

Configure in the physical port configuration mode.

Example

The following example shows how to set the transmission interval on interface g0/1 to 10 seconds.

Switch_config#interface g0/1

Switch_config_g0/1#keepalive 10

## Configuring Port MAC-address learning

Syntax

[no] switchport disable-learning

Configure the learning MAC address on the port.

Parameter

None

Default

Enable the learning MAC address on the port.

Usage guidelines

Configure in the physical port configuration mode.

Example

The following example shows how to disable MAC address learning on the interface g0/1.

Switch_config#interface g0/1

Switch_config_g0/1#switchport disable-learning

## Configuring Port Security

Port security commands include:

- switchport port-security mode

- switchport port-security dynamic

- switchport port-security static

- switchport port-security sticky

### switchport port-security mode

Syntax

- switchport port-security mode {dynamic | static accept|reject | sticky}

- [no] switchport port-security mode

Set the static mode of the security port.

Parameter

None

Default

The port security function is not enabled.

Usage guidelines

Configure the command on the physical port configuration mode.

Example

The following example shows how to configure interface g0/1 in the dynamic interface security mode.

Switch_config#inter g0/1

Switch_config_g0/1#switchport port-security mode dynamic

### switchport port-security dynamic

Syntax

switchport port-security dynamic maximum dynamic_number

[no] switchport port-security dynamic maximum

To configure the maximum number of MAC addresses when the port is in dynamic security mode, run switchport port-security dynamic maximum command.

Parameter

| Parameter | Description | Value ranges |
| --- | --- | --- |
| dynamic_number | Means the maximum number of MAC addresses. | 1-2048 |

Default

The number of MAC addresses that can be learned is 1.

Usage guidelines

Configure the command on the physical port configuration mode.

Example

The following example shows how to apply maximum MAC addresses on interface g0/1 and set the learnable MAC address to 10.

Switch_config#inter g0/1

Switch_config_g0/1# switchport port-security dynamic maximum 10

### switchport port-security static mac-address

Syntax

[no] switchport port-security static mac-address H.H.H

Configure the static MAC address of the security port.

Parameter

None

Default

None

Usage Guidelines

Configure the command on the physical port configuration mode.

Example

The following example shows how to configure mac: 0001.0002.0003 to static security mac.

Switch_config#inter g0/1

Switch_config_g0/1# switchport port-security static mac-address 1.2.3

### switchport port-security sticky

Syntax

Switchport port-security sticky {maximum sticky_number| mac-address H.H.H| aging-time aging_time}

[no] switchport port-security sticky {maximum | mac-address H.H.H| aging-time }

Configure the port-security sticky of MAC address.

Parameter

| Parameter | Description |
| --- | --- |
| sticky_number | The maximum number of addresses that can be learned on the port. Default is 100, range: 1-2048 |
| H.H.H | Mac address |
| aging_time | Aging time. Unit: minutes. The default is 0, the range: 0-100 |

Default

The port sticky of MAC address is not enabled.

Usage Guidelines

Configure the command on the physical port configuration mode.

Example

The following example shows how to manually configure mac: 4433.0002.0021 to security sticky mac address.

Switch_config#inter g0/1

Switch_config_g0/1# switchport port-security sticky mac-address 4433.0002.0021

## Configuring Port Binding

Syntax

switchport port-security bind|block {ip|arp|both-arp-ip ip-addr| mac mac-addr }

no switchport port-security bind|block {ip|arp| both-arp-ip ip-addr mac mac-addr}

To bind a MAC address to an IP address, run the first one of the above commands.

To cancel the address binding one by one or to exit the port binding state by cancelling all addresses on the port, run the no form of the command.

Parameter

| Parameter | Description | Value ranges |
| --- | --- | --- |
| ip-addr | IP address | A.B.C.D |
| Mac-addr | Mac address | H.H.H |

Default

None

Description

This command works in layer-2 port configuration mode.

The port binding function is forbidden by default. However, if one address is bound, the port is then in binding state unless you use the negative form of this command to clear all bound address items.

Example

The following example shows how to bind IP address 1.2.3.4 to MAC address 0001.0001.1111 on interface g0/1 to decline the IP packets and ARP packets from the bound address.

Switch_config#inter g0/1

Switch_config_g0/1# switchport port-security block both-arp-ip 1.2.3.4 mac 0001.0001.1111

## Configuring SVL/IVL

Syntax

[no]vlan shared-learning

The command is used to configure share vlan learning (SVL).

Parameter

None

Default

Configure Independent Vlan Learning (IVL) on the port.

Usage Guidelines

Configure the command in global configuration mode.

Example

The following example shows how to configure shared learning.

Switch_config#vlan shared-learning

## Configuring Link Scan

Syntax

[no] Link scan [normal | fast] interval

To set the scan interval of an interface, run the above command.

Parameter

| Parameter | Description |
| --- | --- |
| [normal ¦ fast] | Normal: standard link scan mode Fas: fast link scan mode |
| interval | Scan interval, unit: 1ms, 10-1000 |

Default

The default scan interval in the standard mode is 500ms;

The default scan interval in the fast mode is 10ms.

Usage Guidelines

Configure the command in global configuration mode. The Fast mode is mainly used for collaborating with the protocol, such as RSTP. Discover the interface up/down in the normal mode.

Example

The following example shows how to set the scan interval of a switch to 20ms.

Link scan normal 20

## Configuring Port Enhanced Link Status Check

Syntax

[no] switchport enhanced-link

To enable/disable status check of the port enhanced link, run the above command.

Parameter

None

Default

Disable status check of the port enhanced link.

Usage Guidelines

Configure the command in port configuration mode.

Example

The following example shows how to enable status check of the port g0/1 enhanced link.

Switch_config#interface g0/1

Switch_config_g0/1#switchport enhanced-link

## Configuring System mtu

Syntax

[no] system mtu mtu

The command is used to configure mtu.

Parameter

| Parameter | Description |
| --- | --- |
| mtu | Configures system mtu, the value ranges from 1500 to 9216. |

Default

The default mtu is 1500 bytes.

Usage Guidelines

Configure the command in the global configuration mode.

Example

The following example shows how to configure mtu to 2000 bytes.

Switch#config

Switch_config#system mtu 2000