GVRP Configuration Commands

Table of Contents

Chapter 1	GVRP Configuration Commands	1

1.1 GVRP Configuration Commands	1

1.1.1 GVRP	1

1.1.2 gvrp dynamic-vlan-pruning	2

1.1.3 Show gvrp statistics	2

1.1.4 Show gvrp status	3

1.1.5 Debug gvrp event	4

1.1.6 Debug gvrp packet	4

1.2 GARP Configuration Commands	5

1.2.1 Garp timer leaveall	5

1.2.2 garp timer hold	6

1.2.3 garp timer join	7

1.2.4 garp timer leave	8

1.2.5 show garp timers	8

1.2.6 Show garp status	9

1.2.7 Debug garp	10

# Chapter 1	GVRP Configuration Commands

## 1.1 GVRP Configuration Commands

### 1.1.1 GVRP

Syntax

To enable GVRP globally on a device and on an interface, use the gvrp command. To disable GRVP, use the no form of this command.

gvrp

no gvrp

Parameter

None

Default

GVRP is disabled globally.

GRVP is enabled on each interface.

Usage Guidelines

GVPRP can be enabled globally or on an interface in the system, and GVRP is not enabled until both of them are enabled.

Example

The following example configures global gvrp on the device and interfaces:

Switch_config#gvrp

Switch_config#

The following example enables gvrp on interface 1:

Switch_config_g0/1#gvrp

Switch_config_g0/1#

### 1.1.2 gvrp dynamic-vlan-pruning

Syntax

gvrp dynamic-vlan-pruning

no gvrp dynamic-vlan-pruning

To set the dynamic VLAN only takes effect on the registered interface, run the first one of the above commands. To return to the default setting, use the no form of this command.

Parameter

None

Default

Dynamic-vlan-prunning is disabled. That is, dynamic VLAN can take effect in all interfaces.

Command Mode

Global configuration mode

Usage Guidelines

After this command is enabled and, if a port has not registered a dynamic VLAN, this port will not belong to the dynamic VLAN even though this port is a trunk port and it allows the dynamic VLAN to pass through.

Example

The following example shows how to make dynamic VLAN validate on its registered port.

Switch_config#gvrp dynamic-vlan-pruning

Switch_config#

### 1.1.3 Show gvrp statistics

Syntax

To show gvrp statistics, use the show gvrp statistics command.

show gvrp statistics [interface intf-id]

Parameter

| Parameter | Description |
| --- | --- |
| intf-id | The concrete physical interface |

Default

None

Usage Guidelines

Show GVRP statistics.

Example

The following example show GVRP statistics of port g0/1:

Switch_config#show gvrp statistics interface g0/1

GVRP statistics on port g0/1

GVRP Status             : Enabled

GVRP Frames Received   : 0

GVRP Frames Transmitted : 20

GVRP Frames Discarded  : 0

GVRP Last Pdu Origin     : 0000.0000.0000

### 1.1.4 Show gvrp status

Syntax

To show GVRP status information, use show gvrp status command.

show gvrp status

Parameter

None

Default

None

Usage Guidelines

Show GVRP status information.

Example

The following command shows GVRP status information of the switch:

Switch_config#show gvrp status

GVRP is enabled

### 1.1.5 Debug gvrp event

Syntax

To enable debugging GVRP event information, use the debug gvrp event command. Use the no form of this command to disable debugging.

debug gvrp event

no debug gvrp event

Parameter

None

Default

None

Usage Guidelines

Enable/disable debugging GVRP event information.

Example

Switch# debug gvrp event

Switch#

### 1.1.6 Debug gvrp packet

Syntax

To enable debugging GVRP packet information, use the debug gvrp event command. Use the no form of this command to disable debugging.

debug gvrp packet

no debug gvrp packet

Parameter

None

Default

None

Usage Guidelines

Enable/disable debugging GVRP packet information.

Example

switch# debug gvrp packet

switch#

## 1.2 GARP Configuration Commands

GARP is the basic module of GVRP/GMRP. It is intended to scheduler GVRP/GMRP operation and provide services.

### 1.2.1 Garp timer leaveall

Syntax

To configure garp leaveall timer, use the garp timer leaveall command. Use the no form of this command to restore the default value.

garp timer leaveall time_value

no garp timer leaveall

Parameter

| Parameter | Description |
| --- | --- |
| timer_value | Global leaveall timer value. Value range: 10–32765 centseconds. |

Default

1000 centiseconds

Usage Guidelines

Bridge will clear all registered VLAN information and send out LeaveAll Message after leaveall timer expires.

Example

The following example configures leaveall timer on the switch to 1200 centiseconds:

Switch_config# garp timer leaveall 1200

Switch_config#

### 1.2.2 garp timer hold

Syntax

garp timer hold time_value

no garp timer hold

To set grap hold timer, run the first one of the above commands. To return to the default setting, use the no form of this command.

Parameter

| Parameter | Description |
| --- | --- |
| timer_value | The hold timer value of the interface. The value ranges from 10 to 32765 centiseconds. |

Default

10 centiseconds

Command Mode

Interface configuration mode

Usage Guidelines

None

Example

The following example shows how to set the garp hold timer on the interface g0/1 to 15 centiseconds.

Switch_config_g0/1#garp timer hold 15

Switch_config_g0/1#

### 1.2.3 garp timer join

Syntax

garp timer join time_value

no garp timer join

To set garp join timer, run the first one of the above commands. To return to the default setting, use the no form of this command.

Parameter

| Parameter | Description |
| --- | --- |
| timer_value | The join timer value of the interface. The value ranges from 10 to 32765 centiseconds. |

Default

20 centiseconds

Command Mode

Interface configuration mode

Usage Guidelines

None

Example

The following example shows how to set garp join timer on interface g0/1 to 25 centiseconds.

Switch_config_g0/1#garp timer join 25

Switch_config_g0/1#

### 1.2.4 garp timer leave

Syntax

garp timer leave time_value

no garp timer leave

To set garp leaver timer, run the first one of the above commands. To return to the default setting, use the no form of this command.

Parameter

| Parameter | Description |
| --- | --- |
| timer_value | The leave timer value of the interface. The value ranges from 10 to 32765 centiseconds. |

Default

60 centiseconds

Command Mode

Interface Configuration Mode

Usage Guidelines

None

Example

The following example shows how to set the value of the garp leave timer on interface g0/1 to 80 centiseconds.

Switch_config_g0/1#garp timer leave 80

Switch_config_g0/1#

### 1.2.5 show garp timers

Syntax

show garp timers [ interface intf_id ]

To show the GARP-configured clock information, run the above command.

Parameter

| Parameter | Description |
| --- | --- |
| intf-id | The concrete physical port. |

Default

None

Usage Guidelines

This command is used to display the GARP-configured clock information, including the global leaveall timer value, the hold/join/leave timer value on the port.

Example

The following example shows how to show the timer configuration information on interface G0/1.

Switch# show garp timers interface g0/1

GARP timers on port 1(G0/1)

Garp Join Time          : 20 centiseconds

Garp Leave Time         : 60 centiseconds

Garp LeaveAll Time      : 1000 centiseconds

Garp Hold Time          : 10 centiseconds

### 1.2.6 Show garp status

Syntax

To  show  the  currently-running  garp  application  example,  use  the  show  garp status command.

show garp status

Parameter

None

Default

None

Usage Guidelines

Display the currently running GARP statistics.

Example

The following example shows the currently running GARP statistics:

Switch_config#show garp status

No GARP application is running.

### 1.2.7 Debug garp

Syntax

To enable the garp event or timer debuging information, use the debug garp event command. Use the no form of this command to disable debugging.

debug garp { event | timer }

no debug garp { event | timer }

Parameter

| Parameter | Parameter description |
| --- | --- |
| event | Event debug |
| timer | Timer debug |

Default

None

Usage Guidelines

Use this command to enable/disable debugging GARP event information.

Example

The following example shows how to enable debugging GARP event information

Switch# debug garp event

Switch#