- Table of Contents

Chapter 1 PDP Configuration Commands	1

1.1 PDP Configuration Commands	1

1.1.1 pdp timer	1

1.1.2 pdp holdtime	2

1.1.3 pdp version	2

1.1.4 pdp run	3

1.1.5 pdp enable	3

1.1.6 show pdp traffic	4

1.1.7 show pdp neighbor	5

# PDP Configuration Commands

## PDP Configuration Commands

PDP commands include:

- pdp timer

- pdp holdtime

- pdp version

- pdp run

- pdp enable

- show pdp traffic

- show pdp neighbor

### pdp timer

Syntax

[no|default] pdp timer seconds

It is used to set the time of the PDP timer.

Parameter

| Parameter | Description |
| --- | --- |
| seconds | Means the interval for PDP to transmit packets outside, which ranges from 5 seconds to 254 seconds. |

Default

The default settings is 60 seconds.

Usage Guidelines

This command is configured in global configuration mode.

Example

The following example shows that a switch transmits PDP packets every 5 seconds.

pdp timer 5

### pdp holdtime

Syntax

[no|default] pdp holdtime seconds

It is used to set the PDP holdtime.

Parameter

| Parameter | Description |
| --- | --- |
| seconds | Means the time needed for deleting the neighbor information, which ranges from 10 to 255 seconds. |

Default

The default settings is 180 seconds.

Usage Guidelines

This command is configured in global configuration mode.

Example

The following example shows that the neighbor information is kept for 15 seconds.

pdp holdtime 15

### pdp version

Syntax

[no] pdp version <1|2>

It is used to set the PDP version.

Parameter

| Parameter | Description |
| --- | --- |
| version | Means the PDP version, which is 1 or 2. |

Default

The default version is version 2.

Usage Guidelines

This command is configured in global configuration mode.

Example

The following example shows that the PDP version of a switch is version 1.

pdp version 1

### pdp run

Syntax

[no] pdp run

It is used to enable PDP.

Parameter

None

Default

The PDP protocol is disabled.

Usage Guidelines

This command is configured in global configuration mode.

Example

The following example shows to disable PDP.

no pdp run

### pdp enable

Syntax

[no] pdp enable

It is used to enable or disable PDP on a port.

Parameter

None

Default

PDP is enabled by default.

Usage Guidelines

This command is set in port configuration mode and takes effect in port or global mode only when PDP is enabled.

Example

The following example shows that PDP is disabled on port g0/1.

switch_config_g0/1#no pdp enable

### show pdp traffic

Syntax

show pdp traffic

It is used to display the number of received PDP packets.

Parameter

None

Default

None

Usage Guidelines

It is used to browse the running of PDP.

Example

Switch_config#show pdp traffic

Packets output: 253491, Input: 0

Hdr syntax: 0, Chksum error: 0

No memory: 0, Invalid packet: 0

### show pdp neighbor

Syntax

show pdp neighbor

It is used to display the PDP neighbor.

Parameter

None

Default

None

Usage Guidelines

It is used to browse the running of PDP neighbor.

Example

Switch_config#show pdp neighbor

Capability Codes: R - Router, T - Trans Bridge, B - Source Route Bridge

S - Switch, H - Host, I - IGMP, r - Repeater

Device-ID   Local-Intf  Hldtme  Port-ID      Platform        Capability

Switch      Gig0/1      169     Gig0/1    COM, RISC  R S