Backuplink Configuration Commands

Table of Contents

Chapter 1 Backup Link Configuration Commands	1

1.1 Global Configuration Commands	1

1.1.1 backup-link-group id	1

1.1.2 backup-link-group id preemption-mode forced {delay value}	2

1.1.3 backup-link-group id preemption-mode bandwidth {delay value}	3

1.1.4 monitor-link-group id	4

1.2 Port Configuration Commands	5

1.2.1 backup-link-group id active	5

1.2.2 backup-link-group id backup	6

1.2.3 share-load vlan vlanmap	7

1.2.4 backup-link-group mmu transmit	8

1.2.5 backup-link-group mmu receive	9

1.2.6 monitor-link-group id uplink	9

1.2.7 monitor-link-group id downlink	10

1.3 Show Configuration Commands	11

1.3.1 show backup-link-group id	11

1.3.2 show monitor-link-group id	12

# Backup Link Configuration Commands

## Global Configuration Commands

### backup-link-group id

Syntax

To set the BackupLink group, run the following command:

backup-link-group id

To delete the BackupLink group, run the following command:

no backup-link-group id

Parameter

| Parameter | Description |
| --- | --- |
| Id | Stands for the instance ID of the backuplink group, ranging from 1 to 8. |

Default

The backuplink group is not configured by default.

Command Mode

Global configuration mode

Usage Guidelines

None

Example

Switch_config#backup-link-group 1

Switch_config#

Related Command

None

### backup-link-group id preemption-mode forced {delay value}

Syntax

To set the port-based preemption mode for the backuplink group, run the following command:

backup-link-group id preemption-mode forced {delay value}

To delete the port-based preemption mode for the backuplink group, run the following command:

no backup-link-group id

Parameter

| Parameter | Description |
| --- | --- |
| Id | Stands for the instance ID of the backuplink group, ranging from 1 to 8. |
| value | Stands for the delay time, ranging from 0s to 60s. |

Default

The backuplink group has not been set with the trait of port-based preemption by default.

Command Mode

Global configuration mode

Usage Guidelines

The backup-link-group id preemption-mode forced {delay value} command can be used to create the backuplink group directly.

Example

Switch_config#backup-link-group 1 preemption-mode forced delay 5

Switch_config#

Related Command

backup-link-group id

backup-link-group id preemption-mode bandwidth {delay value}

### backup-link-group id preemption-mode bandwidth {delay value}

Syntax

To set port bandwidth preemption mode for the backuplink group, run the following command:

backup-link-group id preemption-mode bandwidth {delay value}

To delete port bandwidth preemption mode for the backuplink group, run the following command:

no backup-link-group id

Parameter

| Parameter | Description |
| --- | --- |
| Id | Stands for the instance ID of the backuplink group, ranging from 1 to 8. |
| value | Stands for the delay time, ranging from 0s to 60s. |

Default

The backuplink group has not been set with the trait of port bandwidth preemption by default.

Command Mode

Global configuration mode

Usage Guidelines

None

Example

Switch_config#backup-link-group 1 preemption-mode bandwidth delay 5

Switch_config#

Related Command

backup-link-group id

backup-link-group id preemption-mode forced {delay value}

### monitor-link-group id

Syntax

To set the MonitorLink group, run the following command:

monitor-link-group id

To delete the MonitorLink group, run the following command:

no monitor-link-group id

Parameter

| Parameter | Description |
| --- | --- |
| Id | Stands for the instance ID of the monitorlink group, ranging from 1 to 8. |

Default

The MonitorLink group is not configured by default.

Command Mode

Global configuration mode.

Usage Guidelines

None

Example

Switch _config# monitor-link-group 1

Switch_config#

Related Command

None

## Port Configuration Commands

### backup-link-group id active

Syntax

To set a port to be an active port, run the following command:

backup-link-group id active

To cancel the active port, run the following command:

no backup-link-group id

Parameter

| Parameter | Description |
| --- | --- |
| Id | Stands for the instance ID of the backuplink group, ranging from 1 to 8. |

Default

The primary port is not configured by default.

Command Mode

The physical port configuration mode and the aggregation port configuration mode

Usage Guidelines

If the backuplink group is not established, it will be automatically created when you configure the backuplink group on a port directly.

Example

Switch_config#interface gigaEthernet 0/1

Switch_config_g0/1#backup-link-group 1 active

Switch_config_g0/1#exit

Related Command

backup-link-group id

backup-link-group id backup

### backup-link-group id backup

Syntax

To set a port to be a backup port, run the following command:

backup-link-group id backup

To cancel the backup port, run the following command:

no backup-link-group id

Parameter

| Parameter | Description |
| --- | --- |
| Id | Stands for the instance ID of the backuplink group, ranging from 1 to 8. |

Default

The backup port is not configured by default.

Command Mode

The physical port configuration mode and the aggregation port configuration mode

Usage Guidelines

If the backuplink group is not established, it will be automatically created when you configure the backuplink group on a port directly.

Example

Switch_config#interface gigaEthernet 0/1

Switch_config_g0/1#backup-link-group 1 backup

Switch_config_g0/1#exit

Related Command

backup-link-group id

backup-link-group id active

### share-load vlan vlanmap

Syntax

To set VLAN load balance for the backup port, run the following command:

share-load vlan vlanmap

To delete VLAN load balance for the backup port, run the following command:

no share-load vlan

Parameter

| Parameter | Description |
| --- | --- |
| vlanmap | Stands for the VLAN value, ranging from 1 to 4049. |

Default

VLAN load balance is not set for the backup port by default.

Command Mode

The physical port configuration mode and the aggregation port configuration mode

Usage Guidelines

This command can be set only on the backup port, that is, a port must be set to be a backup port before VLAN load balance is set on the port.

Different BackupLink groups can be set to have the same VLAN group or the overlapped VLAN segments. If there are overlapped VLAN segments, the system will classify these VLANs into different MSTs (STGs) and conduct operations toward a group of ports, the statuses of these ports in different MSTs vary. So, you'd better rule out those overlapped VLAN groups when configuring load balance for VLANs.

Example

Switch_config#interface gigaEthernet 0/1

Switch_config_g0/1# share-load vlan 100-200

Switch_config_g0/1#exit

Related Command

backup-link-group id

backup-link-group id backup

### backup-link-group mmu transmit

Syntax

To set MMU transmission for the ports of the backuplink group, run the following command:

backup-link-group mmu transmit

To delete MMU transmission for the ports of the backuplink group, run the following command:

no backup-link-group mmu

Parameter

None

Default

The MMU transmission function for the ports of the backuplink group is not set by default.

Command Mode

The physical port configuration mode and the aggregation port configuration mode

Usage Guidelines

Only the ports of the backuplink group can be set to transmit, that is, the ports must be set to active or backup.

Example

Switch_config#interface gigaEthernet 0/1

Switch_config_g0/1#backup-link-group mmu transmit

Switch_config_g0/1#exit

Related Command

backup-link-group id

### backup-link-group mmu receive

Syntax

To set MMU reception for ports, run the following command:

backup-link-group mmu receive

To delete MMU reception for ports, run the following command:

no backup-link-group mmu

Parameter

None

Default

The MMU reception function for the ports is not set by default.

Command Mode

The physical port configuration mode and the aggregation port configuration mode

Usage Guidelines

The ports that are set to receive are not necessarily the ports of the backuplink group.

Example

Switch_config#interface gigaEthernet 0/1

Switch_config_g0/1#backup-link-group mmu receive

Switch_config_g0/1#exit

Related Command

None

### monitor-link-group id uplink

Syntax

To set a port to be an uplink port, run the following command:

monitor-link-group id uplink

To cancel the uplink port configuration, run the following command:

no monitor-link-group id

Parameter

| Parameter | Description |
| --- | --- |
| Id | Stands for the instance ID of the monitorlink group, ranging from 1 to 8. |

Default

The uplink port is not configured by default.

Command Mode

The physical port configuration mode and the aggregation port configuration mode

Usage Guidelines

If the Monitorlink group is not established, it will be automatically created when you configure the Monitorlink group on a port directly.

Example

Switch_config#interface gigaEthernet 0/1

Switch_config_g0/1# monitor-link-group 1 uplink

Switch_config_g0/1#exit

Related Command

monitor-link-group id

monitor-link-group id downlink

### monitor-link-group id downlink

Syntax

To set a port to be a downlink port, run the following command:

monitor-link-group id downlink

To cancel the downlink port configuration, run the following command:

no monitor-link-group id

Parameter

| Parameter | Description |
| --- | --- |
| Id | Stands for the instance ID of the monitorlink group, ranging from 1 to 8. |

Default

The downlink port is not configured by default.

Command Mode

The physical port configuration mode and the aggregation port configuration mode

Usage Guidelines

If the Monitorlink group is not established, it will be automatically created when you configure the Monitorlink group on a port directly.

Example

Switch_config#interface gigaEthernet 0/1

Switch_config_g0/1# monitor-link-group 1 downlink

Switch_config_g0/1#exit

Related Command

monitor-link-group id

monitor-link-group id uplink

## Show Configuration Commands

### show backup-link-group id

Syntax

To display the information about the backuplink group, run the following command:

show backup-link-group id

Parameter

| Parameter | Description |
| --- | --- |
| Id | Stands for the instance ID of the backuplink group, ranging from 1 to 8. |

Default

None

Command Mode

Monitoring mode, global configuration mode, node configuration mode or port configuration mode

Usage Guidelines

None

Example

Switch_config# show backup-link-group 1

Active Interface    Backup Interface    State                Vlan State

------------------------------------------------------------------------------

GigaEthernet0/2     GigaEthernet0/4     Forward/Block        Block/Block

Share load vlan: 100-200,port[GigaEthernet0/4] vlan state: Forwarding

Preemption Mode: No Preempt

Preemption Delay: 0 seconds

Related Command

None

### show monitor-link-group id

Syntax

To display the information about the monitorlink group, run the following command:

show monitor-link-group id

Parameter

| Parameter | Description |
| --- | --- |
| Id | Stands for the instance ID of the monitorlink group, ranging from 1 to 8. |

Default

None

Command Mode

Monitoring mode, global configuration mode, node configuration mode or port configuration mode

Usage Guidelines

None

Example

Switch_config#show monitor-link-group 1

uplink interface: GigaEthernet0/2     Forwarding

downlink interface:

GigaEthernet0/1     Forwarding

GigaEthernet0/3     Forwarding

Related Command

None