IP Access List Configuration Commands

Table of Contents

Chapter 1 Configuring Physical Interface IP Access List Command	1

1.1 IP Access List Configuration Commands	1

1.1.1 ip access-group	1

1.1.2 ipv6 access-group	2

# Chapter 1 Configuring Physical Interface IP Access List Command

## IP Access List Configuration Commands

IP access list configuration commands include:

- ip access-group

- ipv6 access-group

### ip access-group

Syntax

To apply an access control list to control packet access, use the ip access-group command in the appropriate configuration mode. To remove the specified access group, use the no form of this command.

In interface configuration mode:

[no] ip access-group name

In global configuration mode:

[no] ip access-group name [vlan {word | add word | remove word}]

Enable the IP access list that created on the port or global, or remove the IP access list that applied on the port or global.

Parameter

| Parameter | Description |
| --- | --- |
| name | Name of an IP access list |
| Vlan | Apply the access list on inbound of VLAN. |
| Word | VLAN range table |
| add | Add VLAN range table |
| remove | Remove VLAN range table |

Command Mode

Global configuration mode or interface configuration mode

Usage Guidelines

Most of the commands in the access list are configured by hardware. The commands that are not supported by hardware do not prompt errors, but have no actual effect. A small amount commands such as time-range is configured by software.

Note:

Ipv4 standard access list support:

Any: any source ip address

source-addr source-mask : source address matching

reverse-mask source-addr source-mask: reverse mask source address matching

ipv4 extended access list support:

any: any ip address

Ip-protocol: IP protocol number

ip -IP protocol

reverse-mask -protocols for the reverse mask configuration

eq/gt/lt/src-portrange/ dst-portrange: tcp/udp port number matching

gre: GRE protocol number matching

icmp: ICMP protocol number matching

igmp: IGMP protocol number matching

ospf: OSPF touting protocol number matching

tcp/udp port number matching can configure source port number matching and destination port number matching at the same time, but only the destination port number matching takes effect. However, if one of the matching methods is eq, the source port number matching and destination port number matching can take effect at the same time.

Example

The following example applies access list filter on packets inbound from Ethernet interface g0/1:

Switch_config#inter g0/1

Switch_config_g0/1# ip access-group filter

### ipv6 access-group

Syntax

To apply an access control list to control packet access, use the ipv6 access-group command in the port configuration mode. To remove the specified access group, use the no form of this command.

In interface configuration mode:

[no] ip access-group name

In global configuration mode:

[no] ipv6 access-group name [vlan {word | add word | remove word}]

Enable the IP6 access list that created on the port or global, or remove the IP6 access list that applied on the port or global.

Parameter

| Parameter | Description |
| --- | --- |
| name | Name of an IP access list |
| Vlan | Apply the access list on inbound of VLAN. |
| Word | VLAN range table |
| add | Add VLAN range table |
| remove | Remove VLAN range table |

Command mode

Global configuration mode or interface configuration mode

Usage Guidelines

Most of the commands in the access list are configured by hardware. The commands that are not supported by hardware do not prompt errors, but have no actual effect. A small amount commands such as time-range is configured by software.

Note:

Ipv6 access list support:

Any: any ipv6 address

Ipv6-addr/ host Ipv6-addr:  ipv6 address matching

Ipv6-protocol: ipv6 protocol number

eq/gt/lt/src-port-range/ dst-port-range: tcp/udp port number matching

dscp/flow-label: packets flied matching

tcp/udp port number matching can configure source port number matching and destination port number matching at the same time, but only the destination port number matching takes effect. However, if one of the matching methods is eq, the source port number matching and destination port number matching can take effect at the same time.

Example

The following example applies access list filter on packets inbound from Ethernet interface g0/1:

Switch_config#inter g0/1

Switch_config_g0/1# ipv6 access-group filter