Interface Cable Diagnostic Configuration Commands

Table of Contents

Chapter 1	Interface Cable Diagnostic Configuration Commands	1

1.1 Interface Cable Diagnostic Configuration Commands	1

1.1.1 cable-diagnostic	1

# Interface Cable Diagnostic Configuration Commands

## Interface Cable Diagnostic Configuration Commands

Interface cable diagnostic configuration commands include:

- cable-diagnostic

### cable-diagnostic

Syntax

[no] cable-diagnostic [ period]

- Enter the configuration port mode. Only used for physical electrical ports.

Parameter

| Parameter | Description |
| --- | --- |
| period | Indicates the diagnostic period (in seconds). Range is from 1 to 32767. |

Default

It is disabled by default. If period is not specified, it will only be detected once. But the diagnostic result will remain until you cancel it with the no form of the command.

Usage Guidelines

You can configure this command in port mode. There may be a delay during the configuration. After configuration, you can see the corresponding test results through show interface xxxx command.

Example

The following example performs cable diagnostic on port g0/4 once:

Switch_config#

Switch_config#interface gigaEthernet0/4

Switch_config_g0/4#cable-diagnostic

Switch_config#

Switch_config_g0/4#show int g0/4

。。。。。。。。。

Cable Ok (4 pairs)

Pair A Ok, length < 1 metres

Pair B Ok, length < 1 metres

Pair C Ok, length < 1 metres

Pair D Ok, length < 1 metres

................................
It shows that the diagnostic status of the four pairs of cables is OK, and the length is less than 1 meter.

Cable status:

l OK: indicates that the wire pair ends normally.

l Open: indicates that the wire pair is open.

l Short: indicates that the wire pair is short circuited.

l Crosstalk (crosstalk): indicates that there is crosstalk between the wire pairs (interference with each other).

l Unknown: Other causes of failure.

The following example performs cable diagnostic on port g0/4 for a period of 3 seconds:

Switch_config#

Switch_config#interface gigaEthernet0/4

Switch_config_g0/4#cable-diagnostic period 3

Switch_config#

Note:

The test results cannot guarantee the accuracy of the cables produced by all manufacturers. The test results are for reference only.

The diagnostic process may affect the normal use of services on the interface, so it is best to disable this function through the no form of the command after diagnostic.