- MAC Address Configuration Commands

- Table of Contents

Chapter 1 MAC Address Configuration Commands	1

1.1 MAC Address Configuration Commands	1

1.1.1 mac address-table static	1

1.1.2 mac address-table aging-time	1

1.1.3 show mac address-table	2

1.1.4 clear mac address-table	3

# MAC Address Configuration Commands

## MAC Address Configuration Commands

### mac address-table static

Syntax

[no] mac address-table static mac-addr vlan vlan-id interface interface-id

To add a static MAC address, run mac address-table static mac-addr vlan vlan-id interface interface-id. To cancel the static MAC address, run no mac address-table static mac-addr vlan vlan-id interface interface-id.

Parameter

| Parameter | Description |
| --- | --- |
| mac-addr | Means an MAC address. Value range: H.H.H |
| vlan-id | A VLAN that the MAC address belongs to Value range: 1-4094 |
| interface-id | Physical port that the MAC address belongs to |

Default value

N/A

Explanation

This command is configured in global configuration mode.

Example

The following example shows how to bind MAC address 0004.5600.67ab to port G0/2 of VLAN1.

Switch_config# mac address-table static 0004.5600.67ab vlan 1 interface g0/2

### mac address-table aging-time

Syntax

mac address-table aging-time [0 | 10-1000000]

To configure the aging time of the MAC address table, run the previous command.

Parameter

| Parameter | Description |
| --- | --- |
| 0 | Means that the MAC address never ages. |
| 10-1000000 | Aging time of the MAC address whose unit is second |

Default value

300s.

Explanation

This command is configured in global configuration mode.

Example

The following example shows how to set the aging time of the MAC address to 100 seconds.

Switch_config# mac address-table aging-time 100

### show mac address-table

Syntax

show  mac address-table [dynamic [interface interface-id | vlan vlan-id] | static | brief | multicast | interface interface-id | vlan vlan-id | H.H.H]

To display the MAC address table of the switch, run show mac address-table {dynamic [interface interface-id | vlan vlan-id] | static | brief | multicast | interface interface-id | vlan vlan-id | H.H.H].

Parameter

| Parameter | Description |
| --- | --- |
| dynamic | Dynamically-learned MAC address table |
| interface-id | Name of an interface |
| vlan-id | VLAN ID。 Value range: 1-4094 |
| static | Static MAC address table |
| brief | Brief information about the MAC address |
| multicast | Multicast MAC address table |
| Interface | Port’s MAC address table |
| Vlan | Vlan mac address table |
| H.H.H | Specific address |

Default value

N/A

Explanation

This command is used to display the MAC address table.

Example

The following example shows how to display all dynamic MAC address tables.

Switch_config#show mac address-table

Mac Address Table (Total 2)

------------------------------------------

Vlan    Mac Address       Type       Ports

----    -----------       ----       -----

1       0026.5a7c.fad3    DYNAMIC    g0/24

1       0000.0000.0004    DYNAMIC    g0/24

### clear mac address-table

Syntax

clear mac address-table dynamic [address mac-addr | interface interface-id | vlan vlan-id]

To delete a dynamic MAC address, run the above-mentioned command:

Parameter

| Parameter | Description |
| --- | --- |
| address mac-addr | Means an MAC address. Value range: H.H.H |
| interface-id | Means a name of a L2 interface. |
| vlan-id | VLAN ID。 Value range: 1-4094 |

Default value

N/A

Remarks

This command is used in EXEC mode.

Example

The following example shows how to clear all dynamically learned MAC addresses on interface g0/2.

Switch# clear mac address-table dynamic interface g0/2