- LLDP Configuration Commands

- Table of Contents

LLDP Configuration Commands	1

Table of Contents	I

Chapter 1 LLDP Configuration Commands	1

1.1 LLDP Commands	1

1.1.1 lldp　run	1

1.1.2 lldp　holdtime	2

1.1.3 lldp　timer	2

1.1.4 lldp　reinit	3

1.1.5 lldp　tlv-select	4

1.1.6 lldp　transmit	5

1.1.7 lldp　receive	6

1.1.8 Show　lldp　errors	6

1.1.9 Show　lldp　interface	7

1.1.10 show lldp　neighbors	8

1.1.11 Show　lldp　neighbors　detail	9

1.1.12 Show　lldp　traffic	11

1.1.13 clear　lldp　counters	12

1.1.14 clear　lldp　table	13

1.1.15 debug　lldp　errors	13

1.1.16 debug　lldp　events	14

1.1.17 debug　lldp　packets	15

1.1.18 debug　lldp　states	16

# LLDP Configuration Commands

## LLDP Commands

### lldp　run

Description

- lldp　run

no　lldp　run

To start up LLDP, run lldp run; to shut down LLDP, run no lldp run.

Parameter

None

Default

Shut down

Explanation

None

Command mode

Global configuration mode

Example

The following command is used to start up LLDP.

Switch_config# lldp　run

### lldp　holdtime

Description

lldp　holdtime　time

no　lldp　holdtime

To configure the ttl value of LLDP, run lldp holdtime time. To resume the default transmission delay, run no lldp holdtime.

Parameter

| Parameter | Description |
| --- | --- |
| time | Storage time of the transmitted message, ranging between zero to 65535 seconds |

Default

120s

Explanation

None

Command mode

Global configuration mode

Example

The following example shows how to set the ttl value of LLDP to 100 seconds.

Switch_config# lldp　holdtime 100

Switch_config#

### lldp　timer

Description

lldp　timer  time

no lldp　timer

To configure the transmission delay of LLDP, run lldp timer time. To resume the default transmission delay, run no lldptimer.

Parameter

| Parameter | Description |
| --- | --- |
| time | Interval for transmitting the LLDP message, ranging between 5 to 65534 seconds. |

Default

30s

Explanation

The transmission interval of the LLDP message must be shorter than its storage time, ensuring multiple updates in the storage time and preventing error which is led by packet loss.

Command mode

Global configuration mode

Example

The following example shows how to configure the transmission interval of LLDP to 24 seconds.

Switch_config# lldp　timer　24

Switch_config#

### lldp　reinit

Description

lldp　reinit　time

no lldp　reinit

To configure the transmission delay of LLDP, run lldp reinit time. To resume the default transmission delay, run no lldp reinit.

Parameter

| Parameter | Description |
| --- | --- |
| time | Transmission delay of LLDP, whose values range from two to five seconds |

Default

２ s

Explanation

None

Command mode

Global configuration mode

Example

The following example shows how to set the transmission delay of LLDP to five seconds.

Switch_config# lldp　reinit　５

Switch_config#

### lldp　tlv-select

Description

lldp　tlv-select　tlv-type

no lldp　tlv-select　tlv-type

To add TLV which is transmitted by the LLDP message, run lldp tlv-select tlv-type. To delete TLV which is transmitted by the LLDP message, run no lldp tlv-select tlv-type.

Parameter

| Parameter | Description |
| --- | --- |
| tlv-type | TLV that can be sent or not whose values are: macphy-confg ,management-address port-description ,port-vlan ,system-capabilities system-description ,system-name |

Default

All TLVs are sent.

Explanation

Three mandatory TLVs must be sent.

Command mode

Global configuration mode

Example

The following example shows how to enable the port description not to be transmitted in the message.

Switch_config#no　lldp　tlv-select　port-description

Switch_config#

### lldp　transmit

Description

lldp　transmit

no lldp　transmit

To set the port to send the LLDP message, run lldp transmit. To forbid receiving the LLDP message, run no lldp transmit.

Parameter

None

Default

Transmittable LLDP message mode

Explanation

Only after the LLDP module is started can the command be valid.

Command mode

Port configuration mode

Example

The following example shows how to set port f0/0 not to send the LLDP message.

Switch_config_f0/0# no lldp　transmit

Switch_config_f0/0#

### lldp　receive

Description

lldp　receive

no lldp　receive

To set the port to the receivable LLDP message mode, run lldp receive. To forbid receiving the LLDP message, run no lldp receive.

Parameter

None

Default

Receivable LLDP message mode

Explanation

Only after the LLDP module is started can the configuration be valid.

Command mode

Port configuration mode

Example

The following example shows how to set port f0/0 to the LLDP message mode.

Switch_config_f0/0# no lldp　receive

Switch_config_f0/0#

### Show　lldp　errors

Description

Show　lldp　errors

It is used to display the error information about the LLDP module.

Parameter

None

Default

None

Explanation

None

Command mode

EXEC/Global configuration mode

Example

The following example shows how to set port f0/0 to the LLDP message mode.

switch_config#show lldp errors

LLDP errors/overflows:

Total memory allocation failures: 0

Total encapsulation failures: 0

Total table overflows: 0

switch_config#

### Show　lldp　interface

Description

Show　lldp　interface　interface-name

To check the transmission and reception mode, run show lldp interface interface name.

Parameter

| Parameter | Description |
| --- | --- |
| interface-name | Name of the interface, such as f0/1 and fastethernet0/1 |

Default

None

Explanation

After LLDP is started, you can check the state of the port.

Command mode

EXEC/global configuration mode

Example

The following example shows how to check the transmission and reception mode of port f0/1.

switch_config#show lldp interface f0/1

FastEthernet0/1:

Rx: enabled

Tx: enabled

switch_config#

### show lldp　neighbors

Description

show lldp　neighbors

It is used to display the simple information about neighbors.

Parameter

None

Default

None

Explanation

The command is used to display the simple information about neighbors.

Command mode

EXEC / global configuration mode

Example

switch_config#show lldp neighbors

Capability Codes:

(R)Router,(B)Bridge,(C)DOCSIS Cable Device,(T)Telephone

(W)WLAN Access Point, (P)Repeater,(S)Station,(O)Other

Device-ID       Local-Intf      Hldtme       Port-ID      Capability

switch          Fas0/2          115          Fas0/32      B

switch          Fas0/32         114          Fas0/2       B

Total entries dispalyed: 2

switch_config#

### Show　lldp　neighbors　detail

Description

Show　lldp　neighbors　detail

It is used to display the detailed information about the neighbor.

Parameter

None

Default

None

Explanation

None

Command mode

EXEC/global configuration mode

Example

switch_config#show lldp neighbors detail

chassis id: 00e0.0f61.ca53

port id: Fas0/32

port description: FastEthernet0/32

system name: switch.

system description: (tm) S3448 Software, Version 2.0.1K

Serial: S35000456

Copyright by Shanghai Baud Data Communication CO. LTD.

Compiled: 2008-11-13 13:33:36 by 16170F032B9F

Time remaining: 98

system capabilities: R B

enabled capabilities: B

Managment Address:

IP: 192.168.213.62

Auto Negotiation -- supported,enabled

Physical media capabilitise:

100baseTX(FD)

100baseTX(HD)

10baseT(FD)

10baseT(HD)

Media Attachment Unit type: 16

------------------------------------------

chassis id: 00e0.0f61.ca35

port id: Fas0/2

port description: FastEthernet0/2

system name: switch.

system description: (tm) S3448 Software, Version 2.0.1K

Serial: S35000456

Copyright by Shanghai Baud Data Communication CO. LTD.

Compiled: 2008-11-13 13:33:36 by 16170F032B9F

Time remaining: 95

system capabilities: R B

enabled capabilities: B

Managment Address:

IP: 90.0.0.66

Auto Negotiation -- supported,enabled

Physical media capabilitise:

100baseTX(FD)

100baseTX(HD)

10baseT(FD)

10baseT(HD)

Media Attachment Unit type: 16

------------------------------------------

Total entries dispalyed: 2

switch#

### Show　lldp　traffic

Description

Show　lldp　traffic

To display all statistics information about LLDP, run show lldp traffic.

Parameter

None

Default

None

Explanation

None

Command mode

EXEC/global configuration mode

Example

switch_config#show lldp traffic

LLDP traffic statistics:

Total frames out: 1599

Total entries aged: 0

Total frames in: 624

Total frames received in error: 0

Total frames discarded: 0

Total TLVs unrecognized: 0

switch_config#

### clear　lldp　counters

Description

clear　lldp　counters

To clear the statistics information, run clear lldp counters.

Parameter

None

Default

None

Explanation

None

Command mode

EXEC

Example

switch#clear lldp counters

switch#

### clear　lldp　table

Description

clear　lldp　table

To remove the neighbor list, run clear lldp table.

Parameter

None

Default

None

Explanation

None

Command mode

EXEC

Example

switch#clear lldp table

switch#

### debug　lldp　errors

Description

debug　lldp　errors

To report some error information about the LLDP module, run debug lldp errors.

Parameter

None

Default

None

Explanation

None

Command mode

EXEC

Example

switch#debug lldp errors

debug lldp errors on

switch#show　debug

lldp errors  debugging is on

switch#Nov 14 09:39:04LLDP recive a bad frame on interface FastEthernet0/2

Nov 14 09:49:44LLDP transmit fail on interface FastEthernet0/2

### debug　lldp　events

Description

debug　lldp　events

To report some special events about the LLDP module, run debug lldp events.

Parameter

None

Default

None

Explanation

None

Command mode

EXEC

Example

switch#debug lldp events

debug lldp events on

switch#show　debug

lldp event debugging is on

switch#config

switch_config#int f0/2

switch_config_f0/2#no lldp tr

switch_config_f0/2#Nov 14 09:39:04 LLDP transmit a end packet on interface FastEthernet0/2

Nov 14 09:44:08LLDP recive a unrecognized tlv frame on interface FastEthernet0/2

### debug　lldp　packets

Description

debug　lldp　packets

To report the message transmission event of the LLDP module, run debug lldp packets.

Parameter

None

Default

None

Explanation

None

Command mode

EXEC

Example

switch#debug lldp packets

debug lldp packets on

switch#show　debug

lldp packet debugging is on

switch#Nov 13 16:38:20 LLDP advertisement packet TX'd on intf FastEthernet0/2

Nov 13 16:38:20 LLDP advertisement packet RX'd on intf FastEthernet0/32

### debug　lldp　states

Description

debug　lldp　states

To report the information about the state of the LLDP port, run debug lldp states.

Parameter

None

Default

None

Explanation

None

Command mode

EXEC

Example

switch#debug lldp states

debug lldp states on

switch#show　debug

lldp state　debugging is on

switch#Nov 13 16:39:51 LLDP transmit state on FastEthernet0/2 set to TX FRAME

Nov 13 16:39:51 LLDP transmit state on FastEthernet0/2 set to IDLE

Nov 13 16:39:51 LLDP receive state on FastEthernet0/32 set to RX FRAME

Nov 13 16:39:51 LLDP receive state on FastEthernet0/32 set to WAIT FOR FRAME