ELPS Configuration Commands

Table of Contents

Chapter 1 ELPS Configuration Commands	1

1.1 Global Commands	1

1.1.1 elps id	1

1.1.2 working-vlanmap vlanmap	2

1.1.3 protection-mode {1plus1-bidirectional | 1plus1-unidirectional | 1to1-bidirectional}	3

1.1.4 revertive-mode {revertive | nonrevertive}	4

1.1.5 detect-fault {physical-port-check| continuity-check | both-check}	5

1.1.6 WTR-time	6

1.1.7 hold-off-time	7

1.2 Port Configuration Commands	8

1.2.1 elps id {working-transport | protection-transport}	8

1.2.2 elps id  mep md md-string ma ma-string level level-id local local-id remote remote-id	9

1.3 Control Commands	10

1.3.1 elps id LockOut	10

1.3.2 elps id ForcedSwitch	11

1.3.3 elps id ManualSwitch	12

1.3.4 elps id ManualSwitch-Working	13

1.3.5 elps id Exercise	14

1.3.6 elps id CLEAR	14

1.4 Show	15

1.4.1 show elps	15

# ELPS Configuration Commands

## Global Commands

### elps id

To set an instance of ELPS node and enter the node mode, run the following command:

elps id

To cancel an instance of ring, run the following command:

no elps id

Parameter

| Parameter | Description |
| --- | --- |
| id | Stands for the node instance ID, which ranges from 0 to 7. |

Default value

By default, the ELPS node instance is not configured.

Command mode

Global configuration mode

Explanation

N/A.

Example

S1_config#elps 1

S1_config_elps1#

Related command

N/A.

### working-vlanmap vlanmap

To set the working vlan map of the ELPS node, run the following command:

working-vlanmap vlanmap

Parameter

| Parameter | Description |
| --- | --- |
| vlanmap | Stands for the VLAN range table (1-4094), which is similar with (1,3,5,7), (1,3-5,7) or (1-7). |

Default value

By default, the working vlan map of a node is not configured.

Command mode

EPLS node configuration mode

Explanation

1. Configuring working VLAN does not mean creating this VLAN, so you have to establish the control VLAN manually.。

2. After working-vlanmap, protection-mode, revertive-mode and detect-fault are configured, the ELPS configuration mode exits and the ELPS node is started, the working-vlanmap cannot be modified.

Example

S1_config#elps 1

S1_config_elps1#working-vlanmap 1-10

S1_config_elps1#

Related command

protection-mode {1plus1-bidirectional | 1plus1-unidirectional | 1to1-bidirectional}

revertive-mode {revertive | nonrevertive}

detect-fault {physical-port-check| continuity-check | both-check}

### protection-mode {1plus1-bidirectional | 1plus1-unidirectional | 1to1-bidirectional}

To set the protection mode of the ELPS node to be the 1plus1-bidirectional mode, run the following command:

protection-mode 1plus1-bidirectional

To set the protection mode of the ELPS node to be the 1plus1-unidirectional mode, run the following command:

protection-mode 1plus1-unidirectional

To set the protection mode of the ELPS node to be the 1to1-bidirectional mode, run the following command:

protection-mode 1to1-bidirectional

Parameter

N/A.

Default value

By default, the ELPS node’s protection mode is not configured.

Command mode

ELPS node configuration mode

Explanation

1. When the ELPS node's protection mode is set to 1plus1-bidirectional or 1plus1-unidirectional, the revertive mode of node is non-revertive by default; the protection mode is 1to1-bidirectional, it is revertive by default.

2. After working-vlanmap, protection-mode, revertive-mode and detect-fault are configured, the ELPS configuration mode exits and the ELPS node is started, the working-vlanmap cannot be modified.

Example

S1_config#elps 1

S1_config_elps1#working-vlanmap 1-10

S1_config_elps1#protection-mode 1plus1-bidirectional

S1_config_elps1#

Related command

working-vlanmap vlanmap

revertive-mode {revertive | nonrevertive}

detect-fault {physical-port-check| continuity-check | both-check}

### revertive-mode {revertive | nonrevertive}

To set the mode of the ELPS to revertive, run the following command:

revertive-mode revertive

To set the mode of the ELPS to non-revertive, run the following command:

revertive-mode nonrevertive

Parameter

N/A.

Default value

By default, the ELPS node’s mode is not configured.

Command mode

ELPS node configuration mode

Explanation

1. When the ELPS node's protection mode is set to 1plus1-bidirectional or 1plus1-unidirectional, the revertive mode of node is non-revertive by default; the protection mode is 1to1-bidirectional, it is revertive by default. The two cases change after the configuration of the revertive mode.

2. After working-vlanmap, protection-mode, revertive-mode and detect-fault are configured, the ELPS configuration mode exits and the ELPS node is started, the working-vlanmap cannot be modified.

Example

S1_config#elps 1

S1_config_elps1#working-vlanmap 1-10

S1_config_elps1#protection-mode 1plus1-bidirectional

S1_config_elps1#revertive-mode revertive

S1_config_elps1#

Related command

working-vlanmap vlanmap

protection-mode {1plus1-bidirectional | 1plus1-unidirectional | 1to1-bidirectional}

detect-fault {physical-port-check| continuity-check | both-check}

### detect-fault {physical-port-check| continuity-check | both-check}

To set the trouble monitoring mode of the ELPS node to physical-port-check, run the following command:

detect-fault physical-port-check

To set the trouble monitoring mode of the ELPS node to continuity-check, run the following command:

detect-fault continuity-check

To set the trouble monitoring mode of the ELPS node to both-check, run the following command:

detect-fault both-check

Default value

By default, the ELPS node’s trouble monitoring mode is not configured.

Command mode

ELPS node configuration mode

Explanation

1. After working-vlanmap, protection-mode, revertive-mode and detect-fault are configured, the ELPS configuration mode exits and the ELPS node is started, the working-vlanmap cannot be modified.

Example

S1_config#elps 1

S1_config_elps1#working-vlanmap 1-10

S1_config_elps1#protection-mode 1plus1-bidirectional

S1_config_elps1#revertive-mode revertive

S1_config_elps1#detect-fault continuity-check

S1_config_elps1#

Related command

working-vlanmap vlanmap

protection-mode {1plus1-bidirectional | 1plus1-unidirectional | 1to1-bidirectional}

revertive-mode {revertive | nonrevertive}

### WTR-time

To set the WTR time of the ELPS node, run the following command:

WTR-time value

To resume the default WTR time of the ELPS node, run the following command:

no WTR-time

Parameter

| Parameter | Description |
| --- | --- |
| value | Stands for the WTR time, which ranges from 5 to 12 minutes. Its step is 1 minute and its default value is 5 minutes. |

Default value

By default, the WTR-time is 5 minutes.

Command mode

ELPS node configuration mode

Explanation

N/A.

Example

S1_config#elps 1

S1_config_elps1#working-vlanmap 1-10

S1_config_elps1#protection-mode 1plus1-bidirectional

S1_config_elps1#revertive-mode revertive

S1_config_elps1#detect-fault continuity-check

S1_config_elps1#WTR-time 6

S1_config_elps1#

Related command

N/A

### hold-off-time

To set the hold-off time of the ELPS node, run the following command:

hold-off-time value

To resume the default hold-off time of the ELPS node, run the following command:

no hold-off-time

Parameter

| Parameter | Description |
| --- | --- |
| value | Stands for the hold-off time, which ranges from 1 to 10 seconds. Its step is 100ms and its default value is 1 second. |

Default value

By default, the hold-off time is one second.

Command mode

ELPS node configuration mode

Explanation

N/A.

Example

S1_config#elps 1

S1_config_elps1#working-vlanmap 1-10

S1_config_elps1#protection-mode 1plus1-bidirectional

S1_config_elps1#revertive-mode revertive

S1_config_elps1#detect-fault continuity-check

S1_config_elps1#hold-off-time 2

S1_config_elps1#

Related command

N/A

## Port Configuration Commands

### elps id {working-transport | protection-transport}

To set a port where the ELPS working transport entity is located, run the following command:

elps id working-transport

To delete the ELPS working transport entity configuration on a port, run the following command:

no elps id working-transport

To set a port where the ELPS protection transport entity is located, run the following command:

elps id protection-transport

To delete the ELPS protection transport entity configuration on a port, run the following command:

no elps id protection-transport

Parameter

| Parameter | Description |
| --- | --- |
| id | ID of the node |

Default value

No ELPS configuration exists on ports by default.

Command mode

The physical port configuration mode and the converged port configuration mode

Explanation

The port cannot be configured until working-vlanmap, protection-mode, revertive-mode and default-fault are all configured.

Example

S1_config#interface GigaEthernet 0/1

S1_config_g0/1# elps 1 working-transport

S1_config_g0/1#exit

Related command

elps id  mep md md-string ma ma-string level level-id local local-id remote remote-id

### elps id  mep md md-string ma ma-string level level-id local local-id remote remote-id

To set the MEP information about the ELPS port, run the following command:

elps id  mep md md-string ma ma-string level level-id local local-id remote remote-id

To delete the MEP information about the ELPS port, run the following command:

no elps id  mep

Parameter

| Parameter | Description |
| --- | --- |
| id | ID of the node |
| md-string | MEP maintenance domain |
| ma-string | MEP maintenance link |
| level-id | MEP level |
| local-id | Local MEP ID |
| remote-id | Remote MEP ID |

Default value

No MEP information exists on ports by default.

Command mode

The physical port configuration mode and the converged port configuration mode

Explanation

The port cannot be configured until working-vlanmap, protection-mode, revertive-mode and default-fault, transport entity of the ELPS port are all configured.

Example

S1_config#interface GigaEthernet 0/1

S1_config_g0/1#elps 1 working-transport

S1_config_g0/1#elps 1 mep md x ma x level 1 local 1 remote 2

S1_config_g0/1#exit

Related command

elps id {working-transport | protection-transport}

## Control Commands

### elps id LockOut

To set the protection lockout of ELPS, run the following command:

elps id LockOut

Parameter

| Parameter | Description |
| --- | --- |
| Id | ID of the node |

Default value

N/A.

Command mode

Monitoring mode

Explanation

N/A.

Example

N/A.

Related command

elps id ForcedSwitch

elps id ManualSwitch

elps id ManualSwitch-Working

elps id Exercise

elps id CLEAR

### elps id ForcedSwitch

To set the forced switching operation of ELPS, run the following command:

elps id ForcedSwitch

Parameter

| Parameter | Description |
| --- | --- |
| Id | ID of the node |

Default value

N/A.

Command mode

Monitoring mode

Explanation

N/A.

Example

N/A.

Related command

elps id LockOut

elps id ManualSwitch

elps id ManualSwitch-Working

elps id Exercise

elps id CLEAR

### elps id ManualSwitch

To set the manual switching operation of ELPS, run the following command:

elps id ManualSwitch

Parameter

| Parameter | Description |
| --- | --- |
| Id | ID of the node |

Default value

N/A.

Command mode

Monitoring mode

Explanation

N/A.

Example

N/A.

Related command

elps id LockOut

elps id ForcedSwitch

elps id ManualSwitch-Working

elps id Exercise

elps id CLEAR

### elps id ManualSwitch-Working

To switch to the working entity of ELPS manually, run the following command:

elps id ManualSwitch-Working

Parameter

| Parameter | Description |
| --- | --- |
| Id | ID of the node |

Default value

N/A.

Command mode

Monitoring mode

Explanation

N/A.

Example

N/A.

Related command

elps id LockOut

elps id ForcedSwitch

elps id ManualSwitch

elps id Exercise

elps id CLEAR

### elps id Exercise

To set the exercise operation of ELPS, run the following command:

elps id Exercise

Parameter

| Parameter | Description |
| --- | --- |
| Id | ID of the node |

Default value

N/A.

Command mode

Monitoring mode

Explanation

N/A.

Example

N/A.

Related command

elps id LockOut

elps id ForcedSwitch

elps id ManualSwitch

elps id ManualSwitch-Working

elps id CLEAR

### elps id CLEAR

To clear the control command of ELPS, run the following command:

elps id CLEAR

Parameter

| Parameter | Description |
| --- | --- |
| Id | ID of the node |

Default value

N/A.

Command mode

Monitoring mode

Explanation

N/A.

Example

N/A.

Related command

elps id LockOut

elps id ForcedSwitch

elps id ManualSwitch

elps id ManualSwitch-Working

elps id Exercise

## Show

### show elps

To display the summary information about the ELPS node, run the following command:

show elps id

To display the detailed information about the ELPS node, run the following command:

show elps id detail

To display the information about the ELPS port, run the following command:

show elps id interface intf-name

To display the summary information about all ELPS nodes, run the following command:

show elps

Parameter

| Parameter | Description |
| --- | --- |
| id | ID of the node |
| intf-name | Name of an interface |

Default value

N/A.

Command mode

Monitoring mode, global configuration mode, node configuration mode or port configuration mode

Explanation

N/A.

Example

N/A.

Related command

N/A.