- IGMP-SNOOPING Configuration Commands

- Table of Contents

Chapter 1 IGMP-SNOOPING Configuration Commands	1

1.1.1 igmp-snooping	1

1.1.2 igmp-snooping static	2

1.1.3 igmp-snooping immediate-leave	3

1.1.4 igmp-snooping dlf-frames	3

1.1.5 igmp-snooping router age	4

1.1.6 igmp-snooping response time	5

1.1.7 igmp-snooping querier	5

1.1.8 show ip igmp-snooping	6

1.1.9 show ip igmp-snooping timer	7

1.1.10 show ip igmp-snooping groups	7

1.1.11 show ip igmp-snooping statistics	8

1.1.12 debug ip igmp-snooping packet	9

1.1.13 debug ip igmp-snooping timer	9

# IGMP-SNOOPING Configuration Commands

IGMP-SNOOPING configuration commands include：

- ip igmp-snooping

- ip igmp-snooping static

- ip igmp-snooping immediate-leave

- ip igmp-snooping dlf-frames

- ip igmp-snooping router age

- ip igmp-snooping response time

- ip igmp-snooping querier

- show ip igmp-snooping

- show ip igmp-snooping timer

- show ip igmp-snooping groups

- show ip igmp-snooping statistics

- debug ip igmp-snooping packet

- debug ip igmp-snooping timer

### igmp-snooping

Description

To enable the IGMP-snooping of VLAN, use the ip igmp-snooping command. Use the no form of this command to restore the default.

ip igmp-snooping [ vlan vlan_id ]

no ip igmp-snooping [ vlan vlan_id ]

Parameter

| Parameter | Description |
| --- | --- |
| vlan id | VLAN identity。Value is from 1 to 4094。 |

Default

disabled

Instruction

If not specified the vlan parameter, this command enable or disable all vlans in the system (IGMP-snooping currently can be ran on 16 vlans at most at the same time.)

Example

The following command enables IGMP snooping of vlan 1:

switch(config)# ip igmp-snooping vlan 1

switch(config)#

### igmp-snooping static

Description

ip igmp-snooping vlan vlan_id static A.B.C.D interface intf

no ip igmp-snooping vlan vlan_id static A.B.C.D interface intf

Parameter

| Parameter | Description |
| --- | --- |
| vlan id | VLAN identity。Value is from 1 to 4094。 |
| A.B.C.D | Specifies the IP address of multicast |
| inft | Specifies the interface |

Default

None

Instruction

Use this command to configurethe the static multicast address of vlan. Use the no form of this command to delete the address.

Example

The following command adds the static multicast address at 234.5.6.7 to the fast ethernet interface 5 of vlan 2:

switch(config)# ip igmp-snooping vlan 2 static 234.5.6.7 interface FastEthernet0/5

switch(config)#

Ｎｏｔｅ：

224.0.0.0-224.0.0.255, as the multicast address which cannot be routed, cannot be registered to each interface.

### igmp-snooping immediate-leave

Description

To configure the immediate-leave characteristic of vlan, use the ip igmp-snooping vlan command. Use the no form of this command to restore the default value.

ip igmp-snooping vlan  vlan_id  immediate-leave

no ip igmp-snooping vlan  vlan_id  immediate-leave

Parameter

| Parameter | Description |
| --- | --- |
| vlan id | VLAN identity。Value is from 1 to 4094。 |

Default

disabled

Instruction

None

Example

The following command enables immediate-leave characteristic of vlan 1:

switch(config)# ip igmp-snooping vlan 1 immediate-leave

switch(config)#

### igmp-snooping dlf-frames

Description

ip igmp-snooping dlf-frames filter

no ip igmp-snooping dlf-frames

Parameter

| Parameter | Description |
| --- | --- |
| filter | Filters for the multicast packets of the unregistered address |

Default

None

Instruction

Use this command to configure filter mode for the multicast packets of the unregistered destination address. Use the no form of this command to restore the default configuration( broadcasting in vlan)

Example

The following example configures to drop multicast packets of the unregistered destination address in all vlans:

switch(config)# ip igmp-snooping dlf-frames filter

switch(config)#

### igmp-snooping router age

Description

ip igmp-snooping  timer  router-age  timer_value

no ip igmp-snooping  timer  router-age  timer_value

Parameter

| Parameter | Description |
| --- | --- |
| time value | Query timer time. Value is from 10 to 2147483647。 |

Default

260  seconds

Instruction

Use this command to configure query timer time. Use the no form of this command to restore the default value.

Example

The following example configures router-age to 300 seconds

switch(config)# ip igmp-snooping timer router-age 300

switch(config)#

### igmp-snooping response time

Description

To configure the maximum response time of IGMP-snooping, use IGMP-snooping command. Use the no form of this command to restore the default value.

ip igmp-snooping timer response-time timer_value

no ip igmp-snooping timer response-time timer_value

Parameter

| Parameter | Description |
| --- | --- |
| time value | Query timer time 。Value is from 1 to 255。 |

Default

10 seconds

Instruction

None

Example

The following example configures response-time to 20 seconds:

switch(config)# ip igmp-snooping timer response-time 20

switch(config)#

### igmp-snooping querier

Description

To activate IGMP-snooping querier mechanism or configure the source ip address of the spontaneous query packets. Use the no form of this command to restore the default value.

ip igmp-snooping querier [address <ip_addr>]

no ip igmp-snooping querier [address <ip_addr>]

Parameter

| Parameter | Description |
| --- | --- |
| ip_addr | Tthe common unicast IP address |

Default

disabled，the default source IP address is 10.0.0.200。

Instruction

None

Example

The following command activates IGMP quirier:

switch(config)# ip igmp-snooping querier

switch(config)#

### show ip igmp-snooping

Description

show ip igmp-snooping

Parameter

None

Default

None

Instruction

Use this command to show configuration information of IGMP-snooping.

Example

The following example shows the vlan information of the running ipmp-snooping:

switch(config)# show ip igmp-snooping

igmp-snooping router age:260 s

igmp-snooping response time:10 s

vlan 1

------------

running

Router: 90.0.0.120(F0/2)

switch(config)#

### show ip igmp-snooping timer

Description

show ip igmp-snooping timer

Parameter

None

Default

None

Instruction

Use this command to show timer information of IGMP.

Example

The following example shows timer information of igmp-snooping:

switch(config)# show ip igmp-snooping timer

vlan 1 router age : 246

switch(config)#

### show ip igmp-snooping groups

Description

show ip igmp-snooping groups

Parameter

None

Default

None

Instruction

Use this command to configure multicast group information of IGMP-snooping.

Example

The following example shows the multicast group information of igmp-snooping:

switch(config)# show ip igmp-snooping groups

Vlan Source          Group           Type Port(s)

---- --------------- --------------- ---- -------------------------------------

1 0.0.0.0         234.5.6.6       IGMP F0/2

1 0.0.0.0         239.255.255.250 IGMP F0/2

switch(config)#

### show ip igmp-snooping statistics

Description

show ip igmp-snooping statistics

Parameter

None

Default

None

Instruction

Use this command to show IGMP-snooping statistics.

Example

The following example shows igmp-snooping statistics:

switch(config)# show ip igmp-snooping statistics

vlan 1

------------

v1_packets:1

v2_packets:2

v3_packets:0

general_query_packets:1

special_query_packets:2

join_packets:0

leave_packets:0

send_query_packets:0

err_packets:0

switch(config)#

### debug ip igmp-snooping packet

Description

debug ip igmp-snooping packet

no debug ip igmp-snooping packet

Parameter

None

Default

None

Instruction

Use this command to enable/disable the packet debugging switch of IGMP-snooping.

Example

The followig command enables the packet debugging switch of igmp-snooping:

switch(config)# debug ip igmp-snooping packet

switch(config)#

### debug ip igmp-snooping timer

Description

debug ip igmp-snooping timer

no debug ip igmp-snooping timer

Parameter

None

Default

None

Instruction

Use this command to enable/disable the timer debugging switch of IGMP-snooping

Example

The following example enables timer debugging switch of igmp-snooping:

switch(config)# debug ip igmp-snooping timer

switch(config)#