- IGMP Proxy Commands

- Table of Contents

Chapter 1 IGMP Proxy Commands	1

1.1 ip igmp-proxy enable	1

1.2 ip igmp-proxy agent-vlan client-vlan map	2

1.3 ip igmp-proxy source svlan sport	2

1.4 show ip igmp-proxy mcache	3

1.5 show ip igmp-proxy	4

1.6 debug ip igmp-proxy	5

# IGMP Proxy Commands

IGMP-proxy commands include:

Ip igmp-proxy

ip igmp-proxy agent-vlan client-vlan map

ip igmp-proxy source svlan sport

show ip igmp-proxy mcache

show ip igmp-proxy

debug ip igmp-proxy

## ip igmp-proxy enable

- Syntax

ip igmp-proxy enable

no ip igmp-proxy enable

To enable or disable　ＩＧＭＰ　ｐｒｏｘｙ, run ip igmp-proxy enable. To resume the default value, run no ip igmp-proxy enable.

- Parameter

| Parameter | Description |
| --- | --- |
| N/A |  |

- Default value

The IGMP proxy is disabled by default.

- Remarks

N/A

- Example

The following example shows how to enable the IGMP proxy:

switch_config# ip igmp-proxy enable

switch_config#

## ip igmp-proxy agent-vlan client-vlan map

- Syntax

ip igmp-proxy agent-vlan avlan_map  client-vlan map  cvlan_map

no ip igmp-proxy agent-vlan avlan_map  client-vlan map  cvlan_map

- Parameter

| Parameter | Description |
| --- | --- |
| avlan_map | Ａｇｅｎｔ ＶＬＡＮ |
| cvlan_map | Customer VLAN |

- Default value

N/A

- Remarks

1． The represented VLAN cannot be configured before vlan is designated by avlan_map; also, the agent VLAN cannot be configured before cvlan_map.

2. The represented and agent VLANs must accept the control of IGMP-Snooping.

- Example

The following example shows how to set vlan2 to be the agent vlan of vlan1.

switch_config# ip igmp-proxy agent-vlan  2  client-vlan map  1

switch_config#

## ip igmp-proxy source svlan sport

- Syntax

ip igmp-proxy source multi_ip src_ip svlan vlan_id sport intf_name

no ip igmp-proxy source multi_ip src_ip svlan vlan_id sport intf_name

To set the entries of the static multicast source, run the above-mentioned command.

- Parameter

| Parameter | Description |
| --- | --- |
| multi_ip | Multicast IP address |
| src_ip | Multicast source’s IP address |
| vlan_id | Multicast source VLAN |
| intf_name | Physical port of the multicast source |

- Default value

N/A.

- Remarks

Only when there are multicast source entries can IGMP-proxy work normally. What is mentioned here is to add static multicast source entries.

- Example

The following example shows how to add a multicast source entry with 234.5.6.7 as its static multicast address, 1.2.3.4 as its source address, 2 as its source VLAN and gigabit portg0/1 as its source port.

switch_config# ip igmp-proxy source  234.5.6.7  1.2.3.4  svlan  2  sport  g0/1

switch_config#

Note:

The SVLAN mentioned here is the multicast source VLAN and the vlan ID of SVLAN cannot be that of represented VLAN.

## show ip igmp-proxy mcache

- Syntax

show ip igmp-proxy mcache [delete | nonsync | sync | static]

- Parameter

| Parameter | Description |
| --- | --- |
| delete | Displays those entries of which hardware caches are deleted but software caches do not time out. |
| nonsync | Displays those entries that have been processed but not yet synchronized to the hardware cache. |
| sync | Displays those entries already in the hardware cache. |
| static | Displays the entries of the IGMP-proxy static multicast cache. |

- Default value

All entries are to be displayed if no filtration conditions are specified.

- Remarks

The multicast source entries of IGMP-proxy are displayed.

- Example

The following example shows how to display the information about the IGMP-proxy multicast source.

Switch# show ip igmp-proxy mcache

Codes: '+' synchronization, '-' deleted, 'S' static

'^' unsynchronization

Item 1: Group 225.1.1.2

+(192.168.213.163, 2, G3/24)

VLAN 3,4

## show ip igmp-proxy

- Syntax

show ip igmp-proxy

- Parameter

N/A

- Default value

N/A

- Remarks

This command is used to display the configurations about IGMP proxy.

- Example

The following example shows how to display the configurations of IGMP proxy.

switch#show ip igmp-proxy

Global IGMP proxy configuration:

--------------------------------

Globally enable	: Enable

Agent: vlan 2, it has been alloced

----------------

Client static   : vlan 1

Client auto     : vlan

Agent: vlan 3, it has been alloced

----------------

Client static   : vlan 1

Client auto     : vlan

Globally clients list

---------------------

Client          : vlan 1, it has been alloced

Static Multi-source list

---------------------

Static Source   : No Static Source

switch#

## debug ip igmp-proxy

- Syntax

debug ip igmp-proxy [error | event | packet]

no debug ip igmp-proxy [error | event | packet]

- Parameter

| Parameter | Description |
| --- | --- |
| event | Debugs the IGMP-snooping event. |
| packet | Debugs the reception and transmission of IGMP-snooping. |
| error | Debugs the IGMP-snooping errors. |

- Default value

N/A

- Remarks

This command is used to enable or disable the debugging & printing switch of IGMP-proxy.