- Interface Range Commands

- Table of Contents

Chapter 1 Interface Range Command	1

1.1 Interface Range	1

# Chapter 1 Interface Range Command

## Interface Range

Syntax

interface range type slot/<port1-port2 | port3>[, <port1-port2|port3>]

Parameter

| Name | Description | Range |
| --- | --- | --- |
| type | Port type | All legal port types, except management ports on the main board of the rackmount switch. |
| slot | Slot number | All legal slot number. |
| port1 | Beginning value of the port number | All legal port numbers on the slot. |
| port2 | Ending value of the port number | All legal port numbers on the slot except for port 1. |
| port3 | A single port. | All legal port numbers on the slot. |

Default

None

Usage Guidelines

Use this command to enter the interface range mode.

Example

Use the following command to enter the enterface configuration mode, including the Ethernet ports 1, 2, 3 and 4:

switch_config# interface range gigaEthernet 0/1-4

switch_config_if_range#

Note:

There are no space on either side of "-" and ","