- Interface Physical Characteristic Configuration Commands

- Table of Contents

Chapter 1 Interface Physical Characteristic Configuration Commands	1

1.1 Interface Physical Characteristic Configuration Commands	1

1.1.1 speed	1

1.1.2 duplex	2

1.1.3 flow-control	2

# Chapter 1 Interface Physical Characteristic Configuration Commands

## Interface Physical Characteristic Configuration Commands

Interface physical characteristic configuration commands include:

- speed

- duplex

- flow-control

### speed

Syntax

speed {10 | 100 | auto } (T port)

speed {100 | 1000 | auto } (SFP port)

no speed

To configure the speed for the interface, use the speed command.

Parameter

| Parameter | Description |
| --- | --- |
| 10, 100,1000 | Configures the interface speed as 10M, 100M, 1000M. |
| auto | Enable the auto-negotiation capability of the interface. |

Default

TX interface is auto-negotiation operation, 100M optical interface speed is 100M, gigabit optical interface speed is 1000M.

Usage Guidelines

Use this command in the layer 2 interface configuration mode.

Note:

The speed of the optical interface is fixed. By default, the auto-negotiation function on the Gigabit optical interface is enabled. The combo port does not support forced gigabit and full duplex simultaneously. Ordinary TX interface does not support speed 1000.

Example

The following example sets the interface g0/1 speed to 100M:

Switch_config# interface g0/1

Switch_config_g0/1# speed 100

### duplex

Syntax

duplex {auto | full | half }

no duplex

To configure the duplex operation on an interface, use the duplex command.

Parameter

| Parameter | Description |
| --- | --- |
| auto | Specifies the auto-negotiation operation. |
| full | Specifies the full-duplex operation. |
| half | Specifies the half-duplex operation. |

Default

TX interface is auto-negotiation operation, optical interface is full-duplex operation.

Usage Guidelines

Use this command in the layer 2 configuration mode.

Note:

The duplex operation of the optical interface is fixed, that is full-duplex operation. The combo port doesn’t support both gigabit and full-duplex operation. In half-duplex mode, back pressure occurs.

Example

The following example configures interface g0/1 to full-duplex operation:

Switch_config# interface g0/1

Switch_config_g0/1# duplex full

### flow-control

Syntax

flow-control  { on | off | auto}

To configure flow control on an interface, use the flow control command.

Parameter

| Parameter | Description |
| --- | --- |
| on | Enables flow control |
| off | Disables flow control. |
| auto | Auto-negotiation mode. |

Default

Disabled

Usage Guidelines

Use this command in the layer 2 configuration mode.

Note:

Both “flow-control auto” and “flow-control on” are forced to receive flow-control frame. But in “auto” mode, the flow-control frame is sent only after successful negotiation with the opposite end.

Example

The following example enables the flow-control on the g0/1 interface:

Switch_config#int g0/1

Switch_config_g0/1#flow-control on