- MAC Address Table Characteristics Configuration Commands

- Table of Contents

Chapter 1 MAC Address Table Characteristics Configuration Commands	1

1.1 MAC Address Table Characteristic Configuration Commands	1

1.1.1 mac address-table static	1

1.1.2 mac address-table aging-time	1

1.1.3 mac address-table blackhole	2

1.1.4 show mac address-table	2

1.1.5 clear mac address-table	3

# Chapter 1 MAC Address Configuration Commands

## MAC Address Configuration Commands

### mac address-table static

Syntax

To add/delete a static MAC address, use the mac address-table static command.

[no] mac address-table static mac-addr vlan vlan-id interface interface-id

Parameters

| Parameter | Description |
| --- | --- |
| mac-addr | MAC address. Value format: H.H.H. |
| vlan-id | Vlan id of the MAC address, in the range from 1 to 4094. |
| interface-id | Interface id of the MAC address. |

Default

None

Command Mode

Global configuration mode

Example

The following example binds the MAC address 0004.5600.67ab to the interface g0/2 of VLAN 1:

Switch_config# mac address-table static 0004.5600.67ab vlan 1 interface g0/2

### mac address-table aging-time

Syntax

To configure the maximum aging time for MAC address table.

mac address-table aging-time [0 | 10-1000000]

Parameters

| Parameter | Description |
| --- | --- |
| 0 | The aging time for MAC address table is disabled. |
| 10-1000000 | The aging time (in seconds) for MAC address table. |

Default

300s

Command Mode

Global configuration mode

Example

The following example configures the aging time for MAC address table to 100 seconds:

Switch_config# mac address-table aging-time 100

### mac address-table blackhole

Syntax

To add/ delete a blackhole MAC address.

[no] mac address-table blackholemac-addr vlan vlan-id

Parameters

| Parameter | Description |
| --- | --- |
| mac-addr | MAC address. Value format: H.H.H. |
| vlan-id | VLAN ID, in the range from 1 to 4094. |

Default

None

Usage Guidelines

Global configuration mode

Example

The following example shows how to binds the MAC address 0004.5600.67ab to the mac blackhole of VLAN 1:

Switch_config# mac address-table blackhole 0004.5600.67ab vlan 1

### show mac address-table

Syntax

To display the content of the switch MAC address table, use the show mac address-table command.

show  mac address-table [dynamic [interface interface-id | vlan vlan-id] | static | brief | multicast | interface interface-id | vlan vlan-id | H.H.H | blackhole]

Parameters

| Parameter | Description |
| --- | --- |
| dynamic | The dynamic MAC address table. |
| interface-id | Interface name |
| vlan-id | VLAN ID, in the range from 1 to 4094. |
| static | The static MAC address table. |
| brief | Brief information of Mac address |
| multicast | Multicast of mac address table |
| Interface | Interface of mac address table |
| Vlan | Mac address table in VLAN |
| H.H.H | Specific address |
| Blackhole | Blackhole of mac address table |

Default

None

Usage Guidelines

The command is used to display MAC address table.

Example

The following example displays all dynamic MAC address tables:

Switch_config#show mac address-table

Mac Address Table (Total 2)

------------------------------------------

Vlan    Mac Address       Type       Ports

----    -----------       ----       -----

1       0026.5a7c.fad3    DYNAMIC    g0/2

### clear mac address-table

Syntax

To delete a dynamic MAC address, use the clear mac address-table

clear mac address-table dynamic [address mac-addr | interface interface-id | vlan vlan-id]

Parameter

| Parameter | Description |
| --- | --- |
| mac-addr | The MAC address. Value range: H.H.H. |
| interface-id | Layer 2 interface name. |
| vlan-id | VLAN ID, in the range from 1 to 4094. |

Default

None

Command Mode

Use the command in the management mode.

Example

The following example deletes all MAC addresses that acquire dynamically on interface g0/2:

Switch# clear mac address-table dynamic interface g0/2