- MAC Address List Characteristic Configuration Commands

- Table of Contents

Chapter 1 MAC Access List Configuration Commands	1

1.1 MAC Access List Configuration Commands	1

1.1.1 mac access-list	1

1.1.2 permit	1

1.1.3 deny	2

1.1.4 mac access-group	3

# Chapter 1 MAC Access List Configuration Commands

## MAC Access List Configuration Commands

Mac access list configuration commands include:

- mac access-list

- permit

- deny

- mac access-group

### mac access-list

Syntax

To add a MAC access list, use the mac access-list command. To delete a MAC access list, use the no mac access-list command.

[no] mac access-list name

Parameter

| Parameter | Description |
| --- | --- |
| name | mac access list name. |

Default

If there is any entry is configured, one deny any any will be added at the end by default, and it is not displayed.

Command Mode

Global configuration mode

Example

The following example establishes MAC access list named mac-acl:

Switch-config# mac access-list  mac-acl

Switch-config-macl#

### permit

Syntax

To add a permit entry to the MAC access list, use the permit command. To delete a permit entry to the MAC access list, use the no permit command.

[no] permit {any | host src-mac-addr | src-mac-addr src-mac-mask } {any | host dst-mac-addr | dst-mac-addr  dst-mac-mask}[ arp [{any | src-ip-addr} {any | dst-ip-addr }] | ethertype | cos value]

Parameter

| Parameter | Description | Value range |
| --- | --- | --- |
| any | Any value | - |
| host | Host | - |
| src-mac-addr | Source MAC address | H.H.H |
| src-mac-addr-mask | Source MAC address mask | H.H.H |
| dst-mac-addr | Destination MAC address | H.H.H |
| dst-mac-addr-mask | Destination MAC address mask | H.H.H |
| arp | Matched arp message | - |
| src-ip-addr | Source IP address | A.B.C.D |
| dst-ip-addr | Destination IP address | A.B.C.D |
| ethertype | Types of the matching ethernet data packet. | 0x0600-0xFFFF |
| cos value | Header mark of message | 0-7 |

Default

Deny all.

Command Mode

MAC access list configuration mode

Example

The following example shows how to permit host whose source MAC address is 1234.5678.abcd:

Switch-config-macl#permit host 1234.5678.abcd any

### deny

Syntax

To add/ delete a deny entry to the MAC access list, use the following command.

[no] deny {any | host src-mac-addr | src-mac-addr src-mac-mask } {any | host dst-mac-addr | dst-mac-addr  dst-mac-mask}[ arp [{any | src-ip-addr} {any | dst-ip-addr }] | ethertype | cos value]

Parameter

| Parameter | Description | Value range |
| --- | --- | --- |
| any | Any value | - |
| host | Host | - |
| src-mac-addr | Source MAC address | H.H.H |
| src-mac-addr-mask | Source MAC address mask | H.H.H |
| dst-mac-addr | Destination MAC address | H.H.H |
| dst-mac-addr-mask | Destination MAC address mask | H.H.H |
| arp | Matched arp message | - |
| src-ip-addr | Source IP address | A.B.C.D |
| dst-ip-addr | Destination IP address | A.B.C.D |
| ethertype | Types of the matching ethernet data packet. | 0x0600-0xFFFF |
| cos value | Header mark of message | 0-7 |

Default

Deny all.

Comamnd mode

MAC access list configuration mode

Example

The following example shows how to deny host whose source MAC address is 1234.5678.abcd:

Switch-config-macl#deny host 1234.5678.abcd any

### mac access-group

Syntax

Apply the configured MAC access list in interface configuration mode or global configuration mode, or delete the MAC access list applied in interface configuration mode or global configuration mode.

Global configuration mode:

mac access-group name [vlan {word | add word | remove word}]

[no] mac access-group name [vlan ]

Interface configuration mode:

[no] mac access-group name

Parameter

| Parameter | Description |
| --- | --- |
| name | Name of the MAC access list. |
| Vlan | The access list is applied to the inbound VLAN. |
| Word | VLAN range table |
| add | Add vlan range table |
| remove | Remove vlan range table |

Default

No MAC access list is applied.

Command Mode

Use the command in layer-2 global or interface configuration mode. If the access list does not exist, an access list with no rule will be created.

Example

The following example shows how to configure MAC access list named macacl on interface g0/1:

Switch_config_g0/1#mac access-group macacl