- STP Configuration Commands

- Table of Contents

Chapter 1 STP Configuration Commands	1

1.1 SSTP Configuration Commands	1

1.1.1 spanning-tree	1

1.1.2 spanning-tree mode sstp	2

1.1.3 spanning-tree sstp priority	2

1.1.4 spanning-tree sstp hello-time	3

1.1.5 spanning-tree sstp max-age	4

1.1.6 spanning-tree sstp forward-time	5

1.1.7 spanning-tree sstp cost	6

1.1.8 spanning-tree cost	7

1.1.9 spanning-tree sstp port-priority	8

1.1.10 spanning-tree port-priority	8

1.1.11 show spanning-tree	9

1.1.12 spanning-tree management trap	10

1.2 VLAN STP Configuration Commands	11

1.2.1 spanning-tree mode pvst	11

1.2.2 spanning-tree vlan	12

1.2.3 spanning-tree vlan priority	13

1.2.4 spanning-tree vlan forward-time	13

1.2.5 spanning-tree vlan max-age	14

1.2.6 spanning-tree vlan hello-time	15

1.2.7 spanning-tree vlan cost	16

1.2.8 spanning-tree vlan port-priority	17

1.2.9 show spanning-tree vlan	18

1.2.10 show spanning-tree pvst instance-list	19

Chapter 2 RSTP Configuration Commands	21

2.1 RSTP Configuration Commands	21

2.1.1 spanning-tree mode rstp	21

2.1.2 spanning-tree rstp forward-time	21

2.1.3 spanning-tree rstp hello-time	22

2.1.4 spanning-tree rstp max-age	23

2.1.5 spanning-tree rstp priority	24

2.1.6 spanning-tree rstp cost	24

2.1.7 spanning-tree rstp port-priority	25

2.1.8 spanning-tree rstp edge	26

2.1.9 spanning-tree rstp point-to-point	27

2.1.10 spanning-tree rstp migration-check	27

Chapter 3 MSTP Configuration Commands	29

3.1 MSTP Configuration Command	29

3.1.1 spanning-tree mode mstp	29

3.1.2 spanning-tree mstp name	29

3.1.3 spanning-tree mstp revision	30

3.1.4 spanning-tree mstp instance	31

3.1.5 spanning-tree mstp root	32

3.1.6 spanning-tree mstp priority	33

3.1.7 spanning-tree mstp hello-time	34

3.1.8 spanning-tree mstp forward-time	34

3.1.9 spanning-tree mstp max-age	35

3.1.10 spanning-tree mstp diameter	36

3.1.11 spanning-tree mstp max-hops	37

3.1.12 spanning-tree mstp port-priority	37

3.1.13 spanning-tree mstp cost	38

3.1.14 spanning-tree mstp edge	39

3.1.15 spanning-tree mstp point-to-point	40

3.1.16 spanning-tree mstp mst-compatible	40

3.1.17 spanning-tree mstp migration-check	41

3.1.18 spanning-tree mstp restricted-role	42

3.1.19 spanning-tree mstp restricted-tcn	43

3.1.20 show spanning-tree mstp	43

3.1.21 show spanning-tree mstp region	45

3.1.22 show spanning-tree mstp detail	45

3.1.23 show spanning-tree mstp interface	47

3.1.24 show spanning-tree mstp protocol-migration	49

# STP Configuration Commands

## SSTP Configuration Commands

### spanning-tree

Syntax

spanning-tree

no spanning-tree

To enable the default STP mode, run spanning-tree; to disable the STP, run no spanning-tree.

Enable or disable STP in interface configuration mode.

Parameter

None

Default

Enable RSTP mode by default.

Usage guidelines

None

Command Mode

Global configuration mode

Physical port or aggression port configuration mode.

Example

None

### spanning-tree mode sstp

Syntax

spanning-tree mode sstp

no spanning-tree mode

To switch between RSTP and SSTP modes, use the spanning-tree mode command. To return to the default settings, use the no form of this command.

Parameter

None

Default

RSTP

Usage guidelines

None

Command Mode

Global configuration

Example

The following command shows how to enable SSTP mode:

Switch_config# spanning-tree mode sstp

Switch_config#

### spanning-tree sstp priority

Syntax

To set the sstp bridge priority, use the spanning-tree sstp priority command. To return to the default settings, use the no form of this command.

- spanning-tree sstp priority value

no spanning-tree sstp priority

Parameter

| Parameter | Description |
| --- | --- |
| value | Value is from 0 to 61440. |

Default

32768

Usage Guidelines

When setting the priority value, you can make the node as the root of the spanning tree. The configuration value takes 4096 as a step and its value is the multiple of 4096. The configurable values are 0, 4096, 8192, 3*4096, 4*4096, …and 15*4096.

Command mode

Global configuration

Example

This example shows how to set the SSTP priority to 4096:

Switch(config)# spanning-tree sstp priority 4096

Switch(config)#

### spanning-tree sstp hello-time

Syntax

To set the hello-time delay timer, use the spanning-tree sstp hello-time command. To return to the default settings, use the no form of this command.

spanning-tree sstp hello-time time

no spanning-tree sstp hello-time

Parameter

| Parameter | Description |
| --- | --- |
| time | Number of seconds to set the hello-time delay timer; valid values are from 1 to 10 seconds. |

Default

2s

Usage Guidelines

The hello-time configured by the local OLT is valid only when the local OLT is the root OLT.

Command mode

Global configuration

Example

The following example sets the SSTP hello-time to 8 seconds:

Switch(config)# spanning-tree sstp hello-time 8

Switch(config)#

### spanning-tree sstp max-age

Syntax

To set the SSTP max-age timer, use the spanning-tree sstp max-age command. To return to the default settings, use the no form of this command.

spanning-tree sstp max-age time

no spanning-tree sstp max-age

Parameter

| Parameter | Description |
| --- | --- |
| seconds | Number of seconds to set the max-age timer; valid values are from 6 to 40 seconds. |

Default

20s

Usage Guidelines

None

Command mode

Global configuration

Example

This example shows how to set the max-age timer to 24 seconds：

Switch(config)# spanning-tree sstp max-age 24

Switch(config)#

### spanning-tree sstp forward-time

Syntax

To set the forward-delay timer, use the spanning-tree sstp forward-time command in global configuration mode. To return to the default settings, use the no form of this command.

spanning-tree sstp forward-time time

no spanning-tree sstp forward-time

Parameter

| Parameter | Description |
| --- | --- |
| time | Number of seconds to set the forward-delay timer; valid values are from 4 to 30 seconds. |

Default

15 seconds

Usage Guidelines

None

Command mode

Global configuration

Example

The following example shows how to set forward delay timer to 20 seconds:

Switch_config# spanning-tree sstp forward-time 20

Switch_config#

### spanning-tree sstp cost

Syntax

To set the path cost of the interface for SSTP calculations, use the spanning-tree sstp cost command in interface configuration mode. To return to the default value, use the no form of this command.

spanning-tree sstp cost value

no spanning-tree sstp cost

Parameter

| Parameter | Description |
| --- | --- |
| value | Path cost. Valid values are from 1 to 200000000 |

Default

10M Ethernet:100 .

100M Ethernet: 19 .

1000M Ethernet: 4 .

Usage Guidelines

None

Command mode

Interface configuration

Example

This example shows how to set a path cost value of 100 for the spanning tree VLAN associated with the interface G0/1:

Switch_config_g0/1#spanning-tree sstp cost 100

Switch_config_g0/1#

### spanning-tree cost

Syntax

To set the path cost of the interface for Spanning Tree Protocol (STP) calculations, use the spanning-tree cost command in interface configuration mode. To return to the default value, use the no form of this command.

spanning-tree cost value

no spanning-tree cost

Parameter

| Parameter | Description |
| --- | --- |
| value | Path cost; valid values are from 1 to 200000000 |

Default

The default path cost is computed from the bandwidth setting of the interface.

Usage Guidelines

The configuration result of this command is valid to all spanning-tree modes. In STP mode, the path cost of all VLAN spanning-trees on the interface will be updated. In MSTP mode, the path cost of all spanning-tree examples will be updated.

But the configuration result of the command will not influence the independent configuration in various modes. For example, the OLT respectively configured with the spanning-tree sstp cost 100 and the spanning-tree cost 110 in SSTP mode, the port priority will be 100.

Command mode

Interface configuration

Example

This example shows how to set a path cost value of 24 for the spanning tree VLAN associated with the interface g0/1:

Switch(config_f0/0)# spanning-tree cost 24

Switch_config_g0/1#

### spanning-tree sstp port-priority

Syntax

To set the priority value in SSTP mode, use the spanning-tree sstp port-priority command. Use the no form of this command to restore the default value.

spanning-tree sstp port-priority value

no spanning-tree sstp port-priority

parameter

| Parameter | Description |
| --- | --- |
| value | Port priority. Value is from 0 to 240. |

Default

128 (0x80)

Usage Guidelines

The port priority must be set in increments of 16 only.

Command mode

Interface configuration

Example

The following example sets 32 as the priority value on interface g0/1:

Switch_config_g0/1# spanning-tree sstp port-priority 32

Switch_config_g0/1#

### spanning-tree port-priority

Description

To prioritize an interface when two bridges compete for position as the root bridge, use the spanning-tree port-priority command. The priority you set breaks the tie. To return to the default value, use the no form of this command.

spanning-tree port-priority value

no spanning-tree port-priority

Parameter

| Parameter | Description |
| --- | --- |
| value | Port priority. Value is from 0 to 240. Step: 16 |

default

Port priority value is 128

Usage Guidelines

The configuration result of this command is valid to all spanning-tree modes. In STP mode, the priority of all VLAN spanning-trees on the interface will be updated. In MSTP mode, the priority of all spanning-tree examples will be updated.

But the configuration result of the command will not influence the independent configuration in various modes. For example, the OLT respectively configured with the spanning-tree sstp port-priority 128 and the spanning-tree port-priority 48 in SSTP mode, the port priority will be 128.

Command mode

Interface configuration

example

The following example shows how to set the priority value to g0/1:

Switch_config_g0/1#spanning-tree port-priority 16

Switch_config_g0/1#

### show spanning-tree

Syntax

To display spanning-tree information for the specified spanning-tree instances, use the show spanning-tree command.

show spanning-tree [detail | interface intf-i]

Parameter

| Parameter | Description |
| --- | --- |
| intf-i | Interface name, for instance, G0/1. |

Default

None

Usage Guidelines

Show spanning-tree state.

Command mode

EXEC/Global configuration/Interface configuration

Example

Switch_config#show spanning-tree

Spanning tree enabled protocol SSTP

SSTP

Root ID    Priority    32768

Address     00E0.0FCC.F775

This bridge is the root

Hello Time 2 sec  Max Age 20 sec  Forward Delay 15 sec

Bridge ID  Priority    32768

Address     00E0.0FCC.F775

Hello Time 2 sec  Max Age 20 sec  Forward Delay 15 sec

Interface        Role Sts Cost      Pri.Nbr Type

---------------- ---- --- --------- ------- --------------------------------

G0/1            Desg FWD 19        128.16  P2p

Switch_config#

### spanning-tree management trap

Syntax

To enable STP Trap, run command spanning-tree management trap [ newroot | topologychange ]. To return to the default value, use the no form of this command.

[no] spanning-tree management trap [ newroot | topologychange ]

Parameter

| Parameter | Description |
| --- | --- |
| newroot | newRoot Trap type |
| topologychange | topologyChange Trap type |

Default

STP Trap is not enabled.

Usage guidelines

None

Command mode

Global configuration

Example

None

## VLAN STP Configuration Commands

### spanning-tree mode pvst

Syntax

spanning-tree mode pvst

no spanning-tree mode

To enable STP modes, use the spanning-tree mode pvst command. To disable all STP modes, use the no form of this command.

Parameter

None

Default

RSTP mode

Usage guidelines

None

Example

The following command shows how to enable PVST in switch.

Switch_config# spanning-tree mode pvst

Switch_config#

### spanning-tree vlan

Syntax

spanning-tree vlan vlan-list

no spanning-tree vlan vlan-list

To configure a STP instance for a specified VLAN, use spanning-tree vlan vlan-list command. To remove the specified spanning-tree vlan, use no form of the command.

Parameter

| Parameter | Description |
| --- | --- |
| vlan-list | VLAN number list, such as: 1,2,3-10,15. |

Default

Switch can configure spanning-tree for a certain number of VLAN. In default mode, the exceeded VLAN will be prohibited.

Usage guidelines

None

Command Mode

Global configuration mode

Example

The following command shows how to remove the spanning-tree of VLAN10, 11, 15-19, and configure the spanning-tree for VLAN 40-50:

Switch_config#no spanning-tree vlan 10,11,15-19

Switch_config#spanning-tree vlan 40-50

Switch_config#

### spanning-tree vlan priority

Syntax

spanning-tree vlan vlan-list priority value

no spanning-tree vlan vlan-list priority

Use the command to configure the bridge priority value for the spanning-tree in specified VLAN.

Parameter

| Parameter | Description |
| --- | --- |
| vlan-list | VLAN number list, such as: 1,2,3-10,15. |
| value | Priority values, ranging from 0 to 61440, step: 4096 |

Default

In default mode, the bridge priority value of the spanning-tree VLAN is 32768 plus VLAN number.

Usage guidelines

None

Command Mode

Global configuration mode

Example

The following command shows how to configure the bridge priority value of VLAN1-3, 5-10 to 4096:

Switch_config#spanning-tree vlan 1-3,5-10 priority 4096

Switch_config#

### spanning-tree vlan forward-time

Syntax

spanning-tree vlan vlan-list forward-time value

no spanning-tree vlan vlan-list forward-time

To set the Forward DelayParameter of the specified spanning-tree vlan, use spanning-tree vlan vlan-list forward-time value command. To return to the default settings, use the no form of this command.

Parameter

| Parameter | Description |
| --- | --- |
| vlan-list | VLAN number list, such as: 1,2,3-10,15. |
| value | Forward-Delay value, ranging from 4s to 30s. The default value is 15s. |

Default

All VLAN Forward Delay is 15s.

Usage guidelines

None

Command Mode

Global configuration mode

Example

The following command shows how to configure the Forward Delay of VLAN1-3, 5-10 to 19s:

Switch_config#spanning-tree vlan 1-3,5-10 forward-time 19

Switch_config#

### spanning-tree vlan max-age

Syntax

spanning-tree vlan vlan-list max-age value

no spanning-tree vlan vlan-list max-age

To set the Max AgeParameter of the specified spanning-tree vlan, use spanning-tree vlan vlan-list max-age value command. To return to the default settings, use the no form of this command.

Parameter

| Parameter | Description |
| --- | --- |
| vlan-list | VLAN number list, such as: 1,2,3-10,15. |
| value | max-age value, ranging from 6s to 40s. Its default value is 20s. |

Default

All VLAN Max Age is 20s.

Usage guidelines

None

Command Mode

Global configuration mode

Example

The following command shows how to configure the Max Age of VLAN1-3, 5-10 to 19s:

Switch_config#spanning-tree vlan 1-3,5-10 max-age 19

Switch_config#

### spanning-tree vlan hello-time

Syntax

spanning-tree vlan vlan-list hello-time value

no spanning-tree vlan vlan-list hello-time

To set the hello-timeParameter of the specified spanning-tree vlan, use spanning-tree vlan vlan-list hello-time value command. To return to the default settings, use the no form of this command.

Parameter

| Parameter | Description |
| --- | --- |
| vlan-list | VLAN number list, such as: 1,2,3-10,15. |
| value | hello-time value, ranging from 1s to 10s. Its default value is 2s. |

Default

All VLAN Hello-Time is 2s.

Usage guidelines

None

Command Mode

Global configuration mode

Example

The following command shows how to configure the Hello Time of VLAN1-3, 5-10 to 9s:

Switch_config#spanning-tree vlan 1-3,5-10 hello-time 9

Switch_config#

### spanning-tree vlan cost

Syntax

spanning-tree vlan vlan-list cost value

no spanning-tree vlan vlan-list cost

To set the path cost of the interface for Spanning Tree Protocol (STP), use the spanning-tree vlan vlan-list cost value command. To return to the default value, use the no form of this command.

Parameter

| Parameter | Description |
| --- | --- |
| vlan-list | VLAN number list, such as: 1,2,3-10,15. |
| value | The path cost of the interface rangings from 1 to 200000000. |

Default

The path cost depends on the connection rate of the port.

The path cost value of 10M Ethernet is 100.

The path cost value of 100M Ethernet is 19.

The path cost value of 1000M Ethernet is 1.

Usage guidelines

None

Command Mode

Interface configuration mode

Example

The following command shows how to configure the path cost of VLAN1-3, 5-10 in the interface G0/1 to 100:

Switch_config_g0/1#spanning-tree vlan 1-3,5-10 cost 100

Switch_config_g0/1#

### spanning-tree vlan port-priority

Syntax

spanning-tree vlan vlan-list port-priority value

no spanning-tree vlan vlan-list port-priority

To set the interface priority for Spanning Tree Protocol (STP), use the spanning-tree vlan vlan-list port-priority value command. To return to the default value, use the no form of this command.

Parameter

| Parameter | Description |
| --- | --- |
| vlan-list | VLAN number list, such as: 1,2,3-10,15. |
| value | Interface priority, ranging from 0 to 240, step: 16. |

Default

128

Usage guidelines

None

Command Mode

Interface configuration mode

Example

The following command shows how to configure the priority of VLAN1-3, 5-10 in the interface G0/1 to 32:

Switch_config_g0/1#spanning-tree vlan 1-3,5-10 port-priority 32

Switch_config_g0/1#

### show spanning-tree vlan

Syntax

show spanning-tree vlan vlan-list [ detail ]

Use the command to check the spanning-tree status of the specified VLAN.

Parameter

| Parameter | Description |
| --- | --- |
| vlan-list | VLAN number list, such as: 1,2,3-10,15. |
| detail | Displays the detailed information. |

Default

None

Usage guidelines

None

Command Mode

EXEC, global configuration mode or interface configuration mode

Example

The following examples to check the spanning-tree of vlan 1-2:

Switch_config#show spanning-tree vlan 1-2

Spanning tree enabled protocol PVST

VLAN0001

Root ID    Priority    32769

Address     00E0.0FCC.F775

This bridge is the root

Hello Time 2 sec  Max Age 20 sec  Forward Delay 15 sec

Bridge ID  Priority    32769

Address     00E0.0FCC.F775

Hello Time 2 sec  Max Age 20 sec  Forward Delay 15 sec

Interface        Role Sts Cost      Pri.Nbr Type

---------------- ---- --- --------- ------- --------------------------------

G0/1            Desg FWD 19        128.1  P2p

VLAN0002

Root ID    Priority    32770

Address     00E0.0FCC.F775

This bridge is the root

Hello Time 2 sec  Max Age 20 sec  Forward Delay 15 sec

Bridge ID  Priority    32770

Address     00E0.0FCC.F775

Hello Time 2 sec  Max Age 20 sec  Forward Delay 15 sec

Interface        Role Sts Cost      Pri.Nbr Type

---------------- ---- --- --------- ------- --------------------------------

G0/1            Desg FWD 19        128.1  P2p

Switch_config#

### show spanning-tree pvst instance-list

Syntax

show spanning-tree pvst instance-list

Use the command to check how the PVST instance corresponds to the VLAN.

Parameter

None

Default

None

Usage guidelines

None

Command Mode

EXEC, global configuration mode or interface configuration mode

Example

None

# RSTP Configuration Commands

## RSTP Configuration Commands

### spanning-tree mode rstp

Syntax

spanning-tree mode rstp

no spanning-tree mode

To enable the RSTP function, run spanning-tree mode rstp. To disable the RSTP, run no spanning-tree mode.

Parameter

None

Default

RSTP is enabled.

Usage Guidelines

None

Example

The following command shows how to enable rstp on switch.

Switch_config# spanning-tree mode rstp

Switch_config#

### spanning-tree rstp forward-time

Syntax

spanning-tree rstp forward-time time

no spanning-tree rstp forward-time

To configure the forwarding delay of RSTP, run spanning-tree rstp forward-time time. To resume the default forwarding delay of RSTP, run no spanning-tree rstp forward-time.

Parameter

| Parameter | Description |
| --- | --- |
| time | Time of the forwarding delay whose value ranges between 4 and 30 seconds. |

Default

15s

Usage Guidelines

None

Example

The following example shows how to set the forwarding delay of RSTP to 20 seconds.

Switch_config# spanning-tree rstp forward-time 20

Switch_config#

### spanning-tree rstp hello-time

Syntax

spanning-tree rstp hello-time time

no spanning-tree rstp hello-time

To configure the update interval of RSTP, run spanning-tree rstp hello-time time. To resume the default update interval of RSTP, run no spanning-tree rstp hello-time.

Parameter

| Parameter | Description |
| --- | --- |
| time | Update interval. The value ranges: 1-10s. |

Default

2s

Usage Guidelines

The Hello-Time configured on the local switch validates only when the local switch runs as a root switch.

Example

The following example shows how to set the update interval of RSTP to 8 seconds.

Switch_config# spanning-tree rstp hello-time 8

Switch_config#

### spanning-tree rstp max-age

Syntax

spanning-tree rstp max-age time

no spanning-tree rstp max-age

To configure the maximum lifespan of the RSTP BPDU, run spanning-tree rstp max-age time. To resume the default interval time, run no spanning-tree rstp max-age.

Parameter

| Parameter | Description |
| --- | --- |
| time | Maximum interval of the lifespan. Value ranges: 6-40s. |

Default

20s

Usage Guidelines

None

Example

The following example shows how to set the maximum lifespan of RSTP to 24 seconds.

Switch_config# spanning-tree rstp max-age 24

Switch_config#

### spanning-tree rstp priority

Syntax

spanning-tree rstp priority value

no spanning-tree rstp priority

To configure the RSTP priority value, run spanning-tree rstp priority value. To resume the default value of the RSTP priority value, run no spanning-tree rstp priority.

Parameter

| Parameter | Description |
| --- | --- |
| value | Priority level of the bridge. The value ranges: 0-61440, step 4096. |

Default

32768

Usage Guidelines

None

Example

The following example shows how to configure the priority level of the brigde of rstp to 4096.

Switch_config# spanning-tree rstp priority 4096

Switch_config#

### spanning-tree rstp cost

Syntax

To configure the path cost of a port, run spanning-tree rstp cost value. To resume the default value, run no spanning-tree rstp cost.

spanning-tree rstp cost value

no spanning-tree rstp cost

Parameter

| Parameter | Description |
| --- | --- |
| value | Value of the path cost. The value ranges: 1-200000000. |

Default

The path cost depends on the connection rate of the port.

10 Mbps: 2000000

100 Mbps: 200000

1000 Mbps: 20000

Usage Guidelines

None

Example

The following example shows how to set the path cost of port g0/1 to 24:

Switch_config_g0/1# spanning-tree rstp cost 24

Switch_config_g0/1#

### spanning-tree rstp port-priority

Syntax

To configure the priority level of a port, run spanning-tree rstp port-priority value. To resume the default value, run no spanning-tree rstp port-priority.

spanning-tree rstp port-priority value

no spanning-tree rstp port-priority

Parameter

| Parameter | Description |
| --- | --- |
| value | Priority level of a port. The value ranges: 0-240, step 16. |

Default

128

Usage Guidelines

None

Example

The following example shows how to set the path cost of port g0/1 to 16:

Switch_config_g0/1# spanning-tree rstp port-priority 16

Switch_config_g0/1#

### spanning-tree rstp edge

Syntax

To configure the edge port, run spanning-tree rstp edge. To return to the default setting, run no spanning-tree rstp edge.

spanning-tree rstp edge

no spanning-tree rstp edge

Parameter

None

Default

Automatic check

Usage Guidelines

None

Command Mode

Interface configuration

Example

None

### spanning-tree rstp point-to-point

Syntax

To set the point-to-point connection of a port to force-truce, force-false or auto, run this command.

spanning-tree rstp point-to-point [ force-true | force-false | auto ]

Parameter

| Parameter | Description |
| --- | --- |
| force-true | To set the point-to-point connection of a port to force-truce. |
| force-false | To set the point-to-point connection of a port to force-false. |
| auto | Sets the point-to-point connection to be automatic check (default). |

Default

Automatic check

Usage Guidelines

None

Command Mode

Interface Configuration

Example

None

### spanning-tree rstp migration-check

Syntax

To restart checking protocol transfer of RSTP, run spanning-tree rstp migration-check.

spanning-tree rstp migration-check

Parameter

None

Default

None

Usage Guidelines

This command is used to restart the protocol transfer check on a port and to change the port in STP-compatible mode to the RSTP mode, enabling RSTP BPDU to be transmitted.

Command Mode

Global and interface configuration

Example

The following example shows how to run protocol transfer check on interface G0/1:

Switch_config_g0/1#spanning-tree rstp migration-check

Switch_config_g0/1#

# MSTP Configuration Commands

## MSTP Configuration Command

### spanning-tree mode mstp

Syntax

To set the running mode of STP to MSTP, run spanning-tree mode mstp. To disable STP, Run no spanning-tree mode.

spanning-tree mode mstp

no spanning-tree mode

Parameter

None

Default

The MSTP mode is disabled, while the RSTP mode is running.

Usage Guidelines

None

Example

The following example shows how to enable MSTP protocol on the switch:

switch(config)# spanning-tree mode mstp

switch(config)#

### spanning-tree mstp name

Syntax

spanning-tree mstp name string

no spanning-tree mstp name

To configure the regional name of the STP, run spanning-tree mstp name string. To resume the default name, run no spanning-tree mstp.

Parameter

| Parameter | Description |
| --- | --- |
| String | Configures the character string of the name. The character string can have up to 32 characters, capital sensitive. The default value is in the form of character string like the MAC address of the switch. |

Default

Character string form of the switch’s MAC address

Usage Guidelines

None

Example

The following example shows how to set the configuration name of the switch’s STP to reg-01.

switch(config)# spanning-tree mstp name reg-01

switch(config)#

### spanning-tree mstp revision

Syntax

To generate the revision number of STP, run spanning-tree mstp revision value. To return to the default value, run no spanning-tree mstp revision.

spanning-tree mstp revision value

no spanning-tree mstp revision

Parameter

| Parameter | Description |
| --- | --- |
| Value | Revision number: 0 ~65535. Its default value is 0. |

Default

The default value of the revision number is 0.

Usage Guidelines

None

Example

The following commands are used to set the regional revision number of STP to 100.

switch(config)# spanning-tree mstp revision 100

switch(config)#

### spanning-tree mstp instance

Syntax

To map the VLAN to the MSTI, run spanning-tree mstp instance instance-id vlan vlan-list. To re-map the VLAN to the CIST, run no spanning-tree mstp instance instance-id.

spanning-tree mstp instance instance-id vlan vlan-list

no spanning-tree mstp instance instance-id

Parameter

| Parameter | Description |
| --- | --- |
| instance-id | Instance number of the STP, meaning an MSTI which ranges from 1 to 15. |
| vlan-list | VLAN list which is mapped to the STP, ranging from 1 to 4094. |

Default

All VLANs are mapped to the CIST (MST00).

Usage Guidelines

instance-id is an unique value representing an STP instance.

vlan-list represents a VLAN group, such as “1,2,3”, “1-5” and “1,2,5-10”.

Example

The following commands map VLAN1 to instance 1 of STP, and VLAN5,7,10-20 to instance 2 of STP, and then re-map these VLANs to MST00.

Switch_config# spanning-tree mstp instance 1 vlan 2

Switch_config# spanning-tree mstp instance 2 vlan 5,7,10-20

Switch_config# no spanning-tree mstp instance 1

Switch_config# no spanning-tree mstp instance 2

### spanning-tree mstp root

Syntax

spanning-tree mstp instance-id root {primary | secondary}

[ diameter net-diameter [ hello-time seconds ] ]

no spanning-tree mstp instance-id root

To configure the specified MSTP instance to the primary/secondary root, run spanning-tree mstp instance-id root {primary | secondary}. To return to the default setting, run the negative form of the above command.

Both the diameter command and the hello-time command can modify the network diameter and the HelloTime parameter of the MSTP when they are setting the root.

Parameter

| Parameter | Description |
| --- | --- |
| instance-id | MSTP instance, ranging from 0 to 15 |
| primary | Sets the MSTP instance to the primary root. |
| secondary | Sets the MSTP instance to the secondary root. |
| net-diameter | Network diameter, which is optional When the instance-id parameter is 0, it is effective. It ranges from 2 to 7. |
| seconds | Hello time, an optional parameter, which ranges from 1 to 10 seconds |

Default

The priority value of all default roots of all MSTP instances are 32768, the network diameter is 7 and the HelloTime is 2 seconds.

Usage Guidelines

Both the diameter command and the hello-time command are valid only when instanc-id is 0.

Generally, after you run the command to set the primary root, the protocol automatically checks the ID of the current network root and then sets the priority field of the root identifier to 24576 if this value gurantees the current OLT to be the root of the MSTP instance. If the priority value of the root is smaller than 24576, the protocol will automatically set the MSTP priority of the current root to a value which is 4096 smaller than the root’s priority. Here, 4069 is the step of the root priority.

Different from the configuration of the primary root, the protocol directly sets the MSTP priority of the OLT to 28672 after the command for configuring the secondary root is run. Thus, the current OLT can be the secondary root when the priorities of other OLTs are the default value 32768.

Example

The following commands are used to set the OLT to the primary root in the CIST and recalculate the time parameter of the MSTP through network diameter 3 and HelloTime3, and at last set the OLT to the secondary root in the MST01.

Switch_config# spanning-tree mstp 0 root primary diameter 3 hello-time 3

Switch_config# spanning-tree mstp 1 root secondary

### spanning-tree mstp priority

Syntax

To configure the bridge priority of the MSTP instance, run spanning-tree mstp instance-id priority value. To return to the default setting, run no spanning-tree mstp instance-id priority.

spanning-tree mstp instance-id priority value

no spanning-tree mstp instance-id priority

Parameter

| Parameter | Description |
| --- | --- |
| instance-id | MSTP instance number, ranging from 0 to 15 |
| value | Bridge priority, which can be one of the given values: 0, 4096, 8192, 12288, 16384, 20480, 24576, 28672, 32768, 36864, 40960, 45056, 49152, 53248, 57344, 61440. |

Default

The default priority of the bridges of all MSTP instances is 32768.

Usage Guidelines

Each priority value in the MSTP instance is independent and can be configured independently.

Example

The following commands are used to set the priority of the switch in the CIST and MST01 to 4096 and 8192 respectively.

Switch_config# spanning-tree mstp 0 priority 4096

Switch_config# spanning-tree mstp 1 priority 8192

### spanning-tree mstp hello-time

Syntax

spanning-tree mstp hello-time seconds

no spanning-tree mstp hello-time

It is used to configure the hello-time of the MSTP, and its negative form is used to resume the default settings of the HelloTime.

Parameter

| Parameter | Description |
| --- | --- |
| seconds | It ranges from 1 to 10 seconds. Its default value is 2 seconds. |

Default

2 seconds

Usage Guidelines

None

Example

The following commands are used to set the HelloTime of the MSTP to 10.

switch(config)# spanning-tree mstp hello-time 10

switch(config)# no spanning-tree mstp hello-time

### spanning-tree mstp forward-time

Syntax

spanning-tree mstp forward-time seconds

no spanning-tree mstp forward-time

It is used to configure the Forward Delay of the MTSP. Its negative is used to resume the default settings.

Parameter

| Parameter | Description |
| --- | --- |
| seconds | It ranges from 4 to 30 seconds. Its default value is 15 seconds. |

Default

15 seconds

Usage Guidelines

None

Example

The following commands are used to set the Forward Delay parameter of the MTSP to 10.

Switch_config# spanning-tree mstp forward-time 10

Switch_config# no spanning-tree mstp forward-time

### spanning-tree mstp max-age

Syntax

To configure the Max Age parameter of the MSTP, run spanning-tree mstp max-age seconds. To return to the default setting, run the negative form of the command.

spanning-tree mstp max-age seconds

no spanning-tree mstp max-age

Parameter

| Parameter | Description |
| --- | --- |
| Seconds | Range: 6-40 seconds The default value is 20 seconds. |

Default

20 seconds

Usage Guidelines

None

Example

The following commands are used to set the MaxAge parameter of the MSTP to 10.

Switch_config# spanning-tree mstp max-age 10

Switch_config# no spanning-tree mstp max-age

### spanning-tree mstp diameter

Syntax

To configure the network diameter of the MSTP, run spanning-tree mstp diameter net-diameter. To return to the default setting, run no spanning-tree mstp diameter.

spanning-tree mstp diameter net-diameter

no spanning-tree mstp diameter

| Parameter | Description |
| --- | --- |
| net-diameter | Range: 2 - 7 Its default value is 7. |

Default

The default network diameter is 7.

Usage Guidelines

The net-diameter parameter is not saved as an independent settings in the OLT. The time parameter that is modified through network diameter configuration can be saved. The net-diameter parameter is valid in the CIST. After settings, the three time parameters of the STP can be automatically updated to a relatively advantageous value.

It is recommended to set the time parameters of the STP through root configuration or network diameter configuration. In this way, the reasonability of the time parameters can be assured.

Example

The following first command is to set the bridge diameter of MSTP to 5. The second command is to resume the default value of the bridge diameter.

Switch_config# spanning-tree mstp diameter 5

Switch_config# no spanning-tree mstp diameter

### spanning-tree mstp max-hops

Syntax

spanning-tree mstp max-hops hop-count

no spanning-tree mstp max-hops

The spanning-tree mstp max-hops hop-count command is used to set the maximum number of hops of the MSTP BPDU. Its negative is used to resume the default settings.

Parameter

| Parameter | Description |
| --- | --- |
| hop-count | Ranges from: 6 -40 Its default value is 20. |

Default

The default vaue of the maximum hop counts is 20.

Usage Guidelines

None

Example

The first command is to set the maximum hop counts of the MSTP BPDU to 6. The second command is to restore the default value of the maximum hop counts.

Switch_config# spanning-tree mstp max-hops 6

Switch_config# no spanning-tree mstp max-hops

### spanning-tree mstp port-priority

Syntax

To designate the priority of the spanning-tree STP instance, run spanning-tree mstp instance-id port-priority value. To return to the default setting, run the no form of the command.

spanning-tree mstp instance-id port-priority value

no spanning-tree instance-id port-priority

Parameter

| Parameter | Description |
| --- | --- |
| instance-id | Number of the STP instance, ranging from 0 to 31. |
| Value | Port priority, which is one of the following values: 0, 16, 32, 48, 64, 80, 96, 112 128, 144, 160, 176, 192, 208, 224, 240 |

Default

The default priority value of the port in all STP instances is 128.

Usage Guidelines

None

Example

The first command is to set the priority of port G0/1 in the CIST to 16. The second command is to resume the default value.

Switch_config_g0/1# spanning-tree mstp 0 port-priority 16

Switch_config_g0/1# no spanning-tree mstp 0 port-priority

### spanning-tree mstp cost

Syntax

The command spanning-tree mstp instance-id cost value is used to set the path cost of the port in the specified STP instance. Its negative is used to resume the default settings.

spanning-tree mstp instance-id cost value

no spanning-tree mstp instance-id cost

Parameter

| Parameter | Description |
| --- | --- |
| instance-id | Number of the STP instance, ranging from 0 to 15. |
| value | Path cost of the port, ranging from 1 to 200000000 |

Default

It depends on the connection rate of the port:

10 Mbps: 2000000

100 Mbps: 200000

1000 Mbps: 20000

Usage Guidelines

None

Example

The following commands are used to set the path cost of port G0/1 in the CIST to 200.

Switch_config_g0/1# spanning-tree mstp 0 cost 200

Switch_config_g5/1#

### spanning-tree mstp edge

Syntax

spanning-tree mstp edge

no spanning-tree mstp edge

To configure the edge port, run spanning-tree mstp edge. To return to the default setting, run no spanning-tree mstp edge.

Parameter

None

Default

Automatic check edge port

Usage Guidelines

None

Example

None

### spanning-tree mstp point-to-point

Syntax

spanning-tree mstp point-to-point { force-true | force-false | auto }

no spanning-tree mstp point-to-point

To configure the connection type of a port, run spanning-tree mstp point-to-point { force-true | force-false | auto }. To resume the connection type to auto-check, run no spanning-tree mstp point-to-point.

Parameter

| Parameter | Description |
| --- | --- |
| force-true | Sets the port connection mode to point-to-point. |
| force-false | Sets the port connection mode to sharing. |
| auto | Sets the port connection mode to auto-chech (the default mode). |

Default

MSTP will automatically check the port connection mode by default.

Usage Guidelines

None

Example

The following example shows how to set the connection mode of port G0/1 to sharing.

Switch_config_g0/1# spanning-tree mstp point-to-point force-false

Switch_config_g0/1#

### spanning-tree mstp mst-compatible

Syntax

spanning-tree mstp mst-compatible

no spanning-tree mstp mst-compatible

Enable/disable the MST-compatible mode, the global configuration mode.

spanning-tree mstp mst-compatible {enable | disable}

no spanning-tree mstp mst-compatible

Enable/disable the MST-compatible mode, the interface configuration mode.

Parameter

| Parameter | Description |
| --- | --- |
| enable | Enable the MST-compatible mode |
| disable | Disable the MST-compatible mode |

Default

The compitable mode is not activated by default and OLT cannot establish an area with other switches which transmit BPDU in compatible mode.

Usage Guidelines

After the MST-compatible mode is enabled, configure other connected switches that are running other MSTP protocols to the roots of CIST, ensuring that the OLT can enter the MSTP-compatible mode by receiving the message.

Example

The following command is to activate the MST-compatible mode in global configuration mode:

switch(config)#spanning-tree mstp mst-compatible

### spanning-tree mstp migration-check

Syntax

spanning-tree mstp migration-check

Clear the STP information that is checked by the port, and restart the protocol conversion process.

Parameter

None

Default

None

Usage Guidelines

The command is valid in global configuration mode and in port configuration mode.

Example

The following commands are used to check the protocol conversion on all ports first, and then check the protocol conversion on port G0/1 again.

Switch_config# spanning-tree mstp migration-check

Switch_config# interface g0/1

Switch_config_g0/1# spanning-tree mstp migration-check

### spanning-tree mstp restricted-role

Syntax

[no] spanning-tree mstp restricted-role

Enable/disable the role restriction on the port.

Parameter

None

Default

Disable the port’s role restriction.

Command Mode

Interface Configuration

Usage Guidelines

Enable the role restriction and the port will not be chosen as the root port.

Example

None

### spanning-tree mstp restricted-tcn

Syntax

[no] spanning-tree mstp restricted-tcn

Enable/disable the TCN restriction on the port.

Parameter

None

Default

Disable the TCN restriction on the port.

Command Mode

Interface Configuration

Usage Guidelines

Enable the TCN restriction on the port and do not tranmit topology changes to other ports.

Example

None

### show spanning-tree mstp

Syntax

show spanning-tree mstp [ instance instance-id ]

The command above is used to check the MSTP information. If you run the command show spanning-tree mstp, the information about all STP instances is displayed.

Parameter

| Parameter | Description |
| --- | --- |
| instance-id | Number of the STP instance, ranging from 0 to 15 |

Default

None

Usage Guidelines

It is valid in monitoring mode, global configuration mode or port mode.

Example

The following shows how to view all STP instances through the command. Here, MST00 stands for CIST, and the Type field stands for the port connection type.

Switch#show spanning-tree mstp

MST00       Vlans Mapped:  1,4-4094

Bridge      Address  00E0.0F64.8365  Priority  32768 (32768 mst-id 0)

Root        This bridge is the CIST and regional root

Configured  Hello Time 2, Forward Delay 15, Max Age 20, Max Hops 20

Root Times  Hello Time 2, Forward Delay 15, Max Age 20

Interface        Role Sts Cost      Pri.Nbr Type

---------------- ---- --- --------- ------- --------------------------------

G0/1             Desg FWD 200000    128.1   P2p

G0/2             Desg FWD 200000    128.2  Edge

MST01       Vlans Mapped:  2

Bridge      Address  00E0.0F64.8365  Priority  32769 (32768 mst-id 1)

Root        This bridge for MST01

Interface        Role Sts Cost      Pri.Nbr Type

---------------- ---- --- --------- ------- --------------------------------

G0/1             Desg FWD 200000    128.1   P2p

MST02       Vlans Mapped:  3

Bridge      Address  00E0.0F64.8365  Priority  32770 (32768 mst-id 2)

Root        This bridge for MST02

Interface        Role Sts Cost      Pri.Nbr Type

---------------- ---- --- --------- ------- --------------------------------

G0/1             Desg FWD 200000    128.1   P2p

### show spanning-tree mstp region

Syntax

show spanning-tree mstp region

Check the regional configuration information about the MSTP.

Parameter

None

Default

None

Usage Guidelines

None

Example

See the following information. MST Config Table shows the relation between VLAN and STP instance.

switch(config)# show spanning-tree mstp region

MST Region:

Name:    [reg01]

Revision:[0]

MST Config Table:

Instance     VLAN IDs

----------     ----------

0     1,4-4094

1     2

2     3

### show spanning-tree mstp detail

Syntax

show spanning-tree mstp detail

The command above is used to check the detailed information about MSTP.

Parameter

None

Default

None

Usage Guidelines

None

Example

The following example shows the detailed STP information after the command is run, including the port connection type and optional characteristics:

Switch#show spanning-tree mstp detail

MST00       Vlans Mapped:  1,4-4094

Root      Address  00E0.0F64.8365  Priority  32768 (32768 mst-id 0)

Root        This root is the CIST and regional root

Configured  Hello Time 2, Forward Delay 15, Max Age 20, Max Hops 20

Root Times  Hello Time 2, Forward Delay 15, Max Age 20

FastEthernet0/1 of MST00 is designated forwarding

Port Info             Port ID 128.1           Priority 128    Cost 200000

Designated Root       Address 00E0.0F64.8365  Priority 32768  Cost 0

CIST Regional Root    Address 00E0.0F64.8365  Priority 32768  Cost 0

Designated Root     Address 00E0.0F64.8365  Priority 32768  Port ID 128.1

Edge Port:   disabled                 Link Type:  point-to-point (auto)

Bpdu Guard:  disabled (default)       Root Guard: disabled (default)

Loop Guard:  disabled (default)

Timers:  message expires in 0 sec, forward delay 0 sec, up time 662 sec

Number of transitions to forwarding state: 1

Bpdu sent 335, received 5

FastEthernet0/3 of MST00 is backup blocking

Port Info             Port ID 128.3           Priority 128    Cost 200000

Designated Root       Address 00E0.0F64.8365  Priority 32768  Cost 0

CIST Regional Root    Address 00E0.0F64.8365  Priority 32768  Cost 0

Designated Root     Address 00E0.0F64.8365  Priority 32768  Port ID 128.1

Edge Port:   disabled                 Link Type:  point-to-point (auto)

Bpdu Guard:  disabled (default)       Root Guard: disabled (default)

Loop Guard:  disabled (default)

Timers:  message expires in 5 sec, forward delay 15 sec, up time 662 sec

Number of transitions to forwarding state: 0

Bpdu sent 5, received 335

FastEthernet0/47 of MST00 is designated forwarding

Port Info             Port ID 128.47          Priority 128    Cost 200000

Designated Root       Address 00E0.0F64.8365  Priority 32768  Cost 0

CIST Regional Root    Address 00E0.0F64.8365  Priority 32768  Cost 0

Designated Root     Address 00E0.0F64.8365  Priority 32768  Port ID 128.47

Edge Port:   enabled (auto)           Link Type:  point-to-point (auto)

Bpdu Guard:  disabled (default)       Root Guard: disabled (default)

Loop Guard:  disabled (default)

Timers:  message expires in 0 sec, forward delay 0 sec, up time 1485 sec

Number of transitions to forwarding state: 1

Bpdu sent 744, received 0

MST01       Vlans Mapped:  2

Root      Address  00E0.0F64.8365  Priority  32769 (32768 mst-id 1)

Root        This root for MST01

FastEthernet0/1 of MST01 is designated forwarding

Port Info             Port ID 128.1           Priority 128    Cost 200000

Designated Root       Address 00E0.0F64.8365  Priority 32769  Cost 0

Desingated Root     Address 00E0.0F64.8365  Priority 32769  Port ID 128.1

Timers:  message expires in 0 sec, forward delay 0 sec, up time 662 sec

Number of transitions to forwarding state: 1

MST Config Message transmitted 335, received 0

MST02       Vlans Mapped:  3

Root      Address  00E0.0F64.8365  Priority  32770 (32768 mst-id 2)

Root        This root for MST02

FastEthernet0/1 of MST02 is designated forwarding

Port Info             Port ID 128.1           Priority 128    Cost 200000

Designated Root       Address 00E0.0F64.8365  Priority 32770  Cost 0

Desingated Root     Address 00E0.0F64.8365  Priority 32770  Port ID 128.1

Timers:  message expires in 0 sec, forward delay 0 sec, up time 662 sec

Number of transitions to forwarding state: 1

MST Config Message transmitted 335, received 0

### show spanning-tree mstp interface

Syntax

show spanning-tree mstp interface interface-id

The command above is used to check the information about the port which is run under MSTP.

Parameter

| Parameter | Description |
| --- | --- |
| interface-id | Port name, such as “G0/1”, “GigaEthernet0/2” |

Default

None

Usage Guidelines

None

Example

The following example shows the information about port G0/1 after you run the command Switch#show spanning-tree mstp interface g0/1

GigaEthernet0/1 of MST00 is designated forwarding

Port Info             Port ID 128.1           Priority 128    Cost 200000

Designated Root       Address 00E0.0F64.8365  Priority 32768  Cost 0

CIST Regional Root    Address 00E0.0F64.8365  Priority 32768  Cost 0

Designated Bridge     Address 00E0.0F64.8365  Priority 32768  Port ID 128.1

Edge Port:   disabled                 Link Type:  point-to-point (auto)

Bpdu Guard:  disabled (default)       Root Guard: disabled (default)

Loop Guard:  disabled (default)

Timers:  message expires in 0 sec, forward delay 0 sec, up time 851 sec

Number of transitions to forwarding state: 1

Bpdu sent 430, received 5

GigaEthernet0/1 of MST01 is designated forwarding

Port Info             Port ID 128.1           Priority 128    Cost 200000

Designated Root       Address 00E0.0F64.8365  Priority 32769  Cost 0

Desingated Bridge     Address 00E0.0F64.8365  Priority 32769  Port ID 128.1

Timers:  message expires in 0 sec, forward delay 0 sec, up time 851 sec

Number of transitions to forwarding state: 1

MST Config Message transmitted 430, received 0

GigaEthernet0/1 of MST02 is designated forwarding

Port Info             Port ID 128.1           Priority 128    Cost 200000

Designated Root       Address 00E0.0F64.8365  Priority 32770  Cost 0

Desingated Bridge     Address 00E0.0F64.8365  Priority 32770  Port ID 128.1

Timers:  message expires in 0 sec, forward delay 0 sec, up time 851 sec

Number of transitions to forwarding state: 1

MST Config Message transmitted 430, received 0

Instance Role Sts Cost      Pri.Nbr Vlans Mapped

-------- ---- --- --------- ------- --------------------

0        Desg FWD 200000    128.1   1,4-4094

1        Desg FWD 200000    128.1   2

2        Desg FWD 200000    128.1   3

### show spanning-tree mstp protocol-migration

Syntax

The command above is used to check the protocol conversion information when the port is running under MSTP.

show spanning-tree mstp protocol-migration

Parameter

None

Default

None

Usage Guidelines

None

Example

The following example shows the information about protocol conversion after the command show spanning-tree mstp protocol-migration is run. Note that port G0/1 has transferred to the 802.1D STP mode.

Switch#show spanning-tree mstp protocol-migration

MSTP Port Protocol Migration

Interface        Protocol

---------------- ---------- ------------

G0/1             802.1D