LLDP Configuration Commands

Table of Contents

Chapter 1 LLDP Configuration Commands	1

1.1 LLDP Commands	1

1.1.1 lldp run	1

1.1.2 lldp holdtime	1

1.1.3 lldp timer	2

1.1.4 lldp reinit	3

1.1.5 lldp tlv-select	4

1.1.6 lldp dot1-tlv-select	5

1.1.7 lldp dot3-tlv-select	6

1.1.8 lldp med-tlv-select	7

1.1.9 lldp transmit	8

1.1.10 lldp receive	9

1.1.11 lldp management-ip	9

1.1.12 lldp trap-send	10

1.1.13 location elin identifier id WORD	11

1.1.14 location civic identifier id	12

1.1.15 location elin/civic id	14

1.1.16 show lldp errors	15

1.1.17 show lldp interface	15

1.1.18 show lldp neighbors	16

1.1.19 show lldp neighbors detail	17

1.1.20 show lldp traffic	19

1.1.21 show location elin	20

1.1.22 show location civic [identifier id]	20

1.1.23 clear lldp counters	21

1.1.24 clear lldp table	22

# LLDP Configuration Commands

## LLDP Commands

### lldp run

Syntax

- lldp run

no lldp run

To start up LLDP, run lldp run; to Disable LLDP, run no lldp run.

Parameter

None

Default

Disabled

Usage Guidelines

None

Command Mode

Global configuration mode

Example

The following command is used to start up LLDP.

Switch_config# lldp　run

### lldp holdtime

Syntax

lldp holdtime time

no lldp holdtime

To configure the ttl value of LLDP, run lldp holdtime time. To resume the default transmission delay, run no lldp holdtime.

Parameter

| Parameter | Description |
| --- | --- |
| time | Storage time of the transmitted message, ranging between zero to 65535 seconds |

Default

120s

Usage Guidelines

In normal, the remote information stored in the MIB will be updated before its aging. But the update frame may be lost during the transmission process, causing the information in the MIB to age. To prevent this, set the TTL value so that it updates the LLDP frame multiple times during the aging time.

Command Mode

Global configuration mode

Example

The following example shows how to set the ttl value of LLDP to 100 seconds.

Switch_config# lldp　holdtime 100

Switch_config#

### lldp timer

Syntax

lldp timer time

no lldp timer

To configure the transmission interval of LLDP, run lldp timer time. To resume the default transmission delay, run no lldptimer.

Parameter

| Parameter | Description |
| --- | --- |
| time | Interval for transmitting the LLDP message, ranging between 5 to 65534 seconds. |

Default

30s

Usage Guidelines

The transmission interval of the LLDP message must be shorter than its storage time, ensuring multiple updates in the storage time and preventing error which is led by packet loss.

Command Mode

Global configuration mode

Example

The following example shows how to configure the transmission interval of LLDP to 24 seconds.

Switch_config# lldp　timer　24

Switch_config#

### lldp reinit

Syntax

lldp reinit time

no lldp reinit

To configure the transmission delay of LLDP, run lldp reinit time. To resume the default transmission delay, run no lldp reinit.

Parameter

| Parameter | Description |
| --- | --- |
| time | Transmission delay of LLDP, whose values range from two to five seconds |

Default

2s

Usage Guidelines

LLDP information is automatically sent when the status or value of one or more information elements (managed objects) in the local system changes and the transmission timer expires. Since a single information change requires sending LLDP packets, a series of continuous information changes may trigger the transmission of many LLDP frames. Only one change is reported in each frame. To avoid this situation, network management defines waiting time between two consecutively senting LLDP frames.

Command Mode

Global configuration mode

Example

The following example shows how to set the transmission delay of LLDP to five seconds.

Switch_config# lldp　reinit　5

Switch_config#

### lldp tlv-select

Syntax

lldp tlv-select tlv-type

no lldp tlv-select tlv-type

To add TLV which is transmitted by the LLDP message, run lldp tlv-select tlv-type. To delete TLV which is transmitted by the LLDP message, run no lldp tlv-select tlv-type.

Parameter

| Parameter | Description |
| --- | --- |
| tlv-type | TLV that can be sent or not whose values are: management-address management address TLV port-description , port description TLV system-capabilities system capabilities TLV system-description, system description TLV system-name system name TLV |

Default

All TLVs are sent.

Usage Guidelines

Three mandatory TLVs must be sent.

Command Mode

Global configuration mode

Example

The following example shows how to enable the port description not to be transmitted in the message.

Switch_config#no　lldp　tlv-select　port-description

Switch_config#

### lldp dot1-tlv-select

Syntax

lldp dot1-tlv-select tlv-type

no lldp dot1-tlv-select tlv-type

To add TLV which is transmitted by the LLDP message, run lldp dot1-tlv-select　tlv-type. To return to the default setting, use the no form of this command.

Parameter

| Parameter | Description |
| --- | --- |
| tlv-type | Stands for TLV that are available for selective transmission. The value is: port-vlan-id port vlan address TLV protocol-vlan-id port and protocol VLAN ID TLV vlan-name vlan name TLV protocol-identity Protocol-identity TLV |

Default

All TLVs are sent.

Usage Guidelines

The TLV of the protocol identity does not support transmission but supports reception.

Command Mode

Port Configuration Mode

Example

The following example shows how to enable the TLV not to be transmitted by deletion of the VLAN address of a port in the transmitted packet.

switch_config#int g0/1

switch_config_g0/1#no　lldp　dot1-tlv-select　port-vlan-id

switch_config_g0/1#

### lldp dot3-tlv-select

Syntax

lldp　dot3-tlv-select　tlv-type

no lldp　dot3-tlv-select　tlv-type

To add TLV which is transmitted by the LLDP message, run lldp　dot3-tlv-select　tlv-type. To return to the default setting, use the no form of this command.

Parameter

| Parameter | Description |
| --- | --- |
| tlv-type | Stands for TLV that are available for selective transmission. The value is: link-aggregation link aggregation TLV macphy-confg MAC/Phy configuration/status TLV max-frame-size max frame size TLV power Power Via MDI TLV |

Default

All TLVs are sent.

Usage Guidelines

None

Command Mode

Port Configuration Mode

Example

The following example shows how to enable the TLV not to be transmitted by deletion of the MAC/Phy configuration/status of a port in the transmitted packet.

switch_config#int g0/1

switch_config_g0/1#no　lldp　dot3-tlv-select　 macphy-confg

switch_config_g0/1#

### lldp med-tlv-select

Syntax

lldp　med-tlv-select　tlv-type

no lldp　med-tlv-select　tlv-type

To add TLV which is transmitted by the LLDP message, run lldp med-tlv-select tlv-type. To return to the default setting, use the no form of this command.

Parameter

| Parameter | Description |
| --- | --- |
| tlv-type | Stands for TLV that are available for selective transmission. The value is: network-policy network policy TLV inventory detailed inventory management TLV location Location trace TLV power-management Expand Power Via MDI TLV |

Default

All TLVs are sent.

Usage Guidelines

When the TLV of MED need be transmitted, the MED capability TLV must be transmitted. Hence it does not fall into the choice.

Command Mode

Port Configuration Mode

Example

The following example shows how to enable the TLV not to be transmitted by deletion of the detailed list management in a transmitted packet.

switch_config#int g0/1

switch_config_g0/1#no　lldp　med-tlv-select　inventory

switch_config_g0/1#

### lldp transmit

Syntax

lldp transmit

no lldp transmit

To set the port to send the LLDP message, run lldp transmit. To forbid receiving the LLDP message, run no lldp transmit.

Parameter

None

Default

Transmittable LLDP message mode

Usage Guidelines

Only after the LLDP module is started can the command be valid.

Command Mode

Port configuration mode

Example

The following example shows how to set port g0/1 not to send the LLDP message.

switch_config_g0/1# no lldp　transmit

switch_config_g0/1#

### lldp receive

Syntax

lldp　receive

no lldp　receive

To set the port to the receivable LLDP message mode, run lldp receive. To forbid receiving the LLDP message, run no lldp receive.

Parameter

None

Default

Receivable LLDP message mode

Usage Guidelines

Only after the LLDP module is started can the configuration be valid.

Command Mode

Port configuration mode

Example

The following example shows how to set port g0/1 to the LLDP message mode.

switch_config_g0/1# no lldp　receive

switch_config_g0/1#

### lldp management-ip

Syntax

lldp　 management-ip　　A.B.C.D

no　lldp　 management-ip

To configure the management address of the LLDP port, run lldp management-ip A.B.C.D. To resume the default transmission delay, run no lldp management-ip.

Parameter

| Parameter | Description |
| --- | --- |
| A.B.C.D | Stands for the management IP address that will be specified. |

Default

The default management address is the IP of vlan interface corresponding to pvid. If the IP is not existed, the default management address is 0.0.0.0.

Usage Guidelines

The configured management IP address should be the IP address related with a port.

Command Mode

Interface configuration mode

Example

The following example shows how to set the management IP address of the port g0/1 to 90.0.0.99:

switch_config_g0/1# lldp　management-ip 90.0.0.99

switch_config_g0/1#

### lldp trap-send

Syntax

lldp 	tranp-send	lldp-mib

To send trap notification to lldp mib base, run the above command.

lldp 	tranp-send	ptopo-mib

To send trap notification to ptopo mib base, run the above command.

Parameter

None

Default

None

Usage Guidelines

None

Command Mode

Global Configuration mode

Example

The following example shows how to send trap notification to lldp mib base, run the following commands.

switch_config#lldp trap-send lldp-mib

switch_config#

The following example shows how to send trap notification to ptopo mib base, run the following command.

switch_config#lldp trap-send ptopo-mib

switch_config#

### location elin identifier id WORD

Syntax

location elin identifier　id WORD

no location elin identifier　id

To add the elin information, run location elin identifier id WORD; to delete the elin information, run no location elin identifier id.

Parameter

| Parameter | Description |
| --- | --- |
| id | Stands for the ID of the to-be-set elin, which ranges from 1 to 65535. |
| WORD | Stands for the content of the configured elin, which ranges from 10 to 25 bytes. |

Default

None

Usage Guidelines

None

Command Mode

Global configuration mode

Example

The following example shows how to set the identifier to 1 and the content of elin to 1234567890.

switch_config# location elin identifier 1 1234567890

switch_config#

### location civic identifier id

Syntax

location civic identifier　id

no location civic identifier　id

To enter the location configuration mode and set the civic information, run location civic identifier id. To return to the default setting, use the no form of this command.

Parameter

| Parameter | Description |
| --- | --- |
| id | Stands for the ID of the to-be-set civic, which ranges from 1 to 65535. |

Default

None

Usage Guidelines

After the system enters the location configuration mode, you can run the following commands to conduct the corresponding configuration to the civic of the ID. To return to the default setting, use the no form of this command.

| Command | Purpose |
| --- | --- |
| (no) language WORD | Sets the language |
| (no) state WORD | Sets the state’s (administrative district, district, province) name, such as shanghai. |
| (no) county WORD | Sets the name of a county. |
| (no) city WORD | Sets the name of a city. |
| (no) division WORD | Sets the name of a division. |
| (no) neighborhood WORD | Sets the name of neighborhood. |
| (no) street WORD | Sets the name of a street. |
| (no) leading-street-dir WORD | Sets the direction of a main street, such as N (north). |
| (no) trailing-street-suffix WORD | Sets the suffix of a small street, such as SW. |
| (no) street-suffix WORD | Sets the suffix of a street, such as platz. |
| (no) number WORD | Sets the street number, such as number 123. |
| (no) street-number-suffix WORD | Sets the suffix of the street number, such as number 1/2 of A road. |
| (no) landmark WORD | Sets the landmark, such as Colombia University. |
| (no) additional-location WORD | Sets the additional location. |
| (no) name WORD | Sets the information about a resident, such as Joe’s haircut shop. |
| (no) postal-code WORD | Sets the postal code. |
| (no) building WORD | Sets the information about a building. |
| (no) unit WORD | Sets the information about a unit. |
| (no) floor WORD | Sets the information about a floor. |
| (no) room WORD | Sets the information about a room. |
| (no) type-of-place WORD | Sets the type of a place, such as office. |
| (no) postal-community WORD | Sets the name of a postal office. |
| (no) post-office-box WORD | Sets the name of a postal box, such as 12345. |
| (no) additional-code WORD | Sets the additional code. |
| (no) country WORD | Sets the name of a country. |
| (no) script WORD | Sets the script. |

Command Mode

Global Configuration Mode

Example

The following example shows how to set the civic information of identifier 1.

Switch_config#location civic identifier 1

Switch_config_civic#language English

Switch_config_civic#city Shanghai

Switch_config_civic#street Curie

Switch_config_civic#script EN

Switch_config_civic#quit

Switch_config#

### location elin/civic id

Syntax

location elin/civic　id

no location elin/civic

To set the location for a port, run location elin/civic id. To delete the location of a port, run no location elin/civic.

Parameter

| Parameter | Description |
| --- | --- |
| id | Stands for the ID of the to-be-set elin, which ranges from 1 to 65535. |

Default

None

Usage Guidelines

None

Command Mode

Port Configuration Mode

Example

The following example shows how to set the elin and the civic for a port.

Switch_config#int g0/8

Switch_config_g0/8#location elin 1

Switch_config_g0/8#location civic 1

### show lldp errors

Syntax

Show　lldp　errors

It is used to display the error information about the LLDP module.

Parameter

None

Default

None

Usage Guidelines

None

Command Mode

EXEC/Global configuration mode

Example

The following example shows how to view the error information of the lldp module.

switch_config#show lldp errors

LLDP errors/overflows:

Total memory allocation failures: 0

Total encapsulation failures: 0

Total table overflows: 0

switch_config#

### show lldp interface

Syntax

Show　lldp　interface　interface-name

To check the transmission and reception mode, run show lldp interface interface name.

Parameter

| Parameter | Description |
| --- | --- |
| interface-name | Name of the interface, such as g0/1 and GigaEthernet0/1 |

Default

None

Usage Guidelines

After LLDP is started, you can check the state of the port.

Command Mode

EXEC/global configuration mode

Example

The following example shows how to check the transmission and reception mode of port g0/1.

switch_config#show lldp interface g0/1

GigaEthernet0/1:

Rx: enabled

Tx: enabled

switch_config#

### show lldp neighbors

Syntax

show lldp　neighbors

It is used to display the simple information about neighbors.

Parameter

None

Default

None

Usage Guidelines

The command is used to display the simple information about neighbors.

Command Mode

EXEC / global configuration mode

Example

switch_config#show lldp neighbors

Capability Codes:

(R)Router,(B)Bridge,(C)DOCsIs Cable Device,(T)Telephone

(W)WLAN Access Point, (P)Repeater,(s)station,(O)Other

Device-ID       Local-Intf      Hldtme       Port-ID      Capability

switch          Gig0/2          115          Gig0/32      B

switch          Gig0/32         114          Gig0/2       B

Total entries dispalyed: 2

switch_config#

### show lldp neighbors detail

Syntax

Show　lldp　neighbors　detail

It is used to display the detailed information about the neighbor.

Parameter

None

Default

None

Usage Guidelines

None

Command Mode

EXEC/global configuration mode

Example

switch_config#show lldp neighbors detail

chassis id: 00e0.0f61.ca53

port id: Gig0/32

port description: GigaEthernet0/32

system name: switch

system description: s3448 software, Version 2.0.1K

serial: s35000456

Compiled: 2008-11-13 13:33:36 by 16170F032B9F

Time remaining: 98

system capabilities: R B

enabled capabilities: B

Managment Address:

IP: 192.168.213.62

Auto Negotiation -- supported,enabled

Physical media capabilitise:

100baseTX(FD)

100baseTX(HD)

10baseT(FD)

10baseT(HD)

Media Attachment Unit type: 16

------------------------------------------

chassis id: 00e0.0f61.ca35

port id: Gig0/2

port description: GigaEthernet0/2

system name: switch

system description: s3448 software, Version 2.0.1K

serial: s35000456

Compiled: 2008-11-13 13:33:36 by 16170F032B9F

Time remaining: 95

system capabilities: R B

enabled capabilities: B

Managment Address:

IP: 90.0.0.66

Auto Negotiation -- supported,enabled

Physical media capabilitise:

100baseTX(FD)

100baseTX(HD)

10baseT(FD)

10baseT(HD)

Media Attachment Unit type: 16

------------------------------------------

Total entries dispalyed: 2

switch#

### show lldp traffic

Syntax

Show　lldp　traffic

To display all statistics information about LLDP, run show lldp traffic.

Parameter

None

Default

None

Usage Guidelines

None

Command Mode

EXEC/global configuration mode

Example

switch_config#show lldp traffic

LLDP traffic statistics:

Total frames out: 1599

Total entries aged: 0

Total frames in: 624

Total frames received in error: 0

Total frames discarded: 0

Total TLVs unrecognized: 0

switch_config#

### show location elin

Syntax

show　location　 elin

To display the elin configuration of the location, run the above command.

Parameter

None

Default

None

Usage Guidelines

None

Command Mode

EXEC/Global Configuration Mode

Example

Switch_config#show location elin

elin information:

elin 2: 0987654321

elin 1: 1234567890

total: 2

Switch_config#

### show location civic [identifier id]

Syntax

show　 location　civic　[identifier　id]

To display the civic information of the location, run the above command.

Parameter

| Parameter | Description |
| --- | --- |
| id | Stands for the ID of the to-be-set civic, which ranges from 1 to 65535. |

Default

None

Usage Guidelines

None

Command Mode

Exec/ Global Configuration Mode

Example

Switch_config#show location civic

civic address information:

identifier: 2

Language: Chinese

Script: CN

-------------------------------------

identifier: 1

City: Shanghai

Language: English

Script: EN

-------------------------------------

total: 2

Switch_config#

### clear lldp counters

Syntax

clear　lldp　counters

To clear the statistics information, run clear lldp counters.

Parameter

None

Default

None

Usage Guidelines

None

Command Mode

EXEC

Example

switch#clear lldp counters

switch#

switch#show lldp traffic

LLDP traffic statistics:

Total frames out: 0

Total entries aged: 0

Total frames in: 0

Total frames received in error: 0

Total frames discarded: 0

Total TLVs unrecognized: 0

switch#

switch#show lldp errors

LLDP errors/overflows:

Total memory allocation failures: 0

Total encapsulation failures: 0

Total table overflows: 0

switch#

### clear lldp table

Syntax

clear　lldp　table

To remove the neighbor list, run clear lldp table.

Parameter

None

Default

None

Usage Guidelines

None

Command Mode

EXEC

Example

switch#clear lldp table

switch#

switch#show lldp neighbors

Capability Codes:

(R)Router,(B)Bridge,(C)DOCsIs Cable Device,(T)Telephone

(W)WLAN Access Point, (P)Repeater,(s)station,(O)Other

Device-ID       Local-Intf      Hldtme       Port-ID      Capability

Total entries displayed: 0