UDLD Configuration Commands

Table of Contents

Chapter 1 UDLD Configuration Commands	1

1.1 UDLD Configuration Commands	1

1.1.1 udld enable	1

1.1.2 udld aggressive	2

1.1.3 udld port	3

1.1.4 udld port aggressive	3

1.1.5 udld message	4

1.1.6 udld reset	5

1.1.7 show udld	6

# Chapter 1 UDLD Configuration Commands

## UDLD Configuration Commands

UDLD configuration commands include:

- udld enable

- udld aggressive

- udld port

- udld port aggressive

- udld message

- udld reset

- show udld

### udld enable

Syntax

udld enable

To enable global UDLD function in normal mode, use udld enable command.

no udld enable

To disable global UDLD function in normal mode, use no udld enable command.

Parameter

None

Default

None

Usage Guidelines

Enable UDLD function of all interfaces in normal mode. In normal mode, if cannot connect, UDLD will set the interface in the state of undetermined, without disabling the interface protocol. If the UDLD is thought to be a two-way link, then the interface is in state of Bidirectional.

Command Mode

Global configuration mode

Example

The following example shows how to enable UDLD in normal mode:

Switch_config#udld enable

### udld aggressive

Syntax

udld aggressive

To enable global UDLD function in aggressive mode, use udld aggressive command.

no udld aggressive

To disable global UDLD function in aggressive mode, use udld aggressive command.

Parameter

None

Default

None

Usage Guidelines

Enable UDLD function of all interfaces in aggressive mode. In aggressive mode, if UDLD cannot connect and fail to reconnect, there is consider to be serious network problem for communication interruption. Then UDLD will disable the interface protocol and the interface is in the state of errdisable. If the UDLD is thought to be a two-way link, then the interface is in state of Bidirectional.

Command Mode

Global configuration mode

Example

The following example shows how to enable UDLD in aggressive mode:

Switch_config#udld aggressive

### udld port

Syntax

udld port

To enable UDLD function of the port in normal mode, use udld port command.

no udld port

To disable UDLD function of the port in normal mode, use no udld port command.

Parameter

None

Default

None

Usage Guidelines

Enable UDLD function of its interface in normal mode. In normal mode, if cannot connect, UDLD will set the interface in the state of undetermined, without disabling the interface protocol. If the UDLD is thought to be a two-way link, then the interface is in state of Bidirectional.

Command Mode

Interface configuration mode

Example

The following example shows how to enable UDLD of the port in normal mode:

Switch_config_g0/1#udld port

### udld port aggressive

Syntax

udld port aggressive

To enable UDLD function of the port in aggressive mode, use udld port aggressive command.

no udld port aggressive

To disable UDLD function of the port in aggressive mode, use no udld port aggressive command.

Parameter

None

Default

None

Usage Guidelines

Enable UDLD function of its interface in aggressive mode. In aggressive mode, if UDLD cannot connect and fail to reconnect, there is consider to be serious network problem for communication interruption. Then UDLD will disable the interface protocol and the interface is in the state of errdisable. If the UDLD is thought to be a two-way link, then the interface is in state of Bidirectional.

Command Mode

Interface configuration mode

Example

The following example shows how to enable UDLD of the port in aggressive mode:

Switch_config_g0/1#udld port aggressive

### udld message

Syntax

udld message  time

To set udld message intervals in aggressive mode, use above command.

no udld message

To return to the default setting, use the no form of this command.

Parameter

| Parameter | Description |
| --- | --- |
| time | Interval time in aggressive mode, range: 7-90s |

Default

15s

Usage Guidelines

Set the udld message interval in aggressive mode. Only after restarting the aggressive mode, new interval time can be valid.

Command Mode

Global configuration mode

Example

The following example shows how to set the udld message interval in aggressive mode as 7s, which is valid after restarting the aggressive mode:

Switch_config#udld message 7

### udld reset

Syntax

udld reset

To reset the interface disabled by UDLD mode protocol, use above command.

Parameter

None

Default

None

Usage Guidelines

Reset the interface disabled by UDLD mode protocol.

Command Mode

Management configuration mode

Example

The following example shows how to reset the interface disabled by UDLD module:

Switch#udld reset

1 ports shutdown by UDLD were reset.

%%UDLD-2-UDLD_PORT_RESET: UDLD reset interface GigaEthernet0/1.

%%PM-4-ERR_RECOVER: Attempting to recover from udld err-disable state on  GigaEthernet0/1.

### show udld

Syntax

show udld interface   [interface ]

To display the connection information of running UDLD, use above command.

Parameter

| Parameter | Description |
| --- | --- |
| interface | Display UDLD operation information of the specified interface. |

Default

None

Usage Guidelines

Display the operation information of UDLD module. When not input the interface parameter, UDLD operation information of all interfaces will be displayed. When input the interface parameter, only display the UDLD information of the interface whose parameter is input.

Command Mode

Management/Global configuration mode

Example

The following example shows how to display the operation state information of UDLD module in all interfaces:

Switch_config#show udld

Interface GigaEthernet0/1

---

Port enable administrative configuration setting: Enabled

Port enable operational state: Enabled

Current bidirectional state: Unknown

Current operational state: Link down

Message interval: 15

Time out interval: 1

No neighbor cache information stored

Interface GigaEthernet0/2

---

Port enable administrative configuration setting: Enabled

Port enable operational state: Enabled

Current bidirectional state: Unknown

Current operational state: Link down

Message interval: 15

Time out interval: 1

No neighbor cache information stored

Interface GigaEthernet0/3

---

Port enable administrative configuration setting: Enabled

Port enable operational state: Enabled

Current bidirectional state: Unknown

Current operational state: Link down

Message interval: 15

Time out interval: 1

No neighbor cache information stored

....

....

....

The following example shows how to display the operation state information of UDLD module in thterface G0/1:

Switch_config#show udld interface g0/1

Interface GigaEthernet0/1

---

Port enable administrative configuration setting: Enabled

Port enable operational state: Enabled

Current bidirectional state: Unknown

Current operational state: Link down

Message interval: 15

Time out interval: 1

No neighbor cache information stored

Switch_config#