- DoS-Attack Prevention Configuration Commands

- Table of Contents

Chapter 1 DoS Attack Prevention Configuration Commands	3

1.1 DoS-Attack Prevention Configuration Commands	3

1.1.1 dos enable	3

1.1.2 show dos	4

# DoS Attack Prevention Configuration Commands

## DoS-Attack Prevention Configuration Commands

DoS attack prevention configuration commands are shown below:

- dos enable

- show dos

### dos enable

Syntax

dos enable {all | icmp  icmp-value | ip | l4port | mac | tcpflags | tcpfrag tcpfrag-value | tcpsmurf | icmpsmurf | ipsmurf }

no dos enable { all | icmp  icmp-value | ip | l4port | mac | tcpflags | tcpfrag tcpfrag-value | tcpsmurf | icmpsmurf | ipsmurf }

Parameter

| Parameter | Description |
| --- | --- |
| all | Enables to prevent all kinds of DoS attacks. |
| icmp icmp-value | Enables to check ICMP messages. The maximum length of ICMP message is icmp-value. ICMP and ICMPv6 messages with length greater than icmp-value will be discarded |
| ip | Enables to prevent the dos attack messages whose source IP address is same as its destination IP address. |
| l4port | Enables to check the layer-4 messages whose source port is same as its destination port. |
| mac | Enables to prevent the messages whose source MAC address is same as its destination MAC address. |
| tcpflags | Starts to check the TCP packets with illegal flags. |
| tcpfrag tcpfrag-value | Enables to check dos attack messages of tcp subcontract, the minimum tcp header is tcpfrag-value, the default value is 20. |
| tcpsmurf | Enables to prevent TCP messages whose destination address is broadcast address. |
| icmpsmurf | Enables to prevent ICMP messages whose destination address is broadcast address. |
| ipsmurf | Enables to prevent IP messages whose destination address is broadcast address. |

Default

DoS attack prevention is disabled by default.

Usage Guidelines

DoS attack prevention is configured in global mode.

The ip sub-function can defend against LAND attack, preventing IP messages whose source address is the same as destination address.

The icmp sub-function can drop packets: 1. ICMP, ICMPv6 ping request message that is large than icmp-value; 2. Subcontact ICMP, ICMPv6 message. It can defend against PING attack.

The l4port sub-function can drop TCP/UDP packets whose source port number is the same as destination port number.

The mac sub-function can check MAC address, preventing messages whose source MAC address is the same as destination MAC address.

The tcpflags sub-function can drop the following 4 kinds of TCP packets: 1.TCP SYN flag = 1 & source port<1024; 2.TCP control flags = 0 & sequence = 0; 3.TCP FIN URG PSH =1 & sequence = 0; 4.TCP FIN SYN =1.

The tcpfrag sub-function can drop the following 2 kinds of TCP packets: 1. TCP subcontract whose header is smaller than tcpfrag-value; 2. TCP subcontract whose offset is 1. It can defend against TearDrop attack.

The tcpsmurf sub-function can defend against tcpsmurf attack, preventing TCP messages whose destination address is the broadcast address.

The icmpsmurf sub-function can defend against icmpsmurf attack, preventing ICMP messages whose destination address is the broadcast address.

The ipsmurf sub-function can defend against ipsmurf attack, preventing IP messages whose destination address is the broadcast address.

Example

The following example shows how to set DoS attack prevention subfunctions in global mode to prevent IP messages whose source IP address is same as destination IP address.

Switch_config#dos enable ip

The following example shows how to set dos enable tcpflags to check illegal TCPflag messages in global mode.

Switch_config#dos enable tcpflags

### show dos

Syntax

show dos

It is used to show all DoS attack prevention functions that users have set.

Parameter

None

Default value

None

Remarks

EXEC mode

Example

The following example shows how to display all DoS attack prevention functions.

Switch_config#dos enable all

Switch_config#show dos

dos enable icmp

dos enable ip

dos enable l4port

dos enable mac

dos enable tcpflags

dos enable tcpfrag

dos enable tcpsmurf

dos enable icmpsmurf

dos enable ipsmurf

Switch_config#

The following example shows how to set dos enable ip to display the sub-function that users have set.

Switch_config#dos enable ip

Switch_config#show dos

dos enable ip