- Interface Range Command

- Table of Contents

Chapter 1 Interface Range Command	1

1.1 Interface Range	1

# Chapter 1 Interface Range Command

## Interface Range

Description

interface range type slot/<port1 - port2 | port3>[<port1 - port2|port3>]

Parameter

| Name | Description | Description |
| --- | --- | --- |
| type | Interface type | All legal interface types except for the management interface on the main contril board of the rack-mounted switch. |
| slot | Slot number | All legal slot numbers |
| port1 | Beginning value of the port number | All legal port numbers on the slot. |
| port2 | Ending value of the port number | All legal port numbers on the slot except for port 1. |
| port3 | A single port. | All legal port numbers on the slot. |

Default

none

Instruction

Use this command to enter the interface range mode.

Example

Use the following command to enter the enterface configuration mode, including slot 0 and fast Ethernet port 1,2,3,6,8,10,11,12:

switch_config#interface range 1 - 3 , 6 , 8 , 10 - 12

switch_config_if_range#