Port Additional Characteristics Configuration Commands

- Table of Contents

Chapter 1 Port Security	2

1.1 switchport port-security mode static	2

1.2 switchport port-security mode dynamic	2

1.3 switchport port-security static mac-address	2

1.4 switchport port-security dynamic maximum	2

Chapter 2 Port Protection	3

2.1 switchport protected	3

Chapter 3 Port Storm Control	4

3.1 storm-control	4

Chapter 4 Port Rate Limitation	5

4.1 switchport rate-limit	5

# Port Security

## switchport port-security mode static

Command description

switchport port-security mode static {accept | reject}

no switchport port-security mode

Set the static mode of the security port.

## switchport port-security mode dynamic

Command description

switchport port-security mode dynamic

no switchport port-security mode

Add/delete the dynamic mode of the security port.

## switchport port-security static mac-address

Command description

switchport port-security static  mac-address mac-addr

no switchport port-security static  mac-address

Configure the static MAC address of the security port.

## switchport port-security dynamic maximum

Command description

switchport port-security dynamic maximum value

no switchport port-security dynamic maximum

Add/delete the maximum number of dynamic MAC addresses of the security port.

# Port Protection

## switchport protected

Command description

[no] switchport protected

Configure the port isolation function.

Parameter

None

Default

The port is not isolated.

Explanation

The command must be configured in layer-2 port configuration mode.

Example

Configure port f0/1 not to forward the unknown unicast frame.

Switch(config)# interface fastethernet0/1

Switch(config-if)# switchport protected

# Port Storm Control

## storm-control

Command description

Configure the storm control function of the port.

storm-control {broadcast | multicast | unicast} threshold count

no storm-control {broadcast | multicast | unicast} threshold count

Parameter

| Parameter | Description |
| --- | --- |
| broadcast ¦ multicast ¦ unicast | Defines the storm control of the broadcast, multicast and unicast. |
| threshold count | Defines the flow percent of the storm control. The count parameter defines the flow caps that lead to the storm. <1-127>, n*64Kbps(n<=28);(n-27)Mbps(n>28) |

Default

The storm control function is not enabled.

Explanation

The command must be configured in layer-2 port configuration mode.

Example

Set the storm control of the unknown unicast frame on port f0/1 to 192 Kbps.

Switch(config)# interface fastethernet0/1

Switch(config-f0/1)# storm-control unicast  threshold 3

# Port Rate Limitation

## switchport rate-limit

Command description

[no] switchport rate-limit band { ingress|egress}

Configure the flow rate limitation for the port.

Parameter

| Parameter | Description |
| --- | --- |
| Band | Flow rate n*64Kbps(n<=28); (n-27)Mbps(n>28) |
| ingress | Functions at the incoming port. |
| egress | Functions at the outgoing port. |

Default

The port has no port rate limitation.

Explanation

Layer-2 port configuration mode

Example

Set the incoming flow rate limitation on port f0/1 to 1M.

Switch(config)# interface f0/1

Switch(config-if)# switchport rate-limit 28 ingress