Anti-Attack Configuration Commands

- Table of Contents

Chapter 1 Anti-Attack Configuration Commands	1

1.1 Anti-Attack Configuration Commands	1

1.1.1 filter period time	1

1.1.2 filter threshold value	1

1.1.3 filter block-time value	2

1.1.4 filter igmp	3

1.1.5 filter arp	3

1.1.6 filter enable	3

1.1.7 show filter	4

# Chapter 1 Anti-Attack Configuration Commands

## Anti-Attack Configuration Commands

### filter period time

To configure filter period for attack, use the filter period command.

parameter

| parameter | Description |
| --- | --- |
| time | The filter period for attack in seconds. It is considered as attack when the attack source sends packets above the specified number in any filter period time. |

default

10 seconds

Command mode

Global configuration mode

example

Switch_config#filter period 15

Related commands

filter threshold value

### filter threshold value

To configure the filter threshold value, use the filter thresholf value command.

parameter

| parameter | Description |
| --- | --- |
| value | It is considered as attack when the receiving packets excddes the filter threshold value. |

default

1000

command mode

global configuration mode

example

Switch_config#filter threshold 1500

Related commands

filter period time

### filter block-time value

To configure the time to block attack resource, use the filter block-time value command.

parameter

| parameter | description |
| --- | --- |
| Value | Time to block attack source in seconds. |

default

300 seconds

command mode

global configuration mode

example

Switch_config#filter block-time 600

Related commands

filter period time

filter threshold value

### filter igmp

To filter IGMP attack, use the filter igmp command.

parameter

none

Command mode

Global configuration mode

example

Switch_config#filter igmp

Related commands

filter enable

### filter arp

To fliter ARP attack, use the filter arp command.

parameter

none

Command mode

physical interface configuration mode

example

Switch_config_f0/1#filter arp

Related commands

filter enable

### filter enable

To enable filter feature, use the filter enable command.

parameter

none

Command mode

Global configuration mode

example

Switch_config#filter enable

Related commands

filter igmp

filter arp

### show filter

To display working state of the anti-attack feature of the current switch, use the show filter command.

parameter

none

command mode

non-user mode

Switch#show fil

Filter threshold: 1000 packet in any 10 seconds

Filters blocked:

Address         seconds     source interface

00a0.0c13.647d    27.0      FastEthernet1/2

Filters counting:

Address         seconds     count  source interface

00a0.0c43.647d     1.84     371    FastEthernet1/2

Filters blocked: indicates MAC address of the blocked attack source, blocked time and source interface.

Filters counting:  indicates MAC address of the attack source, counting time, the number of the receiving packets and the source interface.