OSPFv3 Configuration Commands

Table of Contents

Chapter 1 OSPFv3 Configuration Commands	1

1.1 OSPFv3 Configuration Commands	1

1.1.1 area default-cost	2

1.1.2 area nssa	3

1.1.3 area range	5

1.1.4 area stub	6

1.1.5 area virtual-link	7

1.1.6 debug ipv6 ospf	9

1.1.7 debug ipv6 ospf events	10

1.1.8 debug ipv6 ospf ifsm	12

1.1.9 debug ipv6 ospf lsa	14

1.1.10 debug ipv6 ospf nfsm	16

1.1.11 debug ipv6 ospf nsm	18

1.1.12 debug ipv6 ospf packet	19

1.1.13 debug ipv6 ospf route	21

1.1.14 default-information originate	23

1.1.15 default-metric	24

1.1.16 filter	25

1.1.17 ipv6 ospf area	26

1.1.18 ipv6 ospf cost	27

1.1.19 ipv6 ospf database-filter all out	27

1.1.20 ipv6 ospf dead-interval	28

1.1.21 ipv6 ospf hello-interval	29

1.1.22 ipv6 ospf mtu-ignore	30

1.1.23 ipv6 ospf neighbor	31

1.1.24 ipv6 ospf network	32

1.1.25 ipv6 ospf priority	33

1.1.26 ipv6 ospf retransmit-interval	34

1.1.27 ipv6 ospf transmit-delay	35

1.1.28 passive-interface	36

1.1.29 redistribute	36

1.1.30 router ospfv3	37

1.1.31 router-id	38

1.1.32 show ipv6 ospf	39

1.1.33 show ipv6 ospf database	40

1.1.34 show ipv6 ospf interface	42

1.1.35 show ipv6 ospf neighbor	43

1.1.36 show ipv6 ospf route	44

1.1.37 show ipv6 ospf virtual-link	46

1.1.38 summary-prefix	47

1.1.39 timers delay	48

1.1.40 timers hold	49

# OSPFv3 Configuration Commands

## OSPFv3 Configuration Commands

The OSPFv3 configuration commands include:

- area default-cost

- area nssa

- area range

- area stub

- area virtual-link

- debug ipv6 ospf

- debug ipv6 ospf events

- debug ipv6 ospf ifsm

- debug ipv6 ospf lsa

- debug ipv6 ospf nfsm

- debug ipv6 ospf nsm

- debug ipv6 ospf packet

- debug ipv6 ospf route

- default-information originate

- default-metric

- filter

- ipv6 ospf area

- ipv6 ospf cost

- ipv6 ospf database-filter all out

- ipv6 ospf dead-interval

- ipv6 ospf hello-interval

- ipv6 ospf mtu-ignore

- ipv6 ospf neighbor

- ipv6 ospf network

- ipv6 ospf priority

- ipv6 ospf retransmit-interval

- ipv6 ospf transmit-delay

- passive-interface

- redistribute

- router ospfv3

- router-id

- show ipv6 ospf

- show ipv6 ospf database

- show ipv6 ospf interface

- show ipv6 ospf neighbor

- show ipv6 ospf route

- show ipv6 ospf virtual-link

- summary-prefix

- timers delay

- timers hold

### area default-cost

To specify the cost of the default summary route in the NSSA or STUB area, run the first one of the following two commands:

area area-id default-cost cost

no area area-id default-cost

Parameter

| Parameter | Description |
| --- | --- |
| area-id | Means the ID of the NSSA or STUB area. |
| cost | Means the cost of the default summary route. |

Default value

The default value is 1.

Command mode

Routing configuration mode

Instruction

The command is helpful only when it is used on the boundary router connecting the NASSA area or the STUB area.

After the area stub default-information-originate command is configured, the cost configured by this command will be used in LSA to set the corresponding cost.

Example

The following example shows how to set the default cost of stub domain 36.0.0.0 to 20:

interface vlan 1

ipv6 enable

ipv6 ospf 1 area 36.0.0.0

!

router ospfv3 1

router-id 2.2.2.2

area 36.0.0.0 stub

area 36.0.0.0 default-cost 20

Related command

area nssa

area stub

### area nssa

To configure a NSSA area, run the first one of the following two commands:

area area-id nssa [default-information-originate [metric value] [metric-type {1 | 2}]] [interval value] [no-redistribute] [no-summary] [range {ipv6-prefix/prefix-length} [advertise | not-advertise]] [translator {always|candidate}]

no area area-id nssa [default-information-originate [metric value] [metric-type {1 | 2}]] [interval value] [no-redistribute] [no-summary] [range {ipv6-prefix/prefix-length} [advertise | not-advertise]] [translator {always|candidate}]

Parameter

| Parameter | Description |
| --- | --- |
| area-id | Sets the ID of the NSSA area. It can be a decimal number or an IP address. |
| default-information-originate | Means to send the default route to the NSSA area. |
| metric value | Stands for the cost of the default route, which ranges from 1 to 16777214. |
| metric-type {1 ¦ 2} | Means the cost type of the default route. |
| interval value | Means the stable time of the NSSA translator role, which ranges from 1 to 65535. |
| no-redistribute | Means not to redistribute a route to the NSSA area. |
| no-summary | Forbids the ABR router to send the summary link to the NSSA area. |
| range | Means to conduct summary when type-7 LSA is translated into type-5 LSA. |
| translator | Stands for the NSSA translator role; if the parameter “always” is used, it means it is always the translator, and if it is the parameter “candidate”, it means it can be chosen as a translator. |

Default value

Non-NSSA area

Command mode

Routing configuration mode

Instruction

All routers and access servers in the NSSA area will be configured by the area nssa command.

To decrease the number of LSA’s, you can run no summary on the ABR router to forbid the summary LSA to enter the NSSA area.

The parameter “no-distribute” is always used for ABR and its purpose is to stop redistributed routes from being sent to the NSSA area.

Example

The following example shows how to set the NSSA area of 36.0.0.0:

interface vlan 1

ipv6 enable

ipv6 ospf 1 area 36.0.0.0

!

router ospfv3 1

router-id 2.2.2.2

area 36.0.0.0 nssa

!

Related command

area stub

### area range

To summarize the routes at the field boundary, run area area-id range {ipv6-prefix /prefix-length} [advertise | not-advertise]. To cancel the previous settings, run no area range.

area area-id range {ipv6-prefix /prefix-length} [advertise | not-advertise]

no area area-id range {ipv6-prefix /prefix-length} [advertise | not-advertise]

Parameter

| Parameter | Description |
| --- | --- |
| area-id | Means the fields where the fields will be summarized. It can be a decimal number or an IPv6 address. |
| ipv6-prefix | Means the prefix of the IPv6 address. |
| prefix-length | Means the length of the IPv6 address’ prefix. |
| advertise | Means that the routes are released after they are summarized. |
| not-advertise | Means that the routes are not released after they are summarized. |

Default value

The command has no effect by default.

Command mode

Routing configuration mode

Instruction

The area range command is not run on the ABR router, enabling ABR to be broadcast to other routers through a summary route. In this way, the route of the field boundary is miniatured. As to the outside of the area, each address range has only one summary route.

The command can be configured on the routers in multiple areas, and OSPF, hence, can summarize many address ranges.

Example

The following example shows how to set the prefix of the summarized IPv6 address in area 1, 2001:0DB8:0:1::/64:

interface vlan 1

no ip address

ipv6 enable

ipv6 ospf 1 area 1

!

router ospfv3 1

router-id 192.168.255.5

log-adjacency-changes

area 1 range 2001:0DB8:0:1::/64

### area stub

To configure a STUB area, run the first one of the following two commands. To cancel the configuration, run the other command.

area area-id stub [no-summary]

no area area-id stub [no-summary]

Parameter

| Parameter | Description |
| --- | --- |
| area-id | Sets the ID of the STUB area. It can be a decimal number or an IP address. |
| no-summary | Forbids the ABR router to send the summary link to the STUB area. |

Default value

Non-stub area

Command mode

Routing configuration mode

Instruction

All routers and access servers in the STUB area will be configured by the area stub command. The ABR router adopts the default-cost option to set the cost from the internal router to the STUB area.

To decrease the number of LSA’s, you can run no summary on the ABR router to forbid the summary LSA to enter the STUB area.

Example

The following example shows how to set the STUB area of 36.0.0.0:

interface vlan 1

ipv6 enable

ipv6 ospf 1 area 36.0.0.0

!

router ospfv3 1

router-id 2.2.2.2

area 36.0.0.0 stub

!

Related command

area nssa

### area virtual-link

To configure a virtual link, run the first one of the following two commands.

area area-id virtual-link neighbor-ID [dead-interval dead-value][ hello-interval hello-value][ retransmit-interval retrans-value][ transmit-delay dly-value]

no area area-id virtual-link neighbor-ID

Parameter

| Parameter | Description |
| --- | --- |
| area-id | Specifies the transit-area of the virtual link. |
| neighbor-id | OSPF router ID of the peer router of virtual link |
| dead-value | Stands for the interval for the local router to regard that the neighbor dies, whose unit is second. The values configured at the two terminals of the virtual link must be same. |
| hello-value | Stands for the interval for the router to transmit the HELLO packet on the virtual link, whose unit is second. The values configured at the two terminals of the virtual link must be same. |
| retrans-value | Interval for the router to transmit the re-transmit packet on the virtual link, whose unit is second The values configured at the two terminals of the virtual link must be same. |
| dly-value | Delay value which is reported by the router to LSA on the virtual link, whose unit is second The values configured at the two terminals of the virtual link must be same. The values configured at the two terminals of the virtual link must be same. |

Default value

The virtual link is not configured.

The default values of other parameters are shown in the following:

Hello-value:  10s,   Dead-value : 40s, Retrans-value : 5s, dly-value : 1s

Command mode

Routing configuration mode

Instruction

In order to create a virtual link, you have to perform configuration at the two terminals of the virtual link. If only one terminal need be configured, the virtual link cannot function.

The area-id parameter cannot be zero because the transit area of the virtual link must not be the backbone area. The area-id configured at the two terminals of the virtual link must be same.

Parameters configured at the two terminals of the virtual link must be same.

After the virtual link is created (the neighborhood is in the FULL state), the virtual link works in the Demand Circuit mode, that is, the periodical Hello packet and the LSA refresh packet are not transmitted.

You can run no area area-id virtual-link neighbor-ID to cancel the previous configuration of the virtual link.

You also can run show ip ospf virtual-link to check the state of the virtual link.

Example

The following example shows how to create a virtual link between router A and router B.

Configuration on router A (router-id: 200.200.200.1) :

!

interface vlan 1

no ip address

ipv6 enable

ipv6 ospf 1 area 1

!

router ospfv3 1

router-id 200.200.200.1

area 1 virtual-link 200.200.200.2

!

Configuration on router B (router-id: 200.200.200.2) :

!

interface vlan 1

no ip address

ipv6 enable

ipv6 ospf 1 area 1

!

router ospfv3 1

router-id 200.200.200.2

area 1 virtual-link 200.200.200.1

!

Related command

show ipv6 ospf virtual-link

### debug ipv6 ospf

To open all debugging switches of the OSPFv3 module, run the first one of the following two commands:

debug ipv6 ospf

no debug ipv6 ospf

Parameter

None

Default value

None

Command mode

EXEC

Instruction

This command can be used to collect all debugging information about the OSPFv3 for the R&D engineers and technical support staff.

Example

Router# debug ipv6 ospf

LSA[Area(0.0.0.1):Type(0x2009):0.0.0.1:200.200.200.1 *]: Install Intra-Area-Prefix-LSA to Area 0.0.0.1

LSA[Area(0.0.0.1):Type(0x2009):0.0.0.1:200.200.200.1 *]: consider flooding through interface[FastEthernet0/0]

LSA[Area(0.0.0.1):Type(0x2009):0.0.0.1:200.200.200.1 *]: Intra-Area-Prefix-LSA(0x38110c0) originated

VLINK[VLINK1]: local address is 101::1VLINK[VLINK1]: peer 200.200.200.2 link upLSA[Link(FastEthernet0/0):Type(0x0008):0.0.0.4:200.200.200.1 *]: Install Link-LSA to Link FastEthernet0/0

LSA[Link(FastEthernet0/0):Type(0x0008):0.0.0.4:200.200.200.1 *]: consider flooding through interface[FastEthernet0/0]

LSA[Link(FastEthernet0/0):Type(0x0008):0.0.0.4:200.200.200.1 *]: Link-LSA(0x381ec40) originated

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_EVENT.

LSA[Area(0.0.0.1):Type(0x2001):0.0.0.0:200.200.200.1 *]: Install Router-LSA to Area 0.0.0.1

LSA[Area(0.0.0.1):Type(0x2001):0.0.0.0:200.200.200.1 *]: consider flooding through interface[FastEthernet0/0]

LSA[Area(0.0.0.1):Type(0x2001):0.0.0.0:200.200.200.1 *]: Router-LSA(0x381ec20) originated

IFSM[FastEthernet0/0]: Down (InterfaceUp)

IFSM[FastEthernet0/0]: Status change Down -> Waiting

SPF[0.0.0.0]: Calculation timer scheduled [delay 5 secs]

LSA[Area(0.0.0.0):Type(0x2001):0.0.0.0:200.200.200.1 *]: Install Router-LSA to Area 0.0.0.0

LSA[Area(0.0.0.0):Type(0x2001):0.0.0.0:200.200.200.1 *]: consider flooding through interface[VLINK1]

LSA[Area(0.0.0.0):Type(0x2001):0.0.0.0:200.200.200.1 *]: Router-LSA(0x38297e0) originated

IFSM[VLINK1]: Down (InterfaceUp)

IFSM[VLINK1]: Status change Down -> Point-To-Point

ROUTER[1]: Change status to ABR

IFSM[FastEthernet0/0]: Hello timer expire

Packet[SEND]: src(fe80:4::2e0:fff:fe26:2d98) -> dst(ff02::5)

OSPFv3 Header

Version 3  Type 1 (Hello)  Packet length 36

Router ID 200.200.200.1

Area ID 0.0.0.1

Checksum 0x0000  Instance ID 0

OSPFv3 Hello

Interface ID 4

RtrPriority 1  Options 0x000013 (-|R|-|-|E|V6)

HelloInterval 10  RtrDeadInterval 40

DRouter 0.0.0.0  BDRouter 0.0.0.0

# Neighbors 0

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_TIMER.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_EVENT.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_SOCKET.

……

### debug ipv6 ospf events

To enable the event debug switch of the OSPFv3 module, run the first one of the following two commands:

debug ipv6 ospf events {abr|asbr|vlink|os|router}

no debug ipv6 ospf events { abr|asbr|vlink|os|router }

Parameter

| Parameter | Description |
| --- | --- |
| abr | Opens the state change debug switch of ABR. |
| asbr | Opens the state change debug switch of ASBR. |
| vlink | Opens the state change debug switch of the virtual link. |
| os | Opens the state change debug switch of socket. |
| router | Opens the debug switch of OSPF. |

Default value

None

Command mode

EXEC

Instruction

According to the information exported by the command, you can check the OSPF port and the neighbor trigger event.

Example

Router# debug ip ospf events

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_TIMER.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_TIMER.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_EVENT.

ROUTER[1]: Change status to ABR

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_EVENT.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_SOCKET.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_TIMER.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_SOCKET.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_EVENT.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_SOCKET.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_SOCKET.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_SOCKET.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_EVENT.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_SOCKET.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_SOCKET.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_EVENT.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_EVENT.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_TIMER.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_SOCKET.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_TIMER.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_TIMER.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_EVENT.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_SOCKET.

VLINK[VLINK1]: peer 200.200.200.2 link downROUTER[1]: Change status to non-ABR

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_EVENT.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_TIMER.

ROUTER[Process:1]: GC timer expire

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_SOCKET.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_TIMER.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_TIMER.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_TIMER.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_TIMER.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_SOCKET.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_TIMER.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_EVENT.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_TIMER.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_TIMER.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_SOCKET.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_TIMER.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_TIMER.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_TIMER.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_TIMER.

ROUTER[Process:1]: GC timer expire

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_SOCKET.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_SOCKET.

join AllDRouters on FastEthernet0/0OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_EVENT.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_EVENT.

OSPF6D: Received ospfv3 message: OSPFV3_MSG_RCV_EVENT.

……

### debug ipv6 ospf ifsm

To enable the state machine’s debug switch of the OSPFv3 module, run the first one of the following two commands:

debug ipv6 ospf ifsm {status|events|timers}

no debug ipv6 ospf ifsm {status|events|timers}

Parameter

| Parameter | Description |
| --- | --- |
| status | Opens the state debug switch of the interface state machine. |
| events | Opens the event debug switch of the interface state machine. |
| timers | Opens the timer debug switch of the interface state machine. |

Default value

None

Command mode

EXEC

Instruction

According to the information exported by the command, you can check the whole process of the state machine of the OSPF interface.

Example

Router# debug ipv6 ospf ifsm

IFSM[VLINK1]: Down (InterfaceUp)

IFSM[VLINK1]: Status change Down -> Point-To-Point

IFSM[FastEthernet0/0]: Down (InterfaceUp)

IFSM[FastEthernet0/0]: Status change Down -> Waiting

IFSM[FastEthernet0/0]: Hello timer expire

IFSM[VLINK1]: Hello timer expire

IFSM[VLINK1]: ifsm_ignore called

IFSM[VLINK1]: Point-To-Point (NeighborChange)

IFSM[FastEthernet0/0]: ifsm_ignore called

IFSM[FastEthernet0/0]: Waiting (NeighborChange)

IFSM[VLINK1]: LS ack timer expire

IFSM[VLINK1]: LS ack timer expire

IFSM[VLINK1]: Point-To-Point (InterfaceDown)

IFSM[VLINK1]: Status change Point-To-Point -> Down

IFSM[VLINK1]: ifsm_ignore called

IFSM[VLINK1]: Down (NeighborChange)

IFSM[FastEthernet0/0]: Hello timer expire

IFSM[FastEthernet0/0]: Hello timer expire

IFSM[FastEthernet0/0]: Hello timer expire

IFSM[FastEthernet0/0]: Wait timer expire

IFSM[FastEthernet0/0]: DR-Election[1st]: Backup 200.200.200.2

IFSM[FastEthernet0/0]: DR-Election[1st]: DR     200.200.200.2

IFSM[FastEthernet0/0]: Waiting (WaitTimer)

IFSM[FastEthernet0/0]: Status change Waiting -> DROther

IFSM[VLINK1]: Down (InterfaceUp)

IFSM[VLINK1]: Status change Down -> Point-To-Point

IFSM[FastEthernet0/0]: DR-Election[1st]: Backup 200.200.200.1

IFSM[FastEthernet0/0]: DR-Election[1st]: DR     200.200.200.2

IFSM[FastEthernet0/0]: DR-Election[2nd]: Backup 200.200.200.1

IFSM[FastEthernet0/0]: DR-Election[2nd]: DR     200.200.200.2

IFSM[FastEthernet0/0]: DROther (NeighborChange)

IFSM[FastEthernet0/0]: Status change DROther -> Backup

IFSM[FastEthernet0/0]: Hello timer expire

IFSM[FastEthernet0/0]: LS ack timer expire

IFSM[VLINK1]: Hello timer expire

IFSM[FastEthernet0/0]: LS ack timer expire

IFSM[VLINK1]: Point-To-Point (InterfaceDown)

IFSM[VLINK1]: Status change Point-To-Point -> Down

IFSM[VLINK1]: Down (InterfaceUp)

IFSM[VLINK1]: Status change Down -> Point-To-Point

IFSM[FastEthernet0/0]: LS ack timer expire

IFSM[VLINK1]: Hello timer expire

……

### debug ipv6 ospf lsa

To enable the LSA-related debug switch of the OSPFv3 module, run the first one of the following two commands:

debug ipv6 ospf lsa { flooding|install|maxage|refresh}

no debug ipv6 ospf lsa { flooding|install|maxage|refresh}

Parameter

| Parameter | Description |
| --- | --- |
| flooding | Opens the debug switch of LSA exchange. |
| install | Opens the debug switch of LSA installation. |
| maxage | Opens the debug switch of LSA timeout. |
| refresh | Opens the debug switch of LSA-Refresh. |

Default value

None

Command mode

EXEC

Instruction

According to the information exported by the command, you can browse the operation that OSPF performs to LSA and related events.

Example

router# debug ipv6 ospf lsa

LSA[Area(0.0.0.1):Type(0x2009):0.0.0.1:200.200.200.1 *]: Install Intra-Area-Prefix-LSA to Area 0.0.0.1

LSA[Area(0.0.0.1):Type(0x2009):0.0.0.1:200.200.200.1 *]: consider flooding through interface[FastEthernet0/0]

LSA[Area(0.0.0.1):Type(0x2009):0.0.0.1:200.200.200.1 *]: Intra-Area-Prefix-LSA(0x3824ba0) originated

LSA[Area(0.0.0.0):Type(0x2001):0.0.0.0:200.200.200.1 *]: Install Router-LSA to Area 0.0.0.0

LSA[Area(0.0.0.0):Type(0x2001):0.0.0.0:200.200.200.1 *]: consider flooding through interface[VLINK1]

LSA[Area(0.0.0.0):Type(0x2001):0.0.0.0:200.200.200.1 *]: consider flooding to neighbor[200.200.200.2]

LSA[Area(0.0.0.0):Type(0x2001):0.0.0.0:200.200.200.1 *]: Router-LSA(0x3819be0) originated

LSA[Link(FastEthernet0/0):Type(0x0008):0.0.0.4:200.200.200.1 *]: Install Link-LSA to Link FastEthernet0/0

LSA[Link(FastEthernet0/0):Type(0x0008):0.0.0.4:200.200.200.1 *]: consider flooding through interface[FastEthernet0/0]

LSA[Link(FastEthernet0/0):Type(0x0008):0.0.0.4:200.200.200.1 *]: Link-LSA(0x3819bc0) originated

LSA[Area(0.0.0.1):Type(0x2001):0.0.0.0:200.200.200.1 *]: Install Router-LSA to Area 0.0.0.1

LSA[Area(0.0.0.1):Type(0x2001):0.0.0.0:200.200.200.1 *]: consider flooding through interface[FastEthernet0/0]

LSA[Area(0.0.0.1):Type(0x2001):0.0.0.0:200.200.200.1 *]: Router-LSA(0x3824740) originated

LSA[Area(0.0.0.0):Type(0x2001):0.0.0.0:200.200.200.2]: instance(0x380bf60) created with Link State Update

LSA[Area(0.0.0.0):Type(0x2003):0.0.0.1:200.200.200.2]: instance(0x38246c0) created with Link State Update

LSA[Area(0.0.0.0):Type(0x2001):0.0.0.0:200.200.200.2]: flood started

LSA[Area(0.0.0.0):Type(0x2001):0.0.0.0:200.200.200.2]: consider flooding through interface[VLINK1]

LSA[Area(0.0.0.0):Type(0x2001):0.0.0.0:200.200.200.2]: consider flooding to neighbor[200.200.200.2]

LSA[Area(0.0.0.0):Type(0x2001):0.0.0.0:200.200.200.2]: neighbor is not Full state

LSA[Area(0.0.0.0):Type(0x2001):0.0.0.0:200.200.200.2]: Install Router-LSA to Area 0.0.0.0

LSA[Area(0.0.0.0):Type(0x2003):0.0.0.1:200.200.200.2]: flood started

LSA[Area(0.0.0.0):Type(0x2003):0.0.0.1:200.200.200.2]: consider flooding through interface[VLINK1]

LSA[Area(0.0.0.0):Type(0x2003):0.0.0.1:200.200.200.2]: consider flooding to neighbor[200.200.200.2]

LSA[Area(0.0.0.0):Type(0x2003):0.0.0.1:200.200.200.2]: neighbor is not Full state

LSA[Area(0.0.0.0):Type(0x2003):0.0.0.1:200.200.200.2]: Install Inter-Area-Prefix-LSA to Area 0.0.0.0

LSA[Area(0.0.0.0):Type(0x2001):0.0.0.0:200.200.200.1 *]: Install Router-LSA to Area 0.0.0.0

LSA[Area(0.0.0.0):Type(0x2001):0.0.0.0:200.200.200.1 *]: consider flooding through interface[VLINK1]

LSA[Area(0.0.0.0):Type(0x2001):0.0.0.0:200.200.200.1 *]: consider flooding to neighbor[200.200.200.2]

LSA[Area(0.0.0.0):Type(0x2001):0.0.0.0:200.200.200.1 *]: added to neighbor[200.200.200.2]'s retransmit-list

LSA[Area(0.0.0.0):Type(0x2001):0.0.0.0:200.200.200.1 *]: sending update to interface[VLINK1]

LSA[Area(0.0.0.0):Type(0x2001):0.0.0.0:200.200.200.1 *]: Router-LSA refreshed

OSPFv3 LSA Header

LS age 0

LS type 0x2001 (Router-LSA)

Advertising Router 200.200.200.1

Link State ID 0.0.0.0

LS sequence number 0x80000002

LS checksum 0x5ff7

length 40

LSA[Area(0.0.0.0):Type(0x2001):0.0.0.0:200.200.200.1 *]: consider flooding through interface[VLINK1]

LSA[Area(0.0.0.1):Type(0x2001):0.0.0.0:200.200.200.1 *]: Install Router-LSA to Area 0.0.0.1

LSA[Area(0.0.0.1):Type(0x2001):0.0.0.0:200.200.200.1 *]: consider flooding through interface[FastEthernet0/0]

LSA[Area(0.0.0.1):Type(0x2001):0.0.0.0:200.200.200.1 *]: Router-LSA refreshed

OSPFv3 LSA Header

LS age 0

LS type 0x2001 (Router-LSA)

Advertising Router 200.200.200.1

Link State ID 0.0.0.0

LS sequence number 0x80000002

LS checksum 0x5382

length 24

LSA[Area(0.0.0.1):Type(0x2009):0.0.0.1:200.200.200.1 *]: Install Intra-Area-Prefix-LSA to Area 0.0.0.1

LSA[Area(0.0.0.1):Type(0x2009):0.0.0.1:200.200.200.1 *]: consider flooding through interface[FastEthernet0/0]

LSA[Area(0.0.0.1):Type(0x2009):0.0.0.1:200.200.200.1 *]: Intra-Area-Prefix-LSA refreshed

OSPFv3 LSA Header

LS age 0

LS type 0x2009 (Intra-Area-Prefix-LSA)

Advertising Router 200.200.200.1

Link State ID 0.0.0.1

LS sequence number 0x80000002

LS checksum 0x3631

length 64

……

### debug ipv6 ospf nfsm

To enable the state machine’s debug switch of the OSPFv3 neighbor, run the first one of the following two commands:

debug ipv6 ospf nfsm {status|events|timers}

no debug ipv6 ospf nfsm {status|events|timers}

Parameter

| Parameter | Description |
| --- | --- |
| status | Opens the state debug switch of the neighbor state machine. |
| events | Opens the event debug switch of the neighbor state machine. |
| timers | Opens the timer debug switch of the neighbor state machine. |

Default value

None

Command mode

EXEC

Instruction

According to the information exported by the command, you can check the whole process of the OSPF neighbor's state machine.

Example

router# debug ipv6 ospf nfsm

NFSM[200.200.200.2-00000004]: Full (HelloReceived)

NFSM[200.200.200.2-00000004]: nfsm_ignore calledNFSM[200.200.200.2-00000004]: Full (2-WayReceived)

NFSM[200.200.200.2-00000004]: Down (HelloReceived)

NFSM[200.200.200.2-00000004]: Status change Down -> Init

NFSM[200.200.200.2-00000004]: nfsm_ignore called

NFSM[200.200.200.2-00000004]: Init (1-WayReceived)

NFSM[200.200.200.2-00000004]: Init (HelloReceived)

NFSM[200.200.200.2-00000004]: Init (2-WayReceived)

NFSM[200.200.200.2-00000004]: Status change Init -> 2-Way

NFSM[200.200.200.2-00000004]: 2-Way (HelloReceived)

NFSM[200.200.200.2-00000004]: nfsm_ignore called

NFSM[200.200.200.2-00000004]: 2-Way (2-WayReceived)

NFSM[200.200.200.2-00000004]: 2-Way (AdjOK?)

NFSM[200.200.200.2-00000004]: Status change 2-Way -> ExStar

tNFSM[200.200.200.2-00000004]: ExStart (HelloReceived)

NFSM[200.200.200.2-00000004]: nfsm_ignore called

NFSM[200.200.200.2-00000004]: ExStart (2-WayReceived)

NFSM[200.200.200.2-00000004]: DD Retransmit timer expire

NFSM[200.200.200.2-00000004]: ExStart (NegotiationDone)

NFSM[200.200.200.2-00000004]: Status change ExStart -> Exchange

NFSM[200.200.200.2-00000004]: Exchange (ExchangeDone)

NFSM[200.200.200.2-00000004]: Status change Exchange -> Loading

NFSM[200.200.200.2-00000004]: nfsm_ignore called

NFSM[200.200.200.2-00000004]: Loading (LoadingDone)

NFSM[200.200.200.2-00000004]: Status change Loading -> Full

NFSM[200.200.200.2-80000001]: Down (HelloReceived)

NFSM[200.200.200.2-80000001]: Status change Down -> Init

NFSM[200.200.200.2-80000001]: Init (2-WayReceived)

NFSM[200.200.200.2-80000001]: Status change Init -> ExStart

NFSM[200.200.200.2-80000001]: ExStart (NegotiationDone)

NFSM[200.200.200.2-80000001]: Status change ExStart -> Exchange

NFSM[200.200.200.2-80000001]: Exchange (ExchangeDone)

NFSM[200.200.200.2-80000001]: Status change Exchange -> Loading

NFSM[200.200.200.2-80000001]: nfsm_ignore called

NFSM[200.200.200.2-80000001]: Loading (LoadingDone)

NFSM[200.200.200.2-80000001]: Status change Loading -> Full

NFSM[200.200.200.2-00000004]: Full (HelloReceived)

NFSM[200.200.200.2-00000004]: nfsm_ignore called

NFSM[200.200.200.2-00000004]: Full (2-WayReceived)

NFSM[200.200.200.2-00000004]: Full (AdjOK?)

NFSM[200.200.200.2-00000004]: LS update timer expire

NFSM[200.200.200.2-80000001]: LS update timer expire

NFSM[200.200.200.2-00000004]: LS update timer expire

NFSM[200.200.200.2-80000001]: LS update timer expire

NFSM[200.200.200.2-80000001]: Full (HelloReceived)

NFSM[200.200.200.2-80000001]: nfsm_ignore called

NFSM[200.200.200.2-80000001]: Full (2-WayReceived)

NFSM[200.200.200.2-00000004]: Full (HelloReceived)

NFSM[200.200.200.2-00000004]: nfsm_ignore called

NFSM[200.200.200.2-00000004]: Full (2-WayReceived)

NFSM[200.200.200.2-00000004]: LS update timer expire

NFSM[200.200.200.2-80000001]: LS update timer expire

……

### debug ipv6 ospf nsm

To open the debug switch of information transmission between the IPv6 routing table’s management module and the OSPFv3 module, run the first one of the following two commands:

debug ipv6 ospf nsm { redistribute | interface }

no debug ipv6 ospf nsm { redistribute | interface }

Parameter

| Parameter | Description |
| --- | --- |
| redistribute | Opens the debug switch of routing information forwarding. |
| interface | Opens the debug switch of interface events. |

Default value

None

Command mode

EXEC

Instruction

According to the information exported by this command, you can browse information exchange between OSPF and routing management module.

Example

router# debug ipv6 ospf nsm

Sep 17 16:43:53 OSPFv3: Received [NSM_MSG_ROUTE_CHG_NOTIFY:NSM_REDISTRIBUTE_DEL] message

Sep 17 16:43:53 OSPFv3: Received [NSM_MSG_ROUTE_CHG_NOTIFY:NSM_REDISTRIBUTE_DEL] message

Sep 17 16:43:53 OSPFv3: Received [NSM_MSG_ROUTE_CHG_NOTIFY:NSM_REDISTRIBUTE_DEL] message

Sep 17 16:43:53 OSPFv3: Received [NSM_MSG_ROUTE_CHG_NOTIFY:NSM_REDISTRIBUTE_DEL] message

Sep 17 16:43:53 OSPFv3: Receive [NSM_MSG_GLBL_ENAIPV6] message

……

### debug ipv6 ospf packet

To enable the debug switch of OSPFv3 transmission and reception, run the first one of the following two commands:

debug ipv6 ospf packet { hello|dd|ls-request|ls-update|ls-ack }

no debug ipv6 ospf packet { hello|dd|ls-request|ls-update|ls-ack }

Parameter

| Parameter | Description |
| --- | --- |
| hello | Opens the debug switch of Hello packets. |
| dd | Opens the debug switch of DD packets. |
| ls-request | Opens the debug switch of IS-REQUEST packets. |
| ls-update | Opens the debug switch of IS-Update packets. |
| ls-ack | Opens the debug switch of IS-Ack packets. |
| detail | Observes the details of packets. |

Default value

None

Command mode

EXEC

Instruction

According to the information exported by the command, you can check the exchange of the OSPF packets.

Example

router# debug ipv6 ospf packet

Packet[SEND]: src(fe80:4::2e0:fff:fe26:2d98) -> dst(ff02::5)

OSPFv3 Header

Version 3  Type 1 (Hello)  Packet length 40

Router ID 200.200.200.1

Area ID 0.0.0.1

Checksum 0x0000  Instance ID 0

OSPFv3 Hello

Interface ID 4

RtrPriority 1  Options 0x000013 (-|R|-|-|E|V6)

HelloInterval 10  RtrDeadInterval 40

DRouter 200.200.200.2  BDRouter 200.200.200.1

# Neighbors 1

Neighbor 200.200.200.2

Packet[RECV]: src(101::2) -> dst(101::1)

OSPFv3 Header

Version 3  Type 1 (Hello)  Packet length 40

Router ID 200.200.200.2

Area ID 0.0.0.0

Checksum 0x5774  Instance ID 0

OSPFv3 Hello

Interface ID 2147483649

RtrPriority 1  Options 0x000013 (-|R|-|-|E|V6)

HelloInterval 10  RtrDeadInterval 40

DRouter 0.0.0.0  BDRouter 0.0.0.0

# Neighbors 1

Neighbor 200.200.200.1

RECV[Hello]: Neighbor(200.200.200.2) declare 0.0.0.0 as DR, 0.0.0.0 as Backup

Packet[SEND]: src(101::1) -> dst(101::2)

OSPFv3 Header

Version 3  Type 1 (Hello)  Packet length 40

Router ID 200.200.200.1

Area ID 0.0.0.0

Checksum 0x0000  Instance ID 0

OSPFv3 Hello

Interface ID 2147483649

RtrPriority 1  Options 0x000013 (-|R|-|-|E|V6)

HelloInterval 10  RtrDeadInterval 40

DRouter 0.0.0.0  BDRouter 0.0.0.0

# Neighbors 1

Neighbor 200.200.200.2

Packet[RECV]: src(fe80::2e0:fff:fe26:a8) -> dst(ff02::5)

OSPFv3 Header

Version 3  Type 1 (Hello)  Packet length 40

Router ID 200.200.200.2

Area ID 0.0.0.1

Checksum 0xa8a8  Instance ID 0

OSPFv3 Hello

Interface ID 4

RtrPriority 1  Options 0x000013 (-|R|-|-|E|V6)

HelloInterval 10  RtrDeadInterval 40

DRouter 200.200.200.2  BDRouter 200.200.200.1

# Neighbors 1

Neighbor 200.200.200.1

RECV[Hello]: Neighbor(200.200.200.2) declare 200.200.200.2 as DR, 200.200.200.1 as Backup

Packet[SEND]: src(fe80:4::2e0:fff:fe26:2d98) -> dst(ff02::5)

OSPFv3 Header

Version 3  Type 1 (Hello)  Packet length 40

Router ID 200.200.200.1

Area ID 0.0.0.1

Checksum 0x0000  Instance ID 0

OSPFv3 Hello

Interface ID 4

RtrPriority 1  Options 0x000013 (-|R|-|-|E|V6)

HelloInterval 10  RtrDeadInterval 40

DRouter 200.200.200.2  BDRouter 200.200.200.1

# Neighbors 1

Neighbor 200.200.200.2

……

### debug ipv6 ospf route

To enable the debug switch of OSPFv3 routing information, run the first one of the following two commands:

debug ipv6 ospf route { ase|install|spf|ia }

no debug ipv6 ospf route { ase|install|spf|ia }

Parameter

| Parameter | Description |
| --- | --- |
| ase | Opens the debug switch of exterior routing calculation. |
| install | Opens the debug switch of routing installation procedure. |
| spf | Opens the debug switch of SPF calculation. |
| ia | Opens the debug switch of between-domain routing calculation. |

Default value

None

Command mode

EXEC

Instruction

According to the information exported by the command, you can browse the calculation, deletion and addition of OSPF routes.

Example

router# debug ipv6 ospf route

Route[IA:0.0.0.0]: No SPF tree, schedule SPF calculationSPF[0.0.0.1]: SPF calculation timer expire

SPF[0.0.0.1]: SPF calculation (1st STAGE)

SPF[0.0.0.1]: Vertex[200.200.200.1-0.0.0.0]

SPF[0.0.0.1]: SPF calculation (2nd STAGE)

SPF[0.0.0.1]: SPF calculation (END)

Route[IA:0.0.0.1]: Cleanup IA route because of no ABRsRoute[IA:0.0.0.1]: Cleanup IA route because of no ABRsSPF[0.0.0.1]: Calculation completed [0.170000 sec]

SPF[0.0.0.1]: Calculation timer scheduled [delay 9 secs]

SPF[0.0.0.1]: SPF calculation timer expire

SPF[0.0.0.1]: SPF calculation (1st STAGE)

SPF[0.0.0.1]: Vertex[200.200.200.1-0.0.0.0]

SPF[0.0.0.1]: SPF calculation (2nd STAGE)

SPF[0.0.0.1]: SPF calculation (END)

Route[IA:0.0.0.1]: Cleanup IA route because of no ABRsSPF[0.0.0.1]: Calculation completed [0.180000 sec]

SPF[0.0.0.1]: Calculation timer scheduled [delay 10 secs]

SPF[0.0.0.0]: Calculation timer scheduled [delay 5 secs]

Route[IA:0.0.0.1]: 888::/64 calculating Network routeRoute[IA:0.0.0.1]: 888::/64 Can't find route to ABR (200.200.200.2)Route[IA:0.0.0.0]: No SPF tree, schedule SPF calculationSPF[0.0.0.0]: SPF calculation timer expire

SPF[0.0.0.0]: SPF calculation (1st STAGE)

SPF[0.0.0.0]: Vertex[200.200.200.1-0.0.0.0]

SPF[0.0.0.0]:   Link[0] (200.200.200.2-128.0.0.1): Virtual-Link

SPF[0.0.0.0]:     Calculate nexthop for (200.200.200.2-0.0.0.0)

Route[0.0.0.0:SPF]: ADD Stub Route for (200.200.200.2)SPF[0.0.0.0]: Vertex[200.200.200.2-0.0.0.0]

SPF[0.0.0.0]:   Link[0] (200.200.200.1-128.0.0.1): Virtual-Link

SPF[0.0.0.0]:    LSA[Area(0.0.0.0):Type(0x2001):0.0.0.0:200.200.200.1 *] is already in SPF tree

SPF[0.0.0.0]: SPF calculation (2nd STAGE)

SPF[0.0.0.0]: SPF calculation (END)

SPF[0.0.0.0]: Calculation completed [0.580000 sec]

……

### default-information originate

To introduce the default route to the OSPFv3 routing domain, run the following command:

default-information originate [ always | metric value | metric-type {1 | 2} | tag tag]

no default-information originate

Parameter

| Parameter | Description |
| --- | --- |
| Always | Generates and releases an ASE-LSA, which describes the default route, or just releases it out if the default route exists in the routing table. |
| metric value | Stands for the cost of the default route, which ranges from 1 to 16777214. |
| metric-type | Means the cost type of the default route. |
| tag tag | Means the routing identifier, which ranges from 0 to 4294967295. |

Default value

No default route is introduced.

Command mode

Routing configuration mode

Instruction

The redistribute command cannot introduce the default route, and if you want to introduce the default route, you can use this command.

If the always parameter is set, no matter whether the default route exists in the current routing table, ASE-LSA, describing the default route, will be released out; if the always parameter is not set, ASE-LSA will be released out only when the default route exists in the current routing table.

Example

The following example shows how to introduce the default route from the OSPFv3 autonomous system.

router ospfv3 1

router-id 2.2.2.2

default-information originate always

Related command

redistribute

### default-metric

To set the default weight of the introduced route, run the first one of the following two commands:

default-metric value

no default-metric

Parameter

| Parameter | Description |
| --- | --- |
| value | Stands for the to-be-set route weight, ranging between 1 and 16777214. |

Default value

The default route weight is 10.

Command mode

Routing configuration mode

Instruction

The default-metric command is used to set the default routing weight when the route of other routing protocol is guided into the OSPF packet.  When the redistribute command is used to guide the route of other routing protocol, the default routing weight designated by the default-metric command will be guided the specific routing weight will not be specified.

Example

The following example shows how to introduce the static route and set the default route weight of other routing protocol to 3:

interface vlan 1

ipv6 enable

ipv6 ospf 1 area 36.0.0.0

!

router ospfv3 1

router-id 2.2.2.2

default-metric 3

redistribute static

Related command

redistribute

### filter

To set the routing filtration table, run filter {interface-type interface-number | *} {in | out } {access-list access-list-name | gateway access-list-name | prefix-list prefix-list-name}. To resume the default settings, run no filter {interface-type interface-number | *} {in | out } {access-list access-list-name | gateway access-list-name | prefix-list prefix-list-name}.

filter {interface-type interface-number | *} {in | out } {access-list access-list-name | gateway access-list-name | prefix-list prefix-list-name}

no filter {interface-type interface-number | *} {in | out} {access-list access-list-name | gateway access-list-name | prefix-list prefix-list-name}

Parameter

| interface-type | Interface type |
| --- | --- |
| interface-number | Interface number |
| * | All interfaces |
| In | Filtrates the received OSPF routes. |
| out | Filters the transmitted routes, which is not for a specific interface but for all interfaces. |
| access-list-name | Name of the IP access control list |
| access-list-name | Name of the IP access control list |
| prefix-list-name | Name of the prefix list |

Default value

None

Command mode

Routing configuration mode

Example

The following example shows how to filter the received routes according to the mylist ACL.

router ospfv3 1

filter * in access-list mylist

Related command

None

### ipv6 ospf area

To enable the OSPFv3 protocol on an interface and specify an area for this interface, run the first one of the following two commands:

ipv6 ospf process-id area area-id [instance instance-id]

no ipv6 ospf process-id area area-id [instance instance-id]

Parameter

| Parameter | Description |
| --- | --- |
| process-id | Stands for the OSPF process. |
| area-id | Stands for the OSPF area ID, which is specified by the interface. |
| instance-id | Specifies the OSPF instance ID of the interface, whose default value is 0. |

Default value

None

Command mode

Interface configuration mode

Example

The following example shows how to enable OSPFv3 process 1 for interface vlan1 and set its area ID to 0.

interface vlan 1

ipv6 enable

ipv6 ospf 1 area 0

!

router ospfv3 1

router-id 2.2.2.2

Related command

None

### ipv6 ospf cost

To designate the cost for the OSPFv3 protocol running on the interface, run ipv6 ospf cost cost. To resume the default settings, run no ipv6 ospf cost.

ipv6 ospf cost cost [instance instance-id]

no ipv6 ospf cost cost [instance instance-id]

Parameter

| Parameter | Description |
| --- | --- |
| cost | Cost for the OSPF protocol, which is an integer between 1 and 65535 |
| instance-id | Specifies the OSPF instance ID of the interface, whose default value is 0. |

Default value

The default cost for the OSPFv3 protocol running on the interface is obtained based on the rate of the port.

Command mode

Interface configuration mode

Example

The following example shows how to set the cost for the OSPFv3 protocol running on interface vlan1 to 2:

interface vlan 1

ipv6 ospf cost 2

Related command

None

### ipv6 ospf database-filter all out

To designate an interface to filter those to-be-transmitted LSA, run the first one of the following two commands:

ipv6 ospf database-filter all out [instance instance-id]

no ipv6 ospf database-filter all out [instance instance-id]

Parameter

| Parameter | Description |
| --- | --- |
| instance-id | Specifies the OSPF instance ID of the interface, whose default value is 0. |

Default value

The interface does not filter those to-be-transmitted LSAs.

Command mode

Interface configuration mode

Example

The following example shows how to set interface vlan 1 to filter those to-be-transmitted LSAs:

interface vlan 1

ipv6 ospf database-filter all out

Related command

None

### ipv6 ospf dead-interval

To designate the dead interval of the neighboring router, run ipv6 ospf dead-interval seconds. To resume the default value, run ipv6 ospf dead-interval.

ipv6 ospf dead-interval seconds [instance instance-id]

ipv6 ospf dead-interval seconds [instance instance-id]

Parameter

| Parameter | Description |
| --- | --- |
| seconds | Value of the dead interval for the neighboring router, which ranges from 1 to 2147483647 seconds |
| instance-id | Specifies the OSPF instance ID of the interface, whose default value is 0. |

Default value

The dead interval for the neighboring router is four times of hello-interval by default.

Command mode

Interface configuration mode

Instruction

The value of the dead-interval parameter will be written to the HELLO packet and will be transmitted along with the HELLO packet. It must be ensured that the dead-interval parameter must be identical with that between the neighboring routers and the value of the dead-interval parameter must be four times of the value of the hello-interval parameter.

Example

The following example shows how to set the dead interval of the neighboring router on interface vlan1 to 60 seconds.

interface vlan 1

ipv6 ospf dead-interval 60

Related command

None

### ipv6 ospf hello-interval

To designate the interval for transmitting the HELLO packet on the interface, run ipv6 ospf hello-interval seconds. To resume the default settings, run no ipv6 ospf hello-interval.

ipv6 ospf hello-interval seconds [instance instance-id]

no ipv6 ospf hello-interval seconds [instance instance-id]

Parameter

| Parameter | Description |
| --- | --- |
| seconds | Transmission interval of the HELLO packet, ranging from 1 to 65535 seconds |
| instance-id | Specifies the OSPF instance ID of the interface, whose default value is 0. |

Default value

The interval for the P2P or Broadcast interface to send Hello packets is 10 seconds, while the interval for the P2MP or NBMA interface to send Hello packets is 30 seconds.

Command mode

Interface configuration mode

Instruction

The value of the dead-interval parameter will be written to the HELLO packet and will be transmitted along with the HELLO packet. The smaller the hello-interval is, the sooner the change of the network topology will be found. However, much more path cost will be paid. It must be ensured that the parameter must be identical with that between the neighboring routers.

Example

The following example shows that the interval for transmitting the HELLO packet on interface vlan1 is set to 20 seconds.

interface vlan 1

ipv6 ospf hello-interval 20

Related command

ipv6 ospf dead-interval

### ipv6 ospf mtu-ignore

To set the MTU value of the transmitted DD packet to 0 on an interface and meanwhile omit the checkup of the MTU domain of the received DD packet, run the first one of the following two commands:

ipv6 ospf mtu-ignore [instance instance-id]

no ipv6 ospf mtu-ignore [instance instance-id]

Parameter

| Parameter | Description |
| --- | --- |
| instance-id | Specifies the OSPF instance ID of the interface, whose default value is 0. |

Default value

The value of the MTU domain of the DD packet is set to be the MTU value of this interface and MTU checkup is not omitted.

Command mode

Interface configuration mode

Instruction

OSPF judges whether the MTU value of the network segment, where an interface belongs, is consistent by checking the MTU value of the exchanged DD packet.If the MTU value of the received DD packet is bigger than the MTU value of this interface, the OSPF neighborhood cannot be set up.

Example

The following example shows how to set interface vlan1 to omit MTU checkup.

interface vlan 1

ipv6 ospf mtu-ignore

Related command

None

### ipv6 ospf neighbor

To set the OSPF neighbor on the non-broadcast network interface, run the first one of the following two commands:

ipv6 ospf neighbor router-id ipv6-address [cost number] [database-filter all out] [poll-interval seconds] [priority number] [instance instance-id]

no ipv6 ospf neighbor router-id ipv6-address [cost number] [database-filter all out] [poll-interval seconds] [priority number] [instance instance-id]

Parameter

| Parameter | Description |
| --- | --- |
| router-id | Means the router ID of a neighbor. |
| ipv6-address | Means the local address of the neighbor's link. |
| cost number | Means the neighbor’s cost, whose value ranges from 1 to 65535. |
| database-filter all out | Filters the transmitted LSAs. |
| poll-interval seconds | Means the query interval of a neighbor. |
| priority number | Means the neighbor’s priority, whose value ranges from 0 to 255. |
| instance instance-id | Specifies the OSPF instance ID of the interface, whose default value is 0. |

Default value

No neighbors are set.

Command mode

Interface configuration mode

Instruction

You shall specify a neighbor manually on the non-broadcast network. If neighbors invalidates, the Hello packet should be sent to this neighbor in poll interval.

Example

The following example shows how to set the neighbor of interface vlan1.

interface vlan 1

ipv6 ospf neighbor 1.1.1.1 fe80::1

Related command

None

### ipv6 ospf network

To set the network type for the interface, run the first one of the following two commands.

ipv6 ospf network { broadcast | non-broadcast | point_to_multipoint | point-to-point} [instance instance-id]

no ip ospf network { broadcast | nonbroadcast | point_to_multipoint | point-to-point} [instance instance-id]

Parameter

| Parameter | Description |
| --- | --- |
| broadcast | Sets the network type of the interface to broadcast. |
| nonbroadcast | Sets the network type of the interface to NBMA. |
| point-to-multipoint | Sets the network type of the interface to point-to-multipoint. |
| point-to-point | Sets the network type of the interface to point-to-point. |
| instance instance-id | Specifies the OSPF instance ID of the interface, whose default value is 0. |

Command mode

Interface configuration mode

Instruction

The interface in the broadcast network without multi-address access should be set to NBMA. In the NBMA network, the network should be set to point-to-multipoint if any two routers cannot be ensured to be directly reachable.

Example

The following example shows how to set interface vlan1 to NBMA.

interface vlan 1

ipv6 ospf network non-broadcast

Related command

None

### ipv6 ospf priority

To configure the priority for the interface to choose the router, run ipv6 ospf priority priority. To resume the default value, run no ipv6 ospf priority.

ipv6 ospf priority priority [instance instance-id]

no ipv6 ospf priority [instance instance-id]

Parameter

| Parameter | Description |
| --- | --- |
| priority | Priority to choose the router, ranging between 0 and 255 |
| instance-id | Specifies the OSPF instance ID of the interface, whose default value is 0. |

Default value

The default priority for the interface to choose the routers is 1.

Command mode

Interface configuration mode

Instruction

When two routers in the same network segment want to be the selection router, the router with higher priority will be selected. If the priority of the two routers is the same, the router with a larger ID is selected. When the priority of a router is 0, the router cannot be selected as the designated router or the standby designated router. The priority is effective only on the networks except the nonpoint-to-point network.

Example

The following example shows how to set the priority to 8 when interface vlan1 selects the selection router.

interface vlan 1

ipv6 ospf priority 8

Related command

None

### ipv6 ospf retransmit-interval

To designate the retransmission interval for transmitting LSA between the interface and the neighboring router, run ipv6 ospf retransmit-interval seconds. To resume the default value, run no ipv6 ospf retransmit-interval.

ipv6 ospf retransmit-interval seconds [instance instance-id]

no ipv6 ospf retransmit-interval seconds [instance instance-id]

Parameter

| Parameter | Description |
| --- | --- |
| seconds | Transmission interval for transmitting the link state broadcast between the interface and the neighboring router, ranging between 1 and 3600 seconds |
| instance-id | Specifies the OSPF instance ID of the interface, whose default value is 0. |

Default value

The default interval for transmitting the link state broadcast between the interface and the neighboring router is 5 seconds.

Command mode

Interface configuration mode

Instruction

When a router transmits the link-state broadcast to its neighbor, the command will maintain the link-state broadcast until the peer receives the acknowledgement. If the link-state broadcast is not received during the transmission interval, it will be retransmitted. The value of the seconds parameter must be larger than the round-trip time for a packet transmitting between two routers.

Example

The following example shows how the default interval for transmitting the link-state broadcast between interface vlan1 and the neighboring router is set to 8 seconds.

interface vlan 1

ipv6 ospf retransmit-interval 8

Related command

None

### ipv6 ospf transmit-delay

To set the delay for the link-state broadcast to be transmitted on the interface, run ipv6 ospf transit-delay time. To resume the default value, run no ipv6 ospf transit-delay.

ipv6 ospf transit-delay time [instance instance-id]

no ipv6 ospf transit-delay time [instance instance-id]

Parameter

| Parameter | Description |
| --- | --- |
| time | Means the delay of link state broadcast transmission on an interface, which ranges from 1 to 3600 seconds. |
| instance-id | Specifies the OSPF instance ID of the interface, whose default value is 0. |

Default value

The default delay for the link-state broadcast to be transmitted on the interface is 1 second.

Command mode

Interface configuration mode

Example

The following example shows how to set the delay for transmitting the link-state broadcast on interface vlan1 to 3 seconds.

interface vlan 1

Ipv6 ospf transit-delay 3

Related command

None

### passive-interface

To forbid a designated interface to transmit or receive the OSPFv3 packets, run the first one of the following two commands:

passive-interface { interface-type interface-number | all }

no passive -interface { interface-type interface-number | all }

Parameter

| Parameter | Description |
| --- | --- |
| interface-type interface-number | Means the interface type and the interface ID. |
| all | Means all interfaces. |

Default value

The interface is allowed to transmit or receive the OSPFv3 packets.

Command mode

Routing configuration mode

Instruction

Different processes can forbid a same interface to transmit or receive the OSPFv3 packets, but the passive-interface command takes effect only on the OSPFv3 interface.

Example

The following example shows how to forbid interface vlan1 to receive and transmit the OSPFv3 packets in the OSPFv3 process.

router ospfv3 100

passive –interface vlan 1

Related command

None

### redistribute

To configure the route where OSPF forwards other routing protocols, run redistribute. To resume the default settings, run no redistribute.

redistribute protocol [as-number]  [route-map map-tag]

no redistribute protocol [as-number]  [route-map map-tag]

Parameter

| Parameter | Description |
| --- | --- |
| protocol | Means to forward the learned original protocol. |
| as_number | Means the number of the autonomous system which is not for the connect, rip or static parameter. |
| map-tag | Means the name of the route map. |

Default value

Not forward

Command mode

Routing configuration mode

Instruction

None

Example

The following example shows how to forward the static route in OSPF process 1:

interface vlan 1

ipv6 enable

ipv6 ospf 1 area 0

!

router ospfv3 1

router-id 2.2.2.2

redistribute static

### router ospfv3

To enable OSPFv3 and enter the OSPFv3 configuration view, run the first one of the following two commands:

router ospfv3 process-id

no router ospfv3 process-id

Parameter

| Parameter | Description |
| --- | --- |
| process-id | Identifies the OSPF process. It is a positive integer distributed by the local router. |

Default value

None

Command mode

Global configuration mode

Instruction

One router may have multiple OSPFv3 processes.

Example

The following example shows how to set an OSPFv3 process, whose process ID is 109:

router ospfv3 109

Related command

ipv6 ospf area

### router-id

To set the router ID in the autonomous system for the router on which the OSPFv3 protocol is running, run the first one of the following two commands:

router-id router-id

no router-id router-id

Parameter

| Parameter | Description |
| --- | --- |
| router-id | Means the identifier of the router, which is in the IPv4 address format. |

Default value

If an IPv4 address has already configured on a router before OSPFv3 is enabled, the router will automatically choose an IPv4 address as its ID.

Command mode

Routing configuration mode

Instruction

The router ID is the unique identifier of a OSPFv3-running router in the autonomous system, so the router IDs of two routers in the autonomous system are different. If a router has no router ID, the OSPFv3 process cannot go on.

Example

The following example shows how to set the router ID of OSPFv3 process 1 to 2.2.2.2:

router ospfv3 1

router-id 2.2.2.2

### show ipv6 ospf

To display the main OSPFv3 information, run the following command:

show ipv6 ospf [process-id]

Parameter

| Parameter | Description |
| --- | --- |
| process-id | Means the OSPF process ID. |

Default value

None

Command mode

EXEC

Instruction

The information exported by the command can help checking the OSPFv3 faults. If the process-id parameter follows the command, the information about the global configuration of the OSPFv3 process is displayed.

Example

The following example shows that the configuration information about all OSPFv3 processes will be displayed.

router# show ipv6 ospf

Routing Process "OSPFv3 0" with ID 1.2.3.4

SPF schedule delay 5 secs, Hold time between SPFs 10 secs Minimum LSA interval 5 secs,

Minimum LSA arrival 1 secs Number of external LSA 3. Checksum Sum 0x2CD6F

Number of areas in this router is 1

Area BACKBONE(0)

Number of interfaces in this area is 1

SPF algorithm executed 3 times

Number of LSA 4. Checksum Sum 0x2A6AC

router#

Relative fields are explained in the following table:

| Domain | Description |
| --- | --- |
| Routing Process "OSPFv3 0" | ID of the process |
| with ID 1.2.3.4 | ID of the router |
| SPF schedule delay 5 secs, Hold time between two SPFs 10 secs | Two timer values relative with OSPF |
| Number of areas is 1 | Number of the currently-configured fields, and parameters configured in each field |
| Number of LSA 4 | Quantity of LSAs in the database |
| Number of external LSA 3 | Quantity of ASE LSAs in the database |
| SPF algorithm executed 3 times | SPF algorithm execution statistics |

### show ipv6 ospf database

To display the database information about the OSPFv3 connection state, run the following command:

show ipv6 ospf database { router | network | inter-prefix | inter-router | external | link | intra-prefix } [ ADVROUTER ]

Parameter

| Parameter | Description |
| --- | --- |
| router | Means the LSA type is the router. |
| network | Means the LSA type is the network. |
| inter-prefix | Means the LSA type is the inter-domain route. |
| inter-router | Means the LSA type is the inter-domain router. |
| external | Means the LSA type is the exterior route. |
| link | Means the LSA type is the link. |
| intra-prefix | Means the LSA type is the inside-domain route. |
| ADVROUTER | Means to declare the router ID. |

Default value

None

Command mode

EXEC

Instruction

The information exported by the command can help to check the database information about the OSPFv3 connection state and to find the reason of the faults.

Example

router#

router#show ipv6 ospf database

Link-LSA (Interface eth0)

Link State ID ADV Router Age Seq# CkSum Prefix

0.0.0.3 1.2.3.4 104 0x80000004 0x889e 0

0.0.0.5 5.6.7.8 142 0x80000003 0xab70 2

Router-LSA (Area 0.0.0.0)

Link State ID ADV Router Age Seq# CkSum Link

0.0.0.1 1.2.3.4 94 0x80000014 0xeaea 1

0.0.0.1 5.6.7.8 105 0x80000019 0x8a32 1

Network-LSA (Area 0.0.0.0)

Link State ID ADV Router Age Seq# CkSum

0.0.0.5 5.6.7.8 105 0x80000001 0xa441

Intra-Area-Prefix-LSA (Area 0.0.0.0)

Link State ID ADV Router Age Seq# CkSum Prefix Reference

0.0.0.1 5.6.7.8 104 0x80000001 0x8d4f 2 Network-LSA

AS-external-LSA

Link State ID ADV Router Age Seq# CkSum

0.0.0.1 5.6.7.8 1229 0x80000002 0xe92d

0.0.0.2 5.6.7.8 1229 0x80000002 0xef25

0.0.0.3 5.6.7.8 1229 0x80000002 0xf51d

router#

Relative fields are explained in the following table:

| Domain | Description |
| --- | --- |
| AREA: 1 | Current area |
| Router Link States/Net Link States/Summary Net Link States | LSA type |
| Link ID | LSA ID |
| ADV Router | Releases the router. |
| Age | Releases the age. |
| Seq # | Generates the sequence ID. |
| Checksum | Means the checksum. |

### show ipv6 ospf interface

To display the information about the OSPFv3 interface, run the following command:

show ipv6 ospf interface [ type ] [ index ]

Parameter

| Parameter | Description |
| --- | --- |
| type | Port type |
| index | Port number |

Default value

None

Command mode

EXEC

Instruction

According to the information displayed by the command, you can check the OSPFv3 configuration and its running state, which helps you to detect the OSPFv3 faults.

Example

router#show ipv6 ospf interface

ethernet0/1 is up, line protocol is up

Interface ID 3, Instance ID 0, Area 0.0.0.0

IPv6 Link-Local Address fe80::248:54ff:fec0:f32d/10

Router ID 1.2.3.4, Network Type BROADCAST, Cost: 10

Transmit Delay is 1 sec, State Backup, Priority 1

Designated Router (ID) 5.6.7.8

Interface Address fe80::203:47ff:fe4c:776e

Backup Designated Router (ID) 1.2.3.4

Interface Address fe80::248:54ff:fec0:f32d

Timer interval configured, Hello 10, Dead 40, Wait 40, Retransmit 5

Hello due in 00:00:01

Neighbor Count is 1, Adjacent neighbor count is 1

router#

Relative fields are explained in the following table:

| Domain | Description |
| --- | --- |
| IPv6 Link-Local Address | Address of port IPv6 link-local |
| Nettype | Network type of the OSPF interface |
| OSPF process is | ID of the OSPF process |
| AREA | Current area |
| Router ID | ID of the router where the process belongs |
| Cost | Cost of the OSPF interface of the router |
| Transmit Delay is | Transmission delay |
| Priority | Priority for the interface of the router |
| Hello interval | Transmission interval of the Hello packet |
| Dead timer | Dead time |
| Retransmit | Retransmission interval |
| OSPF INTF State is | State of the OSPF port |
| Designated Router id | ID of the designated router and the IP address of its port |
| Backup Designated router id | ID of the backup designated router and the IP address of its port |
| Neighbor Count is | Number of the neighboring routers |
| Adjacent neighbor count is | Number of neighbors that have established the neighborhood relation |
| Adjacent with neighbor | Neighbor lists that have established the neighborhood relation |

### show ipv6 ospf neighbor

To display the information about OSPFv3 neighbor, run the following command.

show ipv6 ospf neighbor [interface_type interface_number | router-id | detail]

Parameter

| Parameter | Description |
| --- | --- |
| interface_type | Port type |
| interface_number | Port number |
| router-id | Router ID |
| detail | Displays the detailed information. |

Default value

None

Command mode

EXEC

Instruction

The information displayed by the command can help you to check whether the OSPFv3 neighbor configuration is right and to detect the OSPFv3 faults.

Example

router#show ipv6 ospf neighbor

OSPFv3 Process 1

Area 1

Neighbor ID  Pri  State  Dead Time  Interface  Instance ID

5.6.7.8 1         Full/DR 00:00:38       eth0        0

Relative fields are explained in the following table:

| Domain | Description |
| --- | --- |
| OSPFv3 process | ID of the OSPF process |
| AREA | Local area |
| Neighbor | ID of a neighbor |
| Pri | Priority of a neighbor |
| State | Connection state related with the neighbor |
| DeadTime | Time of neighbor invalidation |
| Address | IP address of the neighbor |
| Interface | Interface used by a router to reach its neighbor |

### show ipv6 ospf route

To display the information about the OSPFv3 routing table, run the following command:

show ipv6 ospf route

Parameter

None

Default value

None

Command mode

EXEC

Instruction

The information displayed by the command can help you browse the OSPFv3 routing table and confirm whether the OSPFv3 trouble diagnosis is correctly carried out.

Example

router#show ipv6 ospf route

Destination Metric

Next-hop Interface

3ffe:1:1::/48 10

-- eth0

3ffe:2:1::/48 10

-- eth0

3ffe:2:2::/48 10

-- eth0

3ffe:3:1::/48 10

-- eth0

3ffe:3:2::/48 10

-- eth0

3ffe:3:3::/48 10

-- eth0

E2 3ffe:100:1::1/128 10/20

fe80::203:47ff:fe4c:776e eth0

E2 3ffe:100:2::1/128 10/20

fe80::203:47ff:fe4c:776e eth0

E2 3ffe:100:3::1/128 10/20

fe80::203:47ff:fe4c:776e eth0

IA 3ffe:101:1::/48 20

fe80::203:47ff:fe4c:776e eth0

IA 3ffe:101:2::/48 20

fe80::203:47ff:fe4c:776e eth0

IA 3ffe:101:3::/48 20

fe80::203:47ff:fe4c:776e eth0

Relative fields are explained in the following table:

| Domain | Description |
| --- | --- |
| Destination | Destination network segment |
| Metric | Cost of a route |
| Next-hop | Address of the next hop |
| Interface | Interface of the next hop |

### show ipv6 ospf virtual-link

To display the information about the OSPFv3 virtual link, run the following command:

show ipv6 ospf virtual-link

Parameter

None

Default value

None

Command mode

EXEC

Instruction

According to the information exported by the command, you can check the state of the OSPFv3 virtual link.

You can run show ipv6 ospf neighbor to check the detailed information about the adjacent neighbor.

Example

router#show ipv6 ospf virtual-link

Virtual Link VLINK1 to router 5.6.7.8 is up

Transit area 0.0.0.1 via interface eth0, instance ID 0

Local address 3ffe:1234:1::1/128

Remote address 3ffe:5678:3::1/128

Transmit Delay is 1 sec, State Point-To-Point,

Timer intervals configured, Hello 10, Dead 40, Wait 40, Retransmit 5

Hello due in 00:00:01

Adjacency state Up

Relative fields are explained in the following table:

| Domain | Description |
| --- | --- |
| neibhbor ID | Neighbor ID of the peer |
| 邻居状态 | Neighborhood state for the neighbor |
| TransArea | Transmission area |
| cost | Minimum cost for reaching the peer in the transmission area If the value of the cost is 0, it means that the peer is unreachable. |
| Hello Interval | Current transmission interval for the Hello packet |
| DeadTime | Time of neighbor invalidation |
| Retrans | Retransmission interval |
| Adjacency state | State of the virtual link interface |

Related command

area vritual-link

show ipv6 ospf neighbor

### summary-prefix

To configure the address for OSPFv3 to create the route aggregation, run summary-prefix. To cancel the address of route aggregation, run no summary-prefix.

summary-prefix ipv6-prefix /prefix-length

no summary-prefix ipv6-prefix /prefix-length

Parameter

| Parameter | Description |
| --- | --- |
| ipv6-prefix | Aggregation address with the designated address range |
| prefix-length | Subnet mask of the aggregation route |

Default value

None

Command mode

Routing configuration mode

Instruction

Multiple groups of addresses are summarized. Routes learned from other routing protocols can also be summarized. After the aggregation, all covered networks cannot be transmitted to other routing fields. The cost of the summary route is the minimum value among the cost values of all summary routes. The command cannot be used to reduce the size of the routing table.

The command is used by OSPFv3 to enable the ASBR to notify an external route of being an aggregation route to replace all external routes.  The command is only used to aggregate the OSPFv3 routes of other routing protocols. You can run area range in OSPFv3 to summarize the routes.

Example

In the following example, the summary address 2001::/64 stands for addresses such as 2001::/80, 2001::1/64 and so on, and only address 2001::/64 is broadcasted.

summary-address 2001::/64

Related command

area range

### timers delay

To designate a delay interval between OSPF receiving a topology change and starting a shortest path priority calculation, run timers delay spf-delay. To resume the default settings, run no timers delay.

timers delay spf-delay

no timers delay

Parameter

| Parameter | Description |
| --- | --- |
| spf-delay | Delay between the topology change and calculation start Its default value is 5 seconds. If the value is 0, there is no delay. That is, the calculation will be promptly started if changes occur. |

Default value

spf-delay: 5 seconds

Command mode

Routing configuration mode

Instruction

The smaller value the delay is set to, the faster the network change is reflected.  However, it will take the processor more time.

Example

The following example shows how to set the time for OSPF to start calculating the delay is 10 seconds.

timers delay 10

### timers hold

To set the interval between two continuous SPF calculations, run timers hold. To resume the default settings, run no timers hold.

timers hold spf-holdtime

no timers hold

Parameter

| Parameter | Description |
| --- | --- |
| spf-holdtime | Minimum value between two continuous calculations It ranges between 0 to 65535 seconds. Its default value is 10 seconds; when it is 0, there is no interval between the two continuous calculations. |

Default value

spf-holdtime: 10 seconds

Command mode

Routing configuration mode

Instruction

The smaller value the delay is set to, the faster the network change is reflected.  However, it will take the processor more time.

Example

The following example shows how to set the interval between two successive SPF calculations to 20 seconds:

timers hold 20