IP ACL Application Configuration

Table of Contents

Chapter 1 IP ACL Application Configuration	1

1.1 Applying the IP Access Control List	1

1.1.1 Applying ACL on Ports	1

# Chapter 1 IP ACL Application Configuration

## Applying the IP Access Control List

### Applying ACL on Ports

After an ACL is established, it can be applied on one or many slots or globally.

Run the following command in port configuration mode:

| Command | Purpose |
| --- | --- |
| config | Enters the global configuration mode. |
| interface g0/1 | Enters the to-be-configured port. |
| [no] {ip ¦ ipv6} access-group name | Applies the established IP access list to an interface or cancels the applied IP on the interface. Name IP: Name of the IP access list. |
| exit | Goes back to the global configuration mode. |
| exit | Goes back to the EXEC mode. |
| write | Saves the settings. |

Run the following command in global configuration mode:

| Command | Purpose |
| --- | --- |
| config | Enters the global configuration mode. |
| [no] {ip ¦ ipv6} access-group name [vlan {word ¦ add word ¦ remove word}] | Applies the established IP access list on the global mode or cancels the applied IP on the global. Egress means that the ACL is applied in an outbound direction. Vlan means that the ACL is applied in an inbound VLAN. Word stands for the VLAN range table. Add means to add the VLAN range table. Remove means to delete the VLAN range table. |
| exit | Goes back to the EXEC mode. |
| write | Saves the settings. |

Note: IP access list can be applied to VLAN in the global mode, but not to VLAN in port configuration mode.