DXP-3800D of PBI Digital Headend Product Upgrade Procedure

Beijing Jaeger Communication Electronic Technology Co. Ltd

Address: Room 401, DingJun Building,No.75 SuZhou Street, Haidian District, Beijing,China,100080

TEL: +86-10-82617178,   FAX: +86-10-82610263

E-mail:overseas@pbicn.com

Content

1. Software Upgrade Mode	3

2. Software Upgrade Procedure	3

2.1 Updated by USB	3

2.2 Updated by Telnet	4

2.2 Updated by Web	6

3. Trouble Shooting	9

4. Contact Us	9

# 1. Software Upgrade Mode

The device can support USB and Telnet, 2 different ways to update the software. The USB mode is much faster and easier for field upgrade. There are two files named “target.tgz” and “3800D.tgz” need to update.

# 2. Software Upgrade Procedure

Important: Before upgrading the digital TV head-end equipment, please check whether the Hardware and Software are compliant in the release note. The upgrade can be launched via IP network. Connect the equipment to a PC via a cross over CAT-5 (RJ45) LAN cable or a normal cable using IP switch/hub. Please ensure that the equipment and the PC are in the same local area network(LAN) before upgrade. The default IP address of equipment is 10.10.70.48, and you can check the IP address from LCD screen on front panel.

Figure1: IP Connection Diagram

# 2.1 Updated by USB

- Copy the “target.tgz” and “3800D.tgz” files in the USB disk, then connect the USB disk with the equipment.Wait for a moment, LCD will display “USB was found” if the USB key connect with the unit well.

- It will upgrade automatically if the software version in the USB disk than the version of the unit inside.

- The unit can be upgraded through “LCD: System->Force Upgrade”, when LCD display “The Firmware Lower” and “Not Upgrade It”.

- The unit will reboot automatically when the software upgrade finished.

- It’s better to check the version through “LCD: System->Version”, whether the update is correctly, It’s also available to check in web page.

# 2.2 Updated by Telnet

  - Turn on the equipment until the booting is completed. You can check the IP address from the LCD screen on front panel, the default IP address is 10.10.70.48. Please make sure that the equipment and your PC are in the same IP network, refer to Figure 1. Important: DON’T switch off the equipment and your PC during the software upgrade.

  - Open the IE browser and type ftp://10.10.70.48 in the address bar and press Enter. If the network configuration is correct, you can open the FTP folder without any error, as shown below.

Figure 2: Open the FTP folder

  - Enter the “ftp://10.10.70.48/pub” folder, then copy the “target.tgz” and “3800D.tgz” files in this folder, as shown below.

  - Open MS-DOS window by typing Start on the lower left quarter of Windows OS. Select Run and key in “cmd” in dialog and press Enter. Type the command “telnet 10.10.70.48”; the current IP address of the equipment under software upgrade as below.

Figure 4: Open MS-DOS Window

  - Type Enter to go into the login window. Use “root” as login name and “12345” as password.

Figure 5: Login Menu

  - Key in the command “upgrade” and press Enter. You can select yes or no for the upgrade.The upgrade process will be launched.

  - When the upgrade is finished, the unit will reboot automatically.

# 2.2 Updated by Web

- Open the Web interface and click the “System” button.

- Enter upgrade web page.

- Click browse button to selected the upgrade files;

- Select any one of the two upgrade files.

- Click upload button for upload the upgrade file.Then please wait.

- Repeat step 4 and step 5 for upload the other upgrade files

- After finishing upload the three files click the “upgrade” button to upgrade.

- Then please wait and the device will reboot automaticly.

# 3. Trouble Shooting

- The IP address of equipment must be in the same local area network(LAN) with the host PC.

- Use a cross over CAT 5 RJ-45 cable to connect the equipment and the host PC directly or connect them using normal RJ-45 cable through a IP hub/switch.

- The host PC used for software upgrade have to have only one network card installed to avoid data connection port error.

- The MS-DOS command “Telnet” is hidden in Windows-Vista OS, please activate it before upgrade. Please refer to Vista operation manual for details.

- The operation procedure to access the FTP server in IE 7.0 browser is different from its previous version. Please follow the on-line guide reported by IE 7.0

# 4. Contact Us

To get further support, please contact PBI Technical Support Center:

PBI Technical Support:

Address: Room 401, DingJun Building,No.75 SuZhou Street, Haidian District, Beijing,China,100080

TEL: +86-10-82617178,   FAX: +86-10-82610263

E-mail: overseas@pbicn.com