DCH-3800EC of PBI Digital Headend Product Upgrade Procedure

Beijing Jaeger Communication Electronic Technology Co. Ltd

Address: Room 401, DingJun Building,No.75 SuZhou Street, Haidian District, Beijing,China,100080

TEL: +86-10-82617178,   FAX: +86-10-82610263

E-mail:overseas@pbicn.com

Content

1. Software Upgrade Mode	3

2. Software Upgrade Procedure	3

2.1 Updated by WEB HTTP	4

2.2 Updated by USB	6

2.3 Updated by Telnet	6

3. How to enable “telnet” command on Win7 or Win8	9

4. Trouble Shooting	10

5. Contact Us	11

# 1. Software Upgrade Mode

The device can support USB, WEB HTTP and Telnet, 3 different ways to update the software. The WEB HTTP mode is more convenient for remote update via IP network. The USB mode is much faster and easier for field upgrade. There is just one file named “target.tgz” need to update.

# 2. Software Upgrade Procedure

Important: Before upgrading the digital TV head-end equipment, please check whether the Hardware and Software are compliant in the release note. The upgrade can be launched via IP network. Connect the equipment to a PC via a cross over CAT-5 (RJ45) LAN cable or a normal cable using IP switch/hub. Please ensure that the equipment and the PC are in the same local area network(LAN) before upgrade. The default IP address of equipment is 10.10.70.48, and you can check the IP address from LCD screen on front panel.

Figure1: IP Connection Diagram

# 2.1 Updated by WEB HTTP

- Enter the System->Upgrade page.

Figure 2: Upgrade Page

- Click the “Browse” and select upgrade file “target.tgz”.

Figure 3: Upload the file to FTP

- Click the “Upgrade” and choose “OK”.

Figure 4: Confirm Page

- Select “OK”, the unit will upgrade.

Figure 5: Upgrade Page

- After upgrade, the device will automatically restart.

# 2.2 Updated by USB

- Copy the “target.tgz” file in the USB disk, then connect the USB disk with the equipment. LCD will display “Upgrade Now?” and "YES(Enter) NO(Exit)" if the USB key connect with the unit well.

- The unit can be upgraded, when press “Enter”.

- The unit will reboot automatically when the software upgrade finished.

- It’s better to check the version through “LCD: System->Version”, whether the update is correctly, It’s also available to check in web page.

# 2.3 Updated by Telnet

  - Turn on the equipment until the booting is completed. You can check the IP address from the LCD screen on front panel, the default IP address is 10.10.70.48. Please make sure that the equipment and your PC are in the same IP network, refer to Figure 1. Important: DON’T switch off the equipment and your PC during the software upgrade.

  - Open the IE browser and type ftp://10.10.70.48 in the address bar and press Enter. If the network configuration is correct, you can open the FTP folder without any error, as shown below.

Figure 6: Open the FTP folder

  - Enter the “ftp://10.10.70.48/pub” folder, then copy the “target.tgz” file in this folder, as shown below.

Figure 7: Copy “target.tgz” file

  - Open MS-DOS window by typing Start on the lower left quarter of Windows OS. Select Run and key in “cmd” in dialog and press Enter. Type the command “telnet 10.10.70.48”; the current IP address of the equipment under software upgrade as below.

Important Note: If the Win7 or Win8 systems does not support “telnet” command, please follow the Chapter 3(Page 9) “How to enable telnet command on Win7 or Win8” to resolve it.

Figure 8: Open MS-DOS Window

  - Type Enter to go into the login window. Use “root” as login name and “12345” as password.

Figure 9: Login Menu

  - Key in the command “upgrade” and press Enter. The upgrade process will be launched.

Figure 10: Select Menu

  - The unit will be upgraded when input ‘y’.

  - When the upgrade is finished, the unit will reboot automatically.

# 3. How to enable “telnet” command on Win7 or Win8

1. Through the “Control Panel” to open “Programs and Features”.

2. Then click the “Turn Windows features on or off” item, the “Windows features” select box will be opened, it will take several seconds to update all the items.

3. Please select the “Telnet Client”, “Telnet Server” and “TFTP Client”, then press “OK” button, it will take tens of seconds to finish the settings. Below screenshot for reference.

Figure 11: Enable Telnet on Win7 or Win8

# 4. Trouble Shooting

- The IP address of equipment must be in the same local area network(LAN) with the host PC.

- Use a cross over CAT 5 RJ-45 cable to connect the equipment and the host PC directly or connect them using normal RJ-45 cable through a IP hub/switch.

- The host PC used for software upgrade have to have only one network card installed to avoid data connection port error.

- The MS-DOS command “Telnet” is hidden in Windows-Vista OS, please activate it before upgrade. Please refer to Vista operation manual for details.

- The operation procedure to access the FTP server in IE 7.0 browser is different from its previous version. Please follow the on-line guide reported by IE 7.0

# 5. Contact Us

To get further support, please contact PBI Technical Support Center:

PBI Technical Support:

Address: Room 401, DingJun Building,No.75 SuZhou Street, Haidian District, Beijing,China,100080

TEL: +86-10-82617178,   FAX: +86-10-82610263

E-mail: overseas@pbicn.com