- 8000D  upgrade  procedure

- I . FTP  upgrade

a. Turn on 8000D-xxx(the unit), wait for normal booting being completed.

b. Find and record the IP address of the IP address of the unit, for example, 10.10.80.147.

c. Connect the IP port of the unit to a MS-Windows PC with a cross over CAT5(RJ45) LAN wire; the PC should has the same IP section with the unit.

d. Run the DOS command ‘cmd.exe’ on the same MS-Windows PC(Notice: form the start icon on the lower left comer on MS-Windows), you will see a pop up DOS window, type the command ‘telnet 10.10.80.147’(the IP of the unit) as below.

e.Then, go to the MS-Windows, and Execute the IE browser, enter ftp://10.10.80.147 (as an example, the IP address from the previous step b) on the address bar, wait for the link connected.

f. Enter into the document ‘pub’, and copy the upgradefile ‘target1.tgz’ from its directory to the FTP server linked, as shown below.

g. Go back to the command line on the CMD windows, type the command ‘upgrade target1.tgz’ and press [Enter], the upgrade process will be taken place and just wait for the job down; it might take a few minutes, depends on the speed of the network linked. Please DON’T switch off the power of your PC nor the unit.

II. USB  upgrade

Note: if the version number is higher than the current version number, USB will automatically upgrade, careful with.

a. Copy the upgradefile ‘target1.tgz’ to a USB disk, make sure the upgradefile’s

version of USB disk is higher than the upgradefile’s version of unit

b. The USB disk is plugged into the unit, if the upgradefile’s version of the USB disk is higher than the upgradefile’s version of the unit, it can AUTO upgrade and then reboot.

Trouble shooting and important note:

- The IP address of the unit must be in the same section of the host PC’s.

- Use a crossover CAT 5 wire to connect the unit and the host PC directly or link through a hub/router.

- The host PC that is being used for upgrade has better to have only one LAN card installed, nor wireless LAN is enable, otherwise communication data might send to that port and not delivered correctly to the unit.

- The MS-DOS command “Telnet” has been hidden in Windows-Vista, please activate it before upgrade. Details shoule refer to Vista operation manual.

- The operation steps to access the FTP server in IE 7.0 is different from its previous version. Please follow the on-line guide line reported IE 7.0