CMS Quick Guide

V1.0

## 1.Overview

This Guide provides basic instructions on how to operate the software, live view, record images and play back the recorded video. For detailed instructions on programming and using the software, please refer to the CMS User Manual.

## 2.Install the software

- Please first install "PSService.exe" in the server or PC.

Default Media Server port is 8000. Please do remember the media port If you changed it during installation process.

- Then install "PCTerminal.exe". Run the wizard.

click the“ Next” button, To next operation.

- If you want to install the storage service in your server,you can install “PSStore.exe” in your server.This is option installation.

(4)Install finished, click finish to exit installation.

## 3.Uninstall the software

Click “Start->Settings->Control Panel->Add or Remove programs”.

Select “Video surveillance System” and “Video surveillance System” to remove the CMS.

- 4.User Login

Language: English.

Server: The IP address which you had installed “PSService.exe”.If you install in your PC,then it is your PC IP address.

Port: Server media port. Default is 8000. If you have changed it when installed “PSService.exe”,please use your changed one.

Username: Default is admin

Password: Default is 123456

- 5.Add Devices

- Please enter “Config->Device management” menu.

- There has two way to add devices: Search Add or Manual Add.

- Search Add

- Click “Config->Device management->Search” button to open search add device interface.

Then check the right device you want. Then click “Add devices” button to add your devices.

- (2)Manual Add

- Click “Config->Device management->Add device” button to open manual add device interface.

Device Name: DVR name shows in your software

Device Description: DVR additional information

Device Type: Device type.

Device address: DVR IP address or Domain address in LAN or WLAN.

Device Port: DVR command port. Default is 8101.

Login user:  Login user name in DVR.

Login password:  Login user name in DVR.

Channel:    DVR channel number.

Alarm Input: DVR alarm input number.

Alarm output: DVR alarm input number.

Click “OK” button to finish manual add device process.

- Device state

- Check the devices state in “Config->Device Status” menu.

- If the device icon is gray color,mean is off line.

- If the device icon is gray color, mean is online. You can preview or operate the device by CMS software.

- 6.Live View

- Please enter “Preview” interface.

- Double click one device in interface right side Devices list to open all live view channels.

- Drag one channel to one image window to live view one channel.

- The max image windows number is 10×10=100 channels at the same time.

- 7. Playback

- (1)Playback video

- Double click the device icon in interface right side.

- Then double click the date in Date box to choose the video record which you want to playback.

- If the date have video record ,it will show green bar in record area as below.

- If not,means the channel in this data don’t have video record.

- Double click to green bar to playback video.

- Download video

- Click “Download” button to download video record.

- 8.Alarm

- Check alarm information in “Instant Alarm” menu.

- Click the alarm list,then you can see the corresponding information in left side.

- 9.System setting

- Set Region

- User management

- Storage management

- Set DVR Sequence

- Set alarm linkage

- (6) Set E-map

- Log management

- System setting