IP-PDU Firmware upgrade instruction

1. Overview

To upgrade the IP-PDU firmware though the Tftpd32 and bin file.

2. Operation in detail:

2.1 Preparations

2.1.1 It will require a network cable and a PC or laptop, connect your PC to the NET port of IP-PDU as below:

Figure 1

2.1.2 Switch to the IP address page from the local LCD screen (by press the MENU button to view different LCD pages)

Figure 2

2.1.3  Open “cmd” form from the RUN dialog and click “Enter”

Figure 3

2.1.4.  Use the Ping command to test the network via the IP-address of IP-PDU as below:

Figure 4

2.2 Upgrade the firmware

2.2.1.  Double click the ftfpd32 , then click “Tftp Client” as below:

Figure 5

2.2.2.  Enter your IP address of your PC or laptop from Server interfaces

2.2.3.  Enter the IP address of the IP-PDU from Host;

2.2.4.  Enter “69” from “Port”

2.2.5.  Select the bin file from the icon which is besides the “Local File”, others remain the same;

2.2.6.  Click “Put” as below:

图6

You progress bar will star running after 2-3 seconds as below:

Figure 7

Click “OK” when loading progress is finished as below figure 8

Figure 8

The buzzer sounds and IP-PDU will restart after upgrading.

Remarks: If the progress bar is not running, click “Break” and upload the bin again.  If you got the same problem, please double check the IP address of your IP-PDU and fin file is correct.