- Interface Physical Characteristic Configuration Commands

- Table of Contents

Chapter 1 Interface Physical Characteristic Configuration Commands	1

1.1 Interface Physical Characteristic Configuration Commands	1

1.1.1 speed	1

1.1.2 duplex	2

1.1.3 flow control	2

# Chapter 1 Interface Physical Characteristic Configuration Commands

## Interface Physical Characteristic Configuration Commands

Interface physical characteristic configuration commands include：

- speed

- duplex

- flow-control

### speed

Description

To set the speed rate of the port, run speed.

speed {10 | 100 | 1000 |auto }

no speed

Parameter

| Parameter | Description |
| --- | --- |
| 10, 100, 1000 | Configures the interface to transmit at 10M、100M、1000M。 |
| auto | Turns on the Fast Ethernet auto-negotiation capability. |

Default

TX interface is auto-negotiation operation, gigabit TX interface is 1000M, gigabit optical interface is 1000M.

Instruction

Use this command in layer-2 interface configuration mode.

Note：

The optical interface speed is fixed. The auto parameter following the speed command indicates that the auto-negotiation feature can be enabled on this interface. If there is no auto parameter behind the speed command, the auto-negotiation cannot be configured on the interface.

Example

The following example sets the interface g0/1 speed to 100M:

Switch (config)# interface GigaEthernet0/1

Switch (Switch_config_g0/1)# speed 100

### duplex

Description

To configure the duplex operation on an interface, use the duplex command.

duplex {auto | full | half }

no duplex

Parameter

| Parameter | Description |
| --- | --- |
| auto | Specifies the auto-negotiation operation. |
| full | Specifies the full-duplex operation. |
| half | Specifies the half-duplex operation. |

Default

The Tx interface is for auto-negotiation operation, while the optical interface is for full-duplex operation.

Instruction

Use this command in the layer 2 configuration mode.

Note：

The duplex operation of the optical interface is fixed, that is, full-duplex operation.

Example

The following example configures interface g0/1 to full-duplex operation:

Switch (config)# interface GigaEthernet0/1

Switch (config-if)# duplex full

### flow control

Description

To configure flow control on an interface, use the flow control command. Use the no form of this command to disable the flow control.

flow-control{on | off}

no flow-control

Parameter

| Parameter | Description |
| --- | --- |
| on | Enables flow control. |
| off | Disables flow control. |

Default

Disabled

Instruction

Use this command in the layer 2 configuration mode.

Example

The following example enables the flow-control on an interface:

Switch (config-if)# flow-control on