- IP ACL Application Configuration Commands

- Table of Contents

IP ACL Application Configuration Commands	1

Table of Contents	I

Chapter 1 IP ACL Application Commands	1

1.1 Configuration Commands for Applying IP ACL	1

1.1.1 ip access-group	1

1.1.2 ipv6 access-group	2

# IP ACL Application Commands

## Configuration Commands for Applying IP ACL

The commands for applying the IP Access Control List (ACL) are shown below:

- ip access-group

- ipv6 access-group

### ip access-group

To designate an access group, run the ip access-group. To cancel the designated access group, run no ip access-group.

To apply a created IP ACL on a port, run the following command:

[no] ip access-group name [egress]

To apply a created IP ACL globally, run the following command:

[no] ip access-group name [egress | vlan {word | add word | remove word}]

You can use this command to apply the established IP ACL to an interface or globally or cancel the applied IP ACL on an interface or globally.

Parameter

| Parameter | Description |
| --- | --- |
| name | Stands for the name of IP ACL. |
| egress | Applies the ACL in an outbound direction. |
| Vlan | Applies the ACL in an inbound VLAN. |
| Word | Stands for the VLAN range table. |
| add | Adds a VLAN range table. |
| remove | Cancels a VLAN range table. |

Command mode

Global or port configuration mode

Explanation

Most rules in the ACL take effect through hardware; those that hardware does not support give no errors but they have no actual effects; a few rules such as time-range take effect through software.

Note:

The IPv4 standard ACL supports the following rules:

any: means any source IP address.

source-addr source-mask: means matching up the source address.

reverse-mask source-addr source-mask: means to use the reverse source address for match-up.

The IPv4 extended ACL supports the following rules:

any: means any IP address.

ip-protocol: means the IP protocol ID.

ip-addr source-mask: means IP address match-up.

Interface interface-id: means layer-3 interface match-up.

eq/gt/lt/src-portrange: means TCP/UDP port ID match-up.

totallen: means the length match-up of IP packets.

established/tos/is-fragment/not-fragment/precedence/ttl/offset-not-zero/offset-zero/donotfragment-set/ donotfragment-notset/icmp-type: means field match-up, among which ttl must be set to equal.

Example

The following example shows how to apply the ACL filter at the ingress direction of interface g0/1.

Switch_config#inter g0/1

Switch_config_g0/1# ip access-group filter

### ipv6 access-group

To designate an access group, run the ipv6 access-group. To cancel the designated access group, run no ipv6 access-group.

To apply a created IPv6 ACL on a port, run the following command:

[no] ipv6 access-group name [egress]

To apply a created IPv6 ACL globally, run the following command:

[no] ipv6 access-group name [egress | vlan {word | add word | remove word}]

You can use this command to apply the established IPv6 ACL to an interface or globally or cancel the applied IPv6 ACL on an interface or globally.

Parameter

| Parameter | Description |
| --- | --- |
| name | Stands for the name of IPv6 ACL. |
| egress | Applies the IPv6 ACL in an outbound direction. |
| Vlan | Applies the IPv6 ACL in an inbound VLAN. |
| Word | Stands for the VLAN range table. |
| add | Adds the VLAN range table. |
| remove | Cancels the VLAN range table. |

Command mode

Global configuration mode or port configuration mode

Explanation

Most rules in the ACL take effect through hardware; those that hardware does not support give no errors but they have no actual effects; a few rules such as time-range take effect through software.

Note:

The IPv6 ACL supports the following rules:

any: means any IPv6 address.

Ipv6-addr/ host Ipv6-addr： means IPv6 address match-up.

Ipv6-protocol: means the IPv6 protocol ID.

eq/gt/lt/src-portrange: means TCP/UDP port ID match-up.

dscp/flow-label: means field match-up.

Example

The following example shows how to apply the ACL filter at the ingress direction of interface g0/1.

Switch_config#inter g0/1

Switch_config_g0/1# ipv6 access-group filter