SNTP Configuration Commands

Table of Contents

Chapter 1 SNTP Configuration Commands	1

1.1 SNTP Configuration Commands	1

1.1.1 sntp master	1

1.1.2 sntp query-interval	2

1.1.3 sntp server	3

1.1.4 sntp peer	3

1.1.5 sntp source	4

1.1.6 sntp authenticate	5

1.1.7 sntp authentication-key	5

1.1.8 sntp trusted-key	6

1.1.9 show sntp	7

1.1.10 debug sntp	8

1.1.11 time-zone	8

1.1.12 time-range	9

1.1.13 absolute	10

1.1.14 periodic	10

# SNTP Configuration Commands

## SNTP Configuration Commands

SNTP configuration commands include:

- sntp master

- sntp query-interval

- sntp server

- sntp peer

- sntp source

- show sntp

- debug sntp

- time-zone

### sntp master

To set the level of the local clock, run sntp master num; to cancel this settings, run no sntp master.

sntp master num

no sntp master

Parameter

| Parameter | Description |
| --- | --- |
| num | Means the level of the clock. |

Default

None

Command mode

Routing configuration mode

Remark

The clock level is disabled by default; if it is enabled, the level ranges between 1 and 15, and will be 8 without configuration.

Example

Router_config#sntp master 5

Related Commands

sntp peer

sntp server

### sntp query-interval

To set the interval of sending the SNTP requests, run sntp query-interval time. To resume the default settings, run no sntp query-interval.

sntp query-interval time

no sntp query-interval

Parameter

| Parameter | Description |
| --- | --- |
| Time | Means the interval of sending the requests. |

Default

The default value is 1, that is, the requests will be sent every minute.

Command mode

Routing configuration mode

Remark

The time ranges between 1 and 1440 minutes.

Example

Router_config#sntp query-interval 5

### sntp server

To set the address of the SNTP server, run sntp server address [key key-number｜version version-number].

sntp server address ［key key-number｜version version-number］

no sntp server address

Parameter

| Parameter | Description |
| --- | --- |
| address | Means the address of the SNTP server. |
| version-number | Means the version ID of SNTP (1-4). |
| key-number | Means the key ID of the SNTP server. |

Default

None

Command mode

Routing configuration mode

Remark

You can set multiple addresses for the SNTP server. There is no SNTP server by default.

Example

Router_config#sntp server 1.1.1.1 4

Related Command

sntp peer

### sntp peer

To set the SNTP peer, run sntp peer address version. To delete the SNTP peer, run no sntp peer address.

sntp peer address version

no sntp peer address

Parameter

| Parameter | Description |
| --- | --- |
| address | Means the address of the SNTP peer. |
| version | Means the version ID of SNTP (1-4). |

Default

None

Command mode

Routing configuration mode

Remark

This command is used to set the address of the SNTP peer.

Example

Router_config#sntp peer 1.1.1.1 4

Related Command

Sntp server

### sntp source

To set the designated local source address when SNTP is triggered, run Sntp source [interface inter｜addr].

Sntp source ［interface inter｜ addr］

No sntp source

Parameter

| Parameter | Description |
| --- | --- |
| inter | Means an interface. |
| Address | Means a designated source address. |

Default

None

Command mode

Routing configuration mode

Remark

This command is used to designate the source address of sending the SNTP packets.

Example

Router_config#sntp source 1.1.1.1

### sntp authenticate

To enable the SNTP authentication, run sntp authenticate.

sntp authenticate

no sntp authenticate

Parameter

None

Default

The authentication is disabled.

Command mode

Routing configuration mode

Remark

This command is used to enable SNTP authentication.

Example

Router_config#sntp authenticate

### sntp authentication-key

To set the local authentication key of SNTP, run sntp authentication-key number md5 type password.

sntp authentication-key number md5 type password

no sntp authentication-key number

Parameter

| Parameter | Description |
| --- | --- |
| number | Key number |
| type | Means the mode to display the password. |
| password | Means the corresponding MD5 password. |

Default

None

Command mode

Routing configuration mode

Remark

This command is used to set the local authentication key.

Example

Router_config#sntp authentication-key 2 md5 0 123456

### sntp trusted-key

To set the trusted local key, run sntp trusted-key number.

sntp trusted-key number

no sntp trusted-key number

Parameter

| Parameter | Description |
| --- | --- |
| number | Key number |

Default

None

Command mode

Routing configuration mode

Remark

This command is used to set the trusted local key.

Example

Router_config#sntp trusted-key 1

### show sntp

To display the SNTP related information, run the following command:

Show sntp

Parameter

None

Default

None

Command mode

EXEC mode

Remark

This command is used to display the current status of SNTP.

Example

Router_config#show sntp

Master Mode: Yes        Master Clock Stratum: 5

Debug Mode: Off        Client Status: idle

Interval to Query SNTP Server: 1 (minutes)

Configured SNTP Server List:

Current SNTP Server :  00010003

Configured SNTP Peer List:

IP: 1.1.1.1     Version: 4

Current SNTP Peer : 1.1.1.1 00010003

The local time (UTC):   Date: 2002-3-25 Time: 17:36:59

### debug sntp

To enable SNTP debugging, run debug sntp. To disable SNTP debugging, run no debug sntp.

Debug sntp

no debug sntp

Parameter

None

Default

None

Command mode

EXEC mode

Remark

You can browse the SNTP operation according to the displayed information.

Example

None

### time-zone

To open the time zone, run time-zone name offset-hour offset-minute. To close the time zone, run no time-zone.

time-zone name offset-hour offset-minute

no time-zone

Parameter

| Parameter | Description |
| --- | --- |
| name | Means the name of a time zone. |
| offset-hour | Means the offset hour between the local time and the UTC time (-12-12). |
| offset-minute | Means the offset minute between the local time and the UTC time (0-59). |

Default

None

Command mode

Routing configuration mode

Remark

This command can be used to transfer UTC to the local time.

Example

Router_config#time-zone BeiJing 8

### time-range

To open the time range, run time-range name. To close the time zone, run no time-range name.

time-range name

no time-range name

Parameter

| Parameter | Description |
| --- | --- |
| name | Means the name of a time range. |

Default

None

Command mode

Routing configuration mode

Remark

This command is used to set a time range.

Example

Router_config#time-range aaa

### absolute

To set the absolute time of a time range, run absolute start hh:mm day month year end hh:mm day month year. To resume the default settings, run no absolute.

absolute start hh:mm day month year end hh:mm day month year

no absolute

Parameter

| Parameter | Description |
| --- | --- |
| hh:mm | Stand for the hour and the minute respectively. |
| day | Stands for a day. |
| month | Stands for a month. |
| year | Stands for a year. |

Default

None

Command mode

Time range configuration mode

Remark

This command is used to set an absolute time range.

Example

Router_config_time_range#absolute start 11:11 2 2 2000 end 22:22 3 3 2002

### periodic

To set the periodic time of a time range, run the first one of the following two commands.

Periodic [Monday｜Tuesday｜Wednesda｜Thursday｜Friday｜Saturday｜Sunday｜daily｜weekdays｜weekend] hh:mm to [Monday｜Tuesday｜Wednesda｜Thursday｜Friday｜Saturday｜Sunday｜daily｜weekdays｜weekend] hh:mm

no Periodic

Parameter

| Parameter | Description |
| --- | --- |
| hh:mm | Stand for the hour and the minute respectively. |

Default

None

Command mode

Time range configuration mode

Remark

This command is used to set an absolute time range.

Example

Router_config_time_range#periodic monday 11:11 to tuesday 11:11