- Basic Configuration Commands

- Table of Contents

Basic Configuration Commands	I

Table of Contents	I

Chapter 1 System Management Commands	1

1.1 File Management Configuration Commands	1

1.1.1 copy	1

1.1.2 delete	2

1.1.3 dir	3

1.1.4 ip address	3

1.1.5 ip default-gateway	4

1.1.6 show configuration	5

1.1.7 format	5

1.1.8 more	6

1.2 Basic System Management Commands	6

1.2.1 boot flash	7

1.2.2 cd	8

1.2.3 chinese	8

1.2.4 date	9

1.2.5 english	10

1.2.6 md	10

1.2.7 pwd	11

1.2.8 rd	11

1.2.9 rename	12

1.2.10 reboot	12

1.2.11 alias	13

1.2.12 boot system flash	14

1.2.13 help	15

1.2.14 history	15

1.2.15 show	17

1.2.16 show alias	18

1.2.17 show break	19

Chapter 2 Terminal Service Configuration Commands	20

2.1 Telnet Configuration Commands	20

2.1.1 telnet	20

2.1.2 ip telnet	22

2.1.3 ctrl-shift-6+x (the current connection is mounted)	24

2.1.4 where	25

2.1.5 resume	26

2.1.6 disconnect	27

2.1.7 clear telnet	28

2.1.8 show telnet	29

2.1.9 debug telnet	30

2.2 Terminal Configuration Commands	30

2.2.1 attach-port	31

2.2.2 autocommand	32

2.2.3 clear line	32

2.2.4 connect	33

2.2.5 disconnect	33

2.2.6 exec-timeout	34

2.2.7 length	34

2.2.8 line	35

2.2.9 location	35

2.2.10 login authentication	36

2.2.11 monitor	36

2.2.12 no debug all	37

2.2.13 password	37

2.2.14 resume	38

2.2.15 show debug	38

2.2.16 show line	39

2.2.17 terminal length	39

2.2.18 terminal monitor	40

2.2.19 terminal width	41

2.2.20 terminal-type	42

2.2.21 where	42

2.2.22 width	43

Chapter 3 Maintenance and Debugging Tool Commands	44

3.1 Network Testing Tool Commands	44

3.1.1 ping	44

3.1.2 traceroute	47

3.2 Fault Diagnosis Commands	49

3.2.1 logging	49

3.2.2 logging buffered	50

3.2.3 logging console	52

3.2.4 logging facility	53

3.2.5 logging monitor	54

3.2.6 logging on	56

3.2.7 logging trap	57

3.2.8 logging command	59

3.2.9 logging source-interface	59

3.2.10 logging history alerts	60

3.2.11 logging history critical	60

3.2.12 logging history debugging	61

3.2.13 logging history emergencies	61

3.2.14 logging history errors	62

3.2.15 logging history informational	63

3.2.16 logging history notifications	63

3.2.17 logging history warnings	64

3.2.18 logging history rate-limit	64

3.2.19 logging history size	65

3.2.20 service timestamps	65

3.2.21 clear logging	66

3.2.22 show break	67

3.2.23 show debug	68

3.2.24 show logging	69

Chapter 4 SSH Configuration Commands	71

4.1.1 ip sshd enable	71

4.1.2 ip sshd timeout	71

4.1.3 ip sshd auth-method	72

4.1.4 ip sshd access-class	73

4.1.5 ip sshd auth-retries	74

4.1.6 ip sshd clear	74

4.1.7 ip sshd silence-period	75

4.1.8 ip sshd sftp	76

4.1.9 ip sshd save	76

4.1.10 ip sshd disable-aes	77

4.1.11 ssh	78

4.1.12 show ssh	79

4.1.13 show ip sshd	79

# System Management Commands

## File Management Configuration Commands

The file management configuration commands include:

- copy

- delete

- dir

- ip address

- ip route

- show configuration

- format

- more

### copy

Syntax

To read a file from the tftp server to a switch, use the copy command.

copy tftp<:filename> {flash<:filename>|rom} [ip_addr]

Parameter

| Parameter | Description |
| --- | --- |
| tftp<:filename> | Read a file from the tftp server. Filename indicates the relevant filename. If not specified the filename, the system will prompt user to input the filename after executing the copy command. |
| flash <:filename> | Write a file to the flash memory of the switch. Filename indicates the relevant filename. If not specified the filename, the system will prompt user to input the filename after executing the copy command. |
| rom | Updates bootrom for the switch. |
| ip_addr | Specifies the IP address of tftp srever. If not specified, the system will prompt user to input the IP address after executing the copy command. |

Default

None

Command Mode

Monitor mode

Usage Guidelines

None

Example

monitor#copy tftp:switch.bin flash:switch.bin 192.2.2.1

The example shows how to read the switch.bin from the tftp server to the flash memory of the switch:

Related Command

None

### delete

Syntax

To delete a file, use the delete command.

delete  file-name

Parameter

| Parameter | Description |
| --- | --- |
| file-name | Specifies the filename (maximum 20 characters) |

Default

If not specified the file-name, the system will delete startup-config by default.

Command Mode

Monitor mode

Usage Guidelines

None

Related Command

None

### dir

Syntax

To display filename, use the dir command.

dir file-name

Parameter

| Parameter | Description |
| --- | --- |
| file-name | Specifies the filename (maximum 20 characters) |

Default

None

Command Mode

monitor mode

Usage Guidelines

None

Related Command

None

### ip address

Syntax

To set an IP address for an Ethernet interface, use the ip address command.

ip address ip-address mask

Parameter

| Parameter | Description |
| --- | --- |
| ip-address | IP address |
| mask | IP network mask |

Default

None

Command Mode

Monitor mode

Usage Guidelines

None

Example

monitor#ip address 192.168.1.1 255.255.255.0

Related Command

ip route

ping

### ip default-gateway

Syntax

To set the default gateway, run the following command:

ip default-gateway  gw_ip_addr

Parameter

| Parameter | Description |
| --- | --- |
| gw_ip_addr | The address of the default gateway. |

Default

None

Command Mode

The monitoring state.

Usage Guidelines

None

Example

monitor#ip default-gateway 192.168.1.3

Related Command

ip address

### show configuration

Syntax

To display the running configuration file, use the show configuration command.

show configuration

Parameter

None

Default

None

Command Mode

Monitor mode

Usage Guidelines

None

Related Command

None

### format

Syntax

To format file system, use the format command.

format

Parameter

None

Default

None

Command Mode

EXEC

Usage Guidelines

All files in the file system will de deleted after executing the format command.

Related Command

None

### more

Syntax

To display the contents of a file, use the more command.

more  file-name

Parameter

| Parameter | Description |
| --- | --- |
| file-name | Specifies the name of a file (maximum 20 characters) |

Default

None

Command Mode

EXEC

Usage Guidelines

If all files are displayable characters, they will be displayed in ASCII format, or they will be displayed binary format.

Related Command

None

## Basic System Management Commands

- bootflash

- cd

- chinese

- english

- date

- md

- pwd

- rd

- rename

- reboot

- show break

- alias

- boot system flash

- help

- show

- history

- show alias

### boot flash

Syntax

To enable the system from the specified file in monitor mode, use the boot flash command.

boot flash filename

Parameter

| Parameter | Description |
| --- | --- |
| filename | Specified file name. |

Default

None

Command Mode

Monitor mode

Usage Guidelines

Use the boot flash command to enable the device after user entering the monitor mode.

Example

monitor#boot flash switch.bin

Related Command

None

### cd

Syntax

To change the current directory, use the cd command.

cd  directory|..

Parameter:

| Parameter | Description |
| --- | --- |
| directory | Name of the directory. (maximum 20 characters) |
| .. | Upper directory. |

Default

None

Command Mode

Monitor mode

Usage Guidelines

None

Example

monitor#cd my_dir

Related Command

pwd

### chinese

Syntax

To switch command prompt to chinese mode, use the chinese command.

Parameter

None

Default

None

Command Mode

Monitor mode

Usage Guidelines

None

Example

None

Related Command

None

### date

Syntax

To set the absolute time, use the date command.

Parameter

None

Default

None

Command Mode

Monitor mode

Usage Guidelines

This command is used to set the abslute time for the system. For the switch with a battery-powered clock, the clock will be powered by the battery. If the clock doesn’t keep good time, you need to change the battery.

For the swich without a battery-powered clock, the system date is configured to July 1st, 1970 after the reboot of the switch, and user needs to set the current time each time when starting the switch.

Example

monitor#date

The current date is 2000-7-27 21:17:24

Enter the new date(yyyy-mm-dd):2000-7-27

Enter the new time(hh:mm:ss):21:17:00

Related Command

### english

Syntax

To switch the command prompt to english mode, use the english command.

Parameter

None

Default

None

Command Mode

Monitor mode

Usage Guidelines

None

Example

None

Related Command

None

### md

Syntax

md  directory

Parameter

| Parameter | Description |
| --- | --- |
| directory | Name of directory (maximum 20 characters) |

Default

None

Command Mode

Monitor mode

Usage Guidelines

To set a directory, use the md command

Related Command

None

### pwd

Parameter

None

Default

None

Command Mode

Monitor mode

Usage Guidelines

To display the current directory, use the pwd command

Related Command

None

### rd

Syntax

rd  directory

Parameter

| Parameter | Description |
| --- | --- |
| directory | Name of the directory( maximum 20 characters) |

Default

None

Command Mode

Monitor mode

Usage Guidelines

The system prompts if the directory is not empty. The system prompts if the directory doesn’t exist. To delete a command, use the rd command.

Related Command

None

### rename

Syntax

To rename a file in a file system, use the rename command.

rename  old_file_name  new_file_name

Parameter

| Parameter | Description |
| --- | --- |
| old_file_name | The original filename. |
| new_file_name | The new filename. |

Default

None

Command Mode

Monitor mode

Usage Guidelines

None

Related Command

None

### reboot

Parameter

None

Default

None

Command Mode

Monitor mode

Usage Guidelines

To reboot a switch, use the reboot command.

Related Command

None

### alias

Syntax

alias alias_name command_line

Parameter

| Parameter | Description |
| --- | --- |
| alias_name | Name the alias name. |
| command_line | The command of naming the alias name. |

Default

None

Comand mode

Global configuration mode

Usage Guidelines

The command can be used to replace "command_line" with "alias_name". For instance, alias update1 copy tftp: switch.bin flash:switch.bin 10.168.30.188. The command " copy tftp: switch.bin flash:switch.bin 10.168.30.188 " will automatically run on the OLT only update 1 is input.

Example

The following example shows how to use the command to replace " copy tftp:BDMSU8508_4.0.0B.bin flash:switch.bin 10.168.30.188" with " update1".

alias update1 copy tftp:BDMSU8508_4.0.0B.bin flash:switch.bin 10.168.30.188

Related Command

None

### boot system flash

Syntax

Run the boot system flash command to specify the systematic mirroring files when the system is started up. Run the no boot system flash command to delete the previous configuration.

boot system flash filename

no boot system flash filename

Parameter

| Parameter | Description |
| --- | --- |
| filename | It is the specified filename, which contains no more than 20 characters. |

Default

None

Command Mode

Global configuration mode

Usage Guidelines

If you have not configured the command, the system will execute the first systematic mirroring file in the flash file system. If you have configured multiple commands, the system will execute the mirroring files one by one. If the file does not exist or the check sum is wrong, the system will execute the next file. If both fail, the system will run at the monitoring state.

Example

config#boot system flash switch.bin

Related Command

None

### help

Syntax

help

Parameter

None

Default

None

Command Mode

EXEC mode

Usage Guidelines

The command is used to display the help system of the switch.

Example

The foloowing example shows how to display the help system of the switch, after you enter the command.

switch# help

Help may be requested at any point in a command by entering a question mark '?', If nothing matches, the help list will be empty and you must backup until entering a '?' shows the available options.

Two styles of help are provided:

1. Full help is available when you are ready to enter a command argument(e.g.'show ?') and describes each possible argument.

2. Partial help is provided when an abbreviated argument is entered and you want to know what arguments match the input (e.g. 'interface e?'.)

Related Command

None

### history

Syntax

The command is used to check the historical commands. Run the [no] history command to delete the historical commands.

[no] history [ + <count> | - <count> | clear]

Parameter

| Parameter | Description |
| --- | --- |
| + <count> | Displays the count<1-20> historical command from the beginning to the end. |
| - <count> | Displays the count<1-20> historical command from the end to the beginning. |

Default

If there are no more than 20 commands executed, all historical command lines will be displayed from the beginning to the end. If there are more than 20 commands executed, the latest 20 command lines will be displayed from the beginning to the end.

Command Mode

Abandom command mode

Usage Guidelines

The modularized switch can save up to 20 historical commands. You can invoke these commands with the “up” or “down” key or directly use it after edition.

Example

The following example shows the latest five commands from the end to the beginning:

switch#history - 5

config

int e1/1

no ip addr

ip addr 192.2.2.49 255.255.255.0

exit

Related Command

None

### show

Syntax

To display the relevant information of the system, which or specific ones of which can be filtered through the filter, run the following command:

show <sub-command> [ | <begin | include | exclude | redirect> <WORD> [SEPARATOR WORD]]

Parameter

| Parameter | Description |
| --- | --- |
| sub-command | Stands for a child command. |
| ¦ | Uses the output filter. |
| begin | Means to show the result of the show command starting with a specific word. |
| include | Means to show the sentences of the result of the show command containing a specific word. |
| exclude | Means not to show the lines of the result of the show command containing a specific word. |
| redirect | Redirects the result of the show command to the file in the designated file system. |
| WORD | Stands for a designated word, which is the designated filename as to the redirect command. |
| SEPARATOR WORD | Stands for the designated separator, which is space by default to separate the words. |

Default

None

Command Mode

EXEC mode or global configuration mode

Usage Guidelines

This command can be used to filter the useless information in the result of the show command, especially when the result is too much to read. For example, if you want to browse a designated MAC address in an MAC address table, which contains a lot of MAC addresses, this command will give you convenience for you.

Example

The following example shows how to display the lines, in which the word “interface” is contained, in the result of show running-config.

Switch#show running-config | include interface

Building configuration...

Current configuration:

!

interface GigaEthernet0/1

interface GigaEthernet0/2

interface GigaEthernet0/3

interface GigaEthernet0/4

interface GigaEthernet0/5

interface GigaEthernet0/6

interface GigaEthernet0/7

interface GigaEthernet0/8

Related Command

None

### show alias

Syntax

To display all alias commands, or the alias commands in a specified mode, use the show aliases command.

show alias [<alias name>]

Parameter

| parameter | description |
| --- | --- |
| alias name | Alias command |

Default

Display all alias commands in the format of alias name=command line.

Command Mode

EXEC mode or global configuration mode

Usage Guidelines

None

Example

The following example shows how to use the command to display all alias names:

switch_config#show alias

hualab=date

router=snmp

Related Command

alias

### show break

Syntax

It is used to display the abnormal information of the system. The system stores all abnormal information in the latest running. The abnormal information contains the times of abnormity, the stack content and the invoked functions when abnormity occurs.

Parameter

None

Default

None

Command Mode

Monitor mode

Usage Guidelines

The command is only used for debugging.

Related Command

None

# Terminal Service Configuration Commands

## Telnet Configuration Commands

The chapter describes telnet and related commands. The telnet command is used to establish a session with the remote server. The telnet command is always working at the UNIX operating systems. Option negotiation is required. Telnet does not provide itself the login authentication. Telnet is different from Rlogin because telnet does not provide itself password check.

The telnet configuration commands include:

- telnet

- ip telnet

- where

- disconnect

- resume

- clear Telnet

- show Telnet

- debug Telnet

### telnet

Syntax

The following is a command for establishing a telnet session:

telnet server-ip-addr/server-host-name [/port port][/source-interface interface] [/local local-ip-addr] [/debug][echo/noecho] [/script scriptname]

Parameter

| Parameter | Description |
| --- | --- |
| server-ip-addr | Dotted-decimal IP address of the remote server |
| server-host-name | Name of the remote server, which is configured by the ip host command |
| port | Telnet port of the remote server |
| interface | Local interface where the telnet connection is originated |
| local-ip-addr | Local IP address where the telnet connection is originated |
| /debug | A negotiation process for openning the debug at the client side and printing the connection |
| /echo /noecho | Enable or disable the local echo. The default value is noecho. |
| scriptname | A script name used for auto login |

Default

The default port number is 23. The interface has no default number.

Command Mode

EXEC mode or global configuration mode

Usage Guidelines

You can use one of the following command lines to establish a remote login.

telnet server-ip-addr/server-host-name

In this case, the application program directly sends the telnet login request to port 23 of the remote server. The local IP address is the IP address which is nearest to the peer and found by the routing table.

telnet server-ip-addr/server-host-name /port port

In this case, the application program sends a telnet login request to the port of the peer.

telnet server-ip-addr/server-host-name /source-interface interface

In this case, the application program uses the IP address on the interface ass the local IP address.

telnet server-ip-addr/server-host-name /debug

In this case, the application program opens the debug and exports the connection at the client side.

telnet server-ip-addr/server-host-name echo/noecho

In this case, the application program enables or disables the local echo. The local echo is disabled by default. The echo is completed at the server side. Only when the server is not in charge of echo is the local echo enabled.

telnet server-ip-addr/server-host-name /script scriptname

Before executing the automatic login command of the script, run the command ip telnet script to configure the script.

The previous commands can be used together.

During the session with the remote server, you can press the Q button to exit the session. If the session is not manually closed, the session will be complete after a 10-second timeout.

Example

Suppose you want to telnet server 192.168.20.124, the telnet port of the server is port 23 and port 2323, and the local two interfaces are e1/1(192.168.20.240) and s1/0(202.96.124.240). You can run the following operations to complete the remote login.

1. telnet 192.168.20.124 /port 2323

In this case, the telnet connection with port 2323 of the peer is to be established. The local IP address of the peer is 192.168.20.240.

2. telnet 192.168.20.124 /source-interface s1/0

In this case, the telnet connection with port 23 of the peer is to be established. The local IP address of the peer is 202.96.124.240.

3. telnet 192.168.20.124 /local 192.168.20.240

In this case, the telnet connection with port 23 of the peer is to be established. The local IP address of the peer is 192.168.20.240.

4. telnet 192.168.20.124 /debug

In this case, the telnet connection negotiation with port 23 of the peer will be printed out.

5. telnet 192.168.20.124 /echo

In this case, the local echo is enabled. If the echo is also enabled at the server side, all input will be echoed twice.

6. telnet 192.168.20.124 /script s1

Use login script S1 for automatic login.

### ip telnet

Syntax

The following are the configuration commands of the telnet session:

ip telnet source-interface vlan value

ip telnet access-class accesslist

ip telnet listen-port start-port [end-port]

ip telnet script scriptname ‘user_prompt’ user_answer ‘pwd_prompt’ pwd_answer

Parameter

| Parameter | Description |
| --- | --- |
| num | Maximum number of Telnet connections. |
| value | Local interface where the telnet request is originated |
| accesslist | Access list name to limit the source address when the local client receives the connection |
| start-port | Starting port number designated at the listening port area |
| end-port | End port number designated at the listening port area |
| scriptname | Name of the login script |
| user_prompt | Username prompt returned by the telnet server |
| user_answer | Username response information from the client side |
| pwd_prompt | Password prompt returned by the telnet server |
| pwd_answer | Password response information submitted by the client side |

Default

None

Command Mode

Global configuration

Usage Guidelines

- Run the following command to configure the local interface for originating the telnet connection:

ip telnet source-interface interface

In this case, all telnet connections originated afterwards are through the interface. The configuration command is similar to the command telnet source-interface interface. However, the telnet command has no interface parameters followed. When the interface is configured and the telnet command has interface parameters, the interface followed the telnet command is used.

- Run the following command to configure the name of the access list which performs limitation on local telnet connection reception.

ip telnet access-class accesslist

In this case, the access list will be checked when the server accepts all telnet connections.

- Run the following command to configure a port, except the default port 23, to receive the telnet service.

ip telnet listen-port start-port [end-port]

Usage Guidelines: If the end port number is not designated, the listening will be executed at a specific port. The number of the designated ports cannot be bigger than 16 and the port number ranges between 3001 and 3999.

- Run the following command to configure the telnet login script.

ip telnet script s1 ‘login:’ switch ‘Password:’ test

Usage Guidelines: When the script is configured, the username prompt and password prompt and their answers must be correctly matched, especially the prompt information is capital sensitive and has inverted comma (‘’). If one of them is wrongly configured, the automatic login cannot be performed.

Note:

You can add the NO prefix on the above four commands and then run them to cancel previous configuration.

Example

1. ip telnet source-interface vlan1

In this case, the interface vlan1 will be adopted to originate all telnet connections afterwards.

2. ip telnet access-class abc

In this case, all the received telnet connections use access list abc to perform the access list check.

3. ip telnet listen-port 3001 3010

Except port 23, all ports from port 3001 to port 3010 can receive the telnet connection.

4. ip telnet script s1 ‘login:’ switch ‘Password:’ test

The login script s1 is configured. The username prompt is login: and the answer is switch. The password prompt is Password: and the answer is test.

### ctrl-shift-6+x (the current connection is mounted)

Syntax

Run the following command to mount the current telnet connection:

ctrl-shift-6+x

Parameter

None

Default

None

Command Mode

Any moment in the current telnet session

Usage Guidelines

You can use the shortcut key to mount the current telnet connection at the client side.

Example

switchA>telnet 192.168.20.1

Welcome to Multi-Protocol 2000 Series switch

switchB>ena

switchB#(press ctrl-shift-6+x)

switchA>

You press ctrl-shift-6+x to mount the telnet connection to switch B and return to the current state of switch A.

### where

Syntax

Run the following command to check the currently mounted telnet session:

where

Parameter

None

Default

None

Command Mode

Global configuration

Usage Guidelines

You can use the command to check the mounted outward telnet connection at the client side. The displayed information contains the serial number, peer address, local address and local port.

Note:

The where command is different from the show telnet command. The former is used at the client side and the displayed information is the outward telnet connection. The latter is used at the server and the displayed information is the inward telnet connection.

Example

switchA>telnet 192.168.20.1

Welcome to Multi-Protocol 2000 Series switch

switchB>ena

switchB#(Press ctrl-shift-6+x)

switchA> telnet 192.168.20.2

Welcome to Multi-Protocol 2000 Series switch

switchC>ena

switchC#(Press ctrl-shift-6+x)

switchA>where

NO.         Remote Addr     Remote Port          Local Addr      Local Port

1       192.168.20.1              23      192.168.20.180           20034

2       192.168.20.2              23      192.168.20.180           20035

Enter where at switch A. The mounted outward connection is displayed.

### resume

Syntax

The following command is used to resume the currently mounted outward telnet connection:

resume no

Parameter

| Parameter | Description |
| --- | --- |
| no | Number of the currently mounted telnet session that is checked through the where command |

Default

None

Command Mode

Global configuration

Usage Guidelines

The command can be used to resume the currently mounted outward telnet connection at the client side.

Example

switchA>telnet 192.168.20.1

Welcome to  Multi-Protocol 2000 Series switch

switchB>ena

switchB#( press ctrl-shift-6+x)

switchA> telnet 192.168.20.2

Welcome to Multi-Protocol 2000 Series switch

switchC>ena

switchC#( press ctrl-shift-6+x)

switchA>where

NO.         Remote Addr     Remote Port          Local Addr      Local Port

1       192.168.20.1              23      192.168.20.180           20034

2       192.168.20.2              23      192.168.20.180           20035

switchA>Resume 1

[Resuming connection 1 to 192.168.20.73 . . . ]

(enter)

switchB#

After you enter where at switch A and the mounted outward connection of switch A is displayed. When entering Resume1, you will be prompted that connection 1 is resumed. The command prompts of switch B are displayed after the Enter key is pressed.

### disconnect

Syntax

The following command is used to clear the currently mounted outward telnet session:

disconnect no

Parameter

| Parameter | Description |
| --- | --- |
| No | Number of the currently mounted telnet session that is checked through the where command |

Default

None

Command Mode

Global configuration mode

Usage Guidelines

The command can be used to clear the currently mounted outward telnet connection at the client side.

Note:

The disconnect command is different from the clear telnet command. The former is used at the client side and clears the outward telnet connection. The latter is used at the server and clears the inward telnet connection.

Example

switchA>telnet 192.168.20.1

Welcome to Multi-Protocol 2000 Series switch

switchB>ena

switchB#(press ctrl-shift-6+x)

switchA> telnet 192.168.20.2

Welcome to Multi-Protocol 2000 Series switch

switchC>ena

switchC#(press ctrl-shift-6+x)

switchA>where

NO.         Remote Addr     Remote Port          Local Addr      Local Port

1       192.168.20.1              23      192.168.20.180           20034

2       192.168.20.2              23      192.168.20.180           20035

switchA>disconnect 1

<Closing connection to 192.168.20.1> <y/n>y

Connection closed by remote host.

switchA>

After you enter where at switch A and the mounted outward connection of switch A is displayed, enter disconnect 1. You will be prompted whether the connection of switch B is closed. After you enter Y, the connection is closed.

### clear telnet

Syntax

The following is a command format to clear the telnet session at the server:

clear telnet no

Parameter

| Parameter | Description |
| --- | --- |
| no | Number of the telnet session that is displayed after the show telnet command is run |

Default

None

Command Mode

EXEC mode

Usage Guidelines

The command is used to clear the telnet session at the server.

Example

clear telnet 1

The telnet session whose sequence number is 1 is cleared at the server.

### show telnet

Syntax

The following is a command format to display the telnet session at the server:

show telnet

Parameter

None

Default

None

Command Mode

All command modes except the user mode

Usage Guidelines

The command is used to display the telnet session at the server. The displayed information includes the sequence number, peer address, peer port, local address and local port.

Example

Switch# show telnet

If you run the previous command, the result is shown as follows:

NO.         Remote Addr     Remote Port          Local Addr      Local Port

1      192.168.20.220            1097      192.168.20.240              23

2      192.168.20.180           14034      192.168.20.240              23

### debug telnet

Syntax

The following is a format of the debug command for the telnet session:

debug telnet

Parameter

None

Default

None

Command Mode

EXEC mode

Usage Guidelines

The command is used to enable the telnet debug.

If the switch of the telnet debug is opened, the negotiation processes of all the incoming telnet sessions are printed on the window that the debug command invokes. The debug telnet command is different from the telnet debug command. The former is to export the debug information of the telnet session connected to the server. The latter is to export the debug information of the telnet session that the client originates.

Example

debug telnet

After using the comamnd, the debug information of the telnet session that is connected to the server is displayed.

## Terminal Configuration Commands

The terminal configuration commands include:

- attach-port

- autocommand

- clear line

- connect

- disconnect

- exec-timeout

- length

- line

- location

- login authentication

- monitor

- no debug all

- password

- resume

- show debug

- show line

- terminal-type

- terminal monitor

- terminal width

- terminal length

- where

- width

### attach-port

Syntax

The following command is to bind the telnet listening port to the line vty number and enable the telnet connection at a specific port generates vty according to the designated sequence number.

[no] attach-port PORT

Parameter

| Parameter | Description |
| --- | --- |
| port | Listening port of the telnet server (3001-3999) |

Default

None

Command Mode

Line configuration mode

Example

The following example shows how to bind listening port 3001 to line vty 2 3.

switch_config# line vty 2 3

switch_config_line#attach-port 3001

### autocommand

Syntax

It is used to set the automatically-run command when user logs in to the terminal. The connection is cut off after the command is executed.

autocommand LINE

no autocommand

Parameter

| Parameter | Description |
| --- | --- |
| LINE | Command to be executed |

Command Mode

Line configuration mode

Example

switch_conf#line vty 1

switch_conf_line#autocommand pad 123456

After you successfully log in, the host whose X.121 address is 123456 will be automatically padded.

### clear line

Syntax

To clear the designated line, use the following command:

clear line [aux | tty | vty] [number]

Parameter

Similar to the line command

Command Mode

EXEC mode

Example

switch#clear line vty 0

### connect

Syntax

To connect the telnet server, use the following command:

connect server-ip-addr/server-host-name {[/port port][/source-interface interface] [/local local-ip-addr]} [/script word]

Parameter

| Parameter | Description |
| --- | --- |
| server-ip-addr/server-host-name | IP address of the server or the host name of the server |
| port | Port number |
| interface | Name of the interface where the connection is originated |
| local-ip-addr | Local IP address where the connection is originated |
| word | Name of the script |

Command Mode

All configuration modes

Example

switch# connect 192.168.20.1

### disconnect

Syntax

To delete the mounted telnet session, use the following command:

disconnect N

Parameter

| Parameter | Description |
| --- | --- |
| N | Number of the mounted telnet session |

Command Mode

All configuration modes

Example

switch#disconnect 1

### exec-timeout

Syntax

To set the maximum spare time for the terminal, use the following command:

[no] exec-timeout [time]

Parameter

| Parameter | Description |
| --- | --- |
| time | Spare time whose unit is second.. Range: 0-864000. |

Default

0 (No time-out limitation)

Command Mode

Line configuration mode

Example

Set the spare time of the line to one hour.

switch_config_line#exec-timeout 3600

### length

Syntax

To set the line number on the screen of the terminal, use the following command:

[no] length [value]

Parameter

| Parameter | Description |
| --- | --- |
| value | A value between 0 and 512 The value 0 means there is no pause. |

Default

24

Command Mode

Line configuration mode

### line

Syntax

To enter the line configuration mode, use the following command:

line [aux | console | tty | vty] [number]

Parameter

| Parameter | Description |
| --- | --- |
| console | Monitoring line, which has only one number 0 |
| vty | Virtual lines such as Telnet, PAD and Rlogin |
| number | Number in the line of the type |

Command Mode

Global configuration mode

Example

The following example shows how to enter the line configuration mode of VTY 0 to 10.

switch_config#line vty 0 10

### location

Syntax

To recoded the description of the current line, use the following command. To return to the default setting, use the no form of this command.

location [LINE]

no location

Parameter

| Parameter | Description |
| --- | --- |
| LINE | Description of the current line |

Command Mode

Line configuration mode

### login authentication

Syntax

To set line login authentication parameters, use the following command. To return to the default setting, use the no form of this command.

[no] line login authentication [default | WORD]

Parameter

| Parameter | Description |
| --- | --- |
| Default | Default authentication mode |
| WORD | Name of the authentication list |

Command Mode

Line configuration mode

Example

switch_conf_line#login authentication test

In the example, the authentication list of the line is set to test.

### monitor

Syntax

To export the log and debugging information to the line, use the following command. To return to the default setting, use the no form of this command.

[no] monitor

Parameter

None

Command Mode

Line configuration mode

Example

switch_config_line#monitor

### no debug all

Syntax

To shut down all debugging output of the current VTY, use the following command.

no debug all

Parameter

None

Command Mode

EXEC mode

Example

switch#no debug all

### password

Syntax

To set the password for the terminal, use the following command. To return to the default setting, use the no form of this command.

password {password | [encryption-type] encrypted-password }

no password

Parameter

| Parameter | Description |
| --- | --- |
| password | Password configured on the line, which is entered in the plaintext form and whose maximum length is 30 bits. |
| [encryption-type] encrypted-password | encryption-type means the encryption type of the password. Currently, MY COMPANY products only support two encryption modes: 0 and 7. The number 0 means the password is not encrypted and the plaintext of password is directly entered. It is the same as the way of directly entering the password. The number 7 means the password is encrypted through an algorithm defined by MY COMPANY. You need to enter the encryption text for the encrypted password. The encryption text can be copied from the configuration files of other switches. |

For password encryption, refer to the explanation of the commands service password-encryption and enable password.

Command Mode

Line configuration mode

Example

switch_conf#line vty 1

switch_conf_line#password test

The previous example shows the login password of VTY1 is set to test.

### resume

Syntax

To resume the mounted telnet session, use the following command.

resume N

Parameter

| Parameter | Description |
| --- | --- |
| N | Number of the mounted telnet session |

Command Mode

All configuration modes

Example

switch#resume 1

### show debug

Syntax

To display all debugging information of the current VTY, use the following command.

show debug

Parameter

None

Command Mode

EXEC mode or global configuration mode

Example

Switch# show debug

http authentication debug is on

http cli debug is on

http request debug is on

http response debug is on

http session debug is on

http erro debug is on

http file debug is on

TELNET:

Incoming Telnet debugging is on

### show line

Syntax

To display the status of the current effective line, use the following command.

show line {[console | vty] [number]}}

Parameter

If there is no parameter followed, the status of all effective lines will be displayed.

The definition of other parameters is similar to that of the line command.

Command Mode

All configuration modes except the user mode

### terminal length

Syntax

It is used to change the line number on the current terminal screen. The parameter can be obtained by the remote host. The rlogin protocol uses the parameter to notify the remote UNIX host. Run the no terminal length command to resume the default value:

terminal length length

no terminal length

Parameter

| Parameter | Description |
| --- | --- |
| Length | Line number displayed on each screen Range: 0-512. |

Default

Pause when 24 lines are displayed on the screen.

Command Mode

Global configuration mode

Usage Guidelines

The command is effective only to the current terminal. When the session is complete, the terminal attribute is invalid.

Example

switch#terminal length 40

Related Command

line

### terminal monitor

Syntax

It is used to display the debugging output information and system faulty information at the current terminal. The no form of the command is used to disable the monitoring:

terminal monitor

no terminal monitor

Parameter

None

Default

The system monitoring port (console) is open by default. Other terminals are closed by default.

Command Mode

Global configuration

Usage Guidelines

The command is effective only to the current terminal. When the session is complete, the terminal attribute is invalid.

Example

switch#terminal monitor

Related Command

line

debug

### terminal width

Syntax

In default settings, the switch is to export 80 characters in each line. If the default settings cannot meet your requirements, you can reset it. The parameter can be obtained by the remote host. Run the terminal width command to set the character number in each line. Run the no terminal width command to resume to the default value.

terminal width number

no terminal width

Parameter

| Parameter | Description |
| --- | --- |
| number | Character number of each line |

Default

Display 80 characters in each line

Command Mode

Global configuration

Usage Guidelines

The command is effective only to the current terminal. When the session is complete, the terminal attribute is invalid.

Example

switch#terminal width 40

Related Command

line

### terminal-type

Syntax

To set the terminal type, use the following command.

[no] terminal-type [name]

Parameter

| Parameter | Description |
| --- | --- |
| Name | Terminal name Terminal types currently supported are VT100, ANSI and VT100J. |

Default

ANSI

Command Mode

Line configuration mode

### where

Syntax

To check the currently mounted outward telnet session at the client side, use the following command.

where

Parameter

None

Command Mode

All configuration modes

Example

switch#where

### width

Syntax

To set the terminal width of the line, use the following command. To return to the default setting, use the no form of this command.

[no] width [value]

Parameter

| Parameter | Description |
| --- | --- |
| Value | A value between 0 and 256 The value 0 means no execution. |

Default

80

Command Mode

Line configuration mode

# Maintenance and Debugging Tool Commands

## Network Testing Tool Commands

### ping

Syntax

It is used to test host accessibility and network connectivity. After the ping command is run, an ICMP request message is sent to the destination host, and then the destination host returns an ICMP response message.

ping [-a][-d][-f] [-i {source-ip-address}] [-m {source-interface}] [-j host1 [host2 host3 …]] 		[–k host1 [host2, host3 …]] [-l length] [-n number] [-r hops] [-s tos] [-t ttl] [-v] [-w waittime] [–b interval] [-c] host

Parameter

| Parameter | Description |
| --- | --- |
| -a | Sets ping all along until it is been interrupted. |
| -d | Sets not apply the routing table. |
| -f | Sets the DF digit (message is not segmented). If the message required to be sent is larger than the MTU of the path, the message will be dropped by the routing switch on the path and the routing switch will then return an ICMP error message to the source host. If network performance has problems, one node in the network may be configured to a small MTU. You can use the –f option to decide the smallest MTU on the path. Default: No resetting |
| -i | Sets the source IP address of the message or the IP address of an interface. Default: Main IP address of the message-sending interface |
| source-ip-address | Source IP address adopted by the message |
| source-interface | Message takes the IP address of the source-interface interface as the source address. |
| -j host1 [host2 host3…] | Sets the relaxation source route. Default: Not set |
| -k host1 [host2 host3…] | Sets the strict source route Default: Not set |
| -l length | Sets the length of ICMP data in the message. Default: 56 bytes |
| -n number | Sets the total number of messages. Default: 5 messages |
| -r hops | Records routes. Up to hops routes are recorded. Default: not record |
| -s tos | Sets IP TOS of the message to tos. Default: 0 |
| -t ttl | Sets IP TTL of the message to ttl. Default: 255 |
| -v | Detailed output |
| -w waittime | Time for each message to wait for response Default: 2 seconds |
| -b interval | Sets the time interval of sending ping packet. Unit: 10ms; Value range: 0-65535; Default Value: 0. |
| -c | Simple output |
| host | Destination host |

Command Mode

EXEC mode, global configuration mode

Usage Guidelines

The command supports that the destination address is the broadcast address or the multicast address. If the destination address is the broadcast address (255.255.255.255) or the multicast address, the ICMP request message is sent on all interfaces that support broadcast or multicast. The routing switch is to export the addresses of all response hosts. By pinging multicast address 224.0.0.1, you can obtain the information about all hosts in directly-connected network segment that support multicast transmission.

Press the Q or q key to stop the ping command.

Simple output is adopted by default.

| Parameter | Description |
| --- | --- |
| ! | A response message is received. |
| . | Response message is not received in the timeout time. |
| U | The message that the ICMP destination cannot be reached is received. |
| Q | The ICMP source control message is received. |
| R | The ICMP redirection message is received. |
| T | The ICMP timeout message is received. |
| P | The ICMP parameter problem message is received. |

The statistics information is exported:

| Parameter | Description |
| --- | --- |
| packets transmitted | Number of transmitted messages |
| packets received | Number of received response messages, excluding other ICMP messages |
| packet loss | Rate of messages that are not responded to |
| round-trip min/avg/max | Minimum/average/maximum time of a round trip (ms) |

Example

switch#ping -l 10000 -n 30 192.168.20.125

PING 192.168.20.125 (192.168.20.125): 10000 data bytes

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

--- 192.168.20.125 ping statistics ---

30 packets transmitted, 30 packets received, 0% packet loss

round-trip min/avg/max = 50/64/110 ms

### traceroute

Syntax

It is used to detect which routes have already reached the destination.

You can transmit to the destination the UDP packets (or ICMP ECHO packets) of different TTLs to confirm which routes have come to the destination. Each router on this path has to deduct 1 from the TTL value before forwarding ICMP ECHO packets. Speaking from this aspect, TTL is an effective hop count. When the TTL value of a packet is deducted to zero, the router sends back to the source system the ICMP timeout message.

By checking the ICMP timetout message sent back by intermedial routers, you can confirm the routers. At the arrival of the destination, the traceroute sends a UPD packet whose port ID is larger than 30000; the destination node hence can only transmit back a Port Unreachable ICMP message. This reception of this message means the arrival of destination.

traceroute [-i source-ip-address ] [-m source-interface}] [-j host1 [host2 host3 …]] 		[–k host1 [host2, host3 …]] [-p port-number] [-q probe-count] [-r hops] [-t ttl] [-w waittime] [-x icmp] host

Parameter

| Parameter | Description |
| --- | --- |
| -i source-ip-address | Sets the source IP address of packet. |
| -m source-interface | Sets the packet-transmitted port. |
| -j host1 [host2 host3…] | Sets the loose source route. Default: Not set |
| -k host1 [host2 host3…] | Sets the strict source route. Default: Not set |
| -p port-number | Sets the ID of destination port that transmits UDP packets. Default: 33434 |
| -q probe-count | Sets the number of packets that you detect each time. Default: 3 packets |
| -r hops | Records the routes (at most hops routes can be recorded). Default: The routes are not recorded. |
| -t ttl | Sets the IP TTL of packets as TTL. Default: the minimum and maximum TTLs are 1 and 30 respectively. |
| -w waittime | Means the time that each packet waits for echo. Default: 3 seconds |
| -x icmp | Sets the detection packet to be the ICMP ECHO packet. Default: UDP packet |
| host | Means the destination host. |

Command Mode

EXEC or global configuration mode

Usage Guidelines

The UDP packet is used for detection by default, but you can run –x icmp to replace it with ICMP ECHO for detection.

If you want to stop traceroute, press q or Q.

By default, the simple output information is as follows.

| Parameter | Description |
| --- | --- |
| !N | Receives an ICMP-route unreachable packet. |
| !H | Receives an ICMP-host unreachable packet. |
| !P | Receives an ICMP-protocol unreachable packet. |
| !F | Receives an ICMP unreachable (need to be fragmented) packet. |
| !S | Receive an ICMP unreachable (failing to detect the source-station route) packet. |

The exported statistics information is as follows:

| Parameter | Description |
| --- | --- |
| hops max | Means the maximum detection hops (the threshold of ICMP). |
| byte packets | Stands for the size of each detection packet. |

Example

switch#traceroute 90.1.1.10

traceroute to 90.1.1.10 (90.1.1.10), 30 hops max, 36 byte packets

1  90.2.2.1  0 ms  0 ms  0 ms

2  90.1.1.10  0 ms  0 ms  0 ms

## Fault Diagnosis Commands

The chapter describes the commands used for fault diagnosis. All the following commands are used to detect the reason of the fault. You can use other commands to remove the fault, such as the debug command.

The fault diagnosis commands include:

- logging

- logging buffered

- logging console

- logging facility

- logging monitor

- logging on

- logging trap

- logging command

- logging source-interface

- logging history alerts

- logging history critical

- logging history debugging

- logging history emergencies

- logging history errors

- logging history informational

- logging history notifications

- logging history warnings

- logging history rate-limit

- logging history size

- service timestamps

- clear logging

- show break

- show debug

- show logging

### logging

Syntax

To record the log information to the syslog server, use the following command. To return to the default setting, use the no form of this command.

logging A.B.C.D

no logging A.B.C.D

Parameter

| Parameter | Description |
| --- | --- |
| A.B.C.D | IP address of the syslog server |
| level | Level of log information on the server Refer to table 1. |

Default:

The log information is not recorded to the server.

Command Mode

Global configuration mode

Usage Guidelines

The logging command is used to record the log information to the designated syslog server. It can be used for many times to designate multiple syslog servers.

Example

logging 192.168.1.1 errors

Related Command

logging trap

### logging buffered

Syntax

To record the log information to the memory of the switch, use the following command. To return to the default setting, use the no form of this command.

logging buffered [size | level | dump ]

no logging buffered

Parameter

| Parameter | Description |
| --- | --- |
| size | Size of memory cache Value range: 4096-2147483647 Unit: byte |
| level | Information level of the log recorded to memory cache Refer to table 1. |
| dump | When the system has abnormality, the information in the current memory is currently recorded to the flash and the information is resumed after the system is restarted. |

Default

The information is not recorded to the memory cache.

Command Mode

Global configuration mode

Usage Guidelines

The command records the log information to the memory cache of the switch. The memory cache is circularly used. After the memory cache is fully occupied, the latter information will cover the previous information.

You can use the show logging command to display the log information recorded in the memory cache of the switch.

Do not use big memory for it causes the shortage of memory.

Table 1    Level of log recording

| Prompt | Level | Description | Syslog Definition |
| --- | --- | --- | --- |
| emergencies | 0 | System unusable | LOG_EMERG |
| alerts | 1 | Immediate action needed | LOG_ALERT |
| critical | 2 | Critical conditions | LOG_CRIT |
| errors | 3 | Error conditions | LOG_ERR |
| warnings | 4 | Warning conditions | LOG_WARNING |
| notifications | 5 | Normal but significant condition | LOG_NOTICE |
| informational | 6 | Informational messages only | LOG_INFO |
| debugging | 7 | Debugging messages | LOG_DEBUG |

Related Command

clear logging

show loggin

### logging console

Syntax

Run the command logging console to control the information volume displayed on the console.

Run the command no logging console to forbid the log information to be displayed on the console:

logging console level

no logging console

Parameter

| Parameter | Description |
| --- | --- |
| level | Information level of the logs displayed on the console Refer to table 2. |

Default

The log level displayed on the console port is debugging by default.

Command Mode

Global configuration mode

Usage Guidelines

After the information level is specified, information of this level or the lower level will be displayed on the console.

Run the command show logging to display the currently configured level and the statistics information recorded in the log.

Table 2     Level of log recording

| Prompt | Level | Description | Syslog Definition |
| --- | --- | --- | --- |
| emergencies | 0 | System unusable | LOG_EMERG |
| alerts | 1 | Immediate action needed | LOG_ALERT |
| critical | 2 | Critical conditions | LOG_CRIT |
| errors | 3 | Error conditions | LOG_ERR |
| warnings | 4 | Warning conditions | LOG_WARNING |
| notifications | 5 | Normal but significant condition | LOG_NOTICE |
| informational | 6 | Informational messages only | LOG_INFO |
| debugging | 7 | Debugging messages | LOG_DEBUG |

Example

logging console alerts

Related Command

logging facility

show logging

### logging facility

Syntax

Run the command logging facility to configure to record specified error information. To restore to local7, run the command no logging facility.

logging facility facility-type

no logging facility

Parameter

| Parameter | Description |
| --- | --- |
| facility-type | Facility type Refer to table 3. |

Default

local7

Command Mode

Global configuration

Usage Guidelines

Table 3 Facility type

| Type | Description |
| --- | --- |
| auth | Authorization system |
| cron | Cron facility |
| daemon | System daemon |
| kern | Kernel |
| local0-7 | Reserved for locally defined messages |
| lpr | Line printer system |
| mail | Mail system |
| news | USENET news |
| sys9 | System use |
| sys10 | System use |
| sys11 | System use |
| sys12 | System use |
| sys13 | System use |
| sys14 | System use |
| syslog | System log |
| user | User process |
| uucp | UNIX-to-UNIX copy system |

Example

logging facility kern

Related Command

logging console

### logging monitor

Syntax

Run the command logging monitor to control the information volume displayed on the terminal line.

Run the command no logging monitor to forbid the log information to be displayed on the terminal line.

logging monitor level

no logging monitor

Parameter

| Parameter | Description |
| --- | --- |
| level | Information level of the logs displayed on the terminal line Refer to table 4. |

Default

Debugging

Command Mode

Global configuration mode

Usage Guidelines

Table 4   Level of log recording

| Prompt | Level | Description | Syslog Definition |
| --- | --- | --- | --- |
| emergencies | 0 | System is unusable | LOG_EMERG |
| alerts | 1 | Immediate action needed | LOG_ALERT |
| critical | 2 | Critical conditions | LOG_CRIT |
| errors | 3 | Error conditions | LOG_ERR |
| warnings | 4 | Warning conditions | LOG_WARNING |
| notifications | 5 | Normal but significant condition | LOG_NOTICE |
| informational | 6 | Informational messages only | LOG_INFO |
| debugging | 7 | Debugging messages | LOG_DEBUG |

Example

logging monitor errors

Related Command

terminal monitor

### logging on

Syntax

Run the command logging on to control the recording of error information.

Run the command no logging on to forbid all records.

logging on

no logging on

Parameter

None

Default

logging on

Command Mode

Global configuration mode

Example

switch_config# logging on

switch_config# ^Z

switch#

Configured from console 0 by DEFAULT

switch# ping 192.167.1.1

switch#ping 192.167.1.1

PING 192.167.1.1 (192.167.1.1): 56 data bytes

!!!!!

--- 192.167.1.1 ping statistics ---

5 packets transmitted, 5 packets received, 0% packet loss

round-trip min/avg/max = 0/4/10 ms

switch#IP: s=192.167.1.111 (local), d=192.167.1.1 (FastEthernet0/0), g=192.167.1.1, len=84, sending

IP: s=192.167.1.1 (FastEthernet0/0), d=192.167.1.111 (FastEthernet0/0), len=84,rcvd

IP: s=192.167.1.111 (local), d=192.167.1.1 (FastEthernet0/0), g=192.167.1.1, len=84, sending

IP: s=192.167.1.1 (FastEthernet0/0), d=192.167.1.111 (FastEthernet0/0), len=84,rcvd

IP: s=192.167.1.111 (local), d=192.167.1.1 (FastEthernet0/0), g=192.167.1.1, len=84, sending

IP: s=192.167.1.1 (FastEthernet0/0), d=192.167.1.111 (FastEthernet0/0), len=84,rcvd

IP: s=192.167.1.111 (local), d=192.167.1.1 (FastEthernet0/0), g=192.167.1.1, len=84, sending

IP: s=192.167.1.1 (FastEthernet0/0), d=192.167.1.111 (FastEthernet0/0), len=84,rcvd

IP: s=192.167.1.111 (local), d=192.167.1.1 (FastEthernet0/0), g=192.167.1.1, len=84, sending

IP: s=192.167.1.1 (FastEthernet0/0), d=192.167.1.111 (FastEthernet0/0), len=84,rcvd

switch_config# no logging on

switch_config# ^Z

switch#

switch# ping 192.167.1.1

PING 192.167.1.1 (192.167.1.1): 56 data bytes

!!!!!

--- 192.167.1.1 ping statistics ---

5 packets transmitted, 5 packets received, 0% packet loss

round-trip min/avg/max = 0/4/10 ms

Related Command

logging

logging buffered

logging monitor

logging console

### logging trap

Syntax

Run the command logging trap to control the information volume recorded to the syslog server.

Run the command no logging trap to forbid the information to be recorded to the syslog server.

logging trap level

no logging trap

Parameter

| Parameter | Description |
| --- | --- |
| level | Information level of the logs displayed on the syslog server Refer to table 5. |

Default

Informational

Command Mode

Global configuration mode

Usage Guidelines

Table 5   Level of log recording

| Prompt | Level | Description | Syslog Definition |
| --- | --- | --- | --- |
| emergencies | 0 | System is unusable | LOG_EMERG |
| alerts | 1 | Immediate action needed | LOG_ALERT |
| critical | 2 | Critical conditions | LOG_CRIT |
| errors | 3 | Error conditions | LOG_ERR |
| warnings | 4 | Warning conditions | LOG_WARNING |
| notifications | 5 | Normal but significant condition | LOG_NOTICE |
| informational | 6 | Informational messages only | LOG_INFO |
| debugging | 7 | Debugging messages | LOG_DEBUG |

Example

logging 192.168.1.1

logging trap notifications

Related Command

logging

### logging command

Syntax

To enable the command execution recording, run logging command. After this function is opened, a log will be generated for each of all entered commands, in which the line to execute this command, the command line, the execution result, the login line and the login address will be recorded.

You can use no logging command to disable this function.

Parameter

None

Default

no logging command

Command Mode

Global configuration mode

Example

Switch_config#logging command

Switch_config#Jul 11 15:26:56 %CMD-6-EXECUTE: `logging command ` return 0, switch(vty 0, 192.168.25.42).

Related Command

logging

### logging source-interface

Syntax

This command is used to set the source port of log exchange.

You can use no logging source-interface to disable this function.

Parameter

None

Default

no logging source-interface

Command Mode

Global configuration mode

Example

Switch_config# logging source-interface vlan 1

Related Command

logging

### logging history alerts

Syntax

This command is used to set the level of the historical log table to alerts (need to act immediately).

Parameter

None

Default

logging history warnings

Command Mode

Global configuration mode

Example

Switch_config#logging history alerts

Related Command

logging

### logging history critical

Syntax

This command is used to set the level of the historical log table to critical.

Parameter

None

Default

logging history warnings

Command Mode

Global configuration mode

Example

Switch_config#logging history critical

Related Command

logging

### logging history debugging

Syntax

This command is used to set the level of the historical log table to debugging.

Parameter

None

Default

logging history warnings

Command Mode

Global configuration mode

Example

Switch_config#logging history debugging

Related Command

logging

### logging history emergencies

Syntax

This command is used to set the level of the historical log table to emergencies (system unavailable).

Parameter

None

Default

logging history warnings

Command Mode

Global configuration mode

Example

Switch_config#logging history emergencies

Related Command

logging

### logging history errors

Syntax

This command is used to set the level of the historical log table to errors.

Parameter

None

Default

logging history warnings

Command Mode

Global configuration mode

Example

Switch_config#logging history errors

Related Command

logging

### logging history informational

Syntax

This command is used to set the level of the historical log table to informational.

Parameter

None

Default

logging history warnings

Command Mode

Global configuration mode

Example

Switch_config#logging history informational

Related Command

logging

### logging history notifications

Syntax

This command is used to set the level of the historical log table to notifications (normal but important condition).

Parameter

None

Default

logging history warnings

Command Mode

Global configuration mode

Example

Switch_config#logging history notifications

Related Command

logging

### logging history warnings

Syntax

This command is used to set the level of the historical log table to warnings.

Parameter

None

Default

logging history warnings

Command Mode

Global configuration mode

Example

Switch_config#logging history warnings

Related Command

logging

### logging history rate-limit

Syntax

This command is used to set the log output rate.

Parameter

| Parameter | Description |
| --- | --- |
| <1-512> | Stands for the number of logs which are exported each second. |

Default

logging history rate-limit 0

Command Mode

Global configuration mode

Example

Switch_config#logging history rate-limit 256

Related Command

logging

### logging history size

Syntax

This command is used to set the number of entries in the historical log table.

Parameter

| Parameter | Description |
| --- | --- |
| <0-500> | Stands for the number of historical log entries. |

Default

logging history size 0

Command Mode

Global configuration mode

Example

Switch_config#logging history size 256

Related Command

logging

### service timestamps

Syntax

Run the command service timestamps to configure the time stamp that is added when the system is debugged or records the log information.

Run the command no service timestamps to cancel the time stamp that is added when the system is debugged or records the log information.

service timestamps [log|debug] [uptime| datetime]

no service timestamps [log|debug]

Parameter

| Parameter | Description |
| --- | --- |
| log | Adds the time stamp before the log information. |
| debug | Adds the time stamp before the debug information. |
| uptime | Duration between the startup of the switch and the current time |
| datetime | Real-time clock time |

Default

Service timestamps log date

Service timestamps debug date

Command Mode

Global configuration mode

Usage Guidelines

The time stamp in the uptime form is displayed like HHHH:MM:SS, meaning the duration from the start-up of the switch to the current time.

The time stamp in the date form is displayed like YEAR-MON-DAY HH:MM:SS, meaning the real-time clock time.

Example

Service timestamps debug uptime

### clear logging

Syntax

It is used to clear the log information recorded in the memory cache.

clear logging

Parameter

None

Command Mode

EXEC mode

Related Command

logging buffered

show logging

### show break

Syntax

It is used to display the information about abnormal breakdown of the switch.

show break

Parameter

None

Default

None

Command Mode

EXEC mode

Usage Guidelines

It is used to display the information about abnormal breakdown of the switch, helping to find the cause of the abnormality.

Example

switch#show break

Exception Type:1400-Data TLB error

BreakNum: 1 s date: 2000-1-1  time: 0:34:6

r0       r1       r2       r3       r4       r5       r6

00008538-01dbc970-0054ca18-00000003-80808080-fefefeff-01dbcca1-

r7       r8       r9       r10      r11      r12      r13

00000000-00009032-00000000-7ffffff0-00008588-44444444-0054c190-

r14      r15      r16      r17      r18      r19      r20

000083f4-000083f4-00000000-00000000-00000000-00000000-00000000-

r21      r22      r23      r24      r25      r26      r27

00000000-0000000a-00000001-00000000-00000000-004d6ce8-01dbd15c-

r28      r29      r30      r31      spr8     spr9     ip

00000002-00467078-00010300-00000300-00000310-00008588-00000370-

Variables :

00008538-44444444-01dbd15c-01dbcaac-00000002-00000000-004d6ce8-

01dbca18-

00008538 ---  do_chram_mem_sys_addr---bspcfg.o

0001060c ---  subcmd---cmdparse.o---libcmd.a

000083e4 ---  do_chram_mem_sys---bspcfg.o

0000fb24 ---  lookupcmd---cmdparse.o---libcmd.a

0000f05c ---  cmdparse---cmdparse.o---libcmd.a

003e220c ---  vty---vty.o---libvty.a

00499820 ---  pSOS_qcv_broadcast---ksppc.o---os\libsys.a

The whole displayed content can be divided into six parts:

1   RROR:file function.map not found

The prompt information means that the system has not been installed the software function.map, which does not affect the system running.

If the version of the software function.map is not consistent with that of the switch, the system prompts that the version is not consistent.

2   Exception Type—Abnormal hex code plus abnormal name

3   BreakNum

It is the current abnormal number. It means the number of abnormalities that the system has since it is powered on in the latest time. It is followed by the time when the abnormality occurs.

4  Content of the register

The common content of the register is listed out.

5  Variable area

The content in the stack is listed out.

6  Calling relationship of the number

If the map file is not installed on the system, only the function's address is displayed. If the map file is installed on the system, the corresponding function name, .o file name and .a file name are displayed.

The calling relationship is from bottom to top.

### show debug

Syntax

It is used to display all the enabled debugging options of the switch.

show debug

Parameter

None

Command Mode

EXEC mode

Example

switch# show debug

Crypto Subsystem:

Crypto Ipsec debugging is on

Crypto Isakmp debugging is on

Crypto Packet debugging is on

Related Command

debug

### show logging

Syntax

It is used to display the state of logging (syslog).

show logging

Parameter

None

Command Mode

EXEC mode

Usage Guidelines

It is used to display the state of logging (syslog), including the login information about the console, monitor and syslog.

Example

switch# show logging

Syslog logging: enabled (0 messages dropped, 0 flushes, 0 overruns)

Console logging: level debugging, 12 messages logged

Monitor logging: level debugging, 0 messages logged

Buffer logging: level debugging, 4 messages logged

Trap logging: level informations, 0 message lines logged

Log Buffer (4096 bytes):

2000-1-4 00:30:11 Configured from console 0 by DEFAULT

2000-1-4 00:30:28 User DEFAULT enter privilege mode from console 0, level = 15

Related Command

clear logging

# SSH Configuration Commands

### ip sshd enable

Syntax

ip sshd enable

no ip sshd enable

Parameter

None

Default

Disable

Usage Guidelines

It is used to generate the rsa encryption key and then monitor the connection to the ssh server. The process of generating encryption key is a process of consuming the calculation time. It takes one or two minutes.

Command Mode

Global configuration mode

Example

In the following example, the SSH service is generated.

device_config#ip sshd enable

### ip sshd timeout

Syntax

ip sshd timout time-length

no ip timeout

Parameter

| Parameter | Description |
| --- | --- |
| time-length | Maximum time from the establishment of connection to the authentication approval Value range: 60-65535 |

Default

180 seconds

Usage Guidelines

To prevent the illegal user from occupying the connection resources, the connections that are not approved will be shut down after the set duration is exceeded.

Command Mode

Global configuration mode

Example

In the following example, the timeout time is set to 360 seconds:

device_config#ip sshd timeout 360

### ip sshd auth-method

Syntax

ip sshd auth-method method

no sshd auth-method

Parameter

| Parameter | Description |
| --- | --- |
| method | Sets authentication method list. The length of the authentication method's name is no more than 20 characters. |

Default

The default authentication method list is used.

Usage Guidelines

The ssh server uses the authentication method list of the login type.

Command Mode

Global configuration mode

Example

In the following example, an auth-ssh authentication method list is configured and it is applied to the ssh server:

device_config#aaa authentication login auth-ssh local

device_config#ip sshd auth-method auth-ssh

### ip sshd access-class

Syntax

ip sshd access-class access-list

no ip sshd access-class

Parameter

| Parameter | Description |
| --- | --- |
| access-list | Standard IP access list. The length of the access list's name is no more than 20 characters. |

Default

No access control list

Usage Guidelines

It is used to configure the access control list for the ssh server. Only the connections complying with the regulations in the access control list can be approved.

Command Mode

Global configuration mode

Example

In the following example, an ssh-accesslist access control list is configured and applied in the ssh server:

device_config# ip access-list standard ssh-accesslist

device_config_std_nacl#deny 192.168.20.40

device_config#ip sshd access-class ssh-accesslist

### ip sshd auth-retries

Syntax

ip sshd auth-retries times

no ip sshd auth-retries

Parameter

| Parameter | Description |
| --- | --- |
| times | Maximum re-authentication times Value range: 0-65535 |

Default

6 times

Usage Guidelines

The connection will be shut down when the re-authentication times exceeds the set times.

Command Mode

Global configuration mode

Example

In the following example, the maximum re-authentication times is set to five times:

device_config#ip sshd auth-retries 5

### ip sshd clear

Syntax

ip sshd clear ID

Parameter

| Parameter | Description |
| --- | --- |
| ID | Number of the SSH connection to the local device Value range: 0-65535 |

Default

None

Usage Guidelines

It is used to mandatorily close the incoming ssh connection with the specified number. You can run the command show ip sshd line to check the current incoming connection’s number.

Command Mode

Global configuration mode

Example

In the following example, the No.0 incoming connection is mandatorily closed:

device_config#ip sshd clear 0

### ip sshd silence-period

Syntax

ip sshd silence-period time-length

no ip sshd silence-period

Parameter

| Parameter | Description |
| --- | --- |
| time-length | Means the time of the silence, which ranges from 0 to 3600. |

Default

60s

Usage Guidelines

This command is used to set the login silence period. After the accumulated login failures exceed a certain threshold, the system regards that there exist attacks and disables the SSH service in a period of time, that is, the system enters the login silence period.

The silence period is set by the ip sshd silence-period command. The default silence period is 60 seconds. The allowable login failures are set by the ip sshd auth-retries command, whose default value is 6.

Command Mode

Global configuration mode

Example

The following example shows how to set the silence period to 200 seconds.

switch_config#ip sshd silence-period 200

### ip sshd sftp

Syntax

ip sshd sftp

no ip sshd sftp

Parameter

None

Default

None

Usage Guidelines

This command is used to enable the SFTP function. The SFTP function refers to the secure file transmission system based on SSH, of which the authentication procedure and data transmission are encrypted. Though it has low transmission efficiency, network security is highly improved.

Command Mode

Global configuration mode

Example

The following example shows how to enable the SFTP function.

switch_config#ip sshd sftp

### ip sshd save

Syntax

ip sshd save

no ip sshd save

Parameter

None

Default

None

Usage Guidelines

This command is used to save the initial key. When the SSH server is restarted, the key will be first read from the flash; if the key reading is successful, the recalculation of key will be avoided and the startup time will be shortened.

Command Mode

Global configuration mode

Example

The following example shows how to enable the key protection function.

switch_config#ip sshd save

### ip sshd disable-aes

Syntax

ip sshd disable-aes

no ip sshd disable-aes

Parameter

None

Default

The AES encryption algorithm is forbidden.

Usage Guidelines

This command is used to decide whether to use the AES algorithm during the encryption algorithm negotiation. The AES algorithms such as aes128-cbc and aes256-cbc are not used by default.

Command Mode

Global configuration mode

Example

The following example shows how to disable the AES encryption algorithm.

switch_config#ip sshd disable-aes

### ssh

Syntax

ssh –l userid –d destIP [-c {des|3des|blowfish }] [-o numberofpasswdprompts] [-p port] [-v {1|2}] [-s password]

Parameter

| Parameter | Description |
| --- | --- |
| –l userid | User account on the server |
| –d destI | Destination IP address in the dotted decimal system |
| -o numberofpasswdprompts | Re-authentication times after the first authentication fails Actual re-authentication times is the set value plus the smallest value set on the server. Its default value is three times. Value range: 0-65535 |
| -p port | Port number that the server monitors Its default value is 22. Value range: 0-65535 |
| -c {des¦3des¦blowfish} | Encryption algorithm used during communication The encryption algorithm is 3des by default. |
| -v version | Specified version No. |
| -s password | Set the password |

Default

None

Usage Guidelines

The command is used to create a connection with the remote ssh server.

Command Mode

Privileged mode

Example

In the following example, a connection with the ssh server whose IP address is 192.168.20.41 is created. The account is zmz and the encryption algorithm is blowfish:

device#ip ssh –l zmz –d 192.168.20.41 –c blowfish

### show ssh

Syntax

show ssh

Parameter

None

Default

None

Usage Guidelines

It is used to display the sessions on the ssh server.

Command Mode

Privileged mode

Example

In the following example, the sessions on the ssh server are displayed:

device#show ssh

### show ip sshd

Syntax

show ip sshd

Parameter

None

Default

None

Usage Guidelines

It is used to display the current state of the ssh server.

Command Mode

Privileged mode

Example

In the following example, the current state of the ssh server is displayed:

device#show ip sshd