- IGMP Proxy Commands

- Table of Contents

Chapter 1 IGMP Proxy Commands	1

1.1 ip igmp-proxy enable	1

1.2 ip igmp-proxy agent-vlan client-vlan map	2

1.3 ip igmp-proxy source svlan sport	2

1.4 show ip igmp-proxy mcache	3

1.5 show ip igmp-proxy	4

1.6 debug ip igmp-proxy	5

# IGMP Proxy Commands

IGMP-proxy commands include:

- Ip igmp-proxy

- ip igmp-proxy agent-vlan client-vlan map

- show ip igmp-proxy

- debug ip igmp-proxy

## ip igmp-proxy enable

- Syntax

ip igmp-proxy enable

no ip igmp-proxy enable

To enable or disable igmp-proxy, run ip igmp-proxy enable. To resume the default value, run no ip igmp-proxy enable.  注意igmp-proxy功能以igmp-snooping为前提，因此配置该命令之前，首先要使能igmp-snooping。

- Parameter

N/A

- Default value

The IGMP proxy is disabled by default.

- Usage guidelines

N/A

- Example

The following example shows how to enable the igmp-proxy:

switch_config# ip igmp-snooping

switch_config# ip igmp-proxy enable

switch_config#

## ip igmp-proxy agent-vlan client-vlan map

- Syntax

ip igmp-proxy agent-vlan avlan_map  client-vlan map  cvlan_map

no ip igmp-proxy agent-vlan avlan_map  client-vlan map  cvlan_map

- Parameter

| Parameter | Description |
| --- | --- |
| avlan_map | Agent VLAN |
| cvlan_map | Customer VLAN |

- Default value

N/A

- Usage guidelines

1. The represented VLAN cannot be configured before vlan is designated by avlan_map; also, the agent VLAN cannot be configured before cvlan_map.

2. The represented and agent VLANs must accept the control of igmp-snooping.

- Example

The following example shows how to set vlan2 to be the agent vlan of vlan1.

switch_config# ip igmp-proxy agent-vlan  2  client-vlan map  1

switch_config#

## show ip igmp-proxy

- Syntax

show ip igmp-proxy

- Parameter

N/A

- Default value

N/A

- Usage guidelines

This command is used to display the configurations about IGMP proxy.

- Example

The following example shows how to display the configurations of IGMP proxy.

switch#show ip igmp-proxy

Global IGMP proxy configuration:

--------------------------------

Globally enable	: Enable

Agent: vlan 2, it has been alloced

----------------

Client static   : vlan 1

Client auto     : vlan

Agent: vlan 3, it has been alloced

----------------

Client static   : vlan 1

Client auto     : vlan

Globally clients list

---------------------

Client          : vlan 1, it has been alloced

Static Multi-source list

---------------------

Static Source   : No Static Source

switch#

## debug ip igmp-proxy

- Syntax

debug ip igmp-proxy [error | event | packet]

no debug ip igmp-proxy [error | event | packet]

- Parameter

| Parameter | Description |
| --- | --- |
| event | Debugs the IGMP-snooping event. |
| packet | Debugs the reception and transmission of IGMP-snooping. |
| error | Debugs the IGMP-snooping errors. |

- Default value

N/A

- Usage guidelines

This command is used to enable or disable the debugging & printing switch of IGMP-proxy.