Reliability Configuration Commands

Table of Contents

Chapter 1 VRRP Configuration Commands	1

1.1 VRRP Configuration Commands	1

1.1.1 vrrp associate	1

1.1.2 vrrp authentication	2

1.1.3 vrrp description	2

1.1.4 vrrp preempt	3

1.1.5 vrrp preempt delay	4

1.1.6 vrrp priority	4

1.1.7 vrrp source-mac-use-system	5

1.1.8 vrrp timers	5

1.1.9 vrrp track	6

1.1.10 show vrrp	8

1.1.11 debug vrrp	9

# Chapter 1 VRRP Configuration Commands

## VRRP Configuration Commands

### vrrp associate

Syntax

To configure the virtual IP address of VRRP, run the following command.

vrrp [vrid] associate virtual-address virtual-mask [secondary]

To return to the default setting, use the no form of this command.

no vrrp [vrid] associate [virtual-address virtual-mask] [secondary]

Parameter

| Parameter | Description |
| --- | --- |
| vrid | The virtual routing switch ID. The value ranges from 0 to 255. The default value is 0. |
| virtual-address | The virtual IP address |
| virtual-mask | The virtual IP address mask |

Default Value

Disabled

Usage Guidelines

The virtual IP address must be in the same network range of the interface IP, or the virtual routing switch won’t work.

When the virtual IP address is identical to the interface IP address, the system will automatically advance routing switch to 255.

The virtual routing switch may configure multiple IP addresses, among which one is as the master address and the others are as secondary address. The virtual IP secondary address is configured by command with secondary parameters.

Example

The following example shows how to enable vrrp group 1 on interface vlan 1 and the configured virtual IP address is 192.168.20.100:

Switch_config_v1#vrrp 1 associate 192.168.20.100 255.255.255.0

### vrrp authentication

Syntax

To configure simple-text authentication method of interface VRRP, run the following command.

vrrp [vrid] authentication WORD

To return to the default setting, use the no form of this command.

no vrrp [vrid] authentication

Parameter

| Parameter | Description |
| --- | --- |
| vrid | ID of the virtual routing switch. The value ranges from 0 to 255. The default value is 0. |
| WORD | The verification plaintext. The length cannot exceed 8 characters. |

Default Value

no authentication

Usage Guidelines

The virtual routing switch group which makes up of the virtual routing switch must have the identical authentication method.

Example

The following example configures authentication method of the virtual routing switch 1 on interface vlan 1 to simple-text and the authentication character string is test:

Switch_config_v1#vrrp 1 authentication test

### vrrp description

Syntax

To configure the description information of VRRP, run the following command:

vrrp [vrid] description WORD

To return to the default setting, use the no form of this command.

no vrrp [vrid] description

Parameter

| Parameter | Description |
| --- | --- |
| vrid | The virtual routing switch ID. The value ranges from 0 to 255. The default value is 0. |
| WORD | The description character string. The length cannot outnumber 64 characters. |

Default Value

No description.

Usage Guidelines

None

Example

The following example shows how to configure the description information of the virtual routing switch 1 on switch VLAN1 interface to GROUP1:

Switch_config_v1# vrrp 1 description GROUP1

### vrrp preempt

Syntax

To configure Virtual Router Redundancy Protocol (VRRP) preempt, run the following command. To return to the default setting, use the no form of this command.

vrrp [vrid] preempt

no vrrp [vrid] preempt

Parameter

| parameter | Description |
| --- | --- |
| vrid | ID of the virtual routing switch. The value ranges from 0 to 255. The default value is 0. |

Default Value

Enabled

Command Mode

VLAN interface configuration mode

Usage Guidelines

When VRRP preempt is enabled, the high priority switch in the virtual routing switch of the same group will preempt that of the low priority. When VRRP preempt is disabled, the high priority switch will not preempt that of the low priority.

Example

The following example disables preempt of virtual routing switch 1 on interface vlan 1:

Switch_config_v1# no vrrp 1 preempt

### vrrp preempt delay

Syntax

To configure the priority preempt delay of VRRP, run the following command.

vrrp [vrid] preempt delay second

To return to the default setting, use the no form of this command.

no vrrp [vrid] preempt delay second

Parameter

| Parameter | Description |
| --- | --- |
| vrid | The virtual routing switch ID, The value ranges from 0 to 255. The default value is 0. |
| second | The priority preempt delay, 0 to 255, unit: second |

Default Value

The priority preempt delay is 0 second.

Usage Guidelines

None

Example

The following example shows how to set the priority preempt delay of the virtual routing switch 1 on the switch VLAN interface to 10 seconds.

Switch_config_v1# vrrp 1 preempt delay 10

### vrrp priority

Syntax

To configure Virtual Router Redundancy Protocol (VRRP) priority value, run the following command. To return to the default setting, use the no form of this command.

vrrp [vrid] priority value

no vrrp [vrid] priority

Parameter

| Parameter | Description |
| --- | --- |
| vrid | ID of virtual routing switch. The value ranges from 0 to 255. The default value is 0. |
| value | Priority value. The value ranges from 1 to 254. |

Default Value

100

Usage Guidelines

None

Example

The following example configures priority value of the virtual routing switch 1 on interface vlan 1 to 120.

Switch_config_v1#vrrp 1 priority 120

### vrrp source-mac-use-system

Syntax

To configure the source mac address of the VRRP protocol packet, run the following command. To return to the default setting, use the no form of this command.

[no] vrrp [vrid] source-mac-use-system

Parameter

| Parameter | Description |
| --- | --- |
| vrid | The virtual routing switch ID. The value ranges from 0 to 255. The default value is 0. |

Default Value

The protocol mac address is used by default.

Usage Guidelines

After the command is configured, VRRP packet will use the system mac as the source mac address of the protocol packet.

Example

The following example shows how the virtual routing switch1 of VLAN1 interface forwards the protocol packet by the system mac address.

Switch_config_v1#vrrp 1 source-mac-use-system

### vrrp timers

Syntax

vrrp [vrid] timers advertise {second | dsec decisecond | csec centisecond}

no vrrp [vrid] timers advertise

To configure announce timer of vrrp, run the following command. To return to the default setting, use the no form of this command.

vrrp [vrid] timers learn

no vrrp [vrid] timers learn

To configure announce timer to the learning mode, run the following command. To return to the default setting, use the no form of this command.

Parameter

| Parameter | Description |
| --- | --- |
| vrid | ID of virtual routing switch. The value ranges from 0 to 255. The default value is 0. |
| second | Announce timer value; unit: second; the value ranges from 1 to 255. |
| decisecond | Announce timer value, unit: 0.1s; the value ranges from 5 to 360. |
| centisecond | Announce timer value, unit: 0.01s; the value ranges from 1 to 3600. |

Default Value

1 sec

Usage Guidelines

The timer value will decide the minimum time that the virtual routing switch recovers from the error. When the link of the master routing switch is down, the backup routing switch will transit to master routing switch in 3*advertisement + skew_time interval. The large value of the advertisement is obviously nor favorable for error recovery. We recommend user to use the default value.

When configured in learning mode, the virtual routing switch will learn the announce clock value from the message of the master routing switch.

Example

The following command configures announce timer value of virtual routing switch 1 on interface vlan 1 to 2 seconds:

Switch_config_v1#vrrp 1 timers advertise 2

### vrrp track

Syntax

To configure VRRP monitoring function, run the following command. To return to the default setting, use the no form of this command.

vrrp [vrid] track interface intf-id value

no vrrp [vrid] track interface intf-id

vrrp [vrid] track ip ip-address value

no vrrp [vrid] track ip ip-address

To configure BFD static detection, run the following command. If next jump is reachable, the objective node route is reachable. To return to the default setting, use the no form of this command.

ip route bfd static next-hop

no ip route bfd static next-hop

To configure BFD detection static routing function, run the following command. If the configured gateway address is reachable, the objective node route is reachable. To return to the default setting, use the no form of this command.

ip route bfd static A.B.C.D

no ip route bfd static A.B.C.D

Parameter

| Parameter | Description |
| --- | --- |
| vrid | The virtual routing switch ID. The value ranges from 1 to 255. The default value is 0. |
| intf-id | The concrete physical port of the monitor |
| ip-address | The IP address of the monitor node |
| value | The priority penalty value. The value ranges from 1 to 254. The configured penalty value should not be greater than the priority of the current vrrp group. |
| A.B.C.D | The gateway address when BFD detecting the static routing. |

Default Value

None

Usage Guidelines

With the monitoring function, VRRP group can appropriately adjust its priority according to the link state change and provide an opportunity of switching the main line state to the backup line state. The link state change refers to the destination link passing through VRRP routing switch is reachable or not, rather than that of VRRP routing switch itself.

VRRP supports two monitoring objects. First, monitoring interface state. When the monitored interface link state is down, lower the priority itself actively; second, monitor the static routing state of the designated node. When the monitored route is unreachable, lower the priority itself actively. Monitoring the static routing state of the designated node needs BFD static route detection function.

Example

The following example shows how to set vlan1 virtual routing switch1 monitoring the physical port g5/1. When the physical port g5/1 state is down, lower itself priority 20.

Switch_config_v1#vrrp 1 track interface GigaEthernet0/1 20

The following example shows how vlan1 port virtual routing switch 1 monitors the static routing state of IP address 1.1.1.254. After the corresponding static route and BFD is configured and the routing state of 1.1.1.254 is unreachable, lower itself priority 20:

Switch_config#ip route 1.1.1.254 255.255.255.255 1.1.1.100

Switch_config#ip route bfd static next-hop

Switch_config_v1#vrrp 1 track ip 1.1.1.254 20

### show vrrp

Syntax

To display Virtual Router Redundancy Protocol (VRRP) information, run the following command. To return to the default setting, use the no form of this command.

show vrrp brief

show vrrp [interface intf-id] detail

Parameter

| Parameter | Description |
| --- | --- |
| intf-id | The concrete physical interface. |

Default Value

None

Usage Guidelines

The command is used to show VRRP information.

Example

The following example displays vrrp information on interface vlan 1:

Switch_config#show vrrp interface vlan 1 detail

VLAN1 - Group 1

VRRP State is Master

Virtual IP address : 192.168.20.110/24

Virtual Mac address : 0000.5e00.0101

Current Priority : 100   (Config 100)

VRRP timer : Advertise 1.0 s (default)   master_down 3.6 s

VRRP current timer : Advertise 1.0 s master_down 0.0 s preempt after 0.0 s

Authentication string is not set

Preempt is set  (delay : 0  s)

Learn Advertise Interval is not set

Master Router IP : 192.168.20.118, priority : 100, advertisment :  1.0 s

### debug vrrp

Syntax

To enable debugging VRRP information, use the debug vrrp command. Use the no form of this command to disable debugging VRRP information.

debug vrrp [interface intf-id vrid] {errors | events | packets | all}

no debug vrrp

Parameter

| Parameter | Description |
| --- | --- |
| intf-id | Concrete physical port |
| vrid | Virtual routing switch ID. The value ranges from 0 to 255. The default value is 0. |

Default Value

Disabled

Usage Guidelines

None

Example

The following command shows how to enable VRRP debugging switch:

Switch# debug vrrp packets