Multicast Configuration commands

Table of contents

Chapter 1 Basic Multicast Commands	1

1.1 Basic Multicast Commands	1

1.1.1 debug ip mpacket	2

1.1.2 debug ip mroute-cache	3

1.1.3 debug ip multicast	4

1.1.4 ip multicast-statistic	5

1.1.5 ip multicast-routing	6

1.1.6 ip multicast route-limit	6

1.1.7 ip multicast boundary	7

1.1.8 ip multicast helper-map	8

1.1.9 ip multicast ttl-threshold	11

1.1.10 show ip mroute-cache	12

1.1.11 show ip mroute mfc	13

Chapter 2 IGMP Configuration Commands	15

2.1 IGMP Configuration Commands	15

2.1.1 clear ip igmp group	15

2.1.2 debug ip igmp	16

2.1.3 debug ip igmp-host	17

2.1.4 ip igmp helper-address	18

2.1.5 ip igmp join-group	19

2.1.6 ip igmp immediate-leave group-list	20

2.1.7 ip igmp last-member-query-interval	21

2.1.8 ip igmp querier-timeout	22

2.1.9 ip igmp query-interval	23

2.1.10 ip igmp query-max-response-time	24

2.1.11 ip igmp static-group	25

2.1.12 ip igmp version	26

2.1.13 show ip igmp groups	27

2.1.14 show ip igmp interface	29

2.1.15 show ip igmp-host	30

Chapter 3 PIM-DM Configuration Commands	32

3.1 PIM-DM Configuration Commands	32

3.1.1 clear ip mroute pim-dm	32

3.1.2 debug ip pim-dm	33

3.1.3 ip pim-dm	36

3.1.4 ip pim-dm dr-priority	37

3.1.5 ip pim-dm hello-interval	38

3.1.6 ip pim-dm state-refresh origination-interval	39

3.1.7 ip pim-dm neighor-filter	40

3.1.8 show ip pim-dm neighbor	41

3.1.9 show ip pim-dm interface	42

3.1.10 show ip rpf pim-dm	43

Chapter 4 PIM-SM Configuration Commands	45

4.1 PIM-SM Configuration Commands	45

4.1.1 ip pim-sm	47

4.1.2 ip pim-sm admin-scope	48

4.1.3 ip pim-sm asrt-hold	49

4.1.4 ip pim-sm bsr-border	50

4.1.5 ip pim-sm dr-pri	51

4.1.6 ip pim-sm hello-intvl	52

4.1.7 ip pim-sm holdtime	53

4.1.8 ip pim-sm horizon-split	54

4.1.9 ip pim-sm jp-hold	55

4.1.10 ip pim-sm jp-intvl	56

4.1.11 ip pim-sm lan-delay	57

4.1.12 ip pim-sm nbma-mode	59

4.1.13 ip pim-sm nbr-filter	60

4.1.14 ip pim-sm nbr-track	62

4.1.15 ip pim-sm override	63

4.1.16 router pim-sm	64

4.1.17 hello-option	65

4.1.18 accept bsm-adv	67

4.1.19 accept crp-adv	68

4.1.20 accept rp-addr	69

4.1.21 accept register	70

4.1.22 anycast-rp	72

4.1.23 reg-rate-limit	75

4.1.24 reg-src	76

4.1.25 spt-threshold	77

4.1.26 ssm	78

4.1.27 c-bsr intf_type intf_name	80

4.1.28 c-bsr admin-scope	81

4.1.29 bsm-policy	82

4.1.30 static-rp	84

4.1.31 c-rp intf_type intf_name	86

4.1.32 intvl-time	88

4.1.33 holdtime	89

4.1.34 log	91

4.1.35 clear ip pim-sm rp-mapping	92

4.1.36 clear ip mroute pim-sm	93

4.1.37 show running-configure	94

4.1.38 show ip pim-sm bsr-router	94

4.1.39 show ip pim-sm sz-info	96

4.1.40 show ip pim-sm interface	97

4.1.41 show ip pim-sm neighbor	99

4.1.42 show ip pim-sm rp	101

4.1.43 show ip pim-sm rp-hash	103

4.1.44 show ip pim-sm anycast-rp	104

4.1.45 show ip pim-sm protocol	105

4.1.46 show ip mroute pim-sm	106

4.1.47 show ip rpf pim-sm	108

4.1.48 debug ip pim-sm alert	109

4.1.49 debug ip pim-sm assert	110

4.1.50 debug ip pim-sm bsr	111

4.1.51 debug ip pim-sm rp	111

4.1.52 debug ip pim-sm entry	112

4.1.53 debug ip pim-sm event	113

4.1.54 debug ip pim-sm hello	114

4.1.55 debug ip pim-sm jp	115

4.1.56 debug ip pim-sm nbr	115

4.1.57 debug ip pim-sm packet	116

4.1.58 debug ip pim-sm register	117

4.1.59 debug ip pim-sm timer	118

# Basic Multicast Commands

## Basic Multicast Commands

Basic multicast commands include:

- debug ip mpacket

- debug ip mrouting

- debug ip mroute-cache

- debug ip multicast

- ip mroute

- ip mroute-cache

- ip multicast-routing

- ip multicast route-limit

- ip multicast boundary

- ip multicast helper-map

- ip multicast rate-limit

- ip multicast ttl-threshold

- show ip mflow

- show ip mroute-cache

- show ip mroute mfc

- show ip mroute static

Note:

- The "router" referred to in this chapter are Layer 3 switches running routing protocols, routers in the general network environment, and other devices running routing protocols.

### debug ip mpacket

Syntax

If you want to track the process for the multicast packet, you can use debug ip mpacket command, and  use the no form of the command to close debug information.

debug ip mpacket [access-list][group-address][detail]

no debug ip mpacket

Parameter

| parameter | Description |
| --- | --- |
| access-list | Range for tracked multicast packets |
| group-address | The tracked multicast packet group address |
| detail | Details for multicast packet processing |

Default value

Disable debug information output

Command mode

EXEC

Usage guidelines

You can use this command to track the main process for multicast packet.

Example

The following example shows some situations for multicast packet processing.

You have received the (100.168.20.151, 224.1.1.1) packet on e0/1 port, and the packet length is 112 bytes.

You have sent the (192.168.20.99, 224.0.0.5) packet on e0/1 port, and the packet length is 64 bytes.

switch#debug ip mpacket

M INPUT :  IP Vlan1 (100.168.20.151, 224.1.1.1) , len=112

M OUTPUT :  IP Vlan1 (192.168.20.99, 224.0.0.5) , len=64

Related command

None

### debug ip mroute-cache

Syntax

Use debug ip mroute-cache command to enable multicast routing cache tracking function, then you can see the change from the multicast routing cache. In addition, you can use the no format of the command to disable the tracking.

debug ip mroute-cache [group-address]

no debug ip mroute-cache

Parameter

| parameter | Description |
| --- | --- |
| group-address | The tracked multicast routing cache group address |

Default value

Disable all tracking functions.

Command mode

EXEC

Usage Guidelines

- You can use this command to see the change of the adding/deleting of multicast routing cache.

example

The following example shows you some changes on the multicast routing cache, and the creating and timeout of (192.168.20.97, 230.0.0.1) cache.

switch#debug ip mroute-cache

MRC:  create (192.168.20.97, 230.0.0.1) mroute-cache

MRC:  expired (192.168.20.97, 230.0.0.1) mroute-cache

Related command

None

### debug ip multicast

Syntax

You can use debug ip multicast command to enable multicast event tracking function, and then see the interaction between the multicast protocol and mrouting. you can use the no format of the command to disable the function.

debug ip multicast [alert | border-router]

no debug ip multicast [alert | border-router]

Parameter

| parameter | Description |
| --- | --- |
| alert | Track the alert interaction among multicast routing components |
| border-router | Track related events of multicast border router MBR |

Default value

Disable all tracking functions.

Command mode

EXEC

Usage guidelines

Defining some standard events between multicast routing protocol and mrouting indicates alert, such as creation alert and deletion alert which related (S, G). You can use debug ip multicast alert to see these alerts.

XXCOM’s multicast routing protocol supports MBR, and each multicast routing protocol is a “component”. You can use debug ip multicast border-router command to see the component’s running information.

example

In the following example, the alert router is turned on for output:

switch#debug ip multicast alert

MBR:  [(S, G) deletion alert], originated by PIM-DM, sent to all components

MBR:  [(S, G) creation alert], originated by NONE, sent to all components

MBR:  src = 192.168.20.110, grp = 239.0.0.100

MBR:  sent to owner PIM-DM first

MBR:  [(S, G) join alert], originated by NONE, sent to PIM-DM

MBR:  src = 192.168.20.110, grp = 239.0.0.100

MBR:  [(S, G) firstuse alert], originated by NONE, sent to PIM-DM

MBR:  src = 192.168.20.110, grp = 239.0.0.100

MBR:  [(S, G) deletion alert], originated by PIM-DM, sent to all components

MBR:  src = 192.168.20.110, grp = 239.0.0.100

### ip multicast-statistic

Syntax

To enable IP multicast packet statistics, run the following command. To return to the default setting, use the no form of this command.

ip multicast-statistic

no ip multicast-statistic

Parameter

None

Default value

Disabled

Command mode

Global configuration mode

Usage guidelines

The multicast statistics provides with evidence of switching rpt of pim-sm to spt. When configured switching threshold, the command needs to be enabled. Once the command is disabled, the switch will not make statistics of the multicast.

Example

The following example shows how to configure the statistics multicast packet of the switch:

switch_config#ip multicast-statistic

Related Command

None

### ip multicast-routing

Syntax

Use this command ip multicast-routing to enable IP multicast packet transferring function, and no ip multicast-routing to disable the function.

ip multicast-routing

no ip multicast-routing

Parameter

None

default value

The default is not to transfer multicast packets.

Command mode

Global configuration mode

Usage Guidelines

If you disable this function, the router will no longer transfer multicast packets. Meanwhile, the multicast routing list and the multicast cache will be empty.

example

- The following example will configure the router to transfer multicast packets:

- switch_config#ip multicast-routing

Related command

show ip mroute mfc

### ip multicast route-limit

Syntax

Use this command ip multicast route-limit to configure the maximum number of multicast routing item, and no ip multicast route-limit to un-limit the number.

ip multicast route-limit size

no ip multicast route-limit size

Parameter

| parameter | Description |
| --- | --- |
| size | Maximum number of multicast routing item, the range is from 1 to 5120. |

default value

The default multicast routing item number is 5120 by default.

Command mode

Global configuration mode

Usage guidelines

If you have configured this function, the multicast routing item number will be limited.

Example

The following example will configure the maximum number of multicast routing list  to 2000:

switch_config#ip multicast route-limit 2000

Related command

show ip mroute mfc

### ip multicast boundary

Syntax

Use this command ip multicast boundary to manage the range for the port allowed processing multicast packets;  it is valid for input/output packets on the port. Use no ip multicast boundary to cancel this command.

ip multicast boundary access-list

no ip multicast boundary

Parameter

| parameter | Description |
| --- | --- |
| access-list | the access-list name used to specify the range for processing multicast packets. |

default

- Process all multicast packets.

Command mode

Interface configuration mode

Usage Guidelines

If the function is configured, the range for the port allowed processing multicast packets will be limited.

example

The following example will configure the range for the port vlan1 allowed processing multicast packets to the range limited by the access-list testacl:

switch_config_v1#ip multicast boundary testacl

Related command

None

### ip multicast helper-map

Syntax

Use this command ip multicast boundary helper-map to configure the connection of two broadcast networks with the multicast routing on the multicast network, and no ip multicast boundary helper-map to cancel this command.

ip multicast boundary helper-map {group-address | broadcast} {broadcast-address |multicast-address} access-list

no ip multicast boundary helper-map {group-address | broadcast} {broadcast-address  | multicast-address}

Parameter

| parameter | Description |
| --- | --- |
| group-address | The multicast packet group address which needed to be converted to the broadcast packet. it is used with the broadcast-address keyword. |
| broadcast | It can convert the broadcast packet to the multicast packet. it is used with the multicast-address keyword. |
| broadcast-address | The target address of broadcast packet which is sent after converting. it is used with the group-address keyword. |
| multicast-address | The target address of multicast packet which is sent after converting. It is used with the broadcast keyword. |
| access-list | IP extended access-list name. You can use it to specify the port number for packet converting. |

default value

not perform the conversion between any multicast packets and broadcast packets.

Command mode

- Interface configuration mode

Usage Guidelines

- If two broadcast networks are connected with a multicast network, you can convert the broadcast flow to multicast flow on the first hop router connected with the source broadcast network, and then convert the multicast flow to broadcast flow on the last hop router connected with the target broadcast network. Thus, you can utilize the multicast network’s multicast characteristic between the two broadcast networks which are required to be connected with each other. Furthermore, it can prevent the packets between two broadcast networks from being sent repeatedly, and utilize the “quick forward” characteristic on the multicast network.

Before using ip multicast helper-map, you should have configured this command ip directed-broadcast on the port.

example

- Configuration on the router is as follow:

If you configure command ip directed-broadcast on interface vlan1 of the first hop router, it will be allowed to process the link broadcast packets.

If you have configured “ip multicast helper-map broadcast 230.0.0.1 testacl1”, you can convert the udp broadcast packet, whose port number is 4000 (“ip forward-protocol” command specified) and the source address is 192.168.20.97/24 (testacl1 specified) , to multicast packet whose target address is 230.0.0.1 (“ip multicast helper-map” command specified).

if you configure command”ip directed-broadcast” on port vlan2 of the last hop router, it will be allowed to process the link broadcast packets.

If you have configured “ip multicast helper-map broadcast 230.0.0.1 172.10.255.255 testacl2”, you can convert the multicast packet, whose port number is 4000(“ip forward-protocol” command specified), the source address is 192.168.20.97/24 (testacl2 specified) and target address is 230.0.0.1 , to broadcast packet whose target address is 170.10.255.255 (“ip multicast helper-map” command specified).

On the first hop router which is connected with the source broadcast network:

interface vlan1

ip directed-broadcast

ip multicast helper-map broadcast 230.0.0.1 testacl

ip pim-dm

!

ip access-list extended testacl

permit udp 192.168.20.97 255.255.255.0 any

!

ip forward-protocol udp 4000

On the last hop router which is connected with the target broadcast network:

interface vlan2

ip directed-broadcast

ip multicast helper-map 230.0.0.1 172.10.255.255 testacl2

ip pim-dm

!

ip access-list extended testacl2

permit udp 192.168.20.97 255.255.255.0 any

!

ip forward-protocol udp 4000

Related command

ip forward-protocol

ip directed-broadcast

### ip multicast ttl-threshold

Syntax

Use this command “ip multicast ttl-threshold” to configure the maximum threshold value of multicast packet ttl on the port, and “no ip multicast ttl-threshold” to restore default.

ip multicast ttl-threshold ttl-value

no ip multicast ttl-threshold

Parameter

| parameter | Description |
| --- | --- |
| ttl-value | The multicast packet ttl threshold value on the port. The range is from 0 to 255. |

default

- The default ttl threshold value on the port is 1.

Command mode

Interface configuration mode

Usage Guidelines

- The ttl value of receiving/sending packet should be largerer than the specified threshold value on the port, you can use this command to configure a router to border router.

example

- The ttl threshold value configured on port vlan1 is 200, it means only the multicast packet with ttl value more than 200 is allowed to be received/sent on the port.

interface vlan1

ip multicast ttl-threshold 200

Related command

None

### show ip mroute-cache

Syntax

use this command “show ip mroute-cache” to display the information on the multicast routing cache.

show ip mroute-cache [group-address [source-address]]

Parameter

| parameter | Description |
| --- | --- |
| group-address | The displayed multicast routing cache group address |
| source-address | The displayed multicast routing cache source address |

default

none

Command mode

EXEC

Usage Guidelines

MRC (Multicast Route Cache) is a global multicast routing cache, and every MRC item contains the (S, G)/ (*, G) information, upstream/downstream interface information received from the multicast routing.

example

- The following example will display multicast routing list information:

switch#show ip mroute-cache

IP Multicast Route Cache

(192.168.20.97, 230.0.0.1)|(192.168.20.97, 230.0.0.1)

Incoming interface:  Vlan2, Last used :  00: 00: 34

Outgoing interface list:

Loopback0

(192.168.20.97, 230.0.0.2)|(192.168.20.97, 230.0.0.2)

Incoming interface:  Vlan2, Last used :  00: 00: 12

Outgoing interface list:

Loopback1

### show ip mroute mfc

Syntax

You can use this command “show ip mroute mfc” to display the multicast forwarding list information. You can add the vrf subcommand to display the multicast routing table under a certain vrf. If you do not add the subcommand, the global multicast routing table will be displayed.

show ip mroute [vrf vrf_name] mfc

Parameter

Vrf vrf_name

default

none

Command mode

- EXEC

Usage guidelines

- MFC (Multicast Forwarding Cache) is a global multicast forwarding list, and the multicast packet is forwarded by it. Every MFC item has (S, G)/ (*, G) information and upstream/downstream interface information.

example

The following example will display multicast routing list information:

switch#show ip mroute mfc

IP Multicast Forwarding Cache

(192.168.20.110/32, 239.0.0.100/32)

Incoming interface:  Vlan2, RPF nbr 0.0.0.0, owned by PIM-DM

Outgoing interface list:

Loopback0, owned by PIM-DM

(192.168.20.110/32, 239.0.0.101/32)

Incoming interface:  Vlan2, RPF nbr 0.0.0.0, owned by PIM-DM

Outgoing interface list:

Loopback0, owned by PIM-DM

(192.168.20.138/32, 239.1.1.1/32)

Incoming interface:  Vlan2, RPF nbr 0.0.0.0, owned by PIM-DM

Outgoing interface list:

Loopback0, owned by PIM-DM

Related command

None

# IGMP Configuration Commands

## IGMP Configuration Commands

IGMP configuration commands include:

- clear ip igmp group

- debug ip igmp

- debug ip igmp-host

- ip igmp helper-address

- ip igmp join-group

- ip igmp immediate-leave group-list

- ip igmp last-member-query-interval

- ip igmp querier-timeout

- ip igmp query-interval

- ip igmp query-max-response-time

- ip igmp static-group

- ip igmp version

- show ip igmp groups

- show ip igmp interface

- show ip igmp-host

### clear ip igmp group

Syntax

If you want to clear the multicast group member information saved in multicast router that supports IGMP, you can use the command “clear ip igmp group”.

clear ip igmp group [type number] [group-address | <cr>]

Parameter

| parameter | Description |
| --- | --- |
| type-number | port type and port number. |
| group-address | Multicast group’s group address to clear information. |

Default value

None

Command mode

EXEC

Usage guidelines

Using this command, you can clear the multicast group member information saved in router when the saved multicast group information has a problem.

Example

The following example shows you how to clear the information  of multicast group 233.33.1.1 on port vlan 1.

clear ip igmp group vlan 1 233.33.1.1

Related command

None

### debug ip igmp

Syntax

If you want to track the process for igmp-router end protocol, you can use this command “debug ip igmp”, and  use the”no” form of the command to disable debug information.

debug ip igmp

no debug ip igmp

Parameter

None

Default value

Disable debug information output

Command mode

EXEC

Usage guidelines

You can use this command to track the main process for igmp-router end protocol to find the reason for protocol processing failure.

Example

igmp-router function module’s debug information usually use the natural language to make description. Due to its simplicity, we will not list all of the debug information.

Related command

None

### debug ip igmp-host

Syntax

If you want to track the process for igmp-host end protocol, you can use this command “debug ip igmp-host”, and  use the”no” form of the command to disable debug information.

debug ip igmp-host [group-address]

no debug ip igmp-host [group-address]

Parameter

None

Default value

Disable debug information output

Command mode

EXEC

Usage guidelines

You can use this command to track the main process for igmp-host end protocol to find the reason for protocol processing failure.

Example

igmp-host function module’s debug information usually use the natural language to make description. Due to its simplicity, we will not list all of the debug information.

Related command

None

### ip igmp helper-address

Syntax

If you want a port to transit IGMP packet, you can use this command to configure the port. One port can configure the command only once time, so the next configured command will overwrite the original command.

ip igmp helper-address destination-address

no ip igmp helper-address destination-address

Parameter

| parameter | Description |
| --- | --- |
| destination-address | the destination address of transitting IGMP packet. |

Default value

The port will not transit IGMP packet

Command mode

Interface configuration mode

Usage guidelines

Use this command “ip igmp helper-address” to transit all received igmp packets.

Example

ip igmp helper-address 192.168.20.10

Related command

None

### ip igmp join-group

Syntax

If you want to add a multicast group on the port, you can use this command to perform it.

ip igmp join-group group-address [{include|exclude} source-address]

no ip igmp join-group group-address [{include|exclude} source-address]

Parameter

| parameter | Description |
| --- | --- |
| group-address | the multicast group required to be added to the port |
| include | The mode of SSM needed to add a multicast group is “include”. |
| exclude | The mode of SSM needed to add a multicast group is “exclude”. |
| source-address | source filter address whose port is added to multicast group. |

Default value

No multicast group will be added to the port.

Command mode

Interface configuration mode

Usage guidelines

Use this command “ip igmp join-group” to dynamically add a multicast group to the port.

Example

ip igmp join-group 230.0.0.1

ip igmp join-group 230.0.0.1 exclude 192.168.20.10

Related command

None

### ip igmp immediate-leave group-list

Syntax

If you want the router port running IGMP version 2 to run the multicast group function “Exit Now”, you can use this command “ip igmp immediate-leave group-list” to perform configuring. In addition, you can use the “no” format of the command to forbid the IGMP host to “leave now”.

ip igmp immediate-leave group-list list-name

no ip igmp immediate-leave

Parameter

| parameter | Description |
| --- | --- |
| list-name | Pre-configured ip standard access-list name |

Default value

The IGMP host is not allowed to run “Leave Now” function.

Command mode

global configuration mode/Interface configuration mode

Usage guidelines

This command is available only for the port of running IGMP version 2. It can be used when the network connecting with the port has only one IGMP host. Through configuring this command, the host can immediately leave from a multicast group without the process for packet exchanging and delaying from the router. Besides, you can configure this command in global configuration mode and Interface configuration mode, but this command configured in global configuration mode will be prior to the command configured in Interface configuration mode. If you have configured the command in global configuration mode, the next command configured in Interface configuration mode will be ignored. On the other hand, the command configured in global configurationmode will overwrite the original command configured in interface configurationmode.

Example

Prefer to “Configure multicast routing”.

Related command

ip access-list

### ip igmp last-member-query-interval

Syntax

To change the query interval of last group member on the current port, use this command “ip igmp last-member-query-interval”. You can use the “no” format of the command to restore default settings.

ip igmp last-member-query-interval time

no ip igmp last-member-query-interval

Parameter

| parameter | Description |
| --- | --- |
| time | the value of last member query interval configured on the port. Its unit is millisecond. The range is from 100 to 65535. |

Default value

The default of the last group member query interval on the port is 1000ms.

Command mode

Interface configuration mode

Usage guidelines

You can use this command “ip igmp last-member-query-interval” to modify the last group member query interval on the port.

Example

The following example will modify the last member query interval on the port  to 2 seconds.

interface vlan 1

ip igmp last-member-query-interval 2000

Related command

None

### ip igmp querier-timeout

Syntax

You can use this command “ip igmp querier-timeout” to modify other routers for IGMP querier timeout. Use the “no” format of this command to restore default.

ip igmp querier-timeout time

no ip igmp querier-timeout

Parameter

| parameter | Description |
| --- | --- |
| time | other querier timeout. Its unit is second. The range is from 60 to 300. |

Default value

125 seconds

Command mode

Interface configuration mode

Usage guidelines

You can use this command “ip igmp querier-timeout” to modify other routers for querier timeout. This command is available only for the port which running IGMP version 2.

Example

The following example shows that the querier-timeout specified on interface vlan 1 is 100 seconds.

interface vlan 1

ip igmp querier-timeout 100

Related command

None

### ip igmp query-interval

Syntax

To set the interval for IGMP General Query packet sending on the port, you can use this command “ip igmp query-interval”. Use the “no” format of  this command to  restore default.

ip igmp query-interval time

no ip igmp query-interval

Parameter

| parameter | Description |
| --- | --- |
| time | interval of sending general query packet. Its unit is second. The range is form 2 to 65535. |

Default value

60 seconds

Command mode

Interface configuration mode

Usage guidelines

You can use this command “ip igmp query-interval” to set the interval for general query packet sending on the port,

Example

The following example shows that the interval of sending general query packet on port is specified vlan 1 to 50 seconds.

interface vlan 1

ip igmp query-interval 50

Related command

None

### ip igmp query-max-response-time

Syntax

To specify the maximum interval for IGMP host to respond General Query packet, you can use this command “ip igmp query-max-resposne-time”. Use the “no” format of  this command to  restore default.

ip igmp query-max-response-time time

no ip igmp query-max-response-time

Parameter

| parameter | Description |
| --- | --- |
| time | value of the maximum response time configured on the port. The range is from 1 to 25. |

Default

10 seconds

Command mode

Interface configuration mode

Usage guidelines

You can use this command “ip igmp query-max-response-time” to specify the maximum interval for IGMP host to respond General Query packet. This command is available only for the port which running IGMP version 2 and 3.

Example

The following example will set the IGMP maximum response time on port vlan 1 as 15 seconds.

interface vlan 1

ip igmp query-max-response-time 15

Related command

None

### ip igmp static-group

Syntax

If you want to configure a static multicast group on the port, you can use this command “ip igmp static-group” to perform it. Use the “no” format of  this command to  restore default.

ip igmp static-group { * | group-address } {include source-address | <cr> }

no ip igmp static-group { * | group-address } {include source-address | <cr> }

Parameter

| parameter | Description |
| --- | --- |
| * | all multicast groups. |
| group-address | specified multicast group address. |
| source-address | specified host source address. |

Default value

In default, no multicast group is static configured on the port.

Command mode

Interface configuration mode

Usage guidelines

Configure the static IGMP multicast group information on the current port.

Notes:

For the same group-address, you can configure several “include source-address” commands for the corresponding static multicast group to have several source-addresses existing. However, for the same group-address, you can’t configure both commands with/without “include source-address”.

Example

Refer to “Configure multicast routing”

Related command

None

### ip igmp version

Syntax

To set the IGMP version number running on the port, you can use this command “ip igmp version”. Use the “no” format of the command to restore default.

ip igmp version version-number

no ip igmp version

Parameter

| parameter | Description |
| --- | --- |
| version-number | The value 1, 2 or 3 indicates separately the IGMP version number 1, 2 or 3. |

Default value

If you don’t configure this command, the default version number for IGMP-Router end protocol running on the port is 3.

Command mode

Interface configuration mode

Usage guidelines

use this command “ip igmp version”can set the IGMP version number running on the port,

Example

The following example will specify the IGMP version number running on port vlan 1  as 2.

interface vlan 1

ip igmp version 2

Related command

None

### show ip igmp groups

Syntax

You can use the following command to see the multicast group member information that is saved on the current router.

show ip igmp groups {interface | group-address | detail}

Parameter

| parameter | Description |
| --- | --- |
| interface | The port where you want to see the multicast group information. If you don’t add this parameter, all multicast groups information on the port will be displayed. |
| group-address | The multicast group address to see. If you don’t add this parameter, all multicast groups information on the router will be displayed. |
| detail | T he router whether you want to see the multicast group information. |

Default value

None

Command mode

EXEC/global configuration mode/Interface configuration mode.

Usage guidelines

You can use this command to see the multicast group member information that is saved on the router.

Example

show ip igmp groups detail

Running this command will display the following message:

...............................

Interface:  VLAN1

Group address:  233.33.1.3

Uptime:  00: 03: 46

Group status:  Static

Group filter mode:  INCLUDE

Last reporter:  0.0.0.0

Group source-list:   (Flags:  S-Static, R-Remote)

Source address:    Uptime     Timer      Fwd   Flags

192.168.20.5      00: 03: 46   stopped    Yes   S

Interface:  VLAN1

Group address:  233.33.1.1

Uptime:  00: 03: 46

Group status:  Static

Group filter mode:  INCLUDE

Last reporter:  0.0.0.0

Group source-list:   (Flags:  S-Static, R-Remote)

Source address:    Uptime     Timer      Fwd   Flags

192.168.20.5      00: 03: 46   stopped    Yes   S

192.168.20.3      00: 03: 46   stopped    Yes   S

192.168.20.1      00: 03: 46   stopped    Yes   S

.....................................

show ip igmp groups 233.33.1.1 detail

Running this command will display the following message:

Interface:  VLAN1

Group address:  233.33.1.1

Uptime:  00: 02: 42

Group status:  Static

Group filter mode:  INCLUDE

Last reporter:  0.0.0.0

Group source-list:   (Flags:  S-Static, R-Remote)

Source address:    Uptime     Timer      Fwd   Flags

192.168.20.5      00: 02: 42   stopped    Yes   S

192.168.20.3      00: 02: 42   stopped    Yes   S

192.168.20.1      00: 02: 42   stopped    Yes   S

show ip igmp groups

Running this command will display the following message:

Interface        Group address    Uptime    Expires   Last Reporter    Flags

VLAN1          239.255.255.250  00: 01: 08  00: 02: 05  192.168.20.141   R

VLAN1          224.2.127.254    00: 01: 09  00: 02: 00  32.1.1.67        R

VLAN1          224.1.1.1        00: 01: 24  stopped   0.0.0.0          S

VLAN1          233.33.1.5       00: 01: 24  stopped   0.0.0.0          S

VLAN1          233.33.1.3       00: 01: 24  stopped   0.0.0.0          S

VLAN1          233.33.1.1       00: 01: 24  stopped   0.0.0.0          S

Interface        Group address    Uptime    Expires   Last Reporter    Flags

Loopback10       239.255.255.250  00: 01: 08  00: 02: 05  192.168.20.141   R

Loopback10       224.2.127.254    00: 01: 09  00: 02: 00  32.1.1.67        R

Related command

None

### show ip igmp interface

Syntax

You can use this command to see information on the current router’s port where IGMP is activated.

show ip igmp interface { interface | <cr> }

Parameter

| parameter | Description |
| --- | --- |
| interface | The specified port to display information. If you don’t add this parameter, all information on ports where IGMP is activated will be displayed. |

Default value

None

Command mode

EXEC/global configuration mode/Interface configuration mode

Usage guidelines

You can use this command to display information on the port where IGMP is activated.

Example

show ip igmp int vlan 1

Running this command will display the following information:

VLAN1 is up, line protocol is up

Internet address is 192.168.20.167

Current IGMP switch version is 3

switch plays role of querier on the interface now

IGMP is enable on the interface

IGMP query-interval is 60 seconds

IGMP max query response time is 10 seconds

IGMP Last member query response time is 1000 milliseconds

IGMP querier timeout is 125 seconds

Multicast routing is enabled on the interface

Related command

None

### show ip igmp-host

Syntax

You can use this command to see IGMP host information on the port ofcurrent router.

show ip igmp-host { interface } [detail]

Parameter

| parameter | Description |
| --- | --- |
| interface | The specified port to display information. |
| detail | Display igmp host detailed information. |

Default value

none

Command mode

EXEC/global configuration mode/Interface configuration mode

Usage guidelines

You can use this command to display basic IGMP host information on the port.

Example

show ip igmp-host interface vlan 1

Running this command will display the following information:

IGMP host Mode is IGMP_V3_switch

IGMP host Query Interval is 23 second

IGMP host Query Response Interval is 125

IGMP host Query Robustness Variable is 2

IGMP host Last Query Interval is 0

IGMP interface timer is 0

IGMP host group joined(number of users):

230.0.0.1(1)

Related command

None

# PIM-DM Configuration Commands

## PIM-DM Configuration Commands

PIM-DM configuration commands include:

- clear ip mroute pim-dm

- clear ip pim-dm interface

- debug ip pim-dm

- ip pim-dm

- ip pim-dm dr-priority

- ip pim-dm hello-interval

- ip pim-dm state-refresh origination-interval

- ip pim-dm neighor-filter

- ip pim-dm state-refresh disable

- ip pim version

- show ip pim-dm neighbor

- show ip pim-dm interface

- show ip rpf pim-dm

- show ip mroute pim-dm

### clear ip mroute pim-dm

Syntax

use the following command in EXEC to clear the (S, G) routing list items submitted by PIM-DM to mrouting:

clear ip mroute pim-dm {* | group [source]}

Parameter

| parameter | Description |
| --- | --- |
| * | Delete all multicast routing list items submitted by pim-dm. |
| group | Delete all list items submitted by pim-dm and satisfied in the specified group. |
| source | (optional) Delete all list items submitted by pim-dm and satisfied in the specified group’s source. |

Default value

None

Command mode

EXEC

Usage guidelines

The command will delete all or part of table lists of local multicast router table, and it is possible to affect the normal multicast packet forwarding. This command can only delete the (S, G) items, whose upstream port is created by PIM-DM multicast routing protocol, and inform mrouting, then mrouting will determine if it should re-establish the corresponding (S, G).

Example

Example1:

Switch#clear ip mroute pim-dm *

All (S, G) items, whose middlestream/upstream port is created by PIM-DM, on local MRT will be cleared.

Example2:

Switch #clear ip mroute pim-dm 239.1.1.1

All (S, G) items with the group address 239.1.1.1, whose middlestream/upstream port is created by PIM-DM, on local MRT will be cleared.

Example3:

Switch #clear ip mroute pim-dm 239.1.1.1 192.168.20.131

All (S, G) items with the address (192.168.20.138, 239.1.1.1), whose middlestream/upstream port is created by PIM-DM, on local MRT will be cleared.

### debug ip pim-dm

Syntax

Use this command to track input/output PIM packets and caused events. Set this command to “no” to stop tracking.

debug ip pim-dm [group | alert | assert | graft | hello | jp | neighbor | packet | sg | state-refresh | timer]

Parameter

| parameter | Description |
| --- | --- |
| group | (optional) Track the specified group status. |
| alert | (optional) Track the alert status received from mrouting. |
| assert | (optional) Track the status of asset information. |
| graft | (optional) Track grafting information. |
| hello | (optional) Tracking neighbors' hello message sending and receiving. |
| jp | (optional) Track join/prune messages. |
| neighbor | (optional) Track changes in neighbors. |
| packet | (optional) Tracking the sending and receiving of messages. |
| sg | (optional) Track changes in sg entries. |
| state-refresh | (optional) Track status refresh packet. |
| timer | (optional) Track the triggering of various timers. |

Default value

None

Command mode

EXEC

Usage guidelines

Receive Alert from mrouting.

Send alert to other components.

Example

The following example shows debugging information of group and alert.

Example 1, the output information is as follows:

Hello packet prompt sent to each port.

Hello packet prompt received from each port.

A new neighbor is found.

Delete neighbor.

Port sending status refresh packet.

Port receiving status refresh packet.

Port is sending Assert packet.

Port is receiving Assert packet.

Port is sending prune packet.

Port is receiving prune packet.

Port is sending graft ack packet.

Port is receiving graft ack packet.

Port is sending graft packet.

Port is receiving graft packet.

Port is sending join/prune packet.

Port is receiving join/prune packet.

When a new (S, G) is created

When deleting (S, G)

switch#debug ip pim-dm

2003-3-26 11: 45: 17 received V2 hello packet on Vlan2 from 192.168.20.133(GenID = 3539)

2003-3-26 11: 45: 17 Vlan2 create new nbr 192.168.20.133

2003-3-26 11: 45: 25 send hello packet to 224.0.0.13 on Loopback1

2003-3-26 11: 50: 29 Vlan2 delete nbr 192.168.20.133

2003-3-26 11: 50: 51 received V2 hello packet on Vlan2 from 192.168.20.152

2003-3-26 11: 50: 51 send hello packet to 224.0.0.13 on Vlan2

2003-3-26 12: 04: 37 PIM-DM:   delete (192.168.20.138, 239.1.1.1) in MRT success

2003-3-26 12: 04: 37 PIM-DM:  clear (192.168.20.138, 239.1.1.1) from MRT successful

2003-3-26 12: 04: 39 PIM-DM:  ignored V2 packet on Vlan2 from 192.168.10.204 (validate source address failed)

2003-3-26 12: 04: 39 PIM-DM:   (192.168.20.138, 239.1.1.1)'s upstream: 192.168.20.132 Adding in MRT success

2003-3-26 12: 04: 39 PIM-DM:  (192.168.20.138, 239.1.1.1) Adding in MRT

Example 2, output received alert message:

switch#debug ip pim-dm alert

2003-3-26 12: 09: 51 receive alert_rt_change alert from mroute

2003-3-26 12: 09: 54 receive alert_rt_change alert from mroute

2003-3-26 12: 11: 08 PIM-DM:  send sg_deletion alert

2003-3-26 12: 11: 19 receive alert_sg_creation alert from mroute

2003-3-26 12: 11: 20 receive alert_sg_prune alert from mroute

2003-3-26 12: 11: 56 receive alert_group_report alert from mroute

2003-3-26 12: 11: 56 receive alert_sg_join alert from mroute

Example 3, track the specified group status:

switch#deb ip pim-dm 239.1.1.1

switch#2003-3-26 12: 35: 27 PIM-DM:  clear (192.168.20.138, 239.1.1.1) forwd pkt count success

2003-3-26 12: 35: 37 PIM-DM:   delete (192.168.20.138, 239.1.1.1) in MRT success

2003-3-26 12: 35: 37 PIM-DM:  clear (192.168.20.138, 239.1.1.1) from MRT successful

2003-3-26 12: 35: 37 PIM-DM:   (192.168.20.138, 239.1.1.1)'s upstream:  192.168.20.132 Adding in MRT success

2003-3-26 12: 35: 37 PIM-DM:   (192.168.20.138, 239.1.1.1)'s downstream:  1.1.1.1 create success

2003-3-26 12: 35: 37 PIM-DM:   (192.168.20.138, 239.1.1.1)'s downstream:  192.167.20.132 create success

2003-3-26 12: 35: 42 PIM-DM:  (192.168.20.138, 239.1.1.1) Adding in MRT

### ip pim-dm

Syntax

This command is used to run PIM-DM on the port. Use no form of this command to disable PIM-DM on the port.

ip pim-dm

no ip pim-dm

Parameter

None

Default

None

Command mode

Interface configuration mode

Usage guidelines

- If the “ip multicast-routing” is not configured before configuring this command, it will display the following warning:  WARNING:  "ip multicast-routing" is not configured, IP Multicast packets will not be forwarded.

- Once this function is disabled, PIMDM will no longer run on the port, but it will not affect other PIM-DM configurations. After re-running PIM-DM on the port, all PIM-DM configurations are still valid.

- Enabling this function means it is available for forwarding multicast packet on the port. Certainly, you have to enable the global multicast packet forwarding function first.

Example

Switch_config#ip multicast-routing

Switch_config#interface vlan 1

Switch_config_v1#ip pim-dm

Related command

ip multicast-routing

show ip pim-dm interface

### ip pim-dm dr-priority

Syntax

Set a router as the priority to specified router (DR). You can set this command to “no” to restore default DR priority on the port.

ip pim-dm dr-priority priority

no ip pim-dm dr-priority

Parameter

| parameter | Description |
| --- | --- |
| priority | Port DR priority. The larger the value is, the higher the priority is. Its range is from 0 to 4294967294, and the default is 1. |

Default value

Default DR priority on PIM port is 1.

Command mode

Interface configuration mode

Usage guidelines

- if all PIM neighbors support DR Priority on the port, select the one with the highest priority as DR. If all have the same priority, just select the one with the highest port IP value as DR.

- If router didn’t advertise its priority in Hello packet and there are several routers have the same situation, just select the router with the highest port IP value as DR.

### ip pim-dm hello-interval

Syntax

This command is used to configure the interval of regularly sent PIM-Hello packets on the port. You can set this command to “no” to restore default interval.

ip pim-dm hello-interval interval

no ip pim-dm hello-interval

Parameter

| parameter | Description |
| --- | --- |
| interval | The interval of regularly sent PIM-Hello packets. Its range is from 0 to 65535, and the default is 30 seconds. |

Default value

30 seconds

Command mode

Interface configuration mode

Usage guidelines

Regularly sending Hello packets can check if the neighbor exists. Generally, if Hello packets is not received after the 3.5 times hello-interval timeout configured by neighbor, the neighbor will be considered disappeared.

For IGMP v1, you can select the specified router (DR) through PIM-DM Hello packet.

Example

Switch_config#interface vlan 1

Switch_config_v1#ip pim-dm hello-interval 30

Related command

ip igmp query-interval

### ip pim-dm state-refresh origination-interval

Syntax

The command allows the router to generate original PIM-DM state refresh packet and configure the state refresh interval. To cancel the generation for original PIM-DM state refresh packet, set this command to “no” .

ip pim-dm state-refresh origination-interval [interval]

no ip pim-dm state-refresh origination-interval

Parameter

| parameter | Description |
| --- | --- |
| interval | For the first port router connected with the source directly, it is the interval of regularly sending state refresh packet. For the following router, it is interval of allowed receiving and processing state refresh packet for the port. This parameter is configured optionally, and its range is from 4 to 100 seconds. The default is 60 seconds. |

Default value

Refer to instructions for use

Command mode

Interface configuration mode

Usage guidelines

Configure this command on the first router’s, neighboring directly on the multicast source, incoming port. By default, it will generate original state refresh packet. During configuring this command on the following router’s port, you can use interval to limit the process for received state refresh packet interval. By default, all routers where is running PIM-DM can process and forward state refresh packet.

Example

switch_config_v1#ip pim-dm state-refresh origination-interval 80

Related command

ip pim-dm state-refresh disable

### ip pim-dm neighor-filter

Syntax

This command is used to prevent some routers from participating PIM-DM operation. Set this command to “no” to cancel the limit.

ip pim-dm neighbor-filter access-list-name

no ip pim-dm neighbor-filter access-list-name

Parameter

| parameter | Description |
| --- | --- |
| access-list-name | Standard access-list, whose definition is to deny PIM packets from the specified source. |

Default

No filter function.

Command mode

Interface configuration mode

Usage guidelines

You can use multiple filter lists. The router denied by anyone of the lists can’t be a neighbor of local PIM-DM.

Example

switch_config_v1#ip pim-dm neighbor-filter nbr_filter

switch_config#ip access-list standard nbr_filter

switch_config_std#deny 192.167.20.132 255.255.255.255

switch_config_std#permit 192.168.20.0 255.255.255.0

Related command

access-list (IP standard)

### show ip pim-dm neighbor

Syntax

To display the PIM-DM neighbor and the selected DR, run the following command:

show ip pim-dm neighbor [interface-type interface-number]

Parameter

| Parameter | Description |
| --- | --- |
| interface-type interface-number | Type and ID of the interface, such as Ethernet1/1 and serial 11/0 |

Default value

None

Command mode

- All modes except the user mode

Usage guidelines

This command is used to check on which LAN routers PIM-DM or PIM-SM is configured.

Example

Example 1:

Switch#show ip pim-dm neighbor

PIM-DM Neighbor Table

Neighbor          Interface         Uptime/Expires      Ver   DR Prior/Mode

Address

192.167.20.132    v1          03: 13: 34 / 00: 00: 00    v2     4/D      (DR)

1.1.1.1           Loopback1       03: 52: 30 / 00: 00: 00    v2     1/D      (DR)

192.168.20.132    v2          19: 35: 56 / 00: 00: 00    v2     1/D

192.168.20.152    v2          00: 00: 04 / 00: 01: 41    v2     1/D

192.168.20.204    v2          00: 00: 36 / 00: 01: 44    v2     20/D     (DR)

Example 2:

switch# show ip pim-dm neighbor vlan 2

PIM-DM Neighbor Table

Neighbor          Interface         Uptime/Expires      Ver   DR Prior/Mode

Address

192.168.20.132    v2          19: 39: 22 / 00: 00: 00    v2     1/D

192.168.20.152    v2          00: 00: 30 / 00: 01: 15    v2     1/D

192.168.20.204    v2          00: 00: 04 / 00: 02: 16    v2     20/D     (DR)

Related command

ip pim-dm

ip pim-dm dr-priority

ip pim-dm hello-interval

ip pim version

ip pim-dm neighor-filter

show ip pim-dm interface

### show ip pim-dm interface

Syntax

To display the state of the PIM-DM interface, run the following command:

show ip pim-dm interface [interface-type interface-number]

Parameter

| Parameter | Description |
| --- | --- |
| interface-type interface-number | (optional)Type and ID of the interface, such as Ethernet1/1 and serial 11/0 |

Default value

None

Command mode

All modes except the user mode

Usage guidelines

Only the state of the PIM-DM interface can be displayed after this command is run. If the PIM-DM interface is not designated, the information about all PIM-DM interfaces’ state will be displayed.

Example

Example 1:

Switch#show ip pim-dm interface

address           Interface         Ver/  Nbr    Hello  DR     DR

Mode  Count  intvl  Prior

192.167.20.132    v1                v2/D  0      30     4      192.167.20.132

1.1.1.1           Loopback1         v2/D  0      30     1      1.1.1.1

192.168.20.132    v2                v2/D  2      30     1      192.168.20.204

Example 2:

Switch#show ip pim-dm interface vlan 2

address           Interface         Ver/  Nbr    Hello  DR     DR

Mode  Count  intvl  Prior

192.168.20.132    v2       v2/D  2      30     1      192.168.20.204

Related command

ip pim-dm

ip pim-dm dr-priority

ip pim-dm hello-interval

ip pim version

ip pim-dm neighor-filter

show ip pim-dm neighbor

### show ip rpf pim-dm

Syntax

To display how the multicast route conducts the Reverse Path Forwarding (RPF), run the following command:

show ip rpf pim-dm source-address

Parameter

| Parameter | Description |
| --- | --- |
| source-address | Displays the RFP information of the designated source address. |

Default value

None

Command mode

All modes except the user mode

Usage guidelines

The PIM-DM protocol can obtain the RPF information from multiple types of routing tables. This command tells you where the RPF information is obtained.

Example

Switch#show ip rpf pim-dm 4.1.1.1

RPF information for (4.1.1.1)

RPF interface:  vlan 2

RPF neighbor:  192.168.20.80

RPF route/mask:  192.168.20.0/24

RPF type:  unicast

Metric preference:  120

Metric:  1

Related command

None

# PIM-SM Configuration Commands

## PIM-SM Configuration Commands

PIM-SM configuration commands include:

- ip pim-sm

- ip pim-sm admin-scope

- ip pim-sm asrt-hold

- ip pim-sm bsr-border

- ip pim-sm dr-pri

- ip pim-sm hello-intvl

- ip pim-sm holdtime

- ip pim-sm horizon-split

- ip pim-sm jp-hold

- ip pim-sm jp-intvl

- ip pim-sm lan-delay

- ip pim-sm nbma-mode

- ip pim-sm nbr-filter

- ip pim-sm nbr-track

- ip pim-sm override

- router pim-sm

- hello-option

- accept bsm-adv

- accept crp-adv

- accept rp-addr

- accept register

- anycast-rp

- reg-rate-limit

- reg-src

- spt-threshold

- ssm

- c-bsr intf_type intf_name

- c-bsr admin-scope

- bsm policy

- static-rp

- c-rp intf_type intf_name

- intvl-time

- holdtime

- log

- show running-config

- show ip pim-sm bsr-switch

- show ip pim-sm sz-info

- show ip pim-sm interface

- show ip pim-sm neighbor

- show ip pim-sm rp

- show ip pim-sm rp-hash

- show ip pim-sm anycast-rp

- show ip pim-sm protocol

- show ip mroute pim-sm

- show ip rpf pim-sm

- debug ip pim-sm alert

- debug ip pim-sm assert

- debug ip pim-sm bsr

- debug ip pim-sm rp

- debug ip pim-sm entry

- debug ip pim-sm event

- debug ip pim-sm hello

- debug ip pim-sm jp

- debug ip pim-sm nbr

- debug ip pim-sm packet

- debug ip pim-sm register

- debug ip pim-sm timer

### ip pim-sm

Syntax

Use the ip pim-sm command to enable pim-sm on the port. Use no ip pim-sm command to disable this function on the port.

ip pim-sm

no ip pim-sm

Parameter

| Parameter | English prompt |
| --- | --- |
| pim-sm | pim-sm -- Enable PIM protocol sparse mode |

Default value

Disable PIM-SM

Command mode

Interface configuration mode

Usage guidelines

Only when ip multicast-routing is configured in the global configuration mode, the pim-sm command can be used to enable pim-sm on the port.

If pim-sm is configured for the port for the first time, the hello mechanism will be started, and IGMP will be enabled on the port. In addition, other port-dependent configurations in the global pim-sm configuration mode will be detected, and corresponding operations will be started (such as enabling the BSR election mechanism). For the first configuration, a hello message will be triggered on the port, and the trigger timer is a random value of 0 to 5 seconds.

If ip mulitcast-routing is not configured in the global configuration mode, when the port is configured with ip pim-sm for the first time, there will be a prompt similar to the following:

WARNING:  "ip multicast-routing" is not configured,

IP Multicast packets will not be forwarded

If other multicast protocols have been configured on the port, pim-sm cannot be configured.

Example

The following example indicate the interface vlan8 will start PIM-SM multicast protocl.

switch_config_v8#ip pim-sm

### ip pim-sm admin-scope

Syntax

To configure the multicast administrator scope, run ip pim-sm admin-scope gaddr gmask. The designated multicast address ranges between 239.0.0.0 and 239.255.255.255. This command is configured only on the edge routers which interlace other SZs.

ip pim-sm admin-scope gaddr gmask

no ip pim-sm admin-scope

Parameter

| Parameter | English prompt | Remark |
| --- | --- | --- |
| admin-scope | admin-scope – pim-sm administrator scope | Only the C-BSRs and the ZBRs need to be configured to know about the existence of the scope zones. Other routers, including the C-RPs, learn of their existence from Bootstrap messages. |
| gaddr | A.B.C.D -- private group address prefix | 239.0.0.0 to 239.255.255.255 |
| gmask | A.B.C.D -- private group mask |  |

Default value

The global domain is 224.0.0.0/4 by default.

Command mode

Port configuration mode

Usage guidelines

This command is configured on the boundary of the PIM-SM administrator scope and used to check the BSM information which is received from the outside of the administrator scope. If the SZ scope of the received BSM information is smaller than or equal to the locally configured administrator scope, the received BSM information will be discarded. Otherwise, the received BSM information will enter the local administrator scope. When the BSM information is forwarded locally, the same principle is followed. At present, Huawei only supports the global domain and the private domain, but does not support the covering of the group address. RP in SZ1 will not notify the global SZ of the C-RP-ADV information. But the global BSM information can get in.

Example

The following example shows how to configure the boundary scope on routerA to pim-sm sz1(239.1.1.1/24).

switchA_config_v8#ip pim-sm admin-scope 239.1.1.1 255.255.255.0

switchA_config_ps# c-bsr interface lo1

switchA_config_ps# c-bsr admin-scope 239.1.1.1 255.255.255.0

Related command

ip pim-sm bsr-border

c-bsr intf_type intf_name

### ip pim-sm asrt-hold

Syntax

To configure the holdtime for the assert state on the port corresponding to PIM-SM, run the following command.

ip pim-sm asrt-hold intvl

no ip pim-sm asrt-hold

Parameter

| Parameter | English prompt | Remark |
| --- | --- | --- |
| asrt-hold | asrt-hold -- assert status holdtime |  |
| intvl | <7-65535> -- assert status hold timer value(uint: second, default: 180) | The default value is 180 seconds. |

Default value

180 seconds

Command mode

Port configuration mode

Usage guidelines

None

Example

The following example shows how to configure the timeout time in assert state on port vlan8 to 200 seconds.

switch_config_v8#ip pim-sm asrt-hold 200

### ip pim-sm bsr-border

Syntax

To prevent BSM (bootstrap message) messages from receiving and sending on the specified port, use the command ip pim bsr-border. To cancel this restriction, you can use the no form of the command.

ip pim-sm bsr-border

no ip pim-sm bsr-border

Parameter

| Parameter | English prompt | Remark |
| --- | --- | --- |
| bsr-border | bsr-border -- BSR border |  |

Default value

Disable.

Command mode

Port configuration mode

Usage guidelines

This command can set interface to BSR boundary didn’t send/receive BSM message. This command change difineition BSR boundary to make BSM message can not effect each other in different PIM-SM domain. Otherwise, PIM may elect RPs in other domains as the RPs of this domain, which will eventually cause functional disorder or lose the ability to isolate between domains.

Example

The following example configures the interface to be the PIM domain border:

switch_config_v8# ip pim-sm bsr-border

Related command

None

### ip pim-sm dr-pri

Syntax

Each pim port (LAN port or other type of port) will select a DR. Refer to the description of $2.3.3 for DR election rules. It should be emphasized that the DR priority value is given first priority to the port configuration, followed by the global configuration, and the default configuration takes the third place. After configuring the DR priority, you need to reset the hello timer and set the first timeout value as the value of triggering update PS_RAND_INTVAL(50).

ip pim-sm dr-pri pri-value

no ip pim-sm dr-pri

Parameter

| Name | English prompt |
| --- | --- |
| dr-pri | dr-pri -- PIM-SM DR priority of the interface |
| Pri-value | <0-4294967295> -- DR priority, preference given to larger value |

Default value

DR priority is 1 by default in global mode.

Command mode

Interface configuration mode

Usage guidelines

The change of the global DR priority may affect the reselection of the DR of the port. The election of DR depends on whether the corresponding port itself and all neighbors under the port support DR priority;  if it does, the DR with the higher DR priority value is selected as the DR. If the priority is the same, the DR with the higher IP address is used as the DR;  DR priority is not fully supported, and the one with the larger IP address is directly used as the DR. If the port DR becomes non-DR or vice versa, it may be necessary to comprehensively consider the assert state machine and the corresponding actions of (s, g) and (*, g).

Example

The following example sets the DR priority value to 200 for the pim-sm interface vlan8:

switch_config#interface vlan8

switch_config_v8#ip address 10.0.1.2 255.255.255.0

switch_config_v8#ip pim-sm dr-pri 200

Related command

None

### ip pim-sm hello-intvl

Syntax

Set the interval of the hello timer on the pim-sm port. The value range is 1 to 65535 in seconds. intvl×3.5 is the timeout period for local reachability.

ip pim-sm hello-intvl seconds

no pim-sm hello-intvl

Parameter

| Name | English prompt |
| --- | --- |
| hello-intvl | hello-intvl -- PIM-SM router hello advertisement interval |
| seconds | <1-65535> -- periodic pim hello message are sent(unit: s) |

Default value

The interval of sending Hello messages is 30s.

Command mode

Interface configuration mode

Usage guidelines

Port configuration takes precedence, configuration takes the second place, otherwise, the default value is select. After the Hello interval is configured, the default holdtime is set as 3.5 times of Hello interval correspondingly, which will be included in the hello message as an option.

Example

The following example sets the hello interval value to 40 for interface vlan8:

switch_config#interface vlan8

switch_config_v8#ip address 10.0.1.2 255.255.255.0

switch_config_v8#ip pim-sm hello-intvl 40

Related command

None

### ip pim-sm holdtime

Syntax

To configure the interval of the hello timer on the PIM-SM port, run the following first command. The value of the interval ranges between 4 and 65535. The unit is seconds.

ip pim-sm holdtime seconds

no pim-sm holdtime

Parameter

| Name | English prompt |
| --- | --- |
| Holdtime | Holdtime -- keep alive time to neighbor |
| seconds | <4-65535> -- keep alive time value(uint: s, default: 3.5x hlo_intvl) |

Default value

105 seconds

Command mode

Interface configuration mode

Usage guidelines

This command is first run in port configuration mode and then in global configuration mode, or the default value is chosen. If the value of the holdtime is smaller than that of Hello interval, the configured value is then invalid. The value of the holdtime is 3.5 times of hello-intvl.

Example

The following example sets the holdtime value to 100 for interface pim-sm configuration:

switch_config#interface vlan8

switch_config_v8#ip address 10.0.1.2 255.255.255.0

switch_config_v8#ip pim-sm holdtime 100

Related command

ip pim-sm hold-intvl

intvl-time hello hlo-intvl

### ip pim-sm horizon-split

Syntax

To configure the horizon split strategy of the BSM packets on a port, run the following first command.

ip pim-sm horizon-split

no ip pim-sm horizon-split

Parameter

| Name | English prompt |
| --- | --- |
| horizon-split | horizon-split -- permit interface horizon split for BSM |

Default value

The horizontal split is disabled by default.

Command mode

Interface configuration mode

Usage guidelines

After this command takes effect, you can set the corresponding label bit of the PIM-SM port. The BSM packets that are received from a port will not be transmitted from the port.

Example

The following example configures the bsm split horizon strategy to be enabled on the vlan8 port:

switch_config#interface vlan8

switch_config_v8#ip address 10.0.1.2 255.255.255.0

switch_config_v8#ip pim-sm horizon-split

Related command

bsm policy

### ip pim-sm jp-hold

Syntax

To configure the holdtime for the join-prune state on the port corresponding to PIM-SM, run the following first command.

ip pim-sm jp-hold intvl

no ip pim-sm jp-hold [intvl]

Parameter

| Name | English prompt | Notes |
| --- | --- | --- |
| jp-hold | jp-hold -- join-prune status holdtime |  |
| intvl | <1-65535> -- jp state keep time value(uint: second, default: 210) | The default value is 210 seconds. |

Default value

210 seconds

Command mode

Port configuration mode

Usage guidelines

None

Example

The following example shows how to configure the timeout time in join state on port vlan8 to 200 seconds.

switch_config_v8#ip pim-sm jp-hold 200

### ip pim-sm jp-intvl

Syntax

To set the interval of transmitting the join or prune packets periodically, run the following first command. The interval, whose unit is second, ranges between 1 and 65535.

ip pim-sm jp-intvl seconds

no ip pim-sm jp-intvl

Parameter

| Name | English prompt | Notes |
| --- | --- | --- |
| jp-intvl | jp-intvl -- Join/Prune message advertisement interval |  |
| Seconds | <1-65535> -- regular Join/Prune message interval(unit: s, default: 60) | The default value is 60 seconds. |

Default value

60 seconds

Command mode

Port configuration mode

Usage guidelines

At each configuration the PIM-SM database must be entirely searched for the (s, g) pairs or the (*, g) pairs;  if the configured port is an upstream one, the interval of the jp timer of the corresponding (s, g) or (*, g) pair should be reset. The interval in port configuration mode is prior to the join/prune interval in global mode. If the Join packets from the downstream neighbor have not been received in three JP timeout periods, the downstream that corresponds to the multicast item will be shifted to the prune state. The default holdtime is 3 minutes. If this value is changed randomly, CPU shock and service-forwarding shock may be caused.

Example

The following example changes the PIM join message interval to 90 seconds:

switch_config_v8# ip pim-sm jp-intvl 90

Related command

ip pim-sm jp-hold

### ip pim-sm lan-delay

Syntax

To designate the prune delay time of the PIM-SM port, run the following first command. The local downstream port receives the prune message sent by the downstream switch, and the downstream port corresponding to the multicast entry is switched to prune after the specified prune pending timer expires. The timeout interval of the PPT timer is negotiated according to delay-intvl.

ip pim-sm lan-delay delay-intvl

no ip pim-sm lan-delay

Parameter

| Name | English prompt | Notes |
| --- | --- | --- |
| lan-delay | lan-delay - pim-sm prune delay |  |
| delay-intvl | <0-32767> -- lan delay(uint: ms, default: 500) | The default value is 500 milliseconds. |

Default value

500ms

Command mode

Interface configuration mode

Usage guidelines

If the local port is the downstream port, the finally calculated prune delay time is based on all maximum values reported by downstream neighbors. In this case, the override timer of transmitting the join packets towards the upstream neighbors will be affected. If the prune_delay option is not supported by all downstream neighbors, the default value will be used as the overtime interval of the prune pending timer. If the interval of the prune delay timer is locally set, it will be reported to upstream neighbors through the HELLO packets.

Example

The following example sets the prune delay value to 200 ms for the pim-sm interface vlan8:

switch_config#interface vlan8

switch_config_v8#ip address 10.0.1.2 255.255.255.0

switch_config_v8#ip pim-sm lan-delay 200

Related command

ip pim-sm override

### ip pim-sm nbma-mode

Syntax

If the following first command is configured on all NBMA interfaces, the central node will forward all information that is transmitted from the sub-nodes and the other sub-nodes can obtain the corresponding information.

ip pim-sm nbma-mode

no ip pim-sm nbma-mode

Parameter

| Name | English prompt | Notes |
| --- | --- | --- |
| nbma-mode | nbma-mode -- Enable NBMA mode |  |

Default value

Disable

Command mode

Interface configuration mode

Usage guidelines

Traditional NBMA networks (frame relay, ATM and SMDS) adopt the point-to-multipoint mode;  when a sub-node need be pruned, it will report this information directly to the central node and other sub-nodes, however, can not receive this information. In this case, other sub-nodes cannot respond and the interface of the central node will be incorrectly pruned.

If the following first command is configured on all NBMA interfaces, the central node will forward all information that is transmitted from the sub-nodes and the other sub-nodes can obtain the corresponding information.

This command cannot be used in multicast LANs, such as Ethernet or FDDI. For NBMA networks such as frame relay, it is necessary to see which neighbors of the corresponding output port need to receive the multicast service data before unicasting the service data to the corresponding neighbors.

This command is only used for networks that do not have multicast capabilities; and, this command can be configured only when pim-sm is configured on the port.

Multicast service data is only sent to downstream neighbors that clearly need the multicast stream (using the multicast group as the destination address and sent in unicast mode), and these neighbors explicitly send join messages to the upstream port. This command can be configured only when pim-sm is configured on the port.

Example

The following example configures an interface to be in NBMA mode:

switch_config#interface vlan8

switch_config_v8#ip address 10.0.1.2 255.255.255.0

switch_config_v8#ip pim-sm nbma-mode

Related command

None

### ip pim-sm nbr-filter

Syntax

To stop a device from being added to PIM (for example, to configure stub multicast routing), run the following first command;  to cancel this function, run the following second command.

ip pim-sm nbr-filter acl-name

no ip pim-sm nbr-filter [acl-name]

Parameter

| Name | English prompt | Notes |
| --- | --- | --- |
| br-filter | nbr-filter -- PIM-SM neighbor filter |  |
| acl-name | WORD -- Standard ACL name |  |

Default value

Disable

Command mode

Interface configuration mode

Usage guidelines

If this command is configured, the neighbors need be filtered when Hello packets are received and then a new neighbor can be created. If it is in deny state, the corresponding new neighbor need not be created. Multiple neighbor access lists can be configured (New CISCO bin only allows to configure a neighbor access list;  old bin allows to configure multiple neighbor access lists). Once a neighbor is filtered, the neighbor is then denied.

Example

The following example shows how to configure stub multicast routing on router A and how router B uses access list 1 to filter all PIM information from router A. SwitchA has an egress port vlan8, and the IP address is 10.0.0.1. switchB is a central routing road, the egress port is vlan8, the address is 10.0.0.2.

switch A Configuration

switch_config# ip multicast-routing

switch_config#interface vlan8

switch_config_v8#ip address 10.0.0.1 255.255.255.0

switch_config#interface vlan9

switch_config_vlan9# ip igmp helper-address 10.0.0.2

switch B Configuration

switch_config# ip multicast-routing

switch_config#interface vlan8

switch_config_v8#ip address 10.0.0.2 255.255.255.0

switch_config_v8# ip pim-sm nbr-filter 1

switch_config#ip access-list standard 1

switch_config_std# deny 10.0.0.1

switch_config_std# permit any

Related command

ip pim-sm jp-intvl

### ip pim-sm nbr-track

Syntax

To forbid the limitation of the JOIN packets globally and enable neighbor tracking, run the following first command:

ip pim-sm nbr-track

no ip pim-sm nbr-track

Parameter

| Name | English prompt | Notes |
| --- | --- | --- |
| nbr-track | nbr-track -- pim-sm interface neighbor join desire tracking |  |

Default value

If the global congfigration mode is not configured, neighbor tracking is disabled.

Command mode

Interface configuration mode

Usage guidelines

This command is used to forbid the limitation function of the join packets and enable neighbor tracking. In this way, (s, g), (*, g), (s, g, rpt) each downstream will retain the corresponding neighbor (referring to the neighbor that sent the join message), and upstream will retain the corresponding RPF nbr. By default, neighbor tracking is disabled, and join message suppression capability is allowed. Only when all neighbors on the port support nbr track, the neighbor tracking capability will take effect, and the corresponding flag bit of the pim-sm port structure will be set.

Example

The following example sets the DR priority value to 200 for the interface vlan8:

switch_config#interface vlan8

switch_config_v8#ip address 10.0.1.2 255.255.255.0

switch_config_v8#ip pim-sm nbr-track

Related command

None

### ip pim-sm override

Syntax

The (s, g) or (*, g) upstream port receives the prune message, but the local is still in the join desire state, then an OT timer with a time interval of RANDOM (override-intvl) is triggered locally. After the timer expires, the local will send a join message to the upstream RPF neighbor. The time is negotiated based on the delay-intvl in the neighbor hello message

ip pim-sm override override-intvl

no ip pim-sm override

Parameter

| Name | English prompt | Notes |
| --- | --- | --- |
| override | Override -- pim-sm override timer |  |
| override-intvl | <0-65535> -- override_interval((uint: ms, default: 2500) |  |

Default value

2.5s

Command mode

Interface configuration mode

Usage guidelines

The finally calculated prune deny time is based on the maximum value among all the values reported by all neighbors. If some neighbor does not support prune deny, the default value is selected. If OT is enabled, the value can be random. If the interval of the prune deny timer is locally set, it will be reported to upstream neighbors through the HELLO packets.

Example

The following example sets the override value to 2000 ms for pim-sm interface vlan8 configuration:

switch_config#interface vlan8

switch_config_v8#ip address 10.0.1.2 255.255.255.0

switch_config_v8#ip pim-sm override 2000

Related command

ip pim-sm lan-delay

### router pim-sm

Syntax

To enter the global PIM-SM view, under which there is common attributes, run the following first command:

router pim-sm [vrf  vrf_name]

no router pim-sm [vrf  vrf_name]

Parameter

| Name | English prompt | Notes |
| --- | --- | --- |
| pim-sm | pim-sm --Enable global PIM sparse protocol configure status |  |
| vrf | vrf -- VPN Routing/Forwarding Instance |  |
| vrf_name | WORD -- VPN Routing/Forwarding Instance name |  |

Default value

The system will not generate the PIM-SM view by default.

Command mode

Global configuration mode

Usage guidelines

If the router pim-sm command is configured globally or the PIM-SM related configuration is first configured on the port, the global PIM-SM view will be created. If there is some PIM-SM related configuration on the port, the global view cannot be deleted.

Example

The following command shows how to create the global PIM-SM view:

switch_config#router pim-sm

switch_config_ps#

switch_config#router pim-sm vrf mxw

switch_config_ps_(mxw)#

Related command

ip pim-sm

### hello-option

Syntax

To configure in the global PIM-SM view, the global DR priority, the prune delay time, the transmission interval of HELLO packets, the prune deny time, neighbor tracking and neighbor timeout time, run the following command:

hello-option { dr-pri pri-value | lan-delay delay-intvl |

override override-intvl | nbr-track | holdtime hold-intvl }

no hello-option {dr-pri | lan-delay  |override  | nbr-track | holdtime }

Parameter

| Name | English prompt | Notes |
| --- | --- | --- |
| dr-pri | dr-pri -- PIM-SM global DR priority |  |
| pri-value | <0-4294967295> -- DR priority, preference given to larger value | If this command is not configured, the default DR priority of this process is 1. |
| lan-delay | lan-delay --pim-sm prune delay | Configures the prune delay time. |
| delay-intvl | <0-32767> -- lan delay(uint: ms, default: 500) | The default prune delay time is 500ms. Unit: 100ms Only 15 bits, the 16th bit is T bit. |
| override | override -- pim-sm override timer | Configures the prune deny. |
| override-intvl | <0-65535> -- override_interval((uint: ms, default: 2500) | The default prune deny time is 2500ms. Unit: 100ms |
| nbr-track | nbr-track -- pim-sm interface neighbor join desire tracking. | If this command is configured, the JOIN packets will not be limited. |
| holdtime | holdtime -- keep alive time to neighbor |  |
| hold-intvl | <4-65535> -- keep alive time value(uint: s, default: 3.5x hlo_intvl) | In normal case, its value is 3.5 times larger than the Hello interval on a port. The default neighbor holdtime is 105 seconds. Unit: s |

Default value

See the table above.

Command mode

Global PIM-SM view

Usage guidelines

If there is no corresponding configuration items on a port, the globally configured attributes will be used as the corresponding attributes on the port. The change of the global DR priority may affect the new DR selection.

The rules of DR selection are shown below:

1. The highest DR priority on a port will be selected as the DR of the network segment to which this port belongs;  if there are many same DR values, the relatively large IP address among the main IP address on the local port and the main IP address of the neighbor will be selected as DR.

2. If there are neighbors on a port or the DR priority is not supported on a port, the relatively large mainaddress will serve as DR.

Note:

If the port DR becomes non-DR or vice versa, it may be necessary to comprehensively consider the assert state machine and the corresponding actions of (s, g), (*, g), (*, *, rp), and also need to consider whether to notify IGMP v1 .

Example

The following example sets the DR priority value to 200 for global pim-sm configuration:

switch_config#router pim-sm

switch_config_ps#hello-option dr-pri 200

### accept bsm-adv

Syntax

You can run accept bsm-adv in global mode to set the filtration list. The filtration list settings is especially for filtrating specific BSM source addresses and receive the designated BSM source address. To cancel the filtration, you can run no accept bsm-adv [list std-acl].

accept bsm-adv list std-acl

no accept bsm-adv list [std-acl]

Parameter

| Name | English prompt | Notes |
| --- | --- | --- |
| accept | Accept -- Acceptable RP filter |  |
| bsm-adv | bsm-adv -- BSM packet source address accept filter |  |
| list | List -- IP access-list for bsm source-list |  |
| std-acl | WORD -- stand access list name | It is used to limit the range of the BSM source address. |

Default value

This filtration is disabled by default.

Command mode

pim-sm global view

Usage guidelines

Only one filtration command can be set.

Example

The following example shows that the BSM notifications only from network segment 192.2.2.0/30 can be received.

switch_config#router pim-sm

switch_config_ps#accept bsm-adv list adv-src

switch_config_ps#exit

switch_config#ip access-list stand adv-src

switch_config_std#permit 192.2.2.0 255.255.255.252

### accept crp-adv

Syntax

To set the filtration list specially for filtrating the address range of specific groups, limiting to receive the C-RP-ADV packets from specific candidate rp unicast, and specifying the group address’ range in the received packets through ACL. To cancel the filtration, you can run no form of the command.

accept crp-adv A.B.C.D [std-acl]

no accept crp-adv A.B.C.D [std-acl]

Parameter

| Name | English prompt | Notes |
| --- | --- | --- |
| accept | accept -- Acceptable RP filter |  |
| crp-adv | crp-adv -- C-RP-ADV accept filter |  |
| A.B.C.D | A.B.C.D -- IP address of candidate RP for group |  |
| std-acl | WORD -- Standard ACL name for group-address |  |

Default value

Disable (not filtrating C-RP-ADV from c-rp)

Command mode

pim-sm global view

Usage guidelines

After this command is set, BSR only processes C-RP-ADV from RP. Additionally, the range of the group address must be allowed by the standard ACL.

Example

The following example states that the switch will accept c-rp messages RP address is 100.1.1.1 for the multicast group 224.2.2.2:

switch_config#router pim-sm

switch_config_ps#accept crp-adv 100.1.1.1 grp-acl

switch_config#ip access-list stand grp-acl

switch_config_std#permit 224.2.2.2 255.255.255.255

### accept rp-addr

Syntax

Run accept-rp in global configuration mode to set the filtration list to filter the specific group address range, deciding whether the join/prune of (*, G) is acceptable and responding to the registration information of specific destination group addresses. To cancel this setting above, run the “no” form of this command.

accept rp-addr A.B.C.D [std-acl]

no accept rp-addr A.B.C.D [std-acl]

Parameter

| Name | English prompt | Notes |
| --- | --- | --- |
| accept | accept -- Acceptable RP filter |  |
| rp-addr | rp-addr -- RP address accept filter |  |
| A.B.C.D | A.B.C.D -- IP address of RP for group |  |
| std-acl | WORD -- ip stand access-list name for group | If omit it, switch will process all the PIM-SM message to any group which is mapped to the RP. |

Default value

Disable (All Join, Prune or Register packets will be processed)

Command mode

pim-sm global view

Usage guidelines

After this command is set, the router processes only those Join packets which are mapped to the designated RP. Another point deserving attention is that the range of the group address must be allowed by the standard access list. The aggregation point of the corresponding group must be the calculated RP, and when the aggregation point matches up with the RP can the access filtration list be applied. If the group address is denied, RP will reject the Join and Register packets;  after the Register packets are received, RP will return a Register Stop packet to the registration packet generator.

This command can be set many times if the rp-addr parameters in this command are different. If the RP that the group address is mapped to is not in the configured range, the RP will be denied directly.

Example

The following example states that the switch will accept join or prune messages destined for the RP at address 100.1.1.1 for the multicast group 224.2.2.2:

switch_config#router pim-sm

switch_config_ps#accept rp 100.1.1.1 no-ssm-range

switch_config#ip access-list extended no-ssm-range

switch_config_std#permit 224.2.2.2

%PIM-6-INVALID_RP_JOIN:  Received (*, 238.1.1.1) Join from 192.17.20.173 for invalid RP 1.1.1.1

switch#show ip mroute

(*, 238.1.1.1), 00: 02: 52/00: 00: 07, RP 1.1.1.1, flags:  S

Incoming interface:  Null, RPF nbr 0.0.0.0

Outgoing interface list:

v8, Forward/Sparse, 00: 02: 52/00: 00: 07

It can be seen that the previous address, *.238.1.1.1, ages after the filtration is set.

### accept register

Syntax

When the Register range list is set on C-RP, the selection is RP and the PIM-SM Register packet is received, the filtration list should be used to filter the Register packets. In this case, you should run accept-register, and if you want to cancel the filtration, run the “no” form of this command.

accept register {list ext-acl | route-map map-name}

no accept register [list ext-acl | route-map map-name]

Parameter

| Name | English prompt | Notes |
| --- | --- | --- |
| accept | accept -- Acceptable RP filter |  |
| register | register -- Registers accept filter |  |
| list | list -- standard access list |  |
| ext-acl | WORD -- IP access-list name for register source |  |
| route-map | route-map -- route map |  |
| map-name | WORD -- route map name |  |

Default value

The access list filtration or the route-map filtration will not be conducted to the Register packets.

Command mode

pim-sm global view

Usage guidelines

This command is used to prevent those unauthenticated data source from sending the Register packets to RP. If an unauthenticated data source sends a Register packet to RP, RP will return a Register Stop packet at once. This command takes effect only on the machine that runs as RP.

Example

The following example shows how to restrict the RP from allowing sources in the Source Specific Multicast (SSM) range of addresses to register with the RP.

switch_config#router pim-sm

switch_config_ps#accept register list no-ssm-range

switch_config#ip access-list extended no-ssm-range

switch_config_std#deny ip any 232.0.0.0 0.255.255.255

switch_config_std#permit ip any any

Related command

reg-src

### anycast-rp

Syntax

Through configuring anycast-rp and the corresponding neighbor address, you can specify the corresponding peer neighbor to share the load of RP. To cancel this setting above, run the “no” form of this command.

anycast-rp A.B.C.D nbr A.B.C.D

no anycast-rp A.B.C.D nbr A.B.C.D

Parameter

| Name | English prompt | Notes |
| --- | --- | --- |
| anycast-rp | anycast-rp -- anycast rp for pim-sm |  |
| A.B.C.D | A.B.C.D -- anycast rp address |  |
| nbr | Nbr -- anycast rp neighbor |  |
| A.B.C.D | A.B.C.D -- anycast rp neighbor address |  |

Default value

This command takes no effect by default.

Command mode

pim-sm global view

Usage guidelines

PIM-SM only regulates the standards of the single RP, but a big data flow can cause single RP overload easily. In this case, this command will be used to solve the overload of RP in the PIM-SM domain.

Note:

  - You'd better configure this command on the equipment with good connectivity in the PIM domain in the backbone network. That is, this command is not suitable to be set on the mute terminal router that connects other PIM equipments in the PIM domain through dial-up.

  - If the inside-domain MSDP is not used, the machine that specifies anycast rp must at the same time specify the address of a neighbor with the same RP address so as to facilitate the processing of the register.

  - The command, anycast-rp A.B.C.D nbr, is used on those devices that have no MSDP settings and provide an address as the static RP. All peer neighbors need be specified. The neighbors are reachable to one another.

  - If MSDP is set, the device, if it has provided the static RP address, need not specify anycast rp nbr.

Example

1. The following example shows how to set the anycast-rp address when MSDP is used.

SP1:

Interface loopback 0

ip address 10.0.0.1 255.255.255.255

ip pim-sm

Interface loopback 1

ip address 10.1.1.1 255.255.255.255

l

ip msdp peer 10.1.1.2 connect-source loopback 1

ip msdp originator-id loopback 1

SP2:

Interface loopback 0

ip address 10.0.0.1 255.255.255.255

ip pim-sm

Interface loopback 1

ip address 10.1.1.2 255.255.255.255

l

ip msdp peer 10.1.1.1 connect-source loopback 1

ip msdp originator-id loopback 1

Designate the static RP address on other devices, for example, do it on router Sa:

Sa:

router pim-sm

static-rp 10.0.0.1

2. The following example shows how to make settings when the MSDP is not used.

SP1:

Interface loopback 0

ip address 10.0.0.1 255.255.255.255

ip pim-sm

Interface loopback 1

ip address 10.1.1.1 255.255.255.255

router pim-sm

anycast-rp 10.0.0.1 nbr 10.1.1.1

anycast-rp 10.0.0.1 nbr 10.1.1.2

static-rp 10.0.0.1

SP2:

Interface loopback 0

ip address 10.0.0.1 255.255.255.255

ip pim-sm

Interface loopback 1

ip address 10.1.1.2 255.255.255.255

router pim-sm

anycast-rp 10.0.0.1 nbr 10.1.1.1

anycast-rp 10.0.0.1 nbr 10.1.1.2

static-rp 10.0.0.1

Designate the static RP address on other devices, for example, do it on router Sa: 
Sa:

router pim-sm

static-rp 10.0.0.1

Related command

None

### reg-rate-limit

Syntax

As to (s, g), if you want to set the regeneration rate limit of the PIM-SM registration packets per second, you should use the reg-rate-limit command;  to cancel this settings, you can run no reg-rate-limit [rate].

reg-rate-limit rate

no reg-rate-limit [rate]

Parameter

| Name | English prompt | Notes |
| --- | --- | --- |
| reg-rate-limit | reg-rate-limit -- Rate limit for data register packets |  |
| rate | <1-65535> -- Packets per second |  |

Default value

The regeneration rate of the registration packets of any (s, g) can be limited to one packet per second.

Command mode

pim-sm global view

Usage guidelines

This command can be used to limit the regeneration rate of registration packets of (s.g) on the DR router. After this command is enabled, the load of the DR router will be limited. At the initial establishment of multicast path, the sudden eruption of large traffic of the multicast source may lead to packet loss due to the rate limit and the multicast receiver cannot receive all the multicast packets.

Example

The following example shows how to do the corresponding settings to let each (s.g) generate two registration packets per second.

switch_config#router pim-sm

switch_config_ps#reg-rate-limit 2

### reg-src

Syntax

To specify an IP address of a port to run as the source address for DR to transmit the PIM-SM registration packets, replacing the default port’s address that connects the data source, run reg-src. If you use the reg-src command, the specified port must be active. To cancel this settings, you can run no reg-src.

reg-src intf-type intf-number

no reg-src [intf-type intf-number]

Parameter

| Name | English prompt | Notes |
| --- | --- | --- |
| reg-src | reg-src -- Source address used by register packets |  |
| intf-type intf-number | Type of the designated port and its name | If a port has no main IP address or has no ID, the settings will take no effect. The L3 protocol of a designated port must be up. |

Default value

By default, the port that connects DR and the data source will be used as the source address of the Register packet to conduct packet encapsulation.

Command mode

pim-sm global view

Usage guidelines

When the default source address of the Register packet is not the only routable destination address for RP to return the Register Stop packet, you should use this command to set a new source address for the Register packet. For example, in cases that the source address of the Register packet will be filtered on RP by ACL or the source address is not the only IP address, the Register Stop packet returned by RP may not reach the corresponding DR correctly and then the PIM-SM registration finally may fail.

If the source address of the Register packet is not specified or the specified source address takes no effect, DR will choose the port, which connects the data source, as the source address of the Register packet. Therefore, it is recommended to set for the PIM-SM domain a unique routable address on the loopback port as the source address of the Register packet.

Example

The following example shows how to designate the address of the loopback3 port of DR as the source address of the Register packet.

switch_config#router pim-sm

switch_config_ps#reg-src loopback 3

### spt-threshold

Syntax

For a specific group, the service message flow will switch to the shortest path tree after reaching the configured threshold. In the pim-sm configuration mode, use the command spt-threshold to configure the leaf nodes of PIM-SM. Use the no form of the command to restore the default value of switching to SPT.

spt-threshold {infinity/kbps} [stand-acl]

no spt-threshold {infinity/kbps} [stand-acl]

Parameter

| Name | English prompt | Notes |
| --- | --- | --- |
| spt-threshold | spt-threshold -- Threshold of source-tree switching |  |
| infinity | Infinity -- Never use source-tree |  |
| kbps | <0-4294967> -- Data packet rate in pkt/s |  |
| stand-acl | WORD -- ip standard access list name for group |  |

Default value

There is no traffic limit for switchover. When the downstream receiver tries to join the data source, the data source will switch over to the SPT forwarding when it receives the data.

Command mode

pim-sm global view

Usage guidelines

If the forwarding rate of a multicast source reaches or exceeds the designated threshold, the leaf node will send a (s, g) Join packet to the multicast source for constructing the source tree—the shortest path tree.

If the threshold is set to infinity, all multicast sources for the designated group take the sharing tree for packet forwarding. The group access list designates which groups use the configured threshold for SPT switchover. If the message flow from the data source is less than the designated threshold, the PIM-SM router of the leaf node will be switched back to the sharing tree after a period of time and then send the Prune message to the source tree.

Example

The following example sets a threshold of 4 kbps, above which traffic to a group from a source will cause the switch to switch to the shortest path tree to that source:

switch_config#router pim-sm

switch_config_ps# spt-threshold 4

Related command

None

### ssm

Syntax

To set the range of a specific multicast group, run ssm command. To cancel the designated SSM range, run no ssm.

ssm {default | range std-acl}

no ssm [default | range std-acl]

Parameter

| Name | English prompt | Notes |
| --- | --- | --- |
| ssm | ssm -- Configure Source Specific Multicast group range |  |
| Default | default -- Use default group range(232/8) for SSM mode |  |
| Range | range -- Group range ACL used for SSM |  |
| std-acl | WORD -- IP named access-list |  |

Default value

disable

Command mode

pim-sm global view

Usage guidelines

When PIM-SM is enabled, the default or configured range of the multicast group address can be used. If the multicast group is in the designated SSM range, the locally corresponding (*, g) must be canceled. This requires the same strategic SSM shall be set in the whole PIM-SM.

Note:

- The same SSM strategy shall be set in the whole PIM-SM, otherwise the configured SSM will take no effect on preventing (*, g) Join for IGMPv3 can also specify the addition of (s, g) Join. Additionally, the (*, g) collision may be caused.

- PIM-SM cannot be used together with other protocols. The configuration of SSM prevents the transmission of (*, g) Join and (*, *, rp) Join, and the PMBR device cannot send specific (s, g) Join to the upstream devices.

- After SSM is set, MSDP cannot generate or receive SAs belonging to the designated range of the multicast group address. Our solution is that MSDP notification will be omitted if the group in the (S, G) items of PIM-SM is in the designated SSM group range, .

- If the group range covers BIDIR group range, the previous configuration will be kept, and display error message to the later (not support now).

Example

The following example shows how to configure SSM service for the IP address range defined by access list 4:

switch_config#router pim-sm

switch_config_ps# ssm range grp_range

switch_config#ip access-list stand grp_range

switch_config_std# permit 224.2.151.141

Related command

None

### c-bsr intf_type intf_name

Syntax

To set a router to be a candidate BSR router, run the first of the following two commands;  to cancel this settings, run the second one of the following two commands.

c-bsr [intf_type] [intf_name] [hash-length [priority ]]

no c-bsr [intf_type][intf_name]

Parameter

| Name | English prompt | Notes |
| --- | --- | --- |
| c-bsr | c-bsr -- Candidate bootstrap router (CBSR) |  |
| intf-type intf-number | Type of the designated port and its name |  |
| hash-length | <0-32> -- Hash Mask Length used by RP selection | Specify the length of the hash mask. |
| priority | <0-255> -- Priority value used by BSR election | BSR election priority |

Default value

Dynamic BSR selection is disabled.

Command mode

pim-sm global view

Usage guidelines

After this command is set, the designated address of a port will be used as the BSR address and it will send BSM (bootstrap messages) to all PIM-SM neighbors on the local machine. Each neighbor will compare the previously received BSM with the currently received BSM, and if the BSR address in the current BSM is larger than or equal to that in the previously received BSM, the locally stored BSM will be updated and the current BSM will be forwarded;  otherwise, the current BSM will be dropped directly. Before the candidate BSR receives the BSM with higher priority, it always regards itself as the BSM router in the corresponding management domain.

Note:

- You'd better configure this command on the equipment with good connectivity in the PIM domain in the backbone network. That is, this command is not suitable to be set on the mute terminal router that connects other PIM equipments in the PIM domain through dial-up.

- On accepting C-RP-Adv, BSR only accepts those contents that satisfy the SZ range;  if the content exceeds the SZ range, it will be omitted.

- You can specify only one port of a device as the BSR address;  if multiple commands are set, the previously configured candidate BSRs will be replaced.

- The condition for this command to be effective is that the IP address of the designated port is in PIM-SM state and the protocol is up.

Example

The following example configures the IP address of the switch on Ethernet interface 0 to be a candidate BSR with priority of 10:

switch_config# router pim-sm

switch_config_ps# c-bsr vlan8 10 100

Related command

c-bsr admin-scope {global |gaddr gmask} [hash-length] [priority]

### c-bsr admin-scope

Syntax

To set a candidate BSR in the administration domain, run the first one of the following two commands.

c-bsr admin-scope {global |gaddr gmask} [hash-length [priority ]]

no c-bsr admin-scope {global |gaddr gmask} [hash-length [priority ]]

Parameter

| Name | English prompt | Notes |
| --- | --- | --- |
| c-bsr | c-bsr -- Candidate bootstrap router (CBSR) |  |
| admin-scope | admin-scope -- pim-sm administrator scope |  |
| global | Global -- global group range | 224.0.0.0/255.0.0.0 |
| gaddr | A.B.C.D -- sz range, private group address prefix | 239.0.0.0 to 239.255.255.255 |
| gmask | A.B.C.D -- private group mask |  |
| hash-length | <0-32> -- Hash Mask Length used by RP selection |  |
| priority | <0-255> -- Priority value used by BSR election |  |

Default value

The global domain is 224.0.0.0/4 by default.

Command mode

pim-sm global view

Usage guidelines

This command is used to set the candidate BSR in the administration domain. This command corresponds to the admin-scope command in the domain boundary and is used to specify the range of the administration domain.

Note:

- If the command c-bsr intf type intf name is not configured, this command takes no effect.

- It is recommended to set this command in the administration range (239.0.0.0~ 239.255.255.255).

Example

The following example shows that C-BSR only takes effect in the administration domain 239.1.1.0/24:

switch_config# router pim-sm

switch_config_ps#c-bsr vlan8 10 250

switch_config_ps#c-bsr admni-scope 239.1.1.1 255.255.255.0

Related command

c-bsr intf_type intf_name [hash-length [priority]]

### bsm-policy

Syntax

To set the BSM reception policy, run the first one of the following two commands in PIM-SM configuration mode. To resume the default settings, run the other command.

bsm-policy { unicast {rcvd | send}| forward-all | backward}

no bsm-policy { unicast {rcvd | send} | forward-all }

Parameter

| Name | English prompt | Notes |
| --- | --- | --- |
| bsm-policy | bsm-policy -- PIM-SM RP-address (static RP) |  |
| unicast | "unicast -- unicast bsm packet | The default value allows the reception of the unicast BSM packets. The backward compatibility supports the transmission of unicast BSM. |
| rcvd | Rcvd -- permit to receive bsm unicast message |  |
| send | Send -- permit to send bsm unicast message(backward compatible) |  |
| forward-all | forward-all -- forward all bsm packet", | The horizontal division port will not forward all BSM packets, including unicast or no-forward-bit reset packets. |

Default value

There is no BSM reception or transmission policy.

Command mode

pim-sm global view

Usage guidelines

Refer to the description of this command.

Example

The following example configures the local switch can receive bsm unicasted from neighbor:

switch_config# router pim-sm

switch_config_ps# bsm-policy unicast rcvd

Related command

None

### static-rp

Syntax

To set the static RP of PIM-SM, run the first one of the following two commands;  to resume the default settings, run the other command.

static-rp rp-addr [std-acl] [override]

no static-rp rp-addr [std-acl] [override]

Parameter

| Name | English prompt | Notes |
| --- | --- | --- |
| static-rp | static-rp -- PIM-SM RP-address (static RP) |  |
| rp-addr | A.B.C.D -- IP address of RP", |  |
| std-acl | WORD -- IP access-list", |  |
| override | Override -- Overrides RP-mapping information by BSR | When static RP goes against dynamic RP, if the parameter is not designated, the dynamic RP will be chosen first, and if no dynamic RP exists or dynamic RP takes no effect can static RP take effect. |

Default value

Disable

Command mode

pim-sm global view

Usage guidelines

The designated IP address of static RP must be legal unicast address and cannot be the loopback address (127.0.0.0/8). If ACL is designated, the configured static RP will serve the matched multicast group;  if ACL is not designated, the configured static RP will serve all multicast groups (224.0.0.0/4). When static RP goes against dynamic RP, the former will be first chosen.

In PIM-SM or BIDIR mode, each group will be provided with an RP. All routers in the same administration domain must follow the identical rule to set RP for the group. RP can be obtained through two mechanisms:  static configuration of the RP address or the BSR mechanism’s dynamic learning of RP address. The static rp-address command can be used to set an RP to be the aggregation point of multiple groups. The ACL configured by static RP defines the RP application range. If the standard ACL is not set, the designated static RP will be applied to all groups. One PIM router can use multiple RPs, but it can use the only RP for a specific group.

If multiple static rp-address commands are set, the matchup rules of group-to-rp are listed below:

- If a group matches up with multiple static RP rules, suitable RPs can be chosen according to the longest matchup principle specified by the standard ACL. As to the static configuration without designated ACL, it can be applied to all groups, but the entries in the ACL must be first set.

- If a group and multiple ACLs accord to the longest matchup principle, the IP addresses of RPs must be compared and those RPs with big IP addresses come prior in choice.

- If the static RP configuration is adopted, the reachability test will not be conducted to the designated RP. If an RP is selected, the RP with a comparatively low RP will not be chosen even though the route of the selected RP does not exist locally.

- Each command can be used to specify a static RP address. If the designated static RP address or ACL rule is same during configuration, the new configuration will replace the previous configurations.

In case static RP and dynamic RP are used together, the rules of group-to-rp are listed below:

- When the override is not specified, the RPs, dynamically learned through the BSR mechanism, come prior to static RPs.

- If dynamic RP is used, the c-rp intf_type intf_name command must be set.

Note:

- The same RP cannot be used simultaneously on BIDIR and PIM-SM.

- The statically configured RP only supports global SZ or provides support even if global SZ has not yet created.

Example

The following example shows how to designate 198.92.37.33 to be the static RP address.

switch_config#router pim-sm

switch_config_ps#static-rp 198.92.37.33

Related command

c-rp intf_type intf_name

### c-rp intf_type intf_name

Syntax

To set a port to be C-RP and to send the unicast notification periodically to a designated BSR router in the PIM-SM domain, run the first one of the following two commands. To resume the default settings, run the other command.

c-rp intf-type intf-name [list std-acl ] [ pri pri-value]

no c-rp intf-type intf-name [list std-acl ] [ pri ]

Parameter

| Name | English prompt | Notes |
| --- | --- | --- |
| c-rp | c-rp -- Candidate RP router (CRP) |  |
| intf-type intf-name | Designating the interface type and the interface’s nam. The IP address of the port will be advertised as a c-rp address. | PIM-SM must be enabled on C-RP. |
| list | list -- IP access-list for group-list | It is the prefix of the group address. |
| std-acl | WORD -- access list name |  |
| pri | pri -- priority of rp address |  |
| pri-value | <0-255> -- priority of rp address candidate | The smaller the value is, the higher the priority is. The default value is 192. |

Default value

Dynamic RP selection is disabled.

Command mode

pim-sm global view

Usage guidelines

This command is used to notify all BSRs on C-RP. The range of the group address is listed in a form of the standard ACL.

Note:

- PIM-SM must be enabled on the port that serves as C-RP.

- You’d better set C-RP on the main PIM-SM domain to avoid static configuration on similar routers or the on-demand dialup stub routers.

- If C-RP is not specified with a multicast group range, C-RP will serve all multicast groups.

- If you want to set a router to be C-RP for multiple group ranges, you need to represent multiple group ranges with multiple rules when configuring STD-AC1 that group-list corresponds to.

- One port can only be set to one C-RP and the following configuration will replace the previous configuration, including the replacement of STD-AC1.

- You can set C-RP for multiple ports on the same PIM-SM router.

- Multiple C-RPs can use the same standard ACL.

- If this command is run many times on a same interface, the previous configuration will be replaced.

- If multiple SZs are known on C-RP, unicast C-RP-Adv will be sent to the BSR of each SZ. It is noted that the established group range cannot exceed the group range of the destination SZ.

- If C-RP itself is the ZBR of an SZ, the Admin Scope Bit in the C-RP-Adv packet must be reset;  otherwise, this bit will not be reset. At present, it is used for BSR to record logs but possible to be used for protocol expansion.

Example

The following example shows how to designate lo172 and lo173 to be C-RP ports, the former limiting to provide RP to the group of prefix 239.1/16.

switch_config#router pim-sm

switch_config_ps# c-rp loopback172 group-list grp-range

switch_config# ip access-list standard grp-range

switch_config_std# permit 239.1.0.0 255.255.0.0

switch_config_ps# c-rp loopback173

Related command

None

### intvl-time

Syntax

To enable the periodical transmission of join/prune packets and set the interval of periodically transmitting the Hello, BSM or C-RP-Adv packets, run the first of the following two commands:

intvl-time { join-prune jp-intvl | hello hlo-intvl | c-bsr cbsr-intvl |crp-adv crp-intvl| spt-check spt-intvl }

no intvl-time { join-prune | hello | c-bsr | crp-adv | spt-check }

Parameter

| Name | English prompt | Notes |
| --- | --- | --- |
| intvl-time | intvl-time-- regular message generate timer interval |  |
| join-prune | join-prune-- pim-sm join/prune message interval |  |
| jp-intvl | <1-65535>-- value for timer (unit: s) | The default interval of transmitting Join/Prune packets is 60 seconds. |
| hello | hello-- pim-sm hello advertisement interval | Sets the interval of transmitting the Hello packets. |
| hlo-intvl | <1-65535>-- periodic pim hello message are sent(unit: s) | The default interval of transmitting Hello packets is 30 seconds. |
| c-bsr | c-bsr-- Candidate bootstrap router (candidate BSR) |  |
| cbsr-intvl | <10-65535>-- periodic pim bsm message are sent(unit: s, default: 60) | The default interval of self-selecting packets is 60 seconds. |
| crp-adv | crp-adv-- pim-sm C-RP-ADV interval | Sets the interval of transmitting the C-RP-Adv packets. |
| crp-intvl | <1-65535>-- periodic pim CRP-ADV message are sent(unit: s, default: 60) | The default interval of transmitting Report packets is 60 seconds. |
| spt-check | spt-check-- spt switch response time interval(uint: 100ms) |  |
| spt-intvl | <1-65535>-- spt switch response time interal(uint: 100ms, default: 2s) |  |

Default value

See the above-mentioned table.

Command mode

pim-sm global view

Usage guidelines

If the holdtime of Join-prune packet is not set and the Join packets from the downstream neighbor have not been received in three JP timeout periods, the downstream that corresponds to the multicast entry will be shifted to the prune state. The default holdtime is 3 minutes. The interval in port configuration mode is prior to the join/prune interval in global mode. The relevant interval and holdtime entries have certain time matching requirements.

When using the no form of the command, the specific time parameters required to be input will not be checked and will be restored to the corresponding default values.

Example

The following example sets the join/prune advertisement interval value to 30 for global pim-sm configuration:

switch_config#router pim-sm

switch_config_ps# intvl-time join-prune 30

Related command

holdtime

### holdtime

Syntax

To set the interval of the timeout timer of PIM-SM, run the first one of the following two commands. To resume the default settings, run the other command.

holdtime {join-prune jp-hold | assert asrt-hold | c-bsr cbsr-hold | crp-adv crp-hold| sz sz-hold}

no {join-prune jp-hold | assert asrt-hold | c-bsr cbsr-hold | crp-adv crp-hold| sz sz-hold}

Parameter

| Name | English prompt | Notes |
| --- | --- | --- |
| holdtime | holdtime-- holdtime for keep the status |  |
| join-prune | join-prune-- pim-sm join/prune statue holdtime |  |
| jp-hold | <1-65535>-- join/prune status keep alive value for timer (unit: s; default, 210) | The default value is 210 seconds. |
| assert | assert-- assert status keep alive time |  |
| asrt-hold | <7-65535>-- periodic pim hello message are sent(unit: s, default 180) | The default value is 180 seconds. |
| c-bsr | c-bsr -- BSR status keep alive time |  |
| cbsr-hold | <1-65535> -- periodic pim bsm message are sent(unit: s, default: 130) | By default, it is as follows: holdtime timeout time= holdtime’s interval*2+10 By default, the holdtime’s interval is 60 seconds and the holdtime’s timeout time is therefore 130 seconds. |
| crp-adv | crp-adv -- pim-sm C-RP-ADV keep alive time |  |
| crp-hold | <1-65535> -- CRP-ADV message alive time intvl(unit: s, default: 150) | The default value is 150 seconds. Because non-BSR updates its timeout time through the BSR’s holdtime packets, the timeout time of C-RP must not be less than the interval of holdtime packet transmission. It is best when the former is 2.5 intervals or beyond. |
| sz | sz -- scope zone keep alive time | The default value is 1300 seconds. |
| sz-hold | <10-429496729> -- scope zone keep alive time value(unit: s, default: 1300) |  |

Default value

See the above-mentioned table.

Command mode

pim-sm global view

Usage guidelines

If the holdtime is set on a port, first comes the configuration of this port and then the global configuration;  finally, if neither configuration is done, the default configuration will be chosen.

When using the no command, the specific time parameters required to be input will not be checked and will be restored to the corresponding default values.

Note:

- When configuring the holdtime of C-RP, you should set the timeout time of C-RP to 2.5 holdtime transmission intervals or beyond to prevent the C-RP loss in the BSR holdtime packet.

- The timeout time of SZ must be longer than the BSR timeout time and you'd better set it to be 10 times of BSR timeout.

Example

The following example shows how to set the holdtime of C-RP to 150 seconds, among which C-RP and C-BSR are not set on Ra.

Ra_config# router pim-sm

Ra_config_ps# holdtime c-rp 200

Related command

intvl-time

### log

Syntax

To enable the log switch to record DR’s change, neighbor’s up or down, address conflict and abnormal packets, run the first one of the following two commands:

log { nbr-change | ipaddr-conflict | pkt-conflict }

no log { nbr-change | ipaddr-conflict | pkt-conflict }

Parameter

| Name | English prompt | Notes |
| --- | --- | --- |
| log | log -- To log conflict |  |
| nbr-change | nbr-change -- neighbor up/down or DR changes |  |
| ipaddr-conflict | ipaddr-conflict -- secondary ip address is conflict with the another neighbor |  |
| pkt-conflict | pkt-conflict -- pim-sm mroute items conflict in the pim-sm pkt |  |

Default value

The log function is disabled.

Command mode

pim-sm global view

Usage guidelines

If there is the log server, the corresponding logs will be recorded to the log server.

Example

The following example configures the switch to log the conflict when the exist secondary ip address is also contained in hello packet when receivd from another neighbor.

switch_config_ps# log nbr-change

Related command

None

### clear ip pim-sm rp-mapping

Syntax

To clear pim-sm RP mapping table information, run the following command..

clear ip pim-sm rp-mapping [A.B.C.D]

Parameter

| Name | English prompt | Notes |
| --- | --- | --- |
| pim-sm | pim-sm-- PIM sparse mode |  |
| rp-mapping | rp-mapping-- Clear RP-mapping table by BSR |  |
| A.B.C.D | A.B.C.D -- RP address |  |

Default value

None

Command mode

EXEC

Usage guidelines

The command is used to clear rp mapping table.

Example

None

### clear ip mroute pim-sm

Syntax

To clear ip multicast route pim-sm, run the following command.

clear ip mroute pim-sm  *

clear ip mroute pim-sm  gaddr [saddr]

Parameter

| Parameter | Description | Notes |
| --- | --- | --- |
| pim-sm | pim-sm -- PIM sparse mode |  |
| * | * -- Delete all PIM-SM multicast routes |  |
| gaddr | A.B.C.D-- Delete the related Group multicast route |  |
| saddr | A.B.C.D-- Delete the related Source multicast route |  |

Default value

None

Command mode

EXEC

Usage guidelines

The command is used to clear ip multicast route pim-sm.

Example

None

### show running-configure

Syntax

To display the global PIM-SM information and the main configuration information about a port, run the following command:

show running-config

Parameter

None

Default value

None

Command mode

Other modes except the user mode

Usage guidelines

According to the output information, you can check the configuration information about the current PIM-SM.

Example

None

### show ip pim-sm bsr-router

Syntax

To display pim-sm BSR information, use the command show ip pim-sm bsr-router. The output content includes the elected BSR information and the locally configured candidate RP announcement information.

show ip pim-sm bsr-router

Parameter

| Parameter | Description | Notes |
| --- | --- | --- |
| pim-sm | pim-sm -- PIM Sparse mode |  |
| bsr-router | bsr-router -- display PIM-SM bootstrap (BSR) information |  |

Default

None

Command mode

Other modes except the user mode

Usage guidelines

None

Example

Output of the command show ip pim-sm bsr-router:

switch #show ip pim-s bsr

PIMv2 BSR information:

I am BSR in sz 224.0.0.0/4

CBSR-STM state (0-c, 1-p, 2-e): 2.

Address of BSR:        172.1.1.172

BSR Priority:          64

Hash Mask Length:      30

Uptime:                05: 18: 00

BSR will expires in 00: 00: 13

Candidate-RP:  173.1.1.173(Loopback173)

Interval of Advertisements:  60 seconds

Next Advertisement will be sent in 00: 00: 07

Definitions of domains above:

| Domain | Description |
| --- | --- |
| sz | Current management domain scope |
| CBSR-STM state | Current local BSR status |
| Address of BSR | BSR address in the current domain |
| BSR Priority | BSR priority |
| Hash Mask Length | Length of the hash mask advertised by the BSR |
| Uptime | Survival time of BSR |
| BSR will expires | BSR timeout remaining time |
| Candidate-RP | Local candidate C-RP address |
| Interval of Advertisements | C-RP-ADV periodic notification interval |
| Next Advertisement | Timeout interval before the next notification |

### show ip pim-sm sz-info

Syntax

To show pim-sm sz information, including the range of sz, bsm packet number in sz, szt overtime, and the remaining time of BSM is forwarded from iif interface.

show ip pim-sm sz-info

Parameter

| Parameter | Description | Notes |
| --- | --- | --- |
| pim-sm | pim-sm -- PIM Sparse mode |  |
| sz-info | sz-info -- display PIM-SM sz information |  |

Default value

None

Command mode

Other modes except the user mode

Usage guidelines

The command is used to show pim-sm scope zone information.

Example

Output examples of the command show ip pim-sm sz-info:

switch#show ip pim-sm sz-info

PIMv2 administor scope information:

sz range:  224.0.0.0/4

group range numbers: 1

BSM pkt numbers: 0, in current forward queue

szt not create, for we are c-bsr or BST not expire for no-cbsr

switch #

Definitions of domains above:

| Parameter | Description |
| --- | --- |
| sz range | sz range |
| group range numbers | group range numbers in sz |

### show ip pim-sm interface

Syntax

Display pim-sm port related information with the following command.

show ip pim-sm interface [Type number] [count][detail]

Parameter

| Parameter | Description | Notes |
| --- | --- | --- |
| pim-sm | pim-sm-- PIM Sparse mode |  |
| interface | interface-- display PIM-SM interfaces information |  |
| Type number | See interfaces provided by CMD module. | (Optional) Interface type and number |
| count | count -- Number of packets received and sent out the interface |  |
| Detail | detail-- PIM srct of each interface |  |

Default

None

Command mode

All modes except user mode

Usage guidelines

Output of command show ip pim-sm interface:

switch#show ip pim-sm interface

Intf       Address            Ver/   Nbr    Hello  DR     DR

Mode   Count  Intvl  Prior  Addr

v8       172.20.21.172      V2/S   0      30     1      172.20.21.172

l174       174.1.1.174        V2/S   0      30     1      174.1.1.174

l173       173.1.1.173        V2/S   0      30     1      173.1.1.173

l172       172.1.1.172        V2/S   0      30     1      172.1.1.172

The following command is the output after specifying the count parameter:

switch# show ip pim-sm interface count

State:  * - Fast Switched, D - Distributed Fast Switched

H - Hardware Switching Enabled

Intf       Address            FS      Mpackets In/Out

v8       172.20.21.172     *      59573120/0

l174       174.1.1.174        *      59567488/0

l173       173.1.1.173        *      59568480/0

l172       172.1.1.172        *      59569696/0

With the detail parameter, the output example of interface vlan8 is displayed:

switch#show ip pim-sm interface vlan8 detail

v8 line protocol is up

Internet address is 172.20.21.172/24

Multicast boundary:  not set

Multicast TTL threshold:  0

PIM-SM:  enable

PIM-SM version: 2, mode: sparse

PIM-SM DR: 172.20.21.172

PIM-SM neighbor count: 0

PIM-SM Hello/Query interval: 30

PIM-SM Hello packets in/out: 0/694

PIM-SM State-Refresh processing: disable

PIM-SM State-Refresh origination: disable

PIM-SM NBMA mode: disable

PIM-SM domain border: disable

PIM-SM bsm packet horizon split: disable

Definitions of domains above:

| Field | Description |
| --- | --- |
| Address | Interface IP address of the next hop switch. |
| Interface | Interface type and number that is configured to run PIM. |
| Mode | Multicast mode in which the software is operating. |
| Neighbor Count | Number of PIM-SM neighbors that have been discovered through this interface. |
| Query Interval | Frequency, in seconds, of PIM-SM switch query messages, as set by the ip pim-sm query-interval interface configuration command. The default is 30 seconds. |
| Hello packets In/Out | Number of hello packets into and out of the interface since the switch has been up. |
| Internet address | IP address of the specified interface. |
| Multicast boundary: | Indicates whether an administratively scoped boundary is configured. |
| Multicast TTL threshold: | The time-to-live (TTL) threshold of multicast packets being forwarded out the interface. |
| PIM: | Indicates whether PIM is enabled or disabled. |
| PIM version: | Indicates whether PIM version 1 or version 2 is configured. |
| PIM mode: | Sparse mode. |
| PIM DR: | The IP address of the designated switch (DR). |
| PIM State-Refresh processing: | Indicates whether the processing of PIM state refresh control messages is enabled. |
| PIM State-Refresh origination: | Indicates whether the origination of the PIM state refresh control messages is enabled. |
| PIM NBMA mode: | Indicates whether the interface is enabled for nonbroadcast multiaccess (NBMA) mode. |
| PIM domain border: | Indicates whether the interface is enabled as a PIM domain border. |
| horizon split | Indicates whether the interface is enabled to retransmit BSM packet. |

### show ip pim-sm neighbor

Syntax

Display pim-sm neighbor information with the following command.

show ip pim neighbor [type number]

Parameter

| Parameter | Description | Notes |
| --- | --- | --- |
| Pim-sm | pim-sm -- PIM Sparse mode |  |
| neighbor | neighbor -- display PIM-SM neighbor information |  |
| Type number | See interfaces provided by CMD module. | (Optional) Interface type and number |

Default

None

Command mode

All modes except user mode

Usage guidelines

According to the output information of this command, the user can see the neighbor information of pim-sm (designated port).

Example

The following is sample output from the show ip pim-sm neighbor command:

switch#show ip pim-s nei

PIM-SMv2 Neighbor Table

Neighbor          Interface              Uptime/Expires      DR

Address                                                           Prior

172.20.21.173      v8            00: 00: 08/00: 01: 37  1(DR)

Definitions of domains above:

| Field | Description |
| --- | --- |
| Nbr Addr | IP address of the PIM-SM neighbor. |
| Interface | Interface type and number on which the neighbor is reachable. |
| Uptime | How long (in hours, minutes, and seconds) the entry has been in the PIM neighbor table. |
| Expires | How long (in hours, minutes, and seconds) until the entry will be removed from the IP multicast routing table. |
| (DR) | Indicates that this neighbor is a designated switch on the LAN. |

### show ip pim-sm rp

Syntax

Display pim-sm local RP-SET dynamic rp related information with the following command.

show ip pim-sm rp [mapping [rp-address]| metric [rp-address]| all-set]

Parameter

| Parameter | Description | Notes |
| --- | --- | --- |
| pim-sm | pim-sm -- PIM Sparse mode |  |
| rp | rp -- display PIM-SM RP information |  |
| mapping | mapping -- Displays all group-to-RP mappings | (Optional) Displays all group-to-RP mappings of which the switch is aware (either configured or learned from Auto-RP). |
| metric | metric -- Displays the unicast routing metric to the RPs | (Optional) Displays the unicast routing metric to the RPs configured statically or learned via Auto-RP or the bootstrap switch (BSR). |
| rp-address | A.B.C.D -- IP address of RP |  |
| All-set | all-set -- Displays the whole rp set |  |

Default value

None

Command mode

All modes except user mode

Usage guidelines

According to the output information of this command, you can see the dynamic rp information in RP-SET.

Example

The following is sample output of the show ip pim-sm rp command:

switch# show ip pim-s rp

GROUP:  227.1.1.1

RP:  173.1.1.173

local c-rp

R173_config#show ip pim-s rp

GROUP:  227.1.1.1

RP:  173.1.1.173

Uptime/Expires:  06: 21: 49/00: 02: 06

The following is sample output of the show ip pim-sm rp all-set command:

switch# show ip pim-s rp all

sz range: 224.0.0.0/4

RP-SET node: 224.0.0.0/4

c-rp: 173.1.1.173(pri: 10)

Definitions of domains above:

| Field | Decsription |
| --- | --- |
| sz range | Scope of management domain sz |
| RP-SET node | RP-SET corresponding node |
| Group | Address of the multicast group about which to display RP information |
| RP | Address of the RP for that group |
| Local c-rp | This candidate rp is configured locally |
| Uptime | Length of time the RP has been up (in days and hours). If less than 1 day, time is expressed in hours: minutes: seconds. |
| expires | Time in (hours, minutes, and seconds) in which the entry will expire. |
| Metric Pref | The preference value used for selecting the unicast routing metric to the RP announced by the designated forwarder (DF). |
| Metric | Unicast routing metric to the RP announced by the DF. |
| Flags | Indicates the flags set for the specified RP. The following are descriptions of possible flags: C—RP is configured. L—RP learned via Auto-RP or the BSR. |
| RPF Type | Routing table from which this route was obtained, either unicast, Distance Vector Multicast Routing Protocol (DVMRP), or static mroute. |
| Interface | Interface type and number that is configured to run PIM-SM. |

### show ip pim-sm rp-hash

Syntax

To calculate and display the rp hash value of the specified group information, use the following command.

show ip pim-sm rp-hash gaddr gmask

Parameter

| Parameter | Description | Notes |
| --- | --- | --- |
| pim-sm | pim-sm-- PIM Sparse mode |  |
| rp-hash | rp-hash-- display the RP according to the hash |  |
| gaddr | A.B.C.D-- Displays the RP information for the specified group address |  |
| gmask | A.B.C.D-- Displays the RP information for the specified group mask |  |

Default value

None

Command mode

All modes except user mode

Usage guidelines

According to the output information of this command, you can see the rp-hash information of the specified group address, including the rp address, priority, and hash value.

Example

The output example of the show ip pim-sm rp-hash command for group 239.1.1.1:

switch#show ip pim-sm rp-hash 239.1.1.1 255.255.255.255

RP:  173.1.1.173 for 239.1.1.1/0, v2

Info Source:  173.1.1.173, pri 10, holdtime 150

PIMv2 Hash Value:

RP 173.1.1.173, via local BSR, pri 10, hash value 1268904765

Definitions of domains above:

| Field | Description |
| --- | --- |
| RP: 173.1.1.173 | Address of the RP for the group specified (239.1.1.1) |
| Info source: 173.1.1.173 | Indicates from which system the switch learned this RP information |
| holdtime | RP timeout value |
| via local BSR | Local is the selected BSR |
| pri | Corresponding priority of rp. |
| hash value | The calculated hash value |

### show ip pim-sm anycast-rp

Syntax

To show the current information of anycast-rp, run the following command.

show ip pim-sm anycast-rp

Parameter

| Parameter | Description | Notes |
| --- | --- | --- |
| pim-sm | pim-sm -- PIM Sparse mode |  |
| anycast-rp | anycast-rp -- display register packet rcv and send for anycast-rp |  |

Default value

None

Command mode

Other modes except the user mode

Usage guidelines

The command is used to show the status of local anycast-rp receiving and sending register packet.

Example

Output examples of the command show ip pim-sm anycast-rp for group 239.1.1.1:

switch# show ip pim-sm anycast-rp

anycast-rp 2.2.2.2 nbr 192.168.18.1 status:  REG_SEND

Definitions of domains above:

| Parameter | Description |
| --- | --- |
| anycast-rp | 2.2.2.2 is the address of anycast-rp |
| nbr | 192.168.18.1 is the neighbor address |
| status: REG_SEND | Forward register packet to the neighbor. |

### show ip pim-sm protocol

Syntax

To show the important information related to pim-sm protocol, run the following command.

show ip pim-sm protocol

Parameter

| Parameter | Description | Notes |
| --- | --- | --- |
| pim-sm | pim-sm -- PIM Sparse mode |  |
| protocol | protocol -- protocol independent multi-cast protocol sparse mode |  |

Default value

None

Command mode

Other modes except the user mode

Usage guidelines

The command shows information about the current status, port number, neighbor number and received packet number and interval of BSM configuration of pim-sm protocol.

Example

Output examples of the command show ip pim-sm protocol:

switch# show ip pim-sm protocol

PIM-SM is active

pim-sm interface number: 3

pim-sm neighbor number: 1

pim-sm receive pkt num: 133;  bad pkt num: 0

PIM-SM current glb jp_intvl:  60(s)

PIM-SM current glb hello intvl:  30(s)

PIM-SM current glb BSM update timer intvl:  60(s)

PIM-SM current glb crp_intvl:  60(s)

PIM-SM current glb spt_intvl:  2(s)

PIM-SM BSM hold timeout:  130(s)

### show ip mroute pim-sm

Syntax

Display the pim-sm routing information, you can display the summary information of (s, g) and (*, g), or display the information of the specified item;  you can also display the detailed information with status flags.

show ip mroute pim-sm [gaddr |saddr][detail][summary]

Parameter

| Parameter | Description | Notes |
| --- | --- | --- |
| pim-sm | pim-sm -- PIM Sparse mode |  |
| gaddr | A.B.C.D -- Group address |  |
| saddr | A.B.C.D -- Source address |  |
| detail | detail-- show pim-sm inner database |  |
| summary | summary-- Displays an abbreviated summary information |  |

Default value

None

Command mode

All modes except user mode

Usage guidelines

None

Example

1. The following example shows pim-sm routing information:

switch# show ip mroute pim-sm 227.1.1.1

IP PIM-SM Multicast Routing Table:

(*, 227.1.1.1)         RP: 173.1.1.173       creat time: 3d02h00m  *G

Upstream interface:  Null0, rpf' nbr:  0.0.0.0

ps imm_olist list:

v8,

Loopback174,

2. The following example shows the summary information of pim-sm routing:

show ip mroute pim-s sum

IP PIM-SM Multicast Routing Table summary:

number of (* * RP) entry:  0

number of (* G) entry:  2

number of (s, g) spt entry:  0

number of (s, g) rpt entry:  0

(*, 237.1.1.1)         RP: 175.1.1.173       creat time: 00: 29: 44  *G

Upstream interface:  Null0, rpf' nbr:  0.0.0.0

ps imm_olist list:

v9,

(*, 239.255.255.250)   RP: 175.1.1.173       creat time: 00: 29: 29  *G

Upstream interface:  Null0, rpf' nbr:  0.0.0.0

ps imm_olist list:

v8,

Definitions of domains above:

| Field | Decsription |
| --- | --- |
| Upstream interface | RPF upstream interface of the entry. |
| ps imm_olist | Directly receiving port of Pim-sm routing entry. |
| rpf' nbr | Upstream RPF neighbor. |
| creat time | The duration of the entry since it was created. |

### show ip rpf pim-sm

Syntax

Displays the reverse path of pim-sm corresponding to the multicast source address with the following command.

show ip rpf pim-sm {source-address } [metric]

Parameter

| Parameter | Description | Notes |
| --- | --- | --- |
| pim-sm | pim-sm -- PIM Sparse mode |  |
| source-address | A.B.C.D -- IP address of multicast source |  |
| metric | metric -- Displays the unicast routing metric |  |

Default

None

Command mode

All modes except user mode

Usage guidelines

Display specify source address converse forwarding information.

Example

Output example of command show ip rpf pim-sm:

switch#show ip rpf pim-sm 172.1.1.1

PIM-SM:  show ip rpf pim-sm 172.1.1.1

PIM-SM RPF information for 172.1.1.1 :

RPF_interface:         v8

RPF_neighbor:          172.20.21.172

RPF route/mask(type):  172.1.1.0/24(rip)

The output example of the show ip rpf pim-sm command with the keyword metric:

switch#show ip rpf pim-sm 172.1.1.1 metric

PIM-SM:  show ip rpf pim-sm 172.1.1.1 metric

PIM-SM RPF information for 172.1.1.1 :

RPF_interface:         v8

RPF_neighbor:          172.20.21.172

RPF route/mask(type):  172.1.1.0/24(rip)

Metric preference:  120

Metric:  1

Definitions of domains above:

| Field | Description |
| --- | --- |
| RPF information for source address | Address of the multicast source of interest. |
| RPF interface | This port is the route egress port to the specified multicast source address, and it is also the port where multicast data arrives |
| RPF neighbor | Hope to receive the specified multicast source data from the neighbor address. |
| RPF route/mask | Route to the specified multicast source |
| RPF type | The route type to reach the specified multicast source (for example, unicast or DVMRP route, static multicast route, etc.) |
| Metric preference | Administrative distance of dependent routing protocol |
| Metric | The metric of the current route in the protocol |

### debug ip pim-sm alert

To display the alert information from mrouting or IP, run the first one of the following two commands.

debug ip pim-sm alert

no debug ip pim-sm alert

Parameter

None

Default value

None

Command mode

EXEC

Usage guidelines

VTY will be exported if the alart information from mrouting or aged (s, g).

### debug ip pim-sm assert

Syntax

To track assert messages and state machine information, run the first one of the following two commands:

debug ip pim-sm assert [packet | state- machine | A.B.C.D ]

no debug ip pim-sm assert [packet | state- machine | A.B.C.D ]

Parameter

| Name | English prompt | Notes |
| --- | --- | --- |
| state- machine | state-machine -- Trace state machine activity information |  |
| packet | packet -- Trace information about packet |  |
| A.B.C.D | A.B.C.D -- Group address for stm and packet output |  |

Default value

None

Command mode

EXEC

Usage guidelines

According to the output information, you can check the current assert event of PIM-SM.

Result

- Show (S, G) Assert State-machine Actions

- Show (*, G) Assert Message State Machine actions

- Show activity after timer timerout

- Show packet activity

### debug ip pim-sm bsr

Syntax

To monitor the BSM event of PIM-SM, the C-RP-ADV event or the BSR state machine, run the first one of the following two commands:

debug ip pim-sm bsr[packet | state- machine]

no debug ip pim-sm bsr [packet | state- machine]

Parameter

| Name | English prompt | Notes |
| --- | --- | --- |
| state- machine | state-machine -- Trace state machine activity information |  |
| packet | packet -- Trace information about packet |  |

Default value

None

Command mode

EXEC

Usage guidelines

According to the output information, you can check the BSM event of PIM-SM, the C-RP-ADV event or the BSR state machine.

Example

None

### debug ip pim-sm rp

Syntax

To monitor the related events and packets about PIM-SM-RP-SET change, run the first one of the following two commands:

debug ip pim-sm rp

no debug ip pim-sm rp

Parameter

None

Default value

None

Command mode

EXEC

Usage guidelines

This command is used to export the reception and transmission of C-RP-ADV and the RP-SET change. As to the change of static RP, no debugging information will be exported at present.

Example

None

### debug ip pim-sm entry

Syntax

To export the creation and update of (*, *, rp), (*, g), (s, g, rpt) and (s, g, spt) and their simultaneous change of them at the time of RP change, run the first one of the following two commands.

debug ip pim-sm entry

no debug ip pim-sm entry

Parameter

None

Default value

None

Command mode

EXEC

Usage guidelines

According to the output information, you can browse the information about PIMSM creation and update of multicast routing entries.

Example

None

### debug ip pim-sm event

Syntax

To export all events that PIMSM main task receives, run the first one of the following two commands.

debug ip pim-sm event

no debug ip pim-sm event

Parameter

None

Default value

None

Command mode

EXEC

Usage guidelines

According to the output information, you can browse all events of current PIMSM.

Example

None

### debug ip pim-sm hello

Syntax

To display the Hello packet received or transmitted by PIM-SM for debugging the neighbor’s information, run the first one of the following two commands.

debug ip pim-sm hello

no debug ip pim-sm hello

Parameter

None

Default value

None

Command mode

EXEC

Usage guidelines

According to the output information, you can browse the Hello packets, including those received or transmitted by PIM-SM, to know what's going on the local machine or the neighbors.

Results

- The ports, source addresses and genid of the currently received or transmitted Hello packets can be displayed.

Example

None

### debug ip pim-sm jp

Syntax

To trace the Join/Prune event of (*, g) or (s, g), run the first one of the following two commands.

debug ip pim-sm jp [packet | state- machine | A.B.C.D ]

no debug ip pim-sm jp [packet | state- machine | A.B.C.D ]

Parameter

| Name | English prompt | Notes |
| --- | --- | --- |
| state- machine | state-machine -- Trace state machine activity information |  |
| packet | packet -- Trace information about packet |  |
| A.B.C.D | A.B.C.D -- Group address for stm and packet output |  |

Default value

None

Command mode

EXEC

Usage guidelines

None

Example

None

### debug ip pim-sm nbr

Syntax

To trace the related events of a neighbor, such as the addition of neighbor, aging deletion or DR selection, run the first one of the following two commands.

debug ip pim-sm nbr

no debug ip pim-sm nbr

Parameter

None

Default value

None

Command mode

EXEC

Usage guidelines

According to the output information of this command, you can browse neighbor change, neighbor refreshment, GENID change and DR selection.

Example

None

### debug ip pim-sm packet

Syntax

To trace the protocol control packets received or transmitted by PIM-SM, run the following command.

debug ip pim-sm packet

Parameter

None

Default value

None

Command mode

EXEC

Usage guidelines

After this command is run, the receiving/sending PIM-SM packet’s type will be displayed.

Example

None

### debug ip pim-sm register

Syntax

To display the registration packet and register state event of PIM-SM, run the first one of the following two commands.

debug ip pim-sm register [packet | state- machine | A.B.C.D ]

no debug ip pim-sm register [packet | state- machine | A.B.C.D ]

Parameter

| Name | English prompt | Notes |
| --- | --- | --- |
| state- machine | state-machine -- Trace state machine activity information |  |
| packet | packet -- Trace information about packet |  |
| A.B.C.D | A.B.C.D -- Group address for stm and packet output |  |

Default value

None

Command mode

EXEC

Usage guidelines

According to the output information, you can check the register event of PIM-SM.

Example

None

### debug ip pim-sm timer

Syntax

To display the change of all PIM-SM timers, including creation, deletion, stop and timeout, run the first one of the following two commands.

debug ip pim-sm timer

no debug ip pim-sm timer

Parameter

None

Default value

None

Command mode

EXEC

Usage guidelines

The PIM-SM timers include the Hello timer, the neighbor timeout timer, pim-sm route downstream timeout timer, pim-sm route entry timeout timer, the Join/Prune timer, the override timer, the prune pending timer, the keepalive timer, the assert timer, periodic query timer for unicast routing changes, the register timer, the register limit timer, management domain timer, BSM timer, BSM forwarding timer, the BSM timer, C-RP periodic notification timer, and C-RP trigger update timer, RP-SET keeps the timer and so on.

Example

None