IPv6 Configuration Comman

Table of Contents

Chapter 1	IPv6 Configuration Commands	1

1.1 IPv6 Configuration Commands	1

1.1.1 ipv6 address	1

1.1.2 ipv6 address anycast	2

1.1.3 ipv6 address autoconfig	3

1.1.4 ipv6 address eui-64	3

1.1.5 ipv6 address link-local	4

1.1.6 ipv6 enable	5

1.1.7 show ipv6 interface	6

Chapter 2	IPv6 Configuration Commands	8

2.1	IPv6 Configuration Commands	8

2.1.1 clear ipv6 traffic	8

2.1.2 debug ipv6 packet	9

2.1.3 ipv6 mtu	10

2.1.4 ipv6 redirects	11

2.1.5 ipv6 access-group	12

2.1.6 ipv6 unreachables	12

2.1.7 ipv6 route default	13

2.1.8 show ipv6 general-prefix	14

2.1.9 show ipv6 pmtu	14

2.1.10 show ipv6 traffic	15

2.2 Network Test Tools Commands	16

2.2.1 ping6	16

2.2.2 traceroute6	18

# Chapter 1	IPv6 Configuration Commands

## 1.1 IPv6 Configuration Commands

IPv6 configuration commands include the following ones:

- ipv6 address

- ipv6 address anycast

- ipv6 address autoconfig

- ipv6 address eui-64

- ipv6 address link-local

- ipv6 enable

- show ipv6 interface

### 1.1.1 ipv6 address

Syntax

To set an IPv6 address in port configuration mode and meanwhile enable IPv6 on a port, run ipv6 address command. To delete the IPv6 address on a port, run no ipv6 address command.

ipv6 address { ipv6-address/prefix-length | general-prefix prefix-name sub-bits/prefix-length }

no ipv6 address [ ipv6-address/prefix-length | general-prefix prefix-name sub-bits/prefix-length ]

- Parameter

| Parameter | Description |
| --- | --- |
| ipv6-address | Means the to-be-added IPv6 address. |
| /prefix-length | Means the IPv6 prefix’ length. It is a decimal value behind the symbol “/”, meaning the successive bits in the network part in an address. |
| Prefix-name | Means a general prefix, defining the network part of the IPv6 address. |
| Sub-bits | Means the host part of the IPv6 address. It combines with the prefix, which is defined by prefix-name, to form an IPv6 address. This parameter must support the IPv6 address format regulated in RFC2373. |

- Default

No default IPv6 address exists on the VLAN port.

- Command Mode

VLAN Interface configuration mode

- Usage Guidelines

If you run no ipv6 address, which has no parameters, all manually configured IPv6 addresses on the VLAN port will be deleted.

- Example

The following example shows how to set an IPv6 address in VLAN port configuration mode and meanwhile enable IPv6 on the VLAN port.

Switch_config_v1# ipv6 address 2001:0:0:0:0DB8:800:200C:417A/64

- Related Commands

ipv6 address anycast

ipv6 address eui-64

ipv6 address link-local

show ipv6 interface

### 1.1.2 ipv6 address anycast

Syntax

To set an anycast address and enable IPv6 on the VLAN port, run the first one of the following two commands:

ipv6 address ipv6-prefix/prefix-length anycast

no ipv6 address [ ipv6-prefix/prefix-length anycast ]

- Parameter

| Parameter | Description |
| --- | --- |
| Ipv6-prefix | Means the network part of the IPv6 address. |
| /prefix-length | Means the IPv6 prefix length. It is a decimal value behind the symbol “/”, meaning the successive bits in the network part in an address. |

- Default

It is set as an anycast address on the VLAN port by default.

- Command Mode

VLAN Interface configuration mode

- Usage Guidelines

If you run no ipv6 address, which has no parameters, all manually configured IPv6 addresses on the VLAN port will be deleted.

- Example

Switch_config_v1# ipv6 address 2001:0DB8:1:1:FFFF:FFFF:FFFF:FFFE/64 anycast

- Related Commands

ipv6 address eui-64

ipv6 address link-local

show ipv6 interface

### 1.1.3 ipv6 address autoconfig

Syntax

To use the stateless auto-configuration protocol to set an IPv6 address, run ipv6 address autoconfig in VLAN port configuration mode.

Ipv6 address autoconfig

no ipv6 address autoconfig

- Parameter

None

- Default

By default, IPv6 address auto-configuration is not used.

- Command Mode

VLAN Interface configuration mode

- Example

Switch_config_v1# ipv6 address autoconfig

### 1.1.4 ipv6 address eui-64

Syntax

To set an IPv6 address in VLAN port configuration mode, run ipv6 address eui-64. To remove the setting, use the no form of the command.

ipv6 address ipv6-prefix/prefix-length eui-64

no ipv6 address [ ipv6-prefix/prefix-length eui-64 ]

- Parameter

| Parameter | Description |
| --- | --- |
| Ipv6-prefix | Means the network part of the IPv6 address. |
| /prefix-length | Means the IPv6 prefix length. It is a decimal value behind the symbol “/”, meaning the successive bits in the network part in an address. |

- Default

The IPv6 address in the eui-64 form is not configured on the VLAN port.

- Command Mode

VLAN Interface configuration mode

- Usage Guidelines

If you run no ipv6 address, which has no parameters, all manually configured IPv6 addresses on the VLAN port will be deleted.

If the prefix-length parameter is bigger than 64 bits, the prefix-length is prior to the length of the VLAN port ID.

- Example

Switch_config_v1# ipv6 address 2001:0:0:0:0DB8::/64 eui-64

- Related Commands

ipv6 address link-local show ipv6 interface

### 1.1.5 ipv6 address link-local

Syntax

To set a link-local address in VLAN port configuration mode and meanwhile enable IPv6 on the VLAN port, run the first one of the following two commands:

ipv6 address ipv6-address link-local

no ipv6 address [ ipv6-address link-local ]

- Parameter

| Parameter | Description |
| --- | --- |
| ipv6-address | Means the to-be-added IPv6 address. The format of this address must abide by the definition in RFC 4291 strictly. |
| link-local | Means a link-local address. The link-local address designated by the ipv6-address command will automatically replace the automatically configured link-local address. |

- Default

No default IPv6 link-local address exists on the VLAN port.

- Command Mode

Interface configuration mode

- Usage Guidelines

If you run no ipv6 address, which has no parameters, all manually configured IPv6 addresses on the VLAN port will be deleted. If you run ipv6 enable, a link-local address will be automatically set. Of course you can set the link-local address manually, the command you will use is ipv6 address link-local.

- Example

The following example shows how to set a link-local address manually on the VLAN port:

Switch_config_v1# ipv6 address FE80::A00:3EFF:FE12:3457 link-local

- Related Commands

ipv6 address eui-64 show ipv6 interface

### 1.1.6 ipv6 enable

Syntax

If the IPv6 address is not set on the VLAN port but users want to enable the IPv6 protocol on this port, run ipv6 enable.

ipv6 enable

no ipv6 enable

- Parameter

None

- Default

The IPv6 protocol is forbidden on the VLAN port.

- Command Mode

VLAN Interface configuration mode

- Usage Guidelines

After the ipv6 enable command is run, the system will add a link-local address on the VLAN port automatically. At the same time, the communication range of the IPv6 protocol on the VLAN port is confined to the links that the VLAN port connects. If the IPv6 address has already configured on the VLAN port explicitly, you cannot forbid IPv6 processing on the VLAN port even though you use the no ipv6 enable command.

- Example

Switch_config# interface vlan 1

Switch_config_v1# ipv6 enable

- Related Commands

ipv6 address link-local ipv6 address eui-64 show ipv6 interface

### 1.1.7 show ipv6 interface

Syntax

To show the information about the VLAN port on which the IPv6 protocol is enabled, run the following command:

show ipv6 interface [ interface-type interface-number ] | [brief]

- Parameter

| Parameter | Description |
| --- | --- |
| interface-type | Stands for the type of the VLAN port. |
| interface-number | Stands for the ID of the VLAN port. |

- Default

Those VLAN ports on which the IPv6 protocol is enabled will all be displayed.

- Command Mode

Global configuration mode

- Usage Guidelines

This command can be used to display the state of IPv6 on the VLAN port, the configured IPv6 address and other IPv6 related parameters.

- Example

The following example shows how to display the IPv6 state on port vlan1:

Switch# show ipv6 interface vlan 1

Vlan1 is up, line protocol is down

IPv6 is enabled, link-local address is FE80::A00:3EFF:FE12:3457 [TENTATIVE]

Global unicast address(es):

5678::111, subnet is 5678::/64 [TENTATIVE]

Joined group address(es):

FF02::1

FF02::2

FF02::1:FF12:3457

FF02::1:FF00:111

MTU is 1500 bytes

ICMP error messages limited to one every 100 milliseconds

ICMP redirects are enabled

ICMP unreachables are enabled

| Field | Description |
| --- | --- |
| Vlan1 is up(down/administratively down) | Indicates whether the physical layer of the VLAN port is accessible or whether it can be shut down manageably. |
| line protocol is up(down) | Indicates whether the line protocol (the software layer) is accessible. |
| IPv6 is enabled | Enables the IPv6 protocol. |
| link-local address | Displays the link-local address of a port. |
| Global unicast address(es) | Displays the unicast address of a port. |
| Joined group address(es) | Displays the multicast address of a port. |
| MTU | Displays the MTU of a port. |
| ICMP error messages | Displays the transmission frequency of ICMPv6 error packets (the minimum interval). |
| ICMP redirects | Displays whether the redirection packet will be sent or not. |
| ICMP unreachables | Displays whether the destination unreachable packet will be enabled or shut down. |

- Related Command

None

# Chapter 2	IPv6 Configuration Commands

## 2.1	IPv6 Configuration Commands

IPv6 configuration commands include the following ones:

- clear ipv6 traffic

- debug ipv6 packet

- ipv6 mtu

- ipv6 redirect

- ipv6 access-group

- ipv6 unreachables

- ipv6 route default

- show ipv6 general-prefix

- show ipv6 pmtu

- show ipv6 traffic

### 2.1.1 clear ipv6 traffic

Syntax

To delete the statistics information about the IPv6 flow, run the following command:

clear ipv6 traffic

- Parameter

None

- Command Mode

EXEC

- Usage Guidelines

This command is used to delete all the statistics information about IPv6 flow.

- Example

The following example shows how to delete the statistics information about IPv6 flow:

Switch# clear ipv6 traffic

Switch# show ipv6 traffic

IPv6 statistics:

Rcvd: 0 total, 0 local destination

0 badhdrs, 0 badvers

0 tooshort, 0 toosmall, 0 toomanyhdrs

0 source-routed, 0 badscope

0 badopts, 0 unknowopts, 0 exthdrtoolong

0 fragments, 0 total reassembled

0 reassembly timeouts, 0 reassembly failures

Sent: 0 generated, 0 forwarded, 0 cant forwarded

0 fragmented into 0 fragments, 0 failed

0 no route

Mcast: 0 received, 0 sent

ICMP statistics:

Rcvd: 0 total, 0 format errors, 0 checksum errors

0 unreachable, 0 packet too big

0 time exceeded, 0 parameter problem

0 echos, 0 echo replies

0 membership query, 0 membership report, 0 membership reduction

0 Switch solicitations, 0 Switch advertisements

0 neighbor solicitations, 0 neighbor advertisements, 0 redirect

Sent: 0 total, 0 bandwidth limit

0 unreachable, 0 packet too big

0 time exceeded, 0 parameter problem

0 echos, 0 echo replies

0 membership query, 0 membership report, 0 membership reduction

0 Switch solicitations, 0 Switch advertisements

0 neighbor solicitations, 0 neighbor advertisements, 0 redirect

- Related Commands

show ipv6 traffic

### 2.1.2 debug ipv6 packet

Syntax

To display the debug information about the IPv6 packet, run the first one of the following two commands:

debug ipv6 packet [ interface interface-type interface-number | access-list [ access-list-name ] | detail ]

no debug ipv6 packet

- Parameter

| Parameter | Description |
| --- | --- |
| Interface-type | Type of the interface (optional) |
| Interface-number | ID of an interface (optional) |
| access-list-namae | Name of ACL (optional) |

- Default

The debug information is disabled in default settings.

- Command Mode

EXEC

- Example

The following example shows how to export the IPv6 debug information:

Switch# debug ipv6 packet

2002-1-1 05:07:16

IPv6: source FE80::A00:3EFF:FE12:3459, dest FF02::1

plen 32, proto 58, hops 255

sending on Ethernet1/0

| Field | Description |
| --- | --- |
| source | Source address in the IPv6 packet |
| dest | Destination address in the IPv6 packet |
| plen | Load length in the IPv6 packet |
| proto | Protocol for the next header encapsulation, which is presented by next-header in the IPv6 packet |
| hops | Value of hop-limit in the IPv6 packet |
| sending (receiving, forwarding) on Ethernet | Displays packet transmission, reception and forwarding on an interface. |

### 2.1.3 ipv6 mtu

Syntax

To set the MTU of the VLAN port, run the first one of the following two commands. To return to the default setting, use the no form of this command.

ipv6 mtu bytes

no ipv6 mtu

Parameter

| Parameter | Description |
| --- | --- |
| bytes | MTU(unit : byte) |

Default

The default value depends on the port type, but the minimum value of any port is 1280 bytes.

Command Mode

VLAN interface configuration mode

Usage Guidelines

When MTU is the default value, RA has the MTU option.

When a switch forwards packet, a packet will not be fragmented just because the MTU of the egress is smaller than the packet’s length. But it will be fragmented only when the transmitted packet is generated.

Example

The following example shows how to set MTU of the interface:

Switch_config_v1# ipv6 mtu 1400

Related Command

show ipv6 interface

### 2.1.4 ipv6 redirects

- Syntax

To control whether to transmit a redirection packet after the packet is forwarded, run ipv6 redirects.

ipv6 redirects

no ipv6 redirects

- Parameter

None

- Default

The redirection packet will be transmitted by default.

- Command Mode

VLAN Interface configuration mode

- Usage Guidelines

The redirection packets are transmitted through the ICMPv6 protocol.

- Example

- The following example shows how to enable a port to transmit the redirection packet.

Switch_config_v1# ipv6 redirects

- The following example shows how to disable a port to transmit the redirection packet.

Switch_config_v1# no ipv6 redirects

Use show ipv6 interface command can check whether the port will transmit the redirection packet.

- Related Commands

show ipv6 interface

### 2.1.5 ipv6 access-group

- Syntax

To filter the received packet of some port, run the first one of the following commands. To return to the default setting, use the no form of this command.

ipv6 access-group access-list-name { in | out }

no ipv6 access-group { in | out }

Parameter

| Parameter | Description |
| --- | --- |
| access-list-name | access list name |
| In | Filter direction, receiving packets |
| Out | Filter direction, forwarding packets |

Default

The filter function is not set by default.

Command Mode

VLAN interface configuration

Usage Guidelines

None

Example

The following example shows how to use the access list test to filter packets when received in port vlan 1:

Switch_config_v1# ipv6 access-group test in

Related Commands

Ipv6 access-list

Show ipv6 interface

### 2.1.6 ipv6 unreachables

Syntax

To enable an interface to generate and transmit the ICMPv6 unreachable packets, run ipv6 unreachable.

ipv6 unreachables

no ipv6 unreachables

- Parameter

None

- Default

The unreachable packets are transmitted by default.

- Command Mode

VLAN Interface configuration mode

- Usage Guidelines

The unreachable packets are transmitted through the ICMPv6 protocol.

- Example

The following example shows how to disable the VLAN interface to transmit the unreachable packets.

Switch_config_v1# no ipv6 unreachables

- Related Commands

None

### 2.1.7 ipv6 route default

Syntax

To configure the default ip6 gateway, use ipv6 route default command.

ipv6 route default [NULL | X:X:X:X::X]

no ipv6 route default [NULL | X:X:X:X::X]

Parameter

| Parameter | Description |
| --- | --- |
| NULL | NULL interface |
| X:X:X:X::X | Gateway address |

Default

None

Command Mode

Global configuration mode

Example

The following example shows how to set the address of 2008::1 as the route default of the switch.

ipv6 route default 2008::1

Related Command

None

### 2.1.8 show ipv6 general-prefix

Syntax

To display the detailed information about the general prefix of IPv6, run the following command:

show ipv6 general-prefix

Parameter

None

Command Mode

EXEC

Example

Switch_config#show ipv6 general-prefix

IPv6 Prefix my-prefix, acquired via manual

2002::/64

Vlan1 (Address command)

| Field | Remarks |
| --- | --- |
| IPv6 Prefix | Stands for the name of the general IPv6 prefix defined by user. |
| Acquire via | Stands for the configuration method of the general prefix. At present, the manual configuration and the automatic DHCP obtainment are supported. |
| Vlan1 (Address command) | Stands for a list of ports that use this general prefix. |

Related Commands

ipv6 general-prefix

### 2.1.9 show ipv6 pmtu

Syntax

To display the cache options of path MTU, run the following command:

show ipv6 pmtu

Parameter

None

Command Mode

EXEC

Example

Switch_config#show ipv6 pmtu

PMTU Expired Destination Address

1300 00:04:00 2002:1::1

1280 00:01:00 2001:2::2

The cache of path MTU stores the path MTU which is applied to reach a destination address. If the to-be-transmitted packets generated by routers or switches are bigger than the path MTU, they will be fragmented during being transmitted.

The switch will create a path MTU record when receiving an ICMPv6 too-big packet.

| Field | Remarks |
| --- | --- |
| MTU | Stands for the value of the path MTU. It is this MTU that is contained in the received ICMPv6 too-big packet. |
| Expired | It means expiration. The expiration time starts counting when the ICMPv6 “too-big” packet is received. When the expired parameter is 0, this path MTU record will be deleted. |
| Destination Address | Stands for the destination address. It is this destination address that is contained in the received ICMPv6 too-big packet. |

Related Commands

ipv6 mtu

### 2.1.10 show ipv6 traffic

Syntax

To display the statistics of IPv6 flow, run the following command:

show ipv6 traffic

Parameter

None

Command Mode

EXEC

Example

Switch#show ipv6 traffic

IPv6 statistics:

Rcvd: 0 total, 0 local destination

0 badhdrs, 0 badvers

0 tooshort, 0 toosmall, 0 toomanyhdrs

0 source-routed, 0 badscope

0 badopts, 0 unknowopts, 0 exthdrtoolong

0 fragments, 0 total reassembled

0 reassembly timeouts, 0 reassembly failures

Sent: 25 generated, 0 forwarded, 0 cant forwarded

0 fragmented into 0 fragments, 0 failed

0 no route

Mcast: 0 received, 25 sent

ICMP statistics:

Rcvd: 25 total, 0 format errors, 0 checksum errors

0 unreachable, 0 packet too big

0 time exceeded, 0 parameter problem

0 echos, 0 echo replies

0 membership query, 0 membership report, 0 membership reduction

0 Switch solicitations, 0 Switch advertisements

0 neighbor solicitations, 0 neighbor advertisements, 0 redirect

Sent: 0 total, 0 bandwidth limit

0 unreachable, 0 packet too big

0 time exceeded, 0 parameter problem

0 echos, 0 echo replies

0 membership query, 0 membership report, 0 membership reduction

0 Switch solicitations, 0 Switch advertisements

0 neighbor solicitations, 0 neighbor advertisements, 0 redirect

Related Commands

clear ipv6 traffic

## 2.2 Network Test Tools Commands

IPv6 network test tools commands include:

- ping6

- traceroute6

### 2.2.1 ping6

Syntax

To test the accessibility and connectivity of the host, run the following command. The command enables the host forwards ICMP packets to another host and get ICMP response packets from it.

ping6 host [-a] [-l length] [-n number] [-v] [–w waittime] [-b internal]

Parameter

| Parameter | Description |
| --- | --- |
| host | Destination host address or host name. |
| -a | Continuous forward icmp echo request packets until the user disable it manually. |
| -l length | Set the length of the ICMO data in the packets. The default is 56 bytes. |
| -n number | Set the number of toal transmitted packets. The default one is 5 packets. |
| -w waittime | The time of waiting each packet response. The default one is 2 seconds. |
| -b interval | Set the time interval of transmitting ping packets. Unit: 10ms; value ranges: 0-65535; the default is 0. |

Command Mode

Exec, global configuration mode

Usage Guidelines

To disable ping, press “q” or “Q”.

The output by default:

| Parameter | Description |
| --- | --- |
| ! | Receive the response packet. |
| . | No response packet in timeout. |
| U | Receive the ICMP destination unreachable packet. |
| R | Receive the ICMP redirection packet. |
| T | Receive the ICMP timeout packet. |
| P | Receive the ICMP parameter packet. |

The output of statistics information:

| Parameter | Description |
| --- | --- |
| packets transmitted | The number of transmitted packets. |
| packets received | The number of received packets, not include other ICMP packets. |
| packet loss | The proportion of packet loss. |
| round-trip min/avg/max | The minimum/average/max response time (ms). |

The routing switch supports the destination address is the link-local address or multicast address. When ping this address, the command must designate the vlan interface first and forward ICMP echo packets on the interface. The routing switch will output all addresses of response hosts.

Example

switch#ping6 2008::2 -l 10000 -n 30

PING 2008::2 (2008::2): 10000 data bytes

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

--- 2008::2 ping6 statistics ---

30 packets transmitted, 30 packets received, 0% packet loss

round-trip min/avg/max = 0/1/20 ms

·ping multicast address:

switch#ping6 ff02::1 vlan 1 -n 2

PING 1 (FF02:1B::1): 56 data bytes

Reply to request 0 from FE80::2E0:FFF:FEDB:583F, <10 ms

Reply to request 0 from FE80::1EAF:F7FF:FE35:D02A, 10 ms

Reply to request 1 from FE80::2E0:FFF:FEDB:583F, <10 ms

Reply to request 1 from FE80::1EAF:F7FF:FE35:D02A, 10 ms

### 2.2.2 traceroute6

The command is used to trace the destination route.

Transmit the UDP packets (or ICMP ECHO packets) with different TTL (Time to Live) to the destination, so that the route of the destination can be traced. Each router in the path has to reduce its TTL by 1 at least before transmitting the ICMP response packets, therefore, TTL is an effective way for accounting. When the TTL of the packet reduce to 0, the router sends back ICMP timeout information to the source system. Transmit first response packet whose TTL is 1 and add 1 to the TTL in subsequent transmission until the receive the response packet or reach the max TTL value.

Trace the router by detecting ICMP Time Exceeded information it sends back. When reaching the destination, the destination node can only sends back ICMP information that the port is unreachable, as traceroute transmit UDP packets whose port number is greater than 30000. The destination is arrived as the report is received.

Syntax

traceroute6 host [-i source-ip-address] [-p port-number] [-q probe-count] [-t ttl] [-w waittime] [-x icmp]

Parameter

| Parameter | Description |
| --- | --- |
| host | Destination host address or host name. |
| -i source-ip-address | Sets the source IP address. |
| -p port-number] | Sets the destination port number transmitting UDP packets. The default is 33434. |
| -q probe-count | Sets the packet number of each detection. The default is 3 packets. |
| -t ttl | Sets the IP TTL of the packet to ttl. The TTL is 1 in minimum by default and 30 in maximum. |
| -w waittime | The time of each packet for waiting response. The default time is 3 seconds. |
| -x icmp | Sets the detection packet as the ICMP ECHO packet. The default is UDP packet. |

Command Mode

EXEC, Global Configuration Mode

Usage Guidelines

The command is used to trace the router by UDP packets. To trace the router by ICMP ECHO packets by command –x icmp. To disable traceroute, press “q” or “Q”.

Output by default:

| Parameter | Description |
| --- | --- |
| !N | Receive ICMP destination unreachable packets (the route is unreachable, code: 0) |
| !P | Receive ICMP destination unreachable packets (forbid out of management, code: 1) |
| !S | Receive ICMP destination unreachable packets (not neighbor, code: 2) |
| !A | Receive ICMP destination unreachable packets (unreachable address, code: 3) |
| ! | Receive ICMP destination unreachable packets (unreachable port, code: 4) |

The output of statistics information:

| Parameter | Description |
| --- | --- |
| hops max | Max trace hop number (The uplimit of ICMP increment) |
| byte datalen | Size of each trace packet |

Example

switch#traceroute6 2008::2

tracert6 to 2008::2 , 30 hops max, 12 byte datalen

1 2008::2 0 ms * 0 ms