NTP Configuration Commands

- Table of Contents

Chapter 1 NTP Configuration Commands	1

1.1 ntp master	1

1.2 ntp authentication enable	2

1.3 ntp authentication key	3

1.4 ntp authentication trusted-key	4

1.5 ntp server	5

1.6 ntp peer	6

1.7 show ntp	7

1.8 debug ntp	8

1.9 time-zone	9

# Chapter 1 NTP Configuration Commands

## 1.1 ntp master

Syntax

ntp master primary command is used to set the device as the original NTP server (stratum=1)

ntp master secondary command is used to set device as the secondary NTP server

no ntp master command is used to disable NTP server

Parameter

None

Default

None

Command Mode

Global Configuration Mode

Usage Guidelines

If the device is not configured with NTP server (ntp server command is not configured), ntp master primary command must be configured. Or the switch cannot provide time synchronization service. ntp master secondary command must be run when the switch configures NTP server. Moreover, the switch can provide time synchronization service to the NTP client in condition its own time synchronization is realized.

Example

Switch_config#ntp master primary

Switch_config#ntp master secondary

Switch_config#no ntp master

Related Command

ntp server

ntp peer

## 1.2 ntp authentication enable

Syntax

ntp authentication enable command

no ntp authentication enable

To enable NTP identity authentication, run the above command. To return to the default setting, use the no form of this command.

Parameter

None

Default

Disabled

Command Mode

Global Configuration Mode

Usage Guidelines

For a secure network, NTP identity authentication must be enabled when operating NTP protocol. The identity authentication ensures that the client only realize time synchronization with the server which passes the identity authentication. Thus, the client will not obtain error time information from the illegal server.

Example

Switch_config#ntp authentication enable

Related Commands

ntp authentication key

ntp authentication trusted-key

## 1.3 ntp authentication key

Syntax

ntp authentication key keyid md5 password

no ntp authentication key keyid

To set NTP identity authentication key, run the first one of the above commands. To return to the default setting, use the no form of this command.

Parameter

| Parameter | Description |
| --- | --- |
| keyid | The serial number of the authentication key. The value ranges from 1 to 4294967295. |
| password | The key of keyed. The length ranges from 1 to 50. |

Default

None

Command Mode

Global Configuration Mode

Usage Guidelines

The command is used to set identity authentication key. The client and the server must set the same key serial number and key value, or they cannot realize time synchronization.

After set NTP authentication key, Set the key as the trusted key by command ntp authentication trusted-key. The trusted key will automatically disappear from the trusted key list when it is deleted.There is no need to run command “no ntp authentication trusted-key”.

The command can set multiple ntp authentication key commands.

Example

Switch_config#ntp authentication key 5 md5 abc123

Switch_config#no ntp authentication key 5

Related Commands

ntp authentication enable

ntp authentication trusted-key

## 1.4 ntp authentication trusted-key

Syntax

ntp authentication trusted-key keyid

no ntp authentication trusted-key keyid

To set the created key as the trusted key, run the first one of the above commands. To return to the default setting, use the no form of this command.

Parameter

| Parameter | Description |
| --- | --- |
| keyid | The serial number of the trusted key. The value ranges from 1 to 4294967295. |

Default

None

Command Mode

Global Configuration Mode

Usage Guidelines

Enable the identity authentication function, the client can only time synchronize with the server providing the trusted key. If the key provided by the server is not trusted, the client cannot synchronize to the NTP server.

The command must be configured after the key is set. The trusted key will automatically disappear from the trusted key list when it is deleted. There is no need to run command “no ntp authentication trusted-key”.

Example

Switch_config#ntp authentication trusted-key 5

Switch_config#no ntp authentication trusted-key 5

Related Commands

ntp authentication enable

ntp authentication key

## 1.5 ntp server

Syntax

ntp server ip-address [version number | key keyid ]*

no ntp server ip-address

To set NTP server, run the first one of the above commands. To return to the default setting, use the no form of this command.

Parameter

| Parameter | Description |
| --- | --- |
| ip-address | NTP Server IP address |
| number | NTP version number, the value ranges from: <1-4>, the default value is 4. |
| keyid | When sending NTP packets to the NTP server, calculate the packet information abstract with the key corresponds to the keyid. The value ranges from 1 to 4294967295. If the parameter is not set, the device will not authenticate the identity of the server, or vice verse. |

Default

None

Command Mode

Global Configuration Mode

Usage Guidelines

After setting the NTP server, the device can time synchronize with the server, but the time of the server will not synchronize to the device.

The command can be used to set multiple ntp server commands. If the NTP server in the public network is applied, at least 4 different NTP servers need to be configured (in order to exclude the wrong clock source).

Example

Switch_config#ntp server 1.1.1.1 version 4 key 5

Related Commands

ntp authentication enable

ntp authentication key

ntp authentication trusted-key

## 1.6 ntp peer

Syntax

ntp peer ip-address [version number | key keyid ]*

no ntp peer ip-address

To designate the NTP peer for the device, run the first one of the above commands. To return to the default setting, use the no form of this command.

Parameter

| Parameter | Description |
| --- | --- |
| ip-address | NTP peer IP address |
| number | NTP version number. The value ranges from 1 to 4. The default value is 4. |
| keyid | When sending NTP packets to the NTP server, calculate the packet information abstract with the key corresponds to the keyid. The value ranges from 1 to 4294967295. If the parameter is not set, the device will not authenticate the identity of the server, or vice verse. |

Default

None

Command Mode

Global Configuration Mode

Usage Guidelines

Designate the NTP peer and the device can time synchronize with the peer in condition that it has synchronized. The command is usually used for mutual backup of the NTP server, not applicable for the client. To designate the NTP server, run the command ntp server.

Example

Switch_config#ntp peer 1.1.1.2 version 3 key 5

Related Commands

ntp authentication enable

ntp authentication key

ntp authentication trusted-key

## 1.7 show ntp

Syntax

show ntp [status]

To show NTP current status, run the above command.

show ntp associations [detail]

To show NTP association status, run the above command.

show ntp timers

To show NTP timer status, run the above command.

Parameter

None

Default

None

Command Mode

EXEC

Usage Guidelines

Show NTP relevant information

Example

Switch#show ntp

Time-zone: GMT+8:00, Shanghai

Current time: 2014-05-21 10:45:26

Clock Status: synchronized

Clock Stratum: 3

Leap Indicator: 0

Reference ID: 211.233.84.186

Clock Jitter: 0.004149

Clock Precision: -18

Clock Offset: 6.561 ms

Root Delay: 172.153 ms

Root Dispersion: 587.873 ms

Packets Sent: 30788

Packets Received: 27969 (bad version: 0)

Reference Time: 2014-05-21 10:41:37

Last Update Time: 2014-05-21 10:37:08

Switch#show ntp associations

ip address      reference clock   st   poll   reach   delay   offset   dispersion

====================================================================

61.110.197.50   204.123.2.5      2   64     377    59.99   0.96     2.7

27.114.150.12   193.190.230.65   2   64     377    489.97  -34.56   3.1

*211.233.84.186  204.123.2.5      2   64     377    19.99   9.15     3.0

198.55.111.50   216.229.0.50     3   64     377    229.98  -40.09   3.4

199.241.31.224  132.163.4.103    2   64     377    198.04  2.51     3.6

204.2.134.163   241.199.164.101  2   64     360    169.97  -17.16   942.8

====================================================================

Note: * system peer(master), poll(s), delay(ms), offset(ms), dispersion(ms)

Total Associations: 6

Related Command

None

## 1.8 debug ntp

Syntax

debug ntp packet

To enable NTP packet debug switch, run the above command.

debug ntp event

To enable NTP event debug switch, run the above command.

debug ntp error

To enable NTP error debug switch, run the above command.

debug ntp all

To enable NTP all debug switches, run the above command.

no debug ntp

To disable all debug switches, run the above command.

Parameter

None

Default

None

Command Mode

EXEC

Usage Guidelines

Check NTP running process by debug information.

Example

None

Related Command

None

## 1.9 time-zone

Syntax

time-zone name offset-hour [offset-minute]

no time-zone

To enable time zone function, run the first one of the above commands.

To return to the default setting, use the no form of this command.

Parameter

| Parameter | Description |
| --- | --- |
| name | Name of the time zone |
| offset-hour | Hour off-set of local time to UTC time (-12~12) |
| offset-minute | Minute offset of local time to UTC time (0~59); the default value is 0. |

Default

None

Command Mode

Global Configuration Mode

Usage Guidelines

The command is used to transfer UTC to the local time.

Example

Switch_config#time-zone Beijing 8

Related Command

None