- Optical Port Additional Function Configuration Commands

- Table of Contents

Chapter 1	Optical Port Additional Function Configuration Commands	1

1.1 Configure the DDM Function	1

1.1.1 ddm enable	1

1.1.2 no ddm	1

1.2 Configure the single-fiber transceiver function of the optical port	2

1.2.1 single-fiber one-way	2

1.3 Optical module auto-configuration	3

1.3.1 fiber-auto-config	3

# Optical Port Additional Function Configuration Commands

## Configure the DDM Function

### ddm enable

Syntax

To enable DDM detection function of the optical port, use ddm enable command.

ddm enable

Parameter

| Parameter | Description |
| --- | --- |
| enable | Enable optical port ddm detection. |

Default

None

Command mode

Global configuration mode

Example

ddm enable

### no ddm

Syntax

no ddm

To disable DDM detection function of the optical port, use no ddm command.

Parameter

None

Default

None

Command mode

Global configuration mode

Example

The following example shows how to disable DDM detection function of the optical port.

Switch-config#no ddm

## Configure the single-fiber transceiver function of the optical port

### single-fiber one-way

Syntax

To configure the transceiver function of the optical fiber, use the following command.

single-fiber one-way <tx|rx>

no single-fiber

Parameter

| Parameter | Description |
| --- | --- |
| tx | Configure the port to work in single-fiber transmission mode. |
| rx | Configure the port to work in single-fiber receiving mode. |

Default

None

Command mode

None

Example

The following example shows how to configure single-fiber transmission mode isolation on port g0/1.

Switch_config_g0/1#single-fiber one-way tx

The following example shows how to cancel single fiber mode of port g0/1.

Switch_config_g0/1#no single-fiber

## Optical module auto-configuration

### fiber-auto-config

Syntax

To enable the storm control function of the port, use the following command.

fiber-auto-config {full}

Parameter

| Parameter | Description |
| --- | --- |
| full | When switching to gigabit mode mode, configure in forced mode. |
| None | When switching to gigabit mode, configure in adaptive mode. |

Default

None

Command mode

Interface configuration mode

Example

The following example enables the optical module auto-configuration on interface g0/1, and use the adaptive mode in gigabit mode.

Switch_config#interface g0/1

Switch_config_g0/1#fiber-auto-config

The following example enables the optical module auto-configuration on interface g0/1, and use the forced mode in gigabit mode.

Switch_config#interface g0/1

Switch_config_g0/1#fiber-auto-config full

The following example cancels optical module auto-configuration of interface g0/1.

Switch_config#interface g0/1

Switch_config_g0/1#no fiber-auto-config

Note:

When using a Gigabit optical conversion module that supports 10/100/1000M mode, you must configure this command in adaptive mode. When using a Gigabit optical conversion module that only supports the 1000M mode, you do not need to configure this command.