- Port Physical Characteristics Configuration

- Table of Contents

Chapter 1 Port Physical Characteristics Configuration	1

1.1 Configuring the Ethernet Interface	1

1.1.1 Configuring Rate	1

1.1.2 Configuring the Duplex Mode of an Interface	1

1.1.3 Configuring Flow Control on the Interface	1

# Port Physical Characteristics Configuration

## Configuring the Ethernet Interface

### Configuring Rate

The Ethernet rate can be realized through auto-negotiation or configuration on the interface.

Run the following command to configure the Ethernet rate:

| Command | Purpose |
| --- | --- |
| Speed {10¦100¦auto} (T port) Speed {100¦1000¦auto} (SFP port) | Set the rate of fast Ethernet to 10M, 100M, 1000M or auto-negotiation. |
| No speed | Resume the default settings—auto-negotiation. |

Note:

The speed of the optical interface is fixed. For example, the rate of GBIC and GE-FX is 1000M; the rate of FE-FX is 100M. If the auto parameter is behind the speed command, it means that you can enable the auto-negotiation function on the optical interface. Otherwise, you cannot enable the auto-negotiation function on the optical interface.

### Configuring the Duplex Mode of an Interface

By default, the Ethernet interface can be auto, half duplex or full duplex. The gigabit combo SFP/TX ports does not support speed 1000 and compulsory duplex mode simultaneously.

| Command | Purpose |
| --- | --- |
| duplex {full ¦ half ¦ auto} | Sets the duplex mode of the Ethernet. |
| No duplex | Resumes the default setting. The duplex mode is auto-negotiation. |

### Configuring Flow Control on the Interface

When the interface is in full-duplex mode, the flow control is achieved through the PAUSE frame defined by 802.3X. When the interface is in half-duplex mode, the flow control is achieved through back pressure.

| Command | Purpose |
| --- | --- |
| flow-control {on ¦ off ¦ auto} | Enable or disable the flow control on the interface. |
| no flow-control | Resume the default settings. The default settings have no flow control. |

Note:

The difference between “flow-control auto” and “flow-control on” is that the flow control frame is compulsory received. The flow control frame is forwarded when the peer negotiation is successful in “auto” mode.