CFM and Y1731 Configuration

- Table of Contents

Chapter 1 Overview	1

1.1 Stipulations	1

1.1.1 Format Stipulation in the Command Line	1

Chapter 2 CFM Configuration	2

2.1 CFM Configuration Task List	2

2.2 CFM Maintenance Task List	2

2.3 CFM Configuration	2

2.3.1 Adding the Maintenance Domain	2

2.3.2 Adding the Maintenance Association	2

2.3.3 Adding MIP (Maintenance Domain Intermediate Point)	2

2.3.4 Adding MEP (Maintenance association End Point)	3

2.3.5 Starting CFM	3

2.4 CFM Maintenance	3

2.4.1 Using the Loopback Function	3

2.4.2 Using the Linktrace Function	3

2.5 Configuration Example	4

Chapter 3 Y1731 Configuration	5

3.1 Configuration Task List	5

3.1.1 Specifying an MEP to Forward AIS Frame	5

3.1.2 Displaying the Information About OAM Protocol	5

3.1.3 Deleting Y1731 Configuration or Statistics Information	6

# Overview

## Stipulations

### Format Stipulation in the Command Line

| Syntax | Meaning |
| --- | --- |
| Bold | Stands for the keyword in the command line, which stays unchanged and must be entered without any modification. It is presented as a bold in the command line. |
| {italic} | Stands for the parameter in the command line, which must be replaced by the actual value. It must be presented by the italic in the brace. |
| <italic> | Stands for the parameter in the command line, which must be replaced by the actual value. It must be presented by the italic in the point bracket. |
| [ ] | Stands for the optional parameter, which is in the square bracket. |
| { x ¦ y ¦ ... } | Means that you can choose one option from two or more options. |
| [ x ¦ y ¦ ... ] | Means that you can choose one option or none from two or more options. |
| { x ¦ y ¦ ... } * | Means that you has to choose at least one option from two or more options, or even choose all options. |
| [ x ¦ y ¦ ... ] * | Means that you can choose multiple options or none from two or more options. |
| &<1-n> | Means that the parameter before the “&” symbol can be entered n times. |
| # | Means that the line starting with the “#” symbol is an explanation line. |

# CFM Configuration

## CFM Configuration Task List

- Adding the Maintenance Domain

- Adding the Maintenance Association

- Adding MIP (Maintenance domain Intermediate Point)

- Adding MEP (Maintenance association End Point)

- Starting CFM

## CFM Maintenance Task List

- Using the Loopback Function

- Using the Linktrace Function

## CFM Configuration

### Adding the Maintenance Domain

Configuration mode: Global

| Command | Purpose |
| --- | --- |
| ethernet cfm md mdnf {string} mdn <char_string> [level <0-7> ¦ creation <MHF_creation_type> ¦ sit <sender_id_type> ¦ ip <IP_address>] | Adds a maintenance domain whose name is char_string. Note: - The system enters the maintenance domain configuration mode after the maintenance domain is added. |

### Adding the Maintenance Association

Configuration mode: maintenance domain

| Command | Purpose |
| --- | --- |
| ma manf {string} man <char_string> ci {100ms ¦ 1s ¦ 10s ¦ 1min ¦ 10min} meps <mepids> [vlan <1-4094> ¦ creation <MHF_creation_type> ¦ sit <sender_id_type> ¦ ip <IP_address>] | Adds a maintenance association whose name is char_string. |

### Adding MIP (Maintenance Domain Intermediate Point)

Configuration mode: physical interface

| Command | Purpose |
| --- | --- |
| ethernet cfm mip add level <0-7> [vlan <1-4094>] | Adds a designated VLAN and hierarchical MIP to the designated physical interface. |

### Adding MEP (Maintenance association End Point)

Configuration mode: physical interface

| Command | Purpose |
| --- | --- |
| ethernet cfm mep add mdnf {string} mdn <char_string> manf {string} man <char_string> mepid <1-8191> [direction {up ¦ down} ¦ ip <ip_address>] | Adds a designated maintenance domain and an MEP to the designated physical interface. |

### Starting CFM

Configuration mode: Global

| Command | Purpose |
| --- | --- |
| ethernet cfm {enable} | Starts CFM. |

## CFM Maintenance

### Using the Loopback Function

Configuration mode: EXEC

| Command | Purpose |
| --- | --- |
| ethernet cfm loopback mdnf {string} mdn <char_string> manf {string} man <char_string> mepid <1-8191> mac <AA:BB:CC:DD:EE:FF> number <1-64> | Uses a designated MEP to conduct loopback towards itself. |

### Using the Linktrace Function

Configuration mode: EXEC

| Command | Purpose |
| --- | --- |
| ethernet cfm linktrace mdnf {string} mdn <char_string> manf {string} man <char_string> mepid <1-8191> mac <AA:BB:CC:DD:EE:FF> [ttl {1-255} ¦ fdb-only {yes}] <char_string> manf {string} man <char_string> mepid <1-8191> mac <AA:BB:CC:DD:EE:FF> ttl <1-255> | Uses a designated MEP to conduct loopback towards itself. |

## Configuration Example

You want to add a maintenance domain whose name is customer and hierarchy is 5, set a customer1 maintenance association for vlan1, configure the transmission interval of CCM of the maintenance association to 1s and add an MEP whose MEPID is 2009 to physical port1.

Switch_config#ethernet cfm md mdnf string mdn customer level 5

Switch_config_cfm#ma manf string man customer1 vlan 1 ci 1s meps 1-2,2009

Switch_config_cfm#interface g0/1

Switch_config_g0/1#ethernet cfm mep add mdnf string mdn customer manf string man customer1 mepid 2009 direction DOWN

Switch_config_g0/1#exit

Switch_config#ethernet cfm enable

# Y1731 Configuration

## Configuration Task List

- Specifying an MEP to Forward AIS Frame

- Enabling Frame Delay Measurement

- Displaying the Information About OAM Protocol

### Specifying an MEP to Forward AIS Frame

{0>通过下面的命令指定一个能发送AIS帧的MEP：<}0{>Run the following commands specify an MEP to transmit AIS frames: <0}

| Procedure | Command | Purpose |
| --- | --- | --- |
| Step1 | config | Enters the global configuration mode. |
| Step2 | ethernet y1731 ais-mep timer time | {0>指定AIS报文发送的间隔时间：<}0{>Designates the transmission interval of AIS packets.<0} <1> -- 1 frame per second <2> -- 1 frame per minute {0>默认值１（每秒１帧）<}0{>The default transmission value is 1 second.<0} |
| Step3 | interface intf-type intf-id | Enters the interface configuration mode. |
| Step4 | ethernet y1731 ais-mep MEGID MEPID | {0>指定能发送AIS帧的mep．<}0{>Specifies an MEP to transmit AIS frames.<0} {0>MEGID为MEP所属的MEG的名字．<}0{>MEGID is the name of MEG to which MEP belongs.<0} {0>MEPID为指定的MEP的标示符．<}0{>MEPID is the identifier of the specified MEP.<0} |

{0>可以输入no　ethernet y1731 ais-mep timer 来恢复AIS帧的发送周期为默认周期．输入no　ethernet y1731　ais-mep　MEGID   MEPID来删除能够发送AIS的MEP。<}0{>You can run no ethernet y1731 ais-mep timer to resume the default transmission period of AIS frames and run no ethernet y1731 ais-mep MEGID MEPID to delete AIS transmitter, MEP.<0}

### Displaying the Information About OAM Protocol

{0>使用show命令来查看Y1731的配置信息：<}0{>Run show to browse Y1731 configuration: <0}

| Command | Purpose |
| --- | --- |
| show ethernet y1731 ais-mep | The above-mentioned command is used to show the MEPs that can transmit AIS frames. |
| show ethernet y1731 detect MEGID [MEPID ] | The above-mentioned command is used to display the detection information about the continuous check of MEG, including whether continuity is lost or whether other faults occur. {0>MEGID为的MEG的名字．<}0{>MEGID is the name of MEG.<0} {0>MEPID为要显示的MEP的符号。<}0{>MEPID is the symbol of to-be-displayed MEP.<0} |
| show ethernet y1731 interface interface-name | Displaying MEP and MIP Configurations on a Port {0>interface-name 为端口标识。<}0{>interface-name stands for port identifier.<0} |
| show ethernet y1731 meglist [ MEGID ] | The above-mentioned command is used to display the configuration of all MEG or the detailed configuration about a certain MEG. {0>MEGID为要显示的MEG的名字．<}74{>MEGID is the name of to-be-displayed MEG.<0} |
| show ethernet y1731 miplist | The above-mentioned command is used to display the information about all configured MIPs. |
| show ethernet y1731 traffic | The above-mentioned command is used to display some statistics information about the Y.1731 module, including statistics of the received and transmitted OAM packets and the system error. |

### {0>清除Y1731协议配置/统计信息<}0{>Deleting Y1731 Configuration or Statistics Information<0}

{0>使用clear命令来清除Y1731的配置/统计信息：<}71{>Run clear to browse Y1731 configuration and statistics information: <0}

| Command | Purpose |
| --- | --- |
| clear ethernet y1731 counters | The above-mentioned command is used to delete the transmission statistics information about the OAM packets and the system error information. |