- Layer 2 Tunnel Protocol Configuration

- Table of Contents

Chapter 1 Configuring Layer-2 Protocol Tunnel	1

1.1 Introduction	1

1.2 Configuring Layer-2 Protocol Tunnel	1

1.3 Configuration Example of Layer 2 Protocol Tunnel	1

# Configuring Layer-2 Protocol Tunnel

## Introduction

Layer-2 protocol tunnel allows users between two sides of the switch to transmit the specified layer 2 protocol on their own network without being influenced by the relevant layer 2 software module of the switch. The switch is a transparent media for users.

## Configuring Layer-2 Protocol Tunnel

Use the command line on the interface of the switch to configure tunnel function of the layer 2 protocol. The configuration steps are as follows:

| Command | Description |
| --- | --- |
| configure | Enters global configuration mode. |
| interface <intf_name> | Enters interface configuration mode of the switch. Only the switch port supports layer 2 protocol tunnel (including physical port and aggregation port). |
| [no] l2protocol-tunnel [stp] | Enables layer 2 protocol of the tunnel function. Currently we only support tunnel function of stp protocol. |
| [CTRL] + Z | Returns to EXEC mode. |
| write | Saves configuration. |

## Configuration Example of Layer 2 Protocol Tunnel

Network environment is as follows：

A1/A2/Gather belong to core network, C1/C2 are switches distributed in two places. Customer wants to combine two of its network to one , that is, the core network is a transparent transmission channel for the customer. If user wants to realize the transparent transmission of STP, then the following configurations should be configured on each switch:

- The f0/2 of Switch A1, f0/1 and f0/2 of Gather, f0/1 of A2 should be configured to trunk mode.

- The f0/1 of switch A1, f0/2 of A2 should be configured to Access, and enables tunnel function of the STP protocol.