Netkit Instructions

1. Select Device：

NMII： NetmateⅡ including NetmateⅡ, NetmateⅡmini, Netmate EXT, Mobilemate, Sitemate, MobileSite

NML：Netmate Lite

20XX：2009, 2012

NMIII： NetmateⅢ,  Sitepro

2. Functions:

：Select the device in the LAN，and list them

：Select the device in the list and click”Open the page”, it enters the WEB page via default browser

：Select the device in the list and click”Network Settings”, user can set the network info there

：Select the device in the list and click”Time Synchronization”, it can the synchronize time with the PC.NML can not realize

：Select the device in the list and click”Upgrade”, it can upgrade all device selected(only NMII & NML is supported)

：Select the device in the list and click”Reboot”, it can reboot the device selected(only support NMII)

3. Add IP Manually

In different level LAN, user cannot search the IP but it can ping, then user can add the IP manually(enter IP, and click add),after addition, user can test by time synchronization or other operation.