EMR2.1 Upgrade Guide

# I. EMR platform Upgrade

1. Check the EMR main control IP from the front panel. Menu structure as follows. Choose the <Network setting>

2. Set the computer IP in the same IP segment.

For example: the EMR IP is 192.168.1.100, set the computer IP to 192.168.1.xxx

3. Make sure the computer connected with the EMR main control card <Ethernet>port (Directly or through the switch).

Computer can ping to the EMR successful.

4. Update EMR platform. The step as follows:

(1)In vxWorks folder, double click the <onlineupdata.bat>，then a CMD window will pop up. Input the EMR’s IP address and press Enter.

Software will upgrade automatically.

The EMR will reboot automatically after updating.

# II. Upgrade the drivers.

1. Use the FTP transfer software such as: FileZilla Client.

2. Open the FileZilla Client, input the device IP address in to the Host position, such as: 192.168.1.100; Username and Password are both: target

3. click the <Quick connect>. The filezilla will connect to the EMR.

Here, on the right side is the drivers in the EMR.

<appInit.txt>, <back_board> files are for EMR platform.

Others are drivers for cards.

3. Delete all the old drivers and upload new ones. (if only adding new card to the EMR, just upload the drivers for this card, no need to delete the other drivers)

<appInit.txt>, <back_board> are necessary files that must be upload.

As for the cards drivers, we can select the drivers for certain cards to upload.

4. Reboot EMR after drivers uploading finished.

Note:

FTP space for the card driver is about 15MB，which is enough for 6 different cards. But when upload more than 6 cards drivers, the space may be not enough. Better to delete the card drivers that not in use.